Source:	Orange

Title:	Follow-up on profiles for JBM behaviour evaluation

## Document for:	Discussion

## Agenda Item:	7.7

- Introduction

This Tdoc is a continuation of [1,2,3] on JBM testing in the scope of the eUET work item [4].

As discussed in [2] we already reported test results in [1]. A methodology for VoLTE realistic profile construction was detailed in [1]. Two test cases have already been reported (with MOS-LQO histograms):

- Simulation based on the reference EVS decoder (with rtpdump support)

- Measurements using an example DUT with the PCAP player feature of Rhode&Schwarz CMW500 equipment (with an RTP stream pre-recorded with packet impairments according to the given profiles).

In the present Tdoc we provide measurements conducted with HEAD acoustics LabCore. The test setup is based on TCN files.

- New test results

We describe first the relevant setup details on measurements on HEAD acoustics LabCore. The reference PCAP file used in [1] was recorded with MFE IX, now we use Labcore. The test signal for delay testing with Labcore (WAV file) combines three pulses (CSS signal) and the actual 160s speech sequence inside a single file. Labcore also uses the format txt file for impairments, and it is easier to modify a txt file than a tcn file.

NOTE: There might be limitations with the underlying netem functionality used in LabCore, this is left to be discussed, especially it may not be possible to have duplicated packets.

The test preparation followed the following principles:

- We recorded on the CMW500 a PCAP file from the uplink of LabCore without impairments (DTX on). This is the reference PCAP file.

- For each double sentence in the test signal we applied single packet loss in each successive packets to find packets that get the worst correction by the decoder PLC. In annex you will find for each double sentence the MOS score for single packet losses apply to each individual packet.

- We applied packet losses only to packets corresponding to active speech. The packet loss rate is 3.2% for active (which gives an overall rate of 2.8% for active + SID packets).

- We applied the same impairments to each double sentence, to check the reproducibility of the measurements.

The txt impairment file we used with labCore for testing the PLC is enclosed to this contribution. Note that impairments are not the same as in [1]. The Annex shows the curves we used to choose the packet loss rate based on LabCore recording.

## Figure 1 shows the percentage of RTP active speech packets lost during the measurement for each sentence.

Figure 1: Percentage of lost RTP active speech packets.

## Figures 2 and 3 show the MOS histogram recorded with a DUT (for EVS and AMR-WB resp.) when we make a communication through LabCore for 20 measurements (100 MOS results for each double sentence).

Figure 2: MOS histogram for EVS SWB audio codec at 24.4 kbits/s.

Figure 3: MOS histogram for AMR WB audio codec at 12.65 kbits/s.

For each double sentence we found similar results for EVS and for AMR-WB. The degradation seems to be enough high to later verify JBM performance. Note that it is very easy to change packet losses in the txt impairment file and replace them by other impairment like packet inversions and/or packet delays.

- Next steps

## We plan to conduct the following experiments to further progress:

- Verify with the official EVS PLC and JBM software, the simulation with the new network impairments.

- Find at least two devices, one that gives good results and one that gives worse results in the field for call quality and compare PLC and JBM results for these devices

- Test DUTs with the following conditions with packet loss, packet inversion and packet delay for different conditions:

    - EVS SWB 24.4 kbits/s, VoLTE

    - EVS SWB 24.4 kbits/s, VoWiFi

    - AMR WB 12.65 kbits/s, VoLTE

    - AMR WB 12.65 kbits/s, VoWiFi

## References

- S4-221445, On profiles for JBM behaviour evaluation, Source: Orange

- S4aA230014, Follow-up on profiles for JBM behaviour evaluation, Source: Orange

- S4-230236, Proposed updates on JBM performance, Source: Orange

- SP-220610, New WID on Enhancements to UE Testing, Source: Orange, HEAD acoustics GmbH, Fraunhofer IIS, ROHDE & SCHWARZ, Amazon

## Annex (MOS results for EVS SWB)

Figure 4: MOS result when we lost the packet number shown in x axis.

Figure 5: MOS result when we lost the packet number shown in x axis.

Figure 6: MOS result when we lost the packet number shown in x axis.

Figure 7: MOS result when we lost the packet number shown in x axis.