Source:	HEAD acoustics GmbH

Title:	Updated SWB Measurement results for eUET

Document for:	Discussion

# Introduction

The eUET work item [1] was approved at SA#96 with the objectives, among other things, to update several clauses in 3GPP TS 26.131 [2] on sensitivity/frequency characteristics in sending and receiving for Headset UE, Desktop UE and handheld hands-free UE. Several missing frequency masks in SWB will be defined, including a possible review of the related test methods in 3GPP TS 26.132 [3].

An initial test series was conducted for desktop hands-free UE in SWB mode to investigate the performance that can be achieved by such devices [4] and then further extended to headset UE [5]. The present document provides new test results for handheld hands-free UE. The measurement results can be used to define tolerance masks for the frequency responses that are under study in this work item.

# Test setups

## Desktop Hands-free UE

The test setup for the measurement series according to clause 5.1.3.2 of TS 26.132 [3] was used, which is mainly provides references to the measurement specifications ETSI TS 103 738/740 [6] for hands-free terminals. In contrast to other types of UE with acoustical interface, a test table is introduced into the setup and affects the acoustic pathes. Figure 1 depicts the positioning of HATS, terminal and the table.

| Figure 1: Setup for desktop hands-free from side (left) and top (right) view (from [6]) |  |
| --- | --- |

## Headset UE

The test setup for the measurement series according to clause 5.1.2 of TS 26.132 [3] (HATS equipped with two type 3.3 ears as specified in Recommendation ITU-T P.57 [7]) was used.

## Handheld Hands-free UE

The test setup for the measurement series according to clause 5.1.3.3 of TS 26.132 [3] was used (HATS equipped with two type 3.3 ears and free-field equalization, as specified in Recommendation ITU-T P.57 [7]), which is illustrated in Figure 2.

Figure 2: Setup for handheld hands-free UE (from [3])

# Devices

## Mobile phones

The mobile phones according to Table 1 were used for the VoLTE connection to the network simulator with EVS-SWB at 24.4 kbit/s. These were either used as stand-alone device (see desktop hands-free) or in combination with additional audio equipment (headset or desktop hands-free device).

Table 1: Mobile phones evaluated in test series

| Mobile Phone ID | Year of release |
| --- | --- |
| 1 | 2014 |
| 2 | 2021 |
| 3 | 2019 |
| 4 | 2020 |
| 5 | 2014 |

## Desktop Hands-free

Desktop hands-free (DHF) UE is not a very common use case. When introducing this type of UE, it was most likely intended to address stand-alone hands-free/group audio terminals, which had direct network access to voice services, i.e., needed a dedicated SIM card in the device. At least today, this class of devices are very rare (or do not even exist), but there are two different options that are used more often:

- Mobile phone is configured in hands-free mode, but in contrast to handheld hands-free mode, it is placed on a table/desktop.

- External devices providing microphone(s) and loudspeaker(s) is connected via electrical interface of a mobile phone. This is related to headset UE, where the combination of a mobile phone and external equipment are regarded as a dedicated type of UE.

Both configurations were evaluated in the measurement series. As device under test (DUT), each mobile phone was either used stand-alone in speakerphone mode or with externally connected hands-free equipment (EQ) provided in Table 2.

Table 2: Hands-free equipment evaluated in test series

| EQ ID | El. Interface | Form factor | Playback |
| --- | --- | --- | --- |
| 1 | USB | Small (10 cm x 10 cm) | Mono |
| 2 | USB | Medium (19 cm x 19 cm) | Mono |
| 3 | USB | Medium/triangular shape (2 x 19 cm + 23 cm) | Stereo |
| 4 | USB | Large (36 cm x 18 cm) | Stereo |

Each combination of mobile phone and hands-free equipment represents a separate "sub-DUT", which is denoted with an unique identifier (ID) as:

DUT[mobile phone ID]-E[hands-free equipment ID]

With:

[mobile phone ID]				Mobile phone ID according to Table 1.

[hands-free equipment ID]		Hands-free equipment ID according to Table 2. If mobile phone is used in hands-free mode without external equipment, this ID is set to 0.

This leads to identifiers like e.g., DUT1-E0, DUT2-E1, etc.; note that not all possible combinations could be tested due to time constraints. All mobile phones and external equipment used are commercially available devices, but none of the combinations were explicitly intended to be used with each other as a bundle (as it is common for headsets, which are often included with mobile phones). All connected audio equipment devices should in general be send/receive fullband speech signals, as no limitation was provided in the data sheets and all devices can operate at multiple sampling rates up to 48 kHz.

In addition, two reference DUTs according to Table 3 were included in the evaluation as well. These are simply passive analogue loudspeakers, which were equalized and calibrated at 10 cm and in the abscense of the table setup. These "devices" are only used in receive direction and should illustrate the impact of the table surface on the frequency response. The fixed level was calibrated to meet the requirement range of RLR (5 +/- 4 dB) as per clause 7.2.3 of [2].

Table 3: Reference DUTs used in test series

| DUT ID | Description |
| --- | --- |
| REF-LS1 | Medium analogue loudspeaker |
| REF-LS2 | Small analogue loudspeaker |

## Headset

Headset (HE) UE is typically assumed as the combination of a mobile phone and a bundled headset (analogue or digital). In the test series, some of such units are included as well, as indicated in the list of headsets in Table 4.

Table 4: Headsets evaluated in test series

| HE ID | El. Interface | Bundled with mobile phone ID |
| --- | --- | --- |
| 1 | Analogue | 5 |
| 2 | USB | - |
| 3 | Analogue | - |
| 4 | USB | 4 |
| 5 | Analogue | - |

NOTE 1:	Since neither headsets nor mobile phones with SWB-capable LC3-Codec were available, Bluetooth was not considered as digital headset interface in the evaluation.

NOTE 2:	All headsets were wired binaural in-ear devices with "dangling" microphones.

Each combination of mobile phone and headset represents a separate "sub-DUT", which is denoted with a unique ID as:

DUT[mobile phone ID]-H[headset ID]

With:

[mobile phone ID]				Mobile phone ID according to Table 1.

[headset ID]	Headet ID according to Table 4.

This leads to identifiers like e.g., DUT1-H1, DUT2-H3, etc.; note that not all possible combinations could be tested due to time constraints.

NOTE 3:	The insertion in-ear headsets into the (round) ear canal entry of type 3.3 ears with a sufficient amount of sealing is typically hard to achieve. The work around usually is to reposition and measure the headset multiple times. Due to time constraints, this could not be realized; instead, each headset positioning was manually optimized for lowest possible leakage. However, it cannot be guaranteed that the headset tip remains stable all over the duration of the measurement.

## Handheld Hands-free

The five mobile phones according to Table 1 were used in loudspeaking mode for the test series with Handheld Hands-free (HH) UE.

Handheld hands-free

# Measurement Results

## Desktop Hands-free

### SND

The measurement results for sending frequency response (SFR) and UE type DHF are provided in Figure 3 to Figure 7 for each EQ and across the different mobile phones used. To provide a rough estimate of the performance, the tolerance mask from DHF in WB mode (clause 6.4.3 of [2]) is shown for reference.

Figure 3: SFR for mobile phones used in DHF mode without additional equipment (E0)

Figure 4: SFR for mobile phones used in DHF mode in combination with E1

Figure 5: SFR for mobile phones used in DHF mode in combination with E2

Figure 6: SFR for mobile phones used in DHF mode in combination with E3

Figure 7: SFR for mobile phones used in DHF mode in combination with E4

For reference and to supplement the SFRs, measurement results for sending loudness rating (SLR) are provided in Table 5 for each evaluated combination of phone and EQ. The requirement range of SLR as per clause 7.2.3 of [2] is 13 dB +/- 4 dB.

| EQ | E0 | E1 | E2 | E3 | E4 |
| --- | --- | --- | --- | --- | --- |
| Phone |  |  |  |  |  |
| DUT1 | 9.1 | 4.6 | 23.8 |  |  |
| DUT2 | 11.1 | -0.7 | 6 | -2 | -0.5 |
| DUT3 | 9.8 | 12 | 5 |  | -1.2 |

Table 5: SLR (in dB) for combinations of mobile phone and audio equipment

### RCV

The measurement results for receiving frequency response (RFR) and UE type DHF at nominal volume setting are provided in Figure 8 to Figure 12 for each HFE and across the different mobile phones used. To provide a rough estimate of the performance, the tolerance mask from DHF in WB mode (clause 6.4.4 of [2]) is shown for reference.

Figure 8: RFR for mobile phones used in DHF mode without additional HFE (E0)

Figure 9: RFR for mobile phones used in DHF mode in combination with E1

Figure 10: RFR for mobile phones used in DHF mode in combination with E2

Figure 11: RFR for mobile phones used in DHF mode in combination with E3

Figure 12: RFR for mobile phones used in DHF mode in combination with E4

Figure 13: RFR for reference loudspeakers

Two findings can be drawn from the RFR results of the reference loudspeakers (Figure 13):

- It seems in general possible that devices with loudspeakers of a certain degree of (medium-to-high) quality can pass the current WB tolerance mask (which might be the basis for the extension to SWB).

- As no signal processing or coding is present, only the (mostly linear) acoustics transfer functions apply here. Since the loudspeakers were equalized in advance to a flat frequency response, RFR curves show approximately the difference between free-field conditions and the table setup.
For both loudspeakers, a level increase at 1250 Hz can be observed, which causes violations of the frequency mask. Other peaks and dips are visible as well; it should be investigated further if these are linear (and can thus be compensated with an additional corresponding equalization) – or if these occur due to resonances, from e.g., the table setup (which cannot be compensated).

For reference and to supplement the RFRs, measurement results for receive loudness rating (RLR) are provided in Table 6. For each combination of phone and EQ (or speakerphone mode), the nominal volume control setting closest to the target RLR of 5 dB was selected. However, it turned out that in some cases the playback levels were not sufficient to achieve this target (RLR > 5 dB), even at maximum volume setting.

Table 6: RLR (in dB) at nominal volume control setting for combinations of mobile phone and EQ

| EQ | E0 | E1 | E2 | E3 | E4 | LS1 | LS2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Phone |  |  |  |  |  |  |  |
| DUT1 | 8.7 | 7.8 | 5.7 |  |  |  |  |
| DUT2 | 5.3 | 5.4 | 6.3 | 7.1 | 6.7 |  |  |
| DUT3 | 4.3 | 5.4 | 4.5 |  | 4.8 |  |  |
| REF |  |  |  |  |  | 5.5 | 5.7 |

## Headset

### SND

The measurement results for SFR and UE type HE are provided in Figure 14 to Figure 19 for each headset ID and across the different mobile phones used. For headset UE in SWB, performance requirements for send frequency response already exist and are the same as for handset UE. For this reason, the tolerance mask as specified in clause 7.4.1 is shown in each figure.

Figure 14: SFR for mobile phones used in HE mode with additional headset H1

Figure 15: SFR for mobile phones used in HE mode with additional headset H2

Figure 16: SFR for mobile phones used in HE mode with additional headset H3

Figure 17: SFR for mobile phones used in HE mode with additional headset H4

Figure 18: SFR for mobile phones used in HE mode with additional headset H5

Figure 19: SFR for mobile phones used in HE mode with additional headset H1

For reference and to supplement the SFRs, measurement results for sending loudness rating (SLR) are provided in Table 7 for each evaluated combination of phone and EQ. The requirement range of SLR as per clause 7.2.5 of [2] is 8 dB +/- 3 dB.

Table 7: SLR (in dB) for combinations of mobile phone and headset

| EQ | H1 | H2 | H3 | H4 | H5 |
| --- | --- | --- | --- | --- | --- |
| Phone |  |  |  |  |  |
| DUT1 |  |  | 7.1 |  |  |
| DUT2 |  | 13.8 | 4.4 | 8.0 |  |
| DUT3 |  |  | 6.2 |  |  |
| DUT4 | 27.1 | 21.0 | 11.8 | 15.6 |  |
| DUT5 | 18.9 | 20.7 | 3.7 | 15.5 | 4.4 |

It can be noticed that DUT4 and DUT5 (except for H3 and H5) seem to expect a much higher sensitivity at the microphone input. On the other hand, H3 provides such a high sensitivity in general across all mobile phones.

### RCV

The measurement results for RFR and UE type HE are provided in Figure 20 to Figure 24 for each headset ID and across the different mobile phones used. For receive frequency response in SWB, neither performance requirements nor objectives are defined yet in clause 7.4.2.2. To provide a rough estimate of the performance, the tolerance mask of the provisional values for objectives (provided in brackets) is shown in the following figures.

Figure 20: RFR for mobile phones used in HE mode with additional headset H1

Figure 21: RFR for mobile phones used in HE mode with additional headset H2

Figure 22: RFR for mobile phones used in HE mode with additional headset H3

Figure 23: RFR for mobile phones used in HE mode with additional headset H4

Figure 24: RFR for mobile phones used in HE mode with additional headset H5

For reference and to supplement the RFRs, measurement results for receive loudness rating (RLR) are provided in Table 8. For each combination of phone and headset, the nominal volume control setting closest to the target RLR of 8 dB (binaural) was selected.

It turned out that in some cases (DUT5-H2, DUT5-H4), the playback levels were quite high and independent of the volume setting. This can be explained by the digital USB-headsets used with DUT5, which was not able to handle the volume control commands via USB.

Table 8: RLR (in dB) at nominal volume control setting for combinations of mobile phone and headsets

| EQ | H1 | H2 | H3 | H4 | H5 |
| --- | --- | --- | --- | --- | --- |
| Phone |  |  |  |  |  |
| DUT1 |  |  | 3.2 |  |  |
| DUT2 |  | 8.8 | 5.5 | 6.5 |  |
| DUT3 |  |  | 4.6 |  |  |
| DUT4 | 6.1 | 7.2 | 7.0 | 8.3 |  |
| DUT5 | 7.0 | -1.1 | 7.3 | -7.9 | 8.5 |

## Handheld Hands-free

### SND

The measurement results for SFR and UE type HH are provided in Figure 25 for each mobile phones used. For handheld hands-free UE in SWB, performance requirements for send frequency response in clause 7.4.5 are only indicated within a provisional range. To provide a rough estimate of the performance, an average tolerance mask of these ranges is shown in each figure.

Figure 25: SFR for mobile phones used in HH mode

For reference and to supplement the SFRs, measurement results for sending loudness rating (SLR) are provided in Table 9 for each evaluated combination of phone and EQ. The requirement range of SLR as per clause 7.2.4 is 13 ± 4 dB, which is met by all devices.

Table 9: SLR (in dB) for each mobile phone

| Phone | SLR |
| --- | --- |
| DUT1 | 12.8 |
| DUT2 | 13.7 |
| DUT3 | 13.1 |
| DUT4 | 10.9 |
| DUT5 | 15.1 |

### RCV

The measurement results for RFR and UE type HH are provided in Figure 26 for each mobile phones used. For receive frequency response in SWB, neither performance requirements nor objectives are defined yet in clause 7.4.6. To provide a rough estimate of the performance, the tolerance mask from HH in WB mode (clause 6.4.6) is shown for reference.

Figure 26: RFR for mobile phones used in HH mode

For reference and to supplement the RFRs, measurement results for receive loudness rating (RLR) are provided in Table 10. For each combination of phone and EQ (or speakerphone mode), the nominal volume control setting closest to the target RLR of 10 dB was selected. The requirement range of RLR as per clause 7.2.4 is 10 ± 9 dB, which is met by all devices.

Table 10: RLR (in dB) at nominal volume control setting for each mobile phone

| Phone | RLR |
| --- | --- |
| DUT1 | 10.2 |
| DUT2 | 8.4 |
| DUT3 | 11.2 |
| DUT4 | 11.4 |
| DUT5 | 10.0 |

# Tolerance Masks for SWB

## Method to derive tolerance masks

TBD

[

Editor's Note: A suitable and understandable way has to be found to derive tolerance masks for SWB, based on:

-	provisional values already available in TS 26.131

-	extension of WB tolerances

-	trade-off between sufficient number of devices passing and minimum quality.

]

## SND

TBD

## RCV

TBD

# Conclusion

For the determination of suitable performance requirements of sending and receiving frequency responses, an additional measurement series was conducted with Handheld Hands-free UE to extend the already collected data for Headset and Desktop hands-free UE . Some remaining actions are planned by the source to complete the test series:

-	Some combinations between mobile phones and external audio equipment (for HE and DHF) are missing and will be completed.

-	Potential outlier results will be reviewed again (and removed/repeated, if needed).

-	More devices to be tested in handheld hands-free mode

-	Derive proposals for SWB frequency masks (section 5)

Desktop hands-free UE:

The measurement data showed that performance is highly varying across the DUTs, which is not a surprising result: the different combinations of mobile phone and hands-free equipment investigated were not intended to jointly work together. The standalone mobile phones performed surprisingly well in sending, but completely fail in receive direction.

For the determination of SWB performance requirements for DHF, the tolerance masks from WB might be a basis. Possible ways forward would be to extend the limits up to 16 kHz.To benefit from the SWB bandwidth of EVS codec, the roll-off of the lower tolerance should start at a much higher frequency than 5 kHz (as in the WB masks).

Headset UE:

Results for HE also show varying results across the combinations of mobile phones and connected headsets. The performance of the two bundled DUT4-H4 and DUT5-H1 seems to be the same level as for non-matching combinations. Results for SFR and RFR show the performance is often comparable for a single headset that is connected to multiple phones.

Handheld Hands-free UE:

Results for SLR/RLR (all values within requirements) and in general more consistent SFR/RFR curves indicate that HH seems to be a use case that is more optimized by the manufacturers than the two aforementioned ones. However, an outlier analysis for some devices (high-frequency boost for DUT5 in SND, DUT1 in RCV) is currently ongoing.

Further proposals:

1)	When presenting first results [4] for DHF at SA4#120-e, it was discussed if (or up to which degree) the combination of several/arbitrary devices considered in the test series is in the scope of 3GPP TS 26.131/132 or not, as they were not bundled in a (commercially available) package, like e.g., mobile phones and an associated headset. In the same way, one could argue that Headset UE only applies for such bundles as well, even though it is not explicitly mentioned.

Thus, it is proposed to revise clause 4.3 ("Acoustical interfaces") of TS 26.131/132 within the work item eUET and improve the definitions of UE types. While it seems clear and obvious, which types of devices are meant for Handset and Hand-held hands-free UE, the aforementioned examples showed that a clearer defintion for the other types might be helpful for the reader of the test specifications.

2)	As the positioning and fitting of in-ear devices to type 3.3 artificial ears are often difficult to handle, it could be considered to revise clause 5.1.2, where currently the deprecated type 2 ear is mentioned as a workaround for in-ear headsets. Instead, the more recent type 4 ears (with anatomically shaped ear canal) should be recommended in this case. However, it has to be discussed first if formally such a modification can be made within the work item eUET.

# References

| [1] | 3GPP SP-220610, „New WID on Enhancements to UE Testing,“ Orange, HEAD acoustics GmbH, Fraunhofer IIS, ROHDE & SCHWARZ, Amazon, 2022. |
| --- | --- |
| [2] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release-17.1. |
| [3] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release-17.1. |
| [4] | 3GPP S4-221019, „Initial measurement results for eUET,“ HEAD acoustics GmbH, 08/2022. |
| [5] | 3GPP S4-221419, „SWB Measurement Results for eUET,“ HEAD acoustics GmbH, 11/2022. |
| [6] | ETSI TS 103740 v1.4.1, „Transmission requirements for wideband mobile wireless terminals (hands-free) from a QoS perspective as perceived by the user,“ 10/2021. |
| [7] | Recommendation ITU-T P.57, „Artificial Ears,“ 06/2021. |