Source:	Nokia Corporation

Title:	[ISAR] Cross Check Report for ISAR Objects (ISM-4)

Document for:	Discussion & Agreement

Agenda Item:	7.7 – ISAR

# 1. Introduction

Nokia Corporation participated in ISAR selection testing as a volunteering Cross-check Listening Laboratory (CLL) to carry out selection testing and the statistical results analysis according to the requirements set forth in the ISAR test plan [1].

The test assigned for Nokia is BS1534-3: Objects (ISM-4), see Table 1 for listening laboratory allocation. The cross-check was performed at Nokia Technologies listening laboratory in Tampere, Finland in March 2024.

This input provides the corresponding CLL report including the statistical analysis of results. Nokia is confirming that the listening lab has met all the obligations set for the CLL.

Table 1 ISAR selection listening tests and corresponding listening laboratories.

| - Experiment | - SPL | - CLL/other SPL |
| --- | --- | --- |
| - BS1534-1: SBA (HOA3) | - Dolby | - Qualcomm |
| - BS1534-2: Multi-channel 7.1+4 | - Fraunhofer IIS | - Ittiam |
| - BS1534-3: Objects | - Fraunhofer IIS | - Nokia |
| - BS1534-4: MASA | - Dolby | - Bytedance |

# 2. Listening experiment

# Organization of tests

IVAS specific ISAR selection Cross-check listening test “BS1534-3: Objects” was conducted at Nokia premises using Nokia internal LTSW (Listening Test SoftWare) platform (Figure 1) using binaural non-head-tracked listening. The listening environment was quiet individual listening booths [2]. The used headphone model was Sennheiser HD650.

Figure 1 Screenshot of the Nokia internal listening test software LTSW

Listening test was conducted according to the ITU-R BS.1534-3 Method for the subjective assessment of intermediate quality level of audio systems [3]. Evaluation item and condition playback orders were randomized for listeners. Listeners did not know which condition matches any of the shown sliders.

## Experimental material

Test material and test material processing was provided by Solution Proponent. Nokia downloaded the test material from filesharing Box-service directory “_EXTERNAL_ISAR_LL  BS1534-3_Nokia”. There were 12 different evaluation test sequences. Each of them had four different conditions.

Table 2 BS1534-3_Nokia subjective evaluation test sequences

| No. | Name | No. | Name |
| --- | --- | --- | --- |
| 1 | ism4_1traj6.c01-04.wav | 7 | ism4_7traj7.c01-04.wav |
| 2 | ism4_2traj1.c01-04.wav | 8 | ism4_8traj2.c01-04.wav |
| 3 | ism4_3traj4.c01-04.wav | 9 | ism4_9traj7.c01-04.wav |
| 4 | ism4_4traj1.c01-04.wav | 10 | ism4_10traj5.c01-04.wav |
| 5 | ism4_5traj5.c01-04.wav | 11 | ism4_11traj3.c01-04.wav |
| 6 | ism4_6traj8.c01-04.wav | 12 | ism4_12traj3.c01-04.wav |

For each evaluation test sequence four conditions were provided:

- Hidden reference (c01): Native coding system (IVAS@512kbps with 4 objects rendered to post renderer pose)

- LP7 anchor (c02): Degraded reference, 7 kHz LP filtered

- 0-DOF (c03) native transcoding reference (IVAS@512kbps binaurally rendered to pre-renderer pose, IVAS@256kbps stereo coded)

- CuT (c04): System under test. (IVAS@512kbps with 4 objects rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR@768kbps candidate)

Listeners and performing the listening test

Ten experienced listeners took part to the listening test. All of them were Nokia employees and have prior experience in spatial audio MUSHRA listening testing.

Each listener first made a practice experiment with a randomly selected evaluation sequence to familiarize with the listening test software and practicalities. Next all samples were played back in reference quality to get familiar with the test material. After these preparations the actual listening test was conducted.

After all results were obtained from listeners, results were calculated. All listeners’ results were found to be acceptable and were included to the final statistical analysis.

# 3. Results

Mean score and descriptive statistics can be seen in Table 3 and Figure 2.

Table 3 Descriptive statistics

| Condition | Votes | min | max | mean | Std. deviation | Confidence interval (+/-) |
| --- | --- | --- | --- | --- | --- | --- |
| Hidden refence | 120 | 64 | 100 | 97,28 | 7,47 | 1,5 |
| LP7 | 120 | 5 | 65 | 27,45 | 10,70 | 1,5 |
| 0-DoF | 120 | 9 | 100 | 57,02 | 20,11 | 3,9 |
| CuT | 120 | 52 | 100 | 89,43 | 11,19 | 2,6 |

Figure 2 Listening evaluation results

Sample-wise results can be seen in Table 4. Sample 12 shows a bit weird result of CuT being better than Hidden refence. After checking individual listener’s results, this is just statistical variance. 4 listener voted Cut being better than direct (~80 and 100) and 6 listeners voted either (100 and 100) or (100 and ~95) for (direct and CuT) respectively. Also, when listening to the sample12 nothing unexpected was noticed.

Table 4 Sample-wise results

Paired t-test results can be seen in Table 5.

Table 5 Paired t-test results

From the results it can be concluded that CuT BT 0-DoF.

# 4. Summary

This input provides the CLL report by Nokia Corporation for ISAR selection test “BS1534-3: Objects”.

According to the result analysis, CuT meets and exceeds (BT) the requirement of NWT 0-DoF.

The source asks the group to agree on inclusion of the CLL results as part of the ISAR report.

# References

[1] S4aA240024, “ISAR Pdoc on Testing Aspects for Phase/Track 2/a”, 18 March 2024

[2] M. Kylliäinen, H. Helimäki, N. Zacharov, and J. Cozens, “Compact high performance listening spaces,” in Proc. Euronoise, Naples, Italy, May 2003

[3] ITU-R BS.1534-3 (10/2015): Method for the subjective assessment of intermediate quality level of audio systems.