3GPP SA4#127-bis-e	S4-240726

April 4 – April 12 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0003 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On the binding between RTP header extensions for delay measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 04/8/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Two RTP header extensions are defined for delay measurement. The binding between the two RTP header extensions is mandatory, which limits the use of the RTP header extensions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the binding from mandatory to optional. - Update the ABNF syntax for extmap attribute. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Limited use of the header extensions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

# Proposed changes

# * * * * 1st change * * * *

### 4.4.5	Semantics

T1: Originate Timestamp. It represents the time when the Requester transmits the RTP packet toward the Responder.

T2: Receive Timestamp. It represents the time when the Responder receives the RTP packet that carries the Originate Timestamp T1.

T3: Transmit Timestamp. It represents the time when the Responder transmits the RTP packet that carries the Originate Timestamp T1, the Receive Timestamp T2, and the Transmit Timestamp T3.

A binding may be created between the RTP HE that includes only T1 and the RTP HE that includes T1, T2 and T3.

### 4.4.6	SDP signaling

The signaling of the delay measurement RTP HE shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC 8285 [11].

For the RTP HE carrying only T1, the ABNF syntax for the "extmap" attribute is as follows:

extensionname = "http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time"

extensionattributes = ["short"/"long"]

If the extensionattributes is absent, the RTP HE follows the one-byte format, i.e., the "short" format. If extensionattributes is "short", the RTP HE follows the one-byte format. If extensionattributes is "long", the RTP HE follows the two-byte format.

NOTE 1:	http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time is the extension URI of the RTP HE, and is currently implemented in WebRTC. This extension URI, instead of URN-based ones, allows for support from WebRTC without any change to the WebRTC implementation.

NOTE 2:	This allows to reuse the "Absolute Sender Time" RTP HE in WebRTC without changes to the SDP syntax implemented in WebRTC.

NOTE 3:	Regardless if this extension attribute is present or not, the use of long or short format is determined as described by section 4.1.2 of RFC 8285, i.e., based on what format other RTP HEs use in the same RTP session, unless both endpoints announced support for handling mixed format with "a=extmap-allow-mixed" as described by section 6 of RFC 8285.

Below is an example (Example 1):

a=extmap:4 http://www.webrtc.org/experiments/rtp-hdrext/abs-send-time

For the RTP HE carrying T1, T2 and T3, the ABNF syntax for the "extmap" attribute is as follows:

extensionname = "urn:3gpp:delay-measurement-response:rel-18"

extensionattributes = [SP binding-info]

format = "short"/"long"

binding-info = dependent-extmap-ID [";"m-line-label] [";"processing-ID]

dependent-extmap-ID = "dependent-extmap-ID="1*5DIGIT

m-line-label = "dependent-rtp-he-m-line-label="token

processing-ID = "processing-ID="token

; token as defined by RFC 4566

# * * * * End of 1st  change * * * *