Source:	Ittiam System Pvt Ltd

Title:	Ittiam Cross-Check Listening Lab Report for ISAR Selection

Document for:	Discussion&Approval

Agenda item:	7.7

# Summary

This document constitutes the listening lab report related to Ittiam’s cross-check listening test in the course of the selection testing of IVAS specific ISAR solutions targeted in Phase/Track 2/a of the ISAR work [1].

# Cross-check listening lab assignment

According to ISAR Pdoc on Testing Aspects for Phase/Track 2/a [2], Ittiam was assigned as CCL for experiment BS1534-2 (Multi-channel 7.1+4).

| Experiment | SPL | CCL/other SPL |
| --- | --- | --- |
| BS1534-1: SBA (HOA3) | Dolby | Qualcomm |
| BS1534-2: Multi-channel 7.1+4 | Fraunhofer IIS | Ittiam |
| BS1534-3: Objects | Fraunhofer IIS | Nokia |
| BS1534-4: MASA | Dolby | Bytedance |

# Ittiam System Listening Lab capabilities

Ittiam System has a rich, multi-year long experience as a listening lab. Our critical listening experiments are carried out in dedicated listening rooms meeting all relevant requirements for headphone listening and listening over multi-loudspeaker playback (e.g. 7.1.4). We maintain a large pool of well-trained critical listeners. Our listening tests including the present test are run completely independent on contract basis. The present test was funded by Dolby.

# Test material

The test material for the listening test was downloaded from Box location \\Box\_EXTERNAL_ISAR_LL\BS1534-2_Ittiam, as instructed by email from Dolby via the 3GPP_TSG_SA_WG4_AUDIO email reflector on March 15.

# Listening Environment, Listening Instrument and Test Interface

The test was carried out in listening rooms at Ittiam meeting the requirements of ITU-R BS.1534 [3]/ITU-R BS.1116 [4].

As specified in the test plan [2] high-quality stereo headphones for binaural listening of type Sony MDR 7506 were used.

The test interface used in the blind grading phase was similar to the example interface described in ITU-R BS.1534 [3]. The test interface ensured that the sample presentation was done in a pseudo-random fashion with different presentation orders for different listeners.

# Blinding

The test was carried out in a blinded fashion without revealing information about the identity of the conditions the test subjects were listening to.

# Test instructions

No specific test instructions were provided as the listeners are well-trained expert listeners who are frequently involved in BS.1534 tests and are familiar with the used test interface.

# Test subjects

10 expert listeners were used for the test. As no listeners needed to be post-screened, no extra listeners were used.

The test subjects were unaware of the systems under test and were also otherwise not involved in the development of any of them.

# Post-screening of listeners

The post-screening rule according to ITU-R BS.1534 [3] was:

- an assessor should be excluded from the aggregated responses if he or she rates the hidden reference condition for > 15% of the test items lower than a score of 90;

- an assessor should be excluded from the aggregated responses if he or she rates the mid-range anchor for more than 15% of the test items higher than a score of 90. If more than 25% of the assessors rate the mid-range anchor higher than a score of 90, this might indicate that the test item was not degraded significantly by the anchor processing. In this case assessors should not be excluded on the basis of scores for that item.

For none of the listeners the post-screening rule applied.

# Peculiarities

No peculiarities or irregularities were encountered during execution of the test.

# Test Results

The following plot shows the test results averaged across the 10 listeners. The error bars on the average scores display the 95% confidence intervals.

Figure 1: Result plot of experiment BS.1534-2 (Multi-channel 7.1+4)

T-test analysis was carried out according to the requirements in clause 3.3 of the test plan [2]. The results are given in the following table. From that analysis comparing condition c04 (CuT1) with condition c03 (0-DOF), it can be inferred that c04 (CuT1) is better than c03 (0-DOF).

Table 1: T-test analysis comparing CuT1 with 0-DOF reference

| Mean Diff. | Stdev Diff. | SEMD | t | Prob. |
| --- | --- | --- | --- | --- |
| -18.17 | 7.82 | 0.71 | -25.45 | 1.00 |

# Conclusion

The source carried out the ISAR cross-check listening test BS.1534-2 in accordance with the ISAR Pdoc on Testing Aspects for Phase/Track 2/a [2].

All listening lab obligations were met.

# References

[1]	Tdoc S4-240403: Work Plan for the ISAR v0.5.0

[2] 	Tdoc S4aA240024: ISAR Pdoc on Testing Aspects for Phase/Track 2/a

[3]	ITU-R BS.1534, "Method for the subjective assessment of intermediate quality level

of audio systems, 2015

[4]	ITU-R BS.1116, "Methods for the subjective assessment of small impairments in audio systems," 2015