3GPP TSG-SA4 Meeting #131	S4-250022

# Geneva, , 17th Feb 2025 - 21st Feb 2025									revision of S4aI250062

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0102 | rev | 4 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

X

| Title: | [AMD-ARCH-MED] Distributing encrypted and high-value content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD-ARCH-MED |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | DRM and Conditional Access: DRM and Conditional Access are commonly used by third-party streaming services. However, in case streaming is done through MBS or MBMS, a more careful management of the keys needs to be checked. Scalability of key delivery is an issue. The support for -encrypted content in Unicast/Multicast and Broadcast is relevant. Integration of Content Protection interfaces in the provisioning, for example using CPIX back-end interfaces is of high relevance for the industry and should accordingly be studied. The impacts of these on media plane (reference points M2 and M4) as well as the media session handling APIs (reference points M3, M5) should also be studied. For details see TR 26.804 clause 5.10 and the conclusions in clause 6.10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For distributing encrypted and high-value content as introduced in clause 5.10 of TR 26.804: i. Functional updates to the definition of the 5GMS AS to support: 1. Ingest, delivery, and contribution of encrypted content 2. Content preparation tasks for: 3. Decrypting content ingested at reference point M2d. 4. (Re-)encrypting content prior to distribution at reference point M4d. ii. Updates to the definitions of reference points to support: 1. Carriage of Content Protection information at reference point M2d. 2. Delivery of Content Protection information in presentation manifests at reference point M4d. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0.4, 5.14 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI250062 [AMD-ARCH-MED] Distributing encrypted and high-value content Qualcomm Germany Thomas Stockhammer E-mail Discussion: none Revisions: S4aI250062_BBC.docx S4aI250062-26501-0102rev3-DRM_huawei.docx Presenter: Thomas Stockhammer Online Discussion: (February 6, 2025) Decision: S4aI250062 is noted due to late submission. S4aI250062; [AMD-ARCH-MED] Distributing encrypted and high-value content; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: none Revisions: - S4aI250062_BBC.docx - S4aI250062-26501-0102rev3-DRM_huawei.docx Presenter: Thomas Stockhammer Online Discussion: (February 6, 2025) Decision: S4aI250062 is noted due to late submission. This document provides a revision taking into account the offline comments as well as the discussion in S4-250028. |

## =====  CHANGE =====

### 4.0.4	Content preparation

The content preparation feature is applicable to both downlink media streaming (where is provisioned as part of the content hosting feature introduced in clause 4.0.2) and uplink media streaming (where is provisioned as part of the content publishing feature introduced in clause 4.0.3). The content preparation feature enables a 5GMS Application Provider to specify content manipulation by network-side components of the 5GMS System according to provisioned Content Preparation Templates.

When a 5GMSd Application Provider has provisioned the content preparation feature for downlink media streaming:

1.	Network-side components of the 5GMS System may manipulate ingested media content and may cache the manipulated content prior to serving it to the 5GMSd Client in the UE. Content Preparation may include encoding, transcoding, packaging, encryption and protecting content using DRM.

When a 5GMSu Application Provider has provisioned the content preparation feature for uplink media streaming:

1.	Network-side components of the 5GMS System may manipulate the media content ingested from the 5GMSu Client in the UE and may cache the manipulated content prior to egesting it to the 5GMSu Application Provider.

## =====  CHANGE =====

## 5.14	Content Preparation of DRM-protected content

### 5.14.1	Scenario

A typical scenario is the following: The 5GMSd Application Server runs a content encoding and/or packaging service. However, in many cases, the content provider requires that the content is distributed with specific content and usage rights, referred to a Digital Rights Management (DRM). The DRM system us an implementation of content keys management cooperating with the device’s media platform to enable playback of encrypted content while protecting the decrypted samples and content keys against potential attacks, consisting of two main components: a DRM server and a DRM client.

In a typical deployment scenario, the DRM System including key and license servers is external in the 5GMSd Application Provider domain, but the encoding, encryption and packaging is assigned to the 5GMSd AS. This may for example be the case to adapt the content to the needs of mobile distribution, in combination with a Content preparation operation.

In order to complete the encryption tasks and manifest generation, the 5GMSd AS needs to communicate with the DRM system to obtain all information for manifest generation, content encoding and packaging. In this case Content Protection Information is exchanged across 5GMSd reference points.

In order to address the scenario above, the following sub-functions of the 5GMSd Application Provider, the 5GMSd Application Server and the 5GMSd client are defined:

-	5GMSd Application Provider:

-	Content Provider: a content provider providing content for distribution via 5GMSd with the provided content having assigned specific content and usage rights.

-	Authorization Server: provides authorization tokens that may be required for requesting a license from a license server.

-	DRM Server: is a component in a DRM system that manages and enforces the access and usage rights of digital content. It typically includes a License Server and a Key Server. In certain cases, the DRM System can have embedded entitlement logic to decide for which request to grant a license and for which not, supported by an authorization server.

-	License Server: A server provides licenses that are data structures in a DRM system specific format that contains one or more content keys and associates them with a policy that governs the usage of the content keys.

-	Key Server: The server being responsible for generating, storing, and managing encryption keys used to encrypt the content. It provides the encryption keys to the encryptor during the content encryption process.

-	5GMSd Application Server:

-	Encoder: The encoder converts ingest media content into a compressed format suitable for 5GMSd distrbution. It typically generates multiple versions of the content at different bitrates to support adaptive streaming. It may also produce different variants of the content, for example an HD version, a UHD version, and HDR Version, etc.

- 	Encryptor: The encryptor applies encryption to the encoded content using encryption keys. This ensures that the content is protected and can only be accessed by authorized users. The encryptor communicates with the Key Server to obtain the encryption keys needed to encrypt the content.

-	Packager: The packager receives encrypted content from the encryptor and formats it into streaming protocols like DASH and HLS. It also inserts DRM metadata into the media segments.

-	Media entry point creator: The media entry point creator generates manifest files (e.g., DASH MPD, HLS M3U8) that describe how the content is organized and how it should be played back. These manifest files include DRM metadata that informs the player about the encryption and how to obtain the decryption keys.

-	Content Hosting Server: A server hosting Streaming resources, i.e. Media Entry Points and Segments primarily, and includes information on the used DRM System.

-	5GMSd Application Server:

-	Media Platform: enables playback of encrypted content while protecting the decrypted samples and content keys against potential attacks. The Media Platform summarizes the functions of media decapsulation, media decryption, media decoding and media rendering as defined in clause 4.2.2.

-	DRM Client: processes licenses and enforcing the associated policies. Either the DRM client handles the decryption of samples, or the DRM client interacts with the hardware elements that address the decryption.

-	Media player: a function using the Media platform and the DRM system to playback encrypted content.

Note that in many DRM workflows, only the key IDs (KIDs) are exchanged rather than the actual encryption keys. This approach enhances security by minimizing the exposure of the actual keys.

Note also that there may be multiple License Servers in case of a multi-DRM system with common encryption.

### 5.14.2	Procedure

A typical example workflow for the scenario described in clause 5.14.1 is shown in figure 5.14.2-1.

Figure 5.14.2-1: Procedure for encoding, packaging and encrypting content for DRM-protected content

The steps are as follows, highlighting in bold relevant information exchanged via 5GMSd reference points:

Content Preparation phase:

1.	A content provider provides encoding instructions and rules for using the content to the content preparation and hosting system via reference points M1d/M3d as part of Content Preparation Template.

2.	The content provider provides raw content to the content preparation and hosting system via reference point M2d.

Trust Establishment phase:

3.	The Encryptor/Packager and the DRM System (typically the License and Authorization Server as well as the Key Server) exchange public signing keys to establish a trusted communication via reference points M1d/M3d as part of Content Preparation Template.

Content Protection Information construction phase:

4.	The Packager generates initial Content Protection Information that includes identification of the receivers and the various stream encoding criteria (usage rules).

5.	The Encryptor retrieves this information from the packager and requests one or several content keys from the key server by sending the signed Content Protection Information to a key server via reference point M2d.

6.	The Key Server generates content keys according to the request. It adds these content keys to the Content Protection Information, signs the Content Protection Information and sends it to the encoder/packager via reference point M2d.

7.	The encryptor/packager extracts the content keys and stores them.

Content Protection Information distribution phase:

8.	The Encryptor/Packager sends the Content Protection Information to the DRM System via reference point M2d.

9. 	The License Server verifies the Content Protection Information and imports keys or key ids into its data base.

10.	The License Server sends the updated Content Protection Information to the Encryptor/Packager and the Manifest Creator including associated DRM System information via reference point M2d.

Presentation manifest and media segment generation phase:

11.	The Manifest Creator generates the presentation manifest (e.g. DASH MPD) and adds the Content Protection Information (DRM System specific information, key identifiers, etc.).

12.	The Manifest Creator uploads the presentation manifest to the Content Hosting.

13.	The Encryptor/Packager generates encrypted segments and adds the Content Protection Information (e.g. DRM System specific information).

14.	The Encryptor/Packager provides the encrypted segments to the Content Hosting.

Client requests and authorisation phase:

15.	The device application requests an authentication for the user with the content provider via M8d.

16:	The application provider providers authorisation tokens via M8d.

17:	The application provides the tokens to the DASH Player and the DRM client.

18:	The media access client requests the presentation manifest including Content Protection Information from the Content Hosting function on the 5GMSd AS via reference point M4d.

19:	The media access client requests one or several licenses from the licensing server using key ids, possibly including authorization tokens via reference point M13d.

20:	The License Servers asks for user verification to the authorisation server.

21: The user is verified by the authorisation server.

22:	The License Server requests the decryption keys from the key server based on the key ids.

23: The Key Server provides the relevant content keys.

24.	The License server provides a DRM license with one or multiple keys to the media access client and DRM Client via reference point M13d.

Content delivery and decryption phase:

25.	The DASH Client requests encrypted segments from the Content Hosting function on the 5GMSd AS via reference point M4d.

26.	The media access client provides the encrypted segments to the Media Platform.

27.	The media access client provides the encrypted samples to the DRM Client.

28.	The DRM System decrypts the samples using the DRM licence and content keys.

29.	The DRM System provides the decrypted samples to the Media Platform for playback.

Content Protection Information

Content Protection Information

Content Protection InformationContent Protection InformationContent Protection Information

Content Protection Information

Content Protection Information

Content Protection Information

Content Protection Information

Content Protection Information

Content Protection Information

Content Protection Information

Content Protection Information

Content Protection Information

Content Protection Information

Content Protection Information