3GPP TSG|WG-SA4 Meeting #131 	S4-250407

Geneva, CH, 17 – 21 February 2025	(revision of S4-250092)

# Source:	Qualcomm

# Title:	New WID on Avatar Communication in AR Calls.

# Document for:	Approval

# Agenda Item:	17.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Avatar Communications in AR Calls

Acronym:	AvCall-MED

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_AVATAR | SA4 | 1000019 | Study of Avatars in Real-Time Communication Services |
| MeCAR | SA4 | 950015 | Media Capabilities for Augmented Reality |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

N/A

# 3	Justification

The integration of Avatars into real-time communication services is pivotal for delivering immersive and engaging user experiences. To expedite the commercial viability of these services and establish a robust foundation for future enhancements, normative work for Avatars must be introduced in Release 19. Building upon the findings of the TR 26.813 study, this effort focuses on the essential technical enablers for avatar-based communication—such as specifying Avatar formats, basic signaling support, and repository management—while ensuring minimal impact on existing architectures and protocols.

Following the conclusions of the study in TR 26.813, the next steps for Avatar standardization in 3GPP SA4 should be carried out in a phased manner to ensure a structured and feasible approach to implementation. The initial focus should be on standardizing 1-to-1 Avatar calls, with animation and rendering taking place either at the sender, network or the receiver.

# 4	Objective

The AvCall work item will focus solely on enabling avatar communication as part of IMS-based AR calls and has the following objectives in Rel-19:

- For receiver-driven avatar animation and rendering:

  - Extend TS 26.264 for Avatar Communication by specifying format, protocols, and basic signaling of base avatars and animation streams in IMS-based AR calls,

  - In collaboration with SA2 and SA3, define an informative management interface in TS 26.264 for Base Avatar Repository (BAR) to enable management and access control to assets of base avatars.

- For sender-based avatar animation and rendering:

  - Leverage existing TS 26.264 capabilities for sender-based Avatar animation, potentially only resulting in guidelines,

  - Consider the need to introduce optional pose data exchange extensions when the receiver’s pose is used at the sender’s side for viewer-dependent rendering,

- For network-based avatar animation and rendering:

  - Extend TS 26.264 for Avatar Communication by specifying format, protocols, and basic signaling of base avatars and animation streams in IMS-based AR calls,

  - Add signaling and format negotiation to enable MF-based animation and rendering,

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 26.264 | - format and signaling of Avatars in AR calls - UE to BAR interface and API | SA#109 | Stage #3 of the Avatar call enhancements |

# 6	Work item Rapporteur(s)

Bouazizi, Imed, Qualcomm, bouazizi AT qti dot qualcomm dot com

# 7	Work item leadership

SA4

# 8	Aspects that involve other WGs

Architecture and security aspects will be done in collaboration with SA2 and SA3 respectively.

# 9	Supporting Individual Members

Supporting IM name

Qualcomm Inc.

China Mobile Comm. Corporation

Vodafone Group Plc.

InterDigital Communications

Samsung Electronics Co., Ltd.