Agenda item: 	10.8

Source: 	Qualcomm Inc.

Title: 	[AIML_IMS-MED] An MF profile for AI/ML task processing

Document for	Discussion and Agreement

# Introduction

The IMS Split Rendering architecture defined in TS 26.567 enables the distribution of media processing tasks between terminal devices and network-based Media Functions (MF). This architecture provides a framework for offloading computationally intensive tasks from resource-constrained devices to capable network nodes.

As ML-based media processing becomes increasingly prevalent in telecommunications services, there is a need to extend the MF capability profiles to support ML inference tasks on IMS call media streams. This enables use cases such as:

- Real-time speech and audio processing (transcription, enhancement, analysis)

- Real-time video processing (object detection, scene analysis, enhancement)

- Multimodal processing combining audio and video streams

- Accessibility services through ML-based transformations

By defining ML processing capabilities within MTSI, MNOs can leverage existing IMS infrastructure to deploy ML-enhanced services while maintaining the benefits of MTSI.

In this contribution, we define an MF capability profile that allows MFs to indicate their support for AI/ML media processing capabilities and their subsequent discovery and assignment for an ongoing IMS session.

# ML Profile

MF shall support the following ML inference and processing capabilities for IMS call media streams:

Core ML Processing Capabilities:

- Real-time audio stream ML processing supporting sampling rates as specified in clause 5.2.1 of TS 26.114

- Real-time video stream ML processing supporting formats as specified in clause 5.2.2 of TS 26.114

- Synchronous and asynchronous ML inference modes

- Multi-stream processing with temporal synchronization

- Dynamic model loading and unloading based on service requirements

ML Framework Support:

The MF shall support at least one of the following ML inference frameworks:

- ONNX Runtime with minimum version 1.18 and Opset version 24

Other custom inference frameworks and runtimes may be supported and shall be declared in the capabilities.

Hardware Acceleration:

- Declaration of available ML acceleration hardware (GPU, NPU, TPU, DSP)

- Support for hardware-specific optimization libraries

- Dynamic resource allocation based on workload

Performance Requirements:

- Maximum end-to-end latency requirements per ML task category:

  - Audio processing: 150ms

  - Video processing: 300ms

  - Multimodal processing: 500ms

- Minimum inference throughput based on media type

- Support for quality-of-service parameters for ML tasks

Privacy and Security:

- On-device model execution without external dependencies

- Privacy-preserving inference modes

The type urn:3gpp:mf:profile:ml-processing shall be used to signal compliance with this profile, e.g. upon registration with the NRF.

# Proposal

We propose to agree the content of section 2 and add the content to the base CR to TS 26.114.

# References

[1]	 3GPP TS 26.567, Split Rendering over IMS