3GPP TSG SA WG4 Meeting 133-e	S4-251319

# online, , 18 - 25 July 2025

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on [FS_Beyond2D] Draft Conclusions

Spec:	3GPP TR 26.956v1.0.0

Agenda item:	9.6

Document for:	Decision

1. Introduction

Study closes at this meeting

2. Reason for Change

No conclusions on recommendations yet available

3. Conclusions

They are ready.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 26.956v1.0.0 at SA4-133-e.

* * * First Change * * * *

# 11	Conclusions and Recommendations

## 11.1 Summary and Conclusions

This technical report addresses the evolution of video services from traditional 2D formats to "beyond 2D" video, which includes immersive and interactive experiences such as stereoscopic 3D, multi-view plus depth, dense dynamic point clouds, dynamic meshes, and emerging research formats like Neural Radiance Fields (NeRF), light fields, and 3D Gaussian Splatting. The report aims to evaluate the feasibility, performance, and interoperability of these formats and codecs within 3GPP services, considering implementation constraints and network requirements.

The report categorizes beyond 2D video formats as follows:

-	Stereoscopic 3D and Extensions: Provides depth perception by presenting slightly different images to each eye. Widely supported by current devices and workflows, with extensions for higher resolutions and additional metadata (e.g., depth, alpha).

-	Multi-view Plus Depth: Offers multiple synchronized camera views, optionally with depth maps, enabling free viewpoint navigation and immersive experiences.

-	Dense Dynamic Point Clouds: Represents scenes or objects as high-density 3D points with attributes (color, normals, etc.), allowing detailed volumetric rendering.

-	Dynamic Meshes: Uses vertices, edges, and faces to define 3D geometry, supporting animation and real-time rendering, commonly used for avatars and digital twins.

-	Light Fields, NeRF, and 3D Gaussian Splatting: Advanced research formats that capture and render scenes with high realism and flexibility, though not yet standardized for commercial deployment.

A generic end-to-end reference model is introduced, covering content capture (via cameras or computer graphics), processing (conversion to beyond 2D formats), encoding (compression for efficient transmission), delivery (over 5G/6G networks), decoding, and rendering on various devices (smartphones, VR/AR headsets, autostereoscopic displays). The model emphasizes interoperability points and the need for systematic evaluation.

The report also defines a comprehensive evaluation and characterization framework, including:

-	Reference Scenarios: Real-time communication, streaming, split rendering, and messaging, each with detailed workflows and constraints.

-	Performance Metrics: Both objective (e.g., PSNR, SSIM, PCQM for point clouds, HV3D for stereoscopic video) and subjective (user studies) metrics are used to assess quality, efficiency, and user experience.

-	Test Sequences: A curated set of reference sequences (e.g., volumetric video of people, multi-view scenes) is provided for benchmarking codecs and workflows.

Existing 3GPP codecs (H.264/AVC, H.265/HEVC, MV-HEVC) are evaluated for their ability to support beyond 2D formats. The report identifies gaps in current capabilities, especially for new formats like point clouds and dynamic meshes. Network requirements such as latency, bandwidth, and real-time processing are also discussed, with an emphasis on leveraging 5G capabilities.

The report concludes that certain beyond 2D video formats are maturing and becoming market-relevant, driven by advances in capture, compression, and display technologies.

## 11.2 Recommendations

Based on the evaluation in this document, the following aspects are recommended:

-	3GPP TS 26.265 defines representation formats to support stereoscopic video. Based on the conclusions in this document, considering extensions for stereoscopic representation formats including intrinsic and extrinsic camera parameters, depth, alpha and possibly improved colour subsampling formats is worth to be studied. It is worthwhile to study this in more details including stereoscopic capturing with optical systems on typical UE form factors and then to

-	Identify relevant new representation formats not yet documented in TS 26.265 and provide the benefits in terms of user experience. The evaluation includes potential capturing and rendering of the formats. Candidates include support for alpha, support for depth together with stereo, additional colour subsampling 4:2:2 or 4:4:4.

-	Identify compression options for the representation formats based on HEVC.

-	Identify the opportunities and needs to integrate the representation formats into different transport systems, including messaging, real-time communication, split rendering and streaming

-	Define the expected traffic characteristics for new representation formats to meet certain quality thresholds

-	Define a conformance environment, including hosting, tooling and process, as well as conforming test vectors to support operation points

-	Identify gaps in existing specifications and provide guidance for potential normative work

-	For other representation formats, in particular point clouds and meshes, it is recommended to continue monitoring the broader adoption of these formats. Once there is sufficient market traction, the baseline established in the study may be used to define the exact representation formats. This report also provides good indication how different HEVC profiles and features may be used to compress such formats. While these formats may provide some new experiences, there is no immediate necessity to add them to 3GPP specifications. Instead, the market should be monitored for traction of these technologies, particularly in content generation.

-	A particular format of interest are 3D Gaussian splats as introduced in clause 4.3.6.3. This format represents a 3D scene with a 3D gaussian primitive, an anisotropic Gaussian ellipsoid. The rendering process is simple and can be executed in real time by projecting the sorted 3D gaussian splats onto a screen and rendering them in a photorealistic manner. This results in real scenes rendered in real time with lighting and reflection effects, enhancing the realism of the rendered image. In addition, 3DGS have the potential to be generated with commonly available optical systems on existing and emerging smartphones, supported by AI-based workflows. All of this makes the formats an attractive candidate, possibly in the 6G era, and further detailed study of this format is recommended including:

-	Identification of the use cases for mobile devices that demonstrate the practical applications of 3DGS contents.

-	A full definition and analysis of the 3DGS representation format including the relevance and complexity of the parameters of the primitives. This includes the size of the 3d model and the associated processing requirements, as well as quality and complexity aspects.

-	Identify the opportunities to generated 3DGS content, in particular with existing optical systems and the ability to integrate AI-based workflows on device and/or in the network.

-	Study the integration of such formats into 3GPP services (e.g. messaging), expected traffic characteristics as well as other aspects related to provide fully interoperable solutions.

-	Develop an end-to-end reference implementation for content delivery, covering the entire pipeline from content creation on a server or on the UE and, through compression, transmission, to rendering on a mobile device platform.

* * * End of Changes * * * *