3GPP TSG-SA WG4 Meeting #133-e					S4-251373

Online, 18 – 25 July 2025

Source:	Nokia

Title:	Observations from a P2P demo call with IVAS codec on mobile devices

Agenda item:	7.5

Document for:	Information

- Introduction

A peer to peer (P2P) demo call using the IVAS codec was conducted. The call was established between two mobile devices over an IMS network.

This document discusses some features that were used during the IVAS call and found useful.

- Observations & Discussion

The IVAS session between the mobile devices was negotiated using the IVAS SDP parameters defined in Annex A of TS 26.253. The session was negotiated to start with mono EVS using the im-s parameter (ivas-mode-switch). Later during the session, the codec mode was switched to immersive IVAS mode. The switching was implemented via re-negotiating the session and setting the im-s parameter properly.

For a more seamless switching and to avoid re-negotiation, a proper switching mechanism between mono EVS and immersive IVAS modes should be considered. Additionally, the im-s parameter name was found to be confusing ([1] proposes to change the parameter name to mono-init, which should be more descriptive).

During the call, the media receiver could control the noise suppression level applied at the media sender side by transmitting noise suppression requests. The requests were transmitted over IMS data channel. The ability for the receiver to control the noise suppression level during the call was found to be a useful feature that increased the end user experience in dynamic conditions. [1] proposes to add noise suppression requests to the IVAS payload via PI data, which would enable the feature to be used also in networks without IMS data channel.

The media receiver could also control the transmission bitrate of the media sender by transmitting bitrate switching requests during the call. The bitrate switching feature was observed to be working as intended and did not negatively affect the end user experience.

- Conclusions

The learnings based on the demo indicated that IVAS features such as noise suppression requests and switching between mono EVS and immersive IVAS modes are desirable and that they have practical use in real life scenarios. For more seamless transition and for a better user experience, a proper implementation of switching between mono EVS and immersive IVAS modes should be considered. Noise suppression requests within the IVAS payload are already proposed in [1], and the observations from the demo give good motivation to include such features for IVAS.

- References

[1]	S4-251135 CR26253-0016r1-AnnexA, “Further corrections to Annex A”