3GPP TSG-SA WG4 Meeting #134	S4-252010

# Dallas, US, 17 – 21 November 2025	rev of S4-251697

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.506 | CR | 0012 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [GA4RTAR, TEI19] Correction on N6-unmarked PDUs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Lenovo, BBC, Interdigital, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | GA4RTAR, TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA4 agreed CR0013 to TS 26.113 which specifies that a single PSI value can be applied to all N6-unmarked PDUs, such as RTCP and STUN PDUs. However, this agreement is not reflected in the relevant stage-2 specification, TS 26.506, which currently permits the association of different PSI values with each application protocol used by N6-unmarked PDUs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is specified that a single PSI value is associated with all application protocols used by N6-unmarked PDUs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 and stage-3 specifications are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.7.2.2	N6-unmarked PDUs

Even when PDU Set marking is enabled for an RTC session, some PDUs sent by the RTC AS might still remain unmarked, i.e., they do not include PDU Set identification information. This can occur either because they use a media transport protocol that does not support marking, or because the RTC AS prefers not to mark them – for example, to avoid the overhead of applying marking to low bit rate streams such as audio. For these N6-unmarked PDUs, the UPF is responsible for determining the appropriate PDU Set Information and assigning the PDUs to a PDU Set, as defined in clause 5.37.5.2 of TS 23.501 [11]. However, the UPF lacks a reliable mechanism to determine the PDU Set Importance (PSI) and can only assign it a preconfigured value, since PSI represents the importance of a PDU Set from the perspective of the RTC AS.

If previously negotiated during the WebRTC signalling phase of the RTC session, the Dynamic Policy invoker (i.e., the RTC Media Session Handler of the RTC Client or the Media Function of the RTC AS) may, when instantiating a new Dynamic Policy, indicate to the RTC AF the PDU Set Importance (PSI) value to be associated with all N6-unmarked PDUs on the application flows of the RTC session.

NOTE 1:	This feature provides the 5G Core with application-specific PSI information to apply to N6-unmarked PDUs.

NOTE 2: 	In this release, protocol level differentiation is not supported for N6-unmarked PDUs.