# ISO/IEC JTC 1/SC 29/WG 03 N01573

ISO/IEC JTC 1/SC 29/WG 03
MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output document

Title:	Potential improvement of the of ISO/IEC 23090-14 2nd edition DAM 1: Support of MPEG-I immersive audio, scene understanding and other extensions

Date of document:	2025-07-16

Source:	WG03

No. of pages:	27  (with cover page)

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

ISO/IEC  23090-14:2024/DAM1:2025 (E)

ISO/IEC JTC1/SC 29

Secretariat: JISC

Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 1: Support of MPEG-I audio, scene understanding and other extensions

DAM Potential Improvements

© ISO/IEC 2025

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Fax: +41 22 749 09 47

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 1: Support of MPEG-I audio, scene understanding and other extensions

Modify clause 8 with the following content

In clause 8.1.1 change

AR anchoring allows for anchoring of a scene or some root nodes of a scene into the user’s physical environment to provide an AR experience. AR anchoring is supported at the scene level and at the node level, as well as an extension through the definition of the MPEG_anchor extension.

To

AR anchoring allows for anchoring of a scene or some root nodes of a scene into the user’s physical environment to provide an AR experience. AR anchoring is supported at the scene level and at the node level, as well as an extension through the definition of the MPEG_anchor extension.

Once anchored, the elements of the scene may interact with the physical environment. The anchoring extension also specifies how the representation of this environment may be provided to manage these interactions and to reach expected AR experience.

In clause 8.1.2, change table 28 to

Table28: Definition of the Anchor object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| trackable | integer | M |  | Index of the trackable in the trackables array that will be used for this anchor. |
| requiresAnchoring | boolean | M |  | Indicates if AR anchoring is required for the rendering of the associated nodes. If TRUE, the application shall skip the virtual assets attached to this anchor until the pose of this anchor in the real world is known. if FALSE, the application shall process the virtual assets attached to this anchor |
| minimumRequiredSpace | vec3 | O | (0,0,0) | Space required to anchor the AR asset (width, height, depth in meters). This space corresponds to an axis-aligned bounding box, placed at the origin of the trackable space. Width is aligned with x axis, height with y axis and depth with z axis, expressed in trackable local space. This value shall be compared to the bounding box of the real-world available space associated with the trackable as estimated by the XR runtime. |
| aligned | enumeration | O | NOT_USED | The aligned flag may take one of the following values: NOT_USED=0, ALIGNED_NOTSCALED=1, ALIGNED_SCALED=2. If ALIGNED_SCALED is set, the bounding box of the virtual assets attached to that anchor is aligned and scaled to match the bounding box of the real-world available space associated with the trackable as estimated by the XR runtime. |
| actions | array(number) | O | N/A | Indices of the actions in the actions array of the interactivity extension to be executed once the pose of this anchor is determined. An example is a setTransform action to place the virtual assets attached to that anchor. |
| light | integer | O | N/A | Reference to an item in the lights array of the MPEG_lights_texture_based extension. |
| recommendedSpatialComputingConfig | object | O | N/A | Provides a set of recommended parameters specifying the needed spatial description. The semantics of this object are given in Table 8.1- 1. |

And add the following table after the table 28

Table 8.1- 1: RecommendedSpatialComputingConfig Object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| scanOptions | array | O | N/A | Array of options (enumeration) for the scan computation: possible values are given in Table 8.1- 2. |
| scanDetails | object | O | N/A | Specifies the required level of detail for the representation. The semantics are presented in Table 8.1- 3. |
| scanUpdate | object | O | N/A | Specifies the frequency at which the spatial description must be updated (Table 8.1- 4). |
| scanVolumes | array | O | N/A | Array of bounding volumes that determine the spaces where scanned objects must be provided. Real scan objects that intersect one or more of the bounding volumes should be provided, and all other objects ignored. The semantics for these volumes are presented in Table 8.1- 5. |
| realSemantic | array | O | N/A | Semantic descriptions of nodes that are needed (“table”, “room”, “chair”, “wall”, “light”, “freespace” …) |
| lightOptions | array | O | N/A | Array of option (enumeration) for the light extraction: possible values are given in Table 8.1- 6. |
| lightUpdate | object | O | N/A | Specifies the frequency at which the extraction of real light must be updated (Table 8.1- 4) |

Table 8.1- 2: Possible values for a scanOptions item

| Enumeration value | Description |
| --- | --- |
| PLANE = 0 | Request plane data for scanned objects |
| PLANAR_MESH | Request planar meshes for scanned objects |
| VISUAL_MESH | Request 3D visualization meshes for scanned objects |
| COLLIDER_MESH | Request 3D collider meshes for scanned objects |
| FREE_VOLUME | Request to get the available space around a trackable |
| BOUNDING_BOX | Request a simplified collider mesh |
| TEXTURED_MESH | Request mesh with a texture |

Table 8.1- 3: Semantics for the scanDetail object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| primitivesNumber | number | O | N/A | Gives the quantity of geometric primitives per cubic meter (m3) |
| textureOption | enumeration | O | LOW_RES | Gives the resolution of the texture of textured meshes. Possible values are 512X512 (0), 1024X1024 (1) or 2048X2048 (2). |

Table 8.1- 4: Semantics of a  scanUpdate or lightUpdate object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| occurrence | enumeration | O | ONCE | An occurrence value: - ONCE = 0: the update is performed only once, for instance in case of a static real scene, - N_FRAME: the update is performed periodically, every N rendering frames, N depending on the dynamism of the real scene. The N value is provided in the frameNumber parameter. - AUTO: the update frequency is managed by the module that computes the representation. This module may perform an analysis to detect significant changes in the real world and start an update. This analysis may be performed from raw images data, like RGB images from a camera or depth images from a depth sensor. |
| numberOfFrames | number | O | N/A | Indicates the periodicity, in number of frames, of the update, when the occurrences value is N_FRAME. |

Table 8.1- 5: semantics for a scanVolumes item

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M |  | SPHERE=0, BOX, FRUSTUM |
| If (type == SPHERE) |  |  |  |  |
| center | array | M |  | 3D coordinates of the center of the sphere |
| radius | number | M |  | Radius of the sphere in meters. |
| If (type == BOX) |  |  |  |  |
| pose | matrix | M |  | 4*4 matrix representing the center position and orientation of the box |
| extents | array | O |  | Edge-to-edge length of the box along each dimension. |
| If (type == FRUSTUM) |  |  |  |  |
| pose | matrix | M |  | 4*4 matrix representing the position and orientation of the tip of the frustum |
| fov | vec4 | M |  | Angles of the four sides of the frustum |
| far | number | M |  | Positive distance of the far plane of the frustum |
| near | number | M |  | Positive distance of the near plane of the frustum |
| 3D coordinates are expressed in the XR space related to trackable associated to the anchor. |  |  |  |  |

Table 8.1- 6: Possible values for a lightOptions item

| Enumeration value | Description |
| --- | --- |
| DirectionalLight = 0 | Request the extraction of directional lights |
| EnvLight | Request the extraction of environment lights |
| PointLight | Request the extraction of point lights |
| SpotLight | Request the extraction of spot lights |
| AreaLight | Request the extraction of area lights |

In clause 8.1.3 change

If the array of action is not empty, the actions are executed once the pose of anchor is determined. If the tracking status is set to FALSE after being TRUE, actions are not canceled.

To

If the array of action is not empty, the actions are executed once the pose of anchor is determined. If the tracking status is set to FALSE after being TRUE, actions are not canceled.

When processing the MPEG_anchor extension, if a recommendedSpatialComputingConfig  object is present, the Presentation Engine checks if it can retrieve the recommended spatial description, specified in the  parameters such as scanOptions, scanDetails, scanUpdate, scanVolumes, scanSemantic, lightUpdate and lightOptions.
If all the recommended parameters are not satisfied, the Presentation Engine may continue the rendering of the scene with the available spatial description, but possibly with a degraded XR experience.

At runtime, the Presentation Engine may then request elements of the spatial description according to the recommendedSpatialComputingConfig parameters.

In clause 8.2.1 change

The MPEG_node_interactivity extension is used to complement the interactivity defined at the scene level. One particular case is the definition of the parameters for the physics engine. That is, when an MPEG_node_interactivity extension contains a trigger of type TRIGGER_COLLISION without being referenced by a trigger of type TRIGGER_COLLISION at the MPEG_scene_interactivity extension, this node shall not be considered for collision detection and instead only be used by the physics engine.

To

The MPEG_node_interactivity extension is used to complement the interactivity extension defined at the scene level. One particular case is the definition of the parameters for a physics engine. That is, when an MPEG_node_interactivity extension contains a trigger of type TRIGGER_COLLISION without being referenced by a trigger of type TRIGGER_COLLISION at the MPEG_scene_interactivity extension, this node shall not be considered for collision detection and instead only be used by the physics engine.

When specified, at node and/or scene level, a physics object specifies a set of parameters for the physic engine, used by the application.

In clause 8.2.2.1, in table 33, change

| if (type== TRIGGER_USER_INPUT){ |  |  |  |  |
| --- | --- | --- | --- | --- |
| userInputDescription | string | M |  | Describes the user body part and gesture related to the input. The format shall follow the OpenXR input path description as defined in [OpenXR] section 6. An example is: “/user/hand/left/grip”. |
| nodes | array | O | N/A | Indices of the nodes in the nodes array to be considered for this user input. |
| } |  |  |  |  |

To

| if (type== TRIGGER_USER_INPUT){ |  |  |  |  |
| --- | --- | --- | --- | --- |
| userInputDescription | string | M |  | Describes the user body part and gesture related to the input. The format shall follow the OpenXR input path description as defined in [OpenXR] section 6. An example is: “/user/hand/left/grip”. The annex I gives extra semantics for user input triggers involving multiple users. |
| nodes | array | O | N/A | Indices of the nodes in the nodes array to be considered for this user input. |
| } |  |  |  |  |

In clause 8.2.2.1, change the table 32 to

Table32: Semantic of the MPEG_scene_interactivity extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| physics | object | O | N/A | Provides a set of parameters at scene level to be used for the physics simulation. The semantics of this object are given in table 8.2-1. |
| triggers | array | M | [] | Contains the definition of all the triggers used in that scene |
| actions | array | M | [] | Contains the definition of all the actions used in that scene |
| behaviors | array | M | [] | Contains the definition of all the behaviors used in that scene. A behavior is composed of a pair of (triggers, actions), control parameters of triggers and actions, a priority weight and an optional interrupt action |

And add the following text after table 32

The semantics of a physics object at scene level, are defined in Table 8.2- 1.

Table 8.2- 1: Semantics of a scene level physics object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| recommendedPhysicsHighPrecision | boolean | O | false | Determines whether the application should enable a more deterministic and precise physic simulation |
| gravity | number | O | -9.81 | Determine the gravity for the whole scene. In meters per second square (m.s-2), as defined in the international unit system. |
| recommendedPhysicsFrameRate | number | O | 50 | Provides the recommended frame rate at which the Physics Engine should operate. In frame per second, as defined in the international unit system. |
| bounceThreshold | number | O | 1 | A contact with a relative velocity below this threshold will not result in a bounce. In meter per second (m.s-1), as defined in the international unit system. |

In clause 8.2.2.2, change the table 52 to

Table 52: Semantic of the MPEG_node_interactivity.trigger extension

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M |  | One element of Table 34 that defines the type of the trigger. |
| if (type == TRIGGER_COLLISION){ |  |  |  |  |
| collider | integer | M |  | the index of the mesh element that provides the collider geometry for the current node. The collider mesh may reference a material. |
| isStatic | boolean | M |  | If True, the collider is defined as a static collider. |
| physics | object | O | N/A | Provides a set of parameters at node level to be used for physics simulation. The semantics of this object are given in table 8.2-2. |
| primitives | array(Primitive) | O | N/A | List of primitives used to activate the proximity or collision trigger. Semantics are presented in Table 35. |
| } |  |  |  |  |
| … |  |  |  |  |

And add the following text after table 52

The semantics of a physics object at node level, are defined in Table 8.2- 2

Table 8.2- 2: Semantics of a node level physics object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| needPreciseCollisionDetection | boolean | O | false | If true, the physics engine should handle the collision detection more accurately by increasing the detection rate for this node. |
| linearDamping | number | O | 0 | A non-negative value, in second-1 (s-1), as defined in the international unit system. It defines the linear drag coefficient which corresponds to the rate of decrease of the linear velocity over time. It is used to compute a new velocity value V(t) at each simulation step (dt): V(t+dt) = V(t)*(1-linearDamping*dt), the velocity being clamped to 0. |
| angularDamping | number | O | 0 | A non-negative value, in second-1 (s-1), as defined in the international unit system. It defines the angular drag coefficient which corresponds to the rate of decrease of the angular velocity over time. It is used to compute a new velocity value V(t) at each simulation step (dt): V(t+dt) = V(t)*(1-angularDamping*dt), the velocity being clamped to 0. |
| useGravity | boolean | M |  | Indicates if gravity affects the object |
| mass | number | M |  | Mass of the object in kilogram, as defined in the international unit system. |
| restitution | number | M |  | Provides the ratio of the final to initial relative velocity between two objects after they collide |
| staticFriction | number | M |  | Unitless static friction coefficient as defined in the Coulomb friction model [6]. Friction is the quantity which prevents surfaces from sliding off each other. Static friction is used when the object is lying still. It will prevent the object from starting to move. |
| dynamicFriction | number | M |  | Unitless static friction coefficient as defined in the Coulomb friction model. When a large enough force is applied to the object, a dynamic friction is used and will attempt to slow down the object while in contact with another. |
| } |  |  |  |  |

In clause 8.2.3 change

If the scene description document contains a description of physics properties based on another physics model, then that physics model shall take precedence in the processing of the scene.

Otherwise, the application shall handle a physics simulation if the usePhysics Boolean is TRUE on any of the collision trigger extensions defined at the node level. When a collision occurs between two nodes, the application should calculate the combination of the restitution, static friction and dynamic friction values based on the values provided by the collision trigger extension of the two nodes.

To

The application shall handle a physics simulation if a physics object is defined at a scene level or/and at any of the nodes. When a collision occurs between two nodes, the application should calculate the combination of the restitution, static friction and dynamic friction values based on the values provided by the physics objects of the two nodes.

In clause 8.4.2.2 change

When present, the MPEG_lights_punctual extension shall be included as a glTF file level extension.

The definition of all objects within MPEG_lights_punctual extension is provided in Table 58.

Table 58 —Definition of glTF file objects of MPEG_lights_punctual extension

To

When present, the MPEG_lights_punctual extension shall be included as a glTF file level extension.

The definition of all objects in the array within the MPEG_lights_punctual extension is provided in Table 58.

Table 58 —Definition of glTF file objects of the array in the MPEG_lights_punctual extension

In clause A.11 change

MPEG_anchor schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_anchor.schema.json.

To

MPEG_anchor schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-2/en/amd/1/MPEG_anchor.schema.json.

In clause A.13 change

MPEG_scene_interactivity schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_scene_interactivity.schema.json

MPEG_node_interactivity schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_node_interactivity.schema.json

To

MPEG_scene_interactivity schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-2/en/amd/1/MPEG_interactivity.schema.json

MPEG_node_interactivity schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-2/en/amd/1/MPEG_node_interactivity.schema.json

In clause A.15, change

MPEG_lights_texture_based schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_lights_texture_based.schema.json

To

MPEG_lights_texture_based schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-2/en/amd/1/MPEG_lights_texture_based.schema.json

In clause A.16, change

MPEG_light_punctual schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/amd2/MPEG_light_punctual.schema.json

To

MPEG_lights_punctual schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-2/en/amd/1/MPEG_lights_punctual.schema.json

Add the following to Annex A

A.17 JSON schema for MPEG_node_mapping

MPEG_node_mapping schema is downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-2/en/amd/1/MPEG_node_mapping/MPEG_node_mapping.schema.json.

In annex F, change

In the example downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-1/en/example_MPEG_media.json, two media items are listed by MPEG_media extension.

To

In the example downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-2/en/amd/1//example_MPEG_media.json, two media items are listed by MPEG_media extension.

In annex F, change

In the example downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-1/en/example_MPEG_scene_dynamic.json, the media object includes the patch document format file name and its track index.

To

In the example downloadable from https://standards.iso.org/iso-iec/23090/-14/ed-2/en/amd/1/example_MPEG_scene_dynamic.json, the media object includes the patch document format file name and its track index.

Add the following sub-section to Annex G

## G.4 	Support for MPEG-I immersive audio

### G.4.1 General

MPEG-I Immersive Audio has been specified in ISO/IEC 23090-4. The specification assumes the presence of an MPEG-I immersive audio renderer that will receive the MPEG-I immersive audio bitstream, a set of MPEG-H audio streams, as well as information about some scene metadata, such as listener’s pose. It will then use the audio scene metadata in the MPEG-I immersive audio bitstream, the decoded audio data, for example, from MPEG-H 3D audio (ISO/IEC 23003-3)streams, and the pose information to render the spatial audio. In case of coded audio data with any other codec, the respective decoder is required.

The support of MPEG-I immersive audio is achieved by referencing an MPEG-I immersive audio stream in a MPEG-I scene description document.

The MPEG-I immersive audio bitstream contains a description of the audio scene that is independent of the main scene description consumed by the Presentation Engine.  An alignment between the Presentation Engine and the MPEG-I immersive audio Renderer is needed, that goes beyond the traditional time alignment but includes also spatial alignment. For that, a mapping need to be established between the node in a MPEG-I scene description document and the node of the audio scene.

The following figure depicts an example of a mapping between a node that contains a car and an external audio node in an MPEG-I immersive audio bitstream, with a simplified geometry of that car and the attached audio sources. This mapping is described in an MPEG node mapping extension.

Figure G.4- 1: MPEG-I audio mapping example

This MPEG node mapping extension, identified by MPEG_node_mapping, can be used in a broader scope: It establishes a mapping between the node in a MPEG-I scene description document and an external entity, e.g. an MPEG-I immersive audio renderer, that handles a dedicated scene graph, separate from the main scene description. When present, the MPEG_node_mapping extension shall be included in a node object.

The architecture for the support of such an external renderer is depicted in the following figure:

Figure G.4- 2: Architecture for external render support

The Generic Render Control API is an abstract API that is offered by external renderers to enable applications, such as Presentation Engines, to control the rendering process by aligning and synchronizing their rendering state to that of the Presentation Engine. This API is used by the Presentation Engine to configure and update the status of the external renderer.

### G.4.2 Generic Render Control API

The following table describes the functionality provided by the generic render control API:

Table G.4- 1: Generic Render Control API

| Method | Description |
| --- | --- |
| init() | Initializes the external renderer by providing the related media source information and their corresponding buffers. It also establishes a session between the Presentation Engine and the external renderer. The inputs to this method call should be: - A media source object that contains a handler to the buffer(s), where the source media will be made available by the MAF. A description of the media source and the contents of each buffer shall also be provided. |
| configure() | Configures the external renderer to establish an initial alignment and synchronization between the Presentation Engine and the external renderer. The parameters to this method may include: - A mapping between the initial timestamp of the common Presentation Engine timeline and that of the media associated with the external renderer. It also provides information about the clock rate of the Presentation Engine. - A list of mapped nodes in the source media rendered by the external renderer. This list shall at least contain one object with a mapping to the main camera of the main scene description. For audio renderers, this may be the audio listener. The information is provided by the MPEG_node_mapping extension in the scene description document. It should also provide the initial pose of the mapped nodes after applying the 4x4 matrix of the transform parameter provided in the node mappings. The external renderer may then subscribe for updates to specific aligned nodes, or it may specifically ask for current state for these nodes, using the referenceId. |
| start() pause() resume() stop() | Allows the Presentation Engine to control the playback of selected media sources associated with the external renderer for interactivity purposes. |
| update() | Used by the Presentation Engine to update node positions and orientations for which there is a mapping with the external renderer. Updates may result from received scene updates, user interactions, animations, physics simulations, or any other events. The Presentation Engine uses this update() method when one or more mapped nodes need to be spatially synchronized, depending on their synchronizationOccurrence and synchronizationOccurenceCombination parameters provided in their MPEG_node_mapping extension. The parameters passed to this method are an array of objects consisting of: - The referenceId of the node to which this update applies - The current pose of the mapped node after applying the 4x4 matrix of the transform parameter provided in the corresponding MPEG_node_mapping |
| updateGraph() | The Presentation Engine uses the updateGraph function to add, update, or remove a set of nodes to the internal representation of the scene that is maintained by the external renderer. Only nodes that have a mapping with the external renderer can be passed through this method. The parameters to this method are an array of objects that include: - The graph operation: ADD, REMOVE, UPDATE - For ADD: the referenceId and the initialization information for the associated media data to the object that is to be added. - For REMOVE: the referenceId of the object to be removed. - For UPDATE: the referenceId of the object to be updated, as well as a dictionary of attributes and their update values. |
| registerCallback() | The Presentation Engine may provide a callback function to the external renderer to allow it to query the status of certain parameters at any time. This may for example include asking for the current user pose. The Presentation Engine shall register a callback function whenever possible. |

The following is a description for the API in IDL (ISO/IEC 19516):

interface GenericRenderControl { void init(); void configure(); void start(); void pause(); void resume(); void stop(); update(); void updateGraph(); void registerCallback(); };

### G.4.3 Semantics

The semantics of the MPEG_node_mapping extension are provided in the following table. When present, the MPEG_node_mapping extension shall be included in a node object.

Table G.4- 2: Semantics of the MPEG_node_mapping extension

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| mappings | array(object) |  | M | An array of mappings associated with the containing node. |
| component | string | “urn:mpeg:sd:component:default” | O | An identifier of the component associated with this mapping. The component may for instance be “urn:mpeg:sd: component:audio-renderer” to indicate that the component is an audio renderer. |
| source | number | N/A | M | The index in the MPEG_media that provides the media resource that contains the mapped element. |
| referenceId | number | N/A | M | An identifier of the element in the referenced resource. |
| transform | array(number) | Identity | O | A 4x4 TRS matrix which transforms the 3D coordinates of the node having this glTF extension expressed in the glTF2.0 scene coordinate system to the 3D coordinates of the node of the external renderer graph referenced by the referenceId identifier expressed in the external renderer scene coordinate system. If the mapped node is a child of an AR Anchor/Trackable, the 4x4 TRS matrix transforms the 3D coordinates of the node having this glTF extension expressed in the AR Anchor/Trackable coordinate system to the 3D coordinates of the node of the external renderer graph referenced by the referenceId identifier expressed in the AR Anchor/Trackable coordinate system. |
| updateRecommendation | object | N/A | O | Indicate update recommendations for the node. The semantics is given in table x2. |
| supportsInteractivity | boolean | false | O | Indicates if interactivity actions applied to the node should be exposed if an API is made available to the Presentation Engine by the renderer of the resource. |

Table G.4- 3: Semantics of an updateRecommendation object

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| synchronizationOccurrences | array(enumeration) | [EVENT] | O | An array of synchronization occurrences. Each element of this array is an Enumerator with the following possible values: - ONCE: the synchronization is done once at the configuration step, - EVENT: the synchronization is done based on the activation of one or more triggers (e.g., visibility, proximity, collision, user input) as those defined in the MPEG_interactivity extension. The indices of the triggers, from the triggers array of the MPEG_scene_interactivity extension, are given in the events parameter, - N_FRAME: the synchronization is periodic every N (1, 2, …) rendering frames. The N value is provided in the frameNumber parameter. |
| events | array(number) | N/A | 0 | Array of indices of triggers from the triggers array of the MPEG_scene_interactivity extension. Required when EVENT is mentioned in the synchronizationOccurrences array. |
| frameNumber | number | N/A | 0 | Indicate the periodicity, in number of frames, of the synchronisation, when N_FRAME is mentioned in the synchronizationOccurrences array. |
| synchronizationOccurrenceCombination | string | “¦” | O | A set of logical operations to apply to the synchronization occurrences. A ‘#’ indicates the occurrence index, ‘&’ indicates a logical AND operation, ‘¦’ a logical OR operation and ‘~’ a NOT operation. Parenthesis are used to group some operations. Such a syntax may give the following string: “#1&~#2¦(#3&#4)”. |

### G.4.4 Processing model

When processing the MPEG_node_mapping extension, the Presentation Engine shall identify nodes in the scene description that have a node mapping. The Presentation Engine shall determine if the component identified by the indicated component parameter supports the Generic Render Control API. If it does, the Presentation Engine shall pass the mapping information to the identified component.

The Presentation Engine shall then use the API to align the rendering with the component as configured over the API.

Add the following sub-section to Annex G

## G.5 	Support for Gaussian Splats

### G.5.1	Introduction

Gaussian Splats (GS) provide an efficient representation for static and dynamic 3D scenes by compactly encoding local geometry and view-dependent appearance. To address interoperability, backward compatibility, and progressive rendering in various applications, this section defines how 3D and 4D Gaussian Splats are stored in Scene Description.

Gaussian Splats are stored as an extension of (dynamic) point clouds in scene description by defining a set of additional application-specific attributes and using “mode: 0” of the glTF primitive for points.

### G.5.2	Semantics

The application-specific Gaussian Splat attributes are defined in Table G.5- 1.

Table G.5- 1: Gaussian Splat attributes

| Name | Description |
| --- | --- |
| _MPEG_GS_ORIENTATION | Reference to accessor or timed accessor of type VEC4. It provides the quaternion rotation (x, y, z, w) defining the local axes orientation of each Gaussian splat. |
| _MPEG_GS_SCALE | Reference to accessor or timed accessor of type VEC3. It provides the ellipsoid scale along local X, Y, Z axes defining the size of each Gaussian splat. |
| _MPEG_GS_SH_COEFF_R | Reference to accessor or timed accessor of type SCALAR. It provides the SH coefficients for the Red color channel of the Gaussian splat. |
| _MPEG_GS_SH_COEFF_G | Reference to accessor or timed accessor of type SCALAR. It provides the SH coefficients for the Green color channel of the Gaussian splat. |
| _MPEG_GS_SH_COEFF_B | Reference to accessor or timed accessor of type SCALAR. It provides the SH coefficients for the Blue color channel of the Gaussian splat. |
| _MPEG_GS_SH_COEFF_FIRST | Reference to accessor or timed accessor of type SCALAR. It provides the first-order spherical harmonic coefficients (9 coefficients) for initial progressive download. |
| _MPEG_GS_SH_COEFF_SECOND | Reference to accessor or timed accessor of type SCALAR. It provides the second-order spherical harmonic coefficients (15 coefficients) for subsequent progressive download. |
| _MPEG_GS_SH_COEFF_THIRD | Reference to accessor or timed accessor of type SCALAR. It provides the third-order spherical harmonic coefficients (21 coefficients) for advanced progressive download. |

The other gaussian splat attributes are mapped to the existing glTF primitive attributes according to the Table G.5- 2:

Table G.5- 2: glTF mapped Gaussian spalt attribute

| GS Property | Corresponding glTF Attribute |
| --- | --- |
| x, y, z | POSITION |
| f_dc_[0-2] | COLOR_n (may need to be normalized) |
| opacity | Alpha channel of COLOR_n |

### G.5.3	Processing Model

When a scene description references Gaussian Splats via the application-specific attributes, the GS MAF pipeline retrieves Gaussian Splat data from buffers through accessors specified by these attributes. For static Gaussian Splats, these attributes are fetched once and used continuously throughout the rendering session. In contrast, dynamic Gaussian Splats utilize timed accessors and circular buffers as defined by this specification.

Each rendering cycle retrieves updated Gaussian Splat attributes from the circular buffer. If no new attribute data is available, the renderer continues to use the last available set of Gaussian Splat attributes. Dynamic updates of Gaussian Splat data within circular buffers shall follow the format and size specified in their corresponding timed accessors.

Progressive download and rendering are explicitly integrated into the Gaussian Splat processing model. Upon initialization, the renderer fetches and processes initial coarse-level attributes (POSITION and COLOR_n) from the corresponding buffer views to provide immediate visual representation. Subsequently, as more detailed data becomes available, the renderer incrementally updates Gaussian Splats by loading ORIENTATION, SCALE, and progressively refined spherical harmonic coefficients through higher-level buffer views. Each buffer view is processed sequentially, enhancing visual fidelity dynamically as additional data is retrieved. The renderer manages these updates efficiently, ensuring smooth transitions and continuous refinement of the displayed scene according to network conditions and rendering capabilities.

Add the following sub-section to Annex G

## G.6 	Support for stereo applications

### G.6.1	Introduction

To integrate the support of stereo codec applications in MPEG Scene Description, a glTF extension, identified by MPEG_Material_stereo, is used.

Each mesh primitive has a reference to a material that defines how the surface of a 3D object looks when rendered. The MPEG_material_stereo extension is applied to the material property. The mesh defines the geometry of the object, while the MPEG_material_stereo extension aggregates all relevant texture information processed for rendering.

### G.6.2	Semantics

The semantics of the MPEG_material_stereo extension is shown in Table G.6- 1.

Table G.6- 1: Semantics of the MPEG_material_stereo extension

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| layers | array[Layer] | N/A | M | Provides an array of layer objects, limited to 2 layers. See Table G.6- 2 for the semantics of a layer object. |
| Legend: For attributes: M=mandatory, O=optional, OD=optional with default value, CM=conditionally mandatory. |  |  |  |  |

In Table G.6- 1, a layer is an object which combines a texture with associated rendering properties. Each texture is described in an item of the layers property of the MPEG_material_stereo extension. Additional properties in the layer object indicate whether the texture corresponds to a primary reference layer or a secondary layer and specify its association with the left or right eye.

Table G.6- 2: Semantics of a MPEG_material_stereo layer object

| Name | Type | Default | Usage | Description |
| --- | --- | --- | --- | --- |
| primaryReference | boolean | N/A | M | Indicates whether this layer component is the primary component (also known as the hero view) among other layers. When primaryComponent is set to True, this indicates the layer component is a primary component. Only one layer among the layers of the media object shall have the primaryReference set to True. |
| texture | integer | N/A | M | Provides the index of a texture. |
| eyeChannel | integer | N/A | M | Indicates if the information refers to left (0) or right (1) eye . |
| Legend: For attributes: M=mandatory, O=optional, OD=optional with default value, CM=conditionally mandatory. |  |  |  |  |

### G.6.3	Processing Model

Add the following Annex I

Annex I 
(normative)

Support for multi-users interactivity

## I.1	Introduction

This annex is about multi-users application where multiple users meet in a shared space. This space may be described with MPEG-SD and may contain interactivity features as specified in section 8.2.

The triggers, as specified in section 8.2.2.1, do not fully support interactions that involve multiple users.

For the TRIGGER_USER_INPUT trigger, the trigger is evaluated for each user and the actions associated with the trigger may be performed multiple times, for each received user’s input that met the trigger condition.

For a TRIGGER_USER_INPUT trigger involving multiple users, the trigger must be evaluated for all the users and the actions associated with the trigger must be performed only once, when all the user’s inputs met the trigger condition (or when at least one depending on the scenario).

This Annex provides semantics that address multi-users triggers.

## I.2	Syntax

The following table give the semantics of a trigger object, as specified in the table 33 of the section 8.2.2.1, with a new “multiUsers” parameters added in the extras field.

Table33: semantics of a trigger

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | enumeration | M |  | One element that defines the type of the trigger. |
| … |  |  |  |  |
| extras | object | O | N/A |  |
| multiUsers | object | O | N/A | Additional parameters to address a TRIGGER_USER_INPUT trigger involving multiple users. See Table I.2- 1 for the details of this object. |

Table I.2- 1: semantics of a multiUsers object

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| multiCondition | enumeration | O | ALL | If ALL, the trigger’s condition must be met by all the users. If ONLYN, the trigger’s condition must be met by exactly N users. If ATLEASTN, the trigger’s condition must be met by at least N users. |
| duration | number | O | 2 | Give a time duration, in second. After the trigger condition is met by a first user, it specifies a time window during which the multiCondition must be met. |
| number | number | O | 1 | Give the number of users when condition is ONLYN or ATLEASTN. |

I.3	Processing model

After receiving a scene description file, the application parses the file and starts collecting information from all the connected users (user input, pose…).

The application then enters the rendering loop which include a scene update phase performed by the scene manager (SM), that processes the scene interactivity by iterating over the behaviors  and manage the other elements of the scene description (animation, lighting, physics…).

The iteration over the behaviors is detailed here after, based on the diagram in the Figure I.3- 1.

During the processing of the scene interactivity, for each behavior and for each trigger referenced in a behavior (1):

- (2) The SM checks if the trigger involves multiple users i.e. a multiUsers parameter is specified in the triggers description.

- (3 to 5) If the trigger is not multi-users, for each user information that meet the trigger’s condition, the SM executes the actions referenced in the behavior.  It then continues the processing with the next behavior (1).

- (6) If the trigger is multi-users, the SM check if the trigger condition is met for at least one user:

  - if yes, (7) the SM checks if the condition specified in the multiUsers object is met, i.e. the multiCondition has been met during the given duration:

    - If yes, the SM executes the actions referenced in the behavior (8).

    - if no, the processing continues with the next behavior (1).

  - if no, the processing continues with the next behavior (1).

Figure I.3- 1: MPEG-SD triggers processing

Add the following reference in the Bibliography section

[6] “What is the Coulomb friction law”: https://www.encyclopedie-environnement.org/en/zoom/what-is-the-coulomb-friction-law/