3GPP TSG- SA4 Meeting # 134	S4-251689

17-21 November 2025, Dallas, Texas, USA

5G-MAG TECH0182r8

Title:	Service APIs for MBS User Services

Response to:

Release:	3GPP Rel-17, Rel-18, Rel-19

Work Item:	5MBS

Source:	5G-MAG MEDIA ACTION GROUP

To:	3GPP TSG CT WG3, 3GPP TSG CT WG4

Cc:	3GPP TSG SA WG4

Contact person:	Jordi J. Gimenez, Head of Technology

gimenez@5g-mag.com

Send any reply LS to:	5G-MAG Project Office, info@5g-mag.com

Attachments:

# 1	Overall description

5G-MAG’s Development Working Group is working on a reference implementation of MBS User Services, as specified by 3GPP in TS 26.517, TS 29.580, TS 29.532, TS 29.580 and TS 29.581. During the course of this effort, a number of issues have been identified with the Nmbsf_MBSUserDataIngestSession service API used by the MBS Application Provider (AF/AS) at reference point Nmb10 to provision MBS User Data Ingest Sessions in the MBSF (as specified in TS 29.580), and with the Nmbstf_DistSession service API used by the MBSF at reference point Nmb2 to configure MBS Distribution Sessions in the MBSTF (as specified in TS 29.581).

Although 5G-MAG’s reference implementation is specifically targeting Release 18, 5G-MAG believes that the some of these issues are also present in the latest Release 17 and Release 19 versions of these specifications.

This LS outlines several issues for consideration by CT3 and CT4, to which 5G-MAG members intend to contribute corresponding CRs for resolution.

# 2	Issues for consideration by CT3

## 2.1	Cardinality of active periods repetition rule is incorrectly specified

In TS 29.580, table 6.2.6.2.2-1 specifies the MBSUserDataIngSession data type and table 6.2.6.2.4-1 specifies the MBSUserDataIngSessionPatch data type. In both cases, the cardinality of the actPeriodsRepRule attribute is specified as "1..N". However, this attribute is not specified as an array in the tables, nor in the OpenAPI YAML. 5G-MAG believes that the cardinality of this attribute should be specified as "0..1" in both tables.

For more detailed analysis, please refer to https://github.com/5G-MAG/Standards/issues/183.

## 2.2	Conditional population of object acquisition identifiers is underspecified

TS 29.580 specifies the ObjectDistrMethInfo data type, including some rules for populating its members. The set of population rules is incomplete with respect the stage 2 definitions in table 6.1-1 of TS 26.502. In particular:

| Distribution method | Operating mode | Description |
| --- | --- | --- |
| OBJECT | OBJECT_SINGLE | Each object ingested by the MBSTF is distributed once. Either pull-based and push-based object acquisition methods may be provisioned in combination with this operating mode. When the pull-based object acquisition method is provisioned, the MBS Distribution Session parameters (see table 4.5.6-2) shall cite a set of one or more object URLs as Object acquisition identifiers. When the push-based object acquisition method is provisioned, the set of Object acquisition identifiers shall be empty. |
|  | OBJECT_COLLECTION | A set of objects described by a manifest (see NOTE) is ingested by the MBSTF and distributed once. Either pull-based and push-based object acquisition methods may be provisioned in combination with this operating mode. When the push-based object acquisition method is provisioned, the object manifest (only) shall be pushed to the MBSTF; the objects referenced by the object manifest shall be pulled. The MBS Distribution Session parameters (see table 4.5.6-2) shall cite a single object manifest URL in Object acquisition identifiers. |
|  | OBJECT_CAROUSEL | A set of one or more objects described by a manifest (see NOTE) is ingested by the MBSTF and distributed according to a repetition pattern specified in the manifest. Any change to an object during the course of the MBS Distribution Session is reflected in the distribution at the next available opportunity. Either pull-based and push-based object acquisition methods may be provisioned in combination with this operating mode. When the push-based object acquisition method is provisioned, the object manifest (only) shall be pushed to the MBSTF; the objects referenced by the object manifest shall be pulled. The MBS Distribution Session parameters (see table 4.5.6-2) shall cite a single object manifest URL in Object acquisition identifiers. |
|  | OBJECT_STREAMING | A sequence of objects is ingested by the MBSTF and streamed in real time, for example according to a schedule described in an Application Service Entry Point document (e.g. DASH MPD). Either pull-based and push-based object acquisition methods may be provisioned in combination with this operating mode. For both object acquisition methods, the MBS Distribution Session parameters (see table 4.5.6-2) shall cite a single Application Service Entry Point URL of each type in Object acquisition identifiers (see NOTE 2). As a consequence, the referenced Application Service Entry Point document(s) shall be included in the MBS User Service Announcement Channel as ancillary objects. All Application Service Entry Points referenced by an MBS Distribution Session shall be for equivalent presentations, i.e., comprising common object streams according to compatible presentation timelines. |
| NOTE: OBJECT_COLLECTION operating mode is a special case of OBJECT_CAROUSEL operating mode in which the objects described by the manifest are distributed only once. The baseline parameters of the object manifest are defined in clause 4.5.10. |  |  |

For more detailed analysis, please refer to https://github.com/5G-MAG/Standards/issues/168.

## 2.3	Optional feature for extended event notifications inconsistently specified

From Release 18 onwards, TS 29.580 specifies optional features in table 6.2.8-1. The second of these is specified with the name MBSEventsExt. This name does not appear anywhere else in the document.

In its last column, table 6.2.6.3.4-1 specifies that some event notifications are applicable to the EventExt feature, but this feature is not specified in the document.

It is assumed that MBSEventsExt and EventExt are referring to the same feature, but there is a naming inconsistency.

For more detailed analysis, please refer to https://github.com/5G-MAG/Standards/issues/180.

## 2.4	OpenAPI YAML for MBS User Data Ingest Session patch operation incorrectly specified

Clause 5.3.2.4 of TS 29.580 specifies the Nmbsf_MBSUserDataIngestSession_Update service operation. One available option is to use the MBSUserDataIngSessionPatch data structure in combination with the HTTP PATCH method.

Clause 5.3.2.4 of TS 29.580 specifies that an individual MBS Distribution Session may be removed from the mbsDisSessInfos map by quoting its reference as the key and by setting the corresponding value in the map to NULL. However, the OpenAPI YAML definition for the mbsDisSessInfos only allows the entire map to be nullified, removing all MBS Distribution Sessions from the MBS User Data Ingest Session. This appears to be inconsistent with the Nmbsf_MBSUserDataIngestSession_Create service operation specified in clause 5.3.2.2 of TS 29.580 which requires at least one MBS Distribution Session to be present in the MBS User Data Ingest Session.

5G-MAG believes that the OpenAPI YAML in this case is incorrectly specified.

For more detailed analysis, please refer to https://github.com/5G-MAG/Standards/issues/182.

# 3	Issues for consideration by CT4

## 3.1	Cardinality of object acquisition identifiers

Table 6.1-1 of TS 26.502 defines certain rules for the cardinality of object acquisition identifiers. In particular:

| Distribution method | Operating mode | Description |
| --- | --- | --- |
| OBJECT | OBJECT_SINGLE | Each object ingested by the MBSTF is distributed once. Either pull-based and push-based object acquisition methods may be provisioned in combination with this operating mode. When the pull-based object acquisition method is provisioned, the MBS Distribution Session parameters (see table 4.5.6-2) shall cite a set of one or more object URLs as Object acquisition identifiers. When the push-based object acquisition method is provisioned, the set of Object acquisition identifiers shall be empty. |

This syntax rule is too complex to be expressed using OpenAPI YAML schema constraints, but it would nevertheless be useful to explicitly specify this restriction in clause 6.1.6.2.5 of TS 29.581 for the benefit of implementers referring only to the stage-3 specification. For example:

"In the case where objDistributionOperatingMode is set to SINGLE and objAcquisitionMethod is set to PUSH, the objAcquisitionIdsPull attribute shall be omitted and the objAcquisitionIdPush attribute shall be omitted, indicating that any object may be pushed to the MBSTF."

For more detailed analysis, please refer to https://github.com/5G-MAG/Standards/issues/168.

## 3.2	Configuring the source address for User Plane multicast IP packets

The MBS Session ID is defined by TS 23.247 as either or both of the following:

- A Temporary Mobile Group Identifier (TMGI) or

- A Source-Specific Multicast (SSM) address. This is the combination of a source IP address and a destination multicast group IP address.

TS 29.571 defines a shared MbsSessionId data type to realise this conceptual identifier at stage-3. The MbsSessionId data type is used to provision the MBSF at reference point Nmb10 (TS 29.580) and to provision the MB-SMF at reference point Nmb1 (TS 29.532).

At reference point Nmb2, only the source-specific multicast address is relevant, since this is what needs to be included in multicast IP datagrams generated by the MBSTF and tunnelled to the MB-UPF. TS 29.581 specifies a UpTrafficFlowInfo data type to enable configuration of MBSTF User Plane traffic by the MBSF, as required by TS 26.502. This provides the following attributes:

- Destination IP address, as required.

- Port number, as also required. (This is assumed to be the destination port number for User Plane traffic, although the specification in TS 29.581 is ambiguous, and could usefully be clarified.)

No provision is made in UpTrafficFlowInfo for the MBSF to indicate the required source IP address. In the absence of this parameter at reference point Nmb2 the MBSF is unable to populate the source IP address in the User Service Announcement it advertises to MBS Clients. The resulting system is therefore not viable for deployment as currently specified.

For a more detailed analysis, please refer to https://github.com/5G-MAG/Standards/issues/172.

## 3.3	Configuring the FLUTE Transport Session Identifier for ALC packets in the User Plane

TS 26.517 specifies the use of the FLUTE protocol per IETF RFC 3926 for the transport of objects such as media segments to the MBS Client at reference point MBS-4-MC. This, in turn is based on the ALC protocol per IETF RFC 3450.

According to RFC 3926, every FLUTE/ALC session is uniquely identified by the combination of its source IP address (part of the Source-Specific Multicast address – see section 3.2 above) and a Transport Session Identifier (TSI). Both of these identifiers need to be included in the MBS User Service Announcement advertised to MBC Clients.

No provision is currently made in TS 29.581 (in the UpTrafficFlowInfo data structure or elsewhere in the DistSession data structure) for the MBSF to indicate the required source FLUTE/ALC Transport Session Identifier. In the absence of this parameter at reference point Nmb2 the MBSF is unable to populate the FLUTE/ALC Transport Session Identifier in the MBS User Service Announcement advertised to MBS Clients. The resulting system is therefore not viable for deployment as currently specified.

For more detailed analysis, please refer to https://github.com/5G-MAG/Standards/issues/174.

## 3.4	Status notification URL specification

TS 29.581 provides the stage-3 specification for the Nmbstf_MBSDistributionSession service. The StatusSubscribe service operation involves the subscriber supplying an object of type StatusSubscribeReqData and the statusNotification callback of the StatusSubscribe service operation specifies the callback URL {$request.body#/notifUri}, referring to a property of the StatusSubscribeReqData supplied in the original subscription. However, this property does not exist in the referenced data type. Rather, this callback URL is available from {$request.body#/subscription/notifyUri}.

For more detailed analysis, please refer to https://github.com/5G-MAG/Standards/issues/181.

# 4	Actions

To 3GPP TSG SA CT3

ACTIONS: 5G-MAG respectfully asks CT3 to take the information in section 2 above into account.

5G-MAG requests the cardinality of the active period repetition rules is corrected in tables 6.2.6.2.2-1 and 6.2.6.2.2-1 of TS 29.580 (Release 18 and Release 19).

5G-MAG requests the specification of rules for populating object acquisition identifiers in table 6.2.6.2.5-1 of TS 29.580 (Release 18 and Release 19).

5G-MAG requests a resolution to the inconsistent name of the extended event notifications feature in clauses 6.2.8 and 6.2.6.3.4 of TS 29.580 (Release 18 and Release 19).

5G-MAG recommends a change to the syntax of the MBSUserDataIngSessionPatch data structure to allow individual MBS Distribution Sessions to be removed in the Nmbsf_‌MBSUserDataIngestSession_‌Update service operation.

To 3GPP TSG SA CT4

ACTIONS: 5G-MAG respectfully asks CT4 to take the information in section 3 above into account.

5G-MAG requests the addition of a sentence in clause 6.1.6.2.5 of TS 29.581 (Release 17, Release 18 and Release 19) to explicitly specify the cardinality of object acquisition identifiers in the specific case outlined in section 2.1.

5G-MAG requests the addition of a source IP address attribute to the UpTrafficFlowInfo data structure in clause 6.1.6.2.13 of TS 29.581 (Release 17, Release 18 and Release 19).

5G-MAG requests the addition of a transport session identifier attribute to the UpTrafficFlowInfo data structure in clause 6.1.6.2.13 of TS 29.581 (Release 17, Release 18 and Release 19) or a more appropriate location.

5G-MAG requests the correction of the status notification URL specified in the OpenAPI YAML of TS 29.581 (Release 17, Release 18 and Release 19) or a more appropriate location.

# 5	Dates of next 5G-MAG WG meetings

5G-MAG meetings of the relevant WGs and Work Items are held regularly.

Regular WG TECH meeting	Fortnightly on Thursdays	Online

Regular WG DEV meeting	Weekly on Fridays	Online

#### About 5G-MAG

The 5G Media Action Group (see http://www.5g-mag.com/) is a non-for-profit industry association driving standards and software for connected media experiences.

In its role as a 3GPP Market Representation Partner, 5G-MAG provides regular feedback to 3GPP based on the implementation, validation and demonstration of several specifications in the context of media and connectivity (see http://www.5g-mag.com/technology).

5G-MAG also established the 5G-MAG Reference Tools Developer Community to accelerate the transition of standards into products through open-source software tools. This activity supports the validation, testing and onboarding of technologies for real-world applications and services (see http://www.5g-mag.com/reference-tools).

5G-MAG Reference Tools Project: https://hub.5g-mag.com/Getting-Started/pages/5g-multicast-broadcast-services/