3GPP TSG-SA4 Meeting #134	S4-251709

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0029 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] WT#6: Latency Measurement and control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The study item identifies the following work topic - WT#6: Latency Measurement and control: Media Service quality is often determined by network latency, and controlling latency is an important topic. The support of latency measurements in 5G Media streaming provides opportunities for service and network providers to judge the quality of the service. The study item identifies the following objectives 1. Document the following additional Key Issues in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture: 2. Study collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. 3. Based on existing architectures, develop one or more deployment architectures that address the key topics and the collaboration models. 4. Map the key topics to basic functions and develop high-level call flows. 5. Identify the issues that need to be resolved. 6. Provide candidate solutions including call flows, protocols and APIs for each of the identified issues. 7. Coordinate work with other 3GPP groups e.g. SA2, SA3, SA5, SA6 and others as needed. 8. Coordinate work with external organizations such as SVTA, CTA WAVE, ISO/IEC JTC29 WG3 (MPEG Systems), 5G-MAG, DVB or IETF, as needed. 9. Identify gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The issues are addressed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[9]	DASH-IF/DVB: "Report on Low-Latency Live Service with DASH", July 2017, available here: https://dash-industry-forum.github.io/docs/Report%20on%20Low%20Latency%20DASH.pdf

[10]	DASH-IF: "IOP Guidelines v5, Low-latency Modes for DASH", available here: https://dash-industry-forum.github.io/docs/CR-Low-Latency-Live-r8.pdf

[90]		"DASH-IF WebRTC-based Streaming", https://dashif.org/news/webrtc/

[105]	Consumer Technology Association Specification CTA-5004-A: "Web Application Video Ecosystem – Common Media Client Data", December 2025.

[DIF-WEBRTC]	DASH-IF Report: DASH and WebRTC-Based Streaming, https://dashif.org/webRTC/report.html, March 2022.

[TIMBRE]	BBC Project Timbre: Investigating mobile coverage for live radio streaming on BBC Sounds, https://www.bbc.co.uk/rd/blog/2024-03-project-timbre-investigating-mobile-coverage-for-live-radio-streaming-on-bbc-sounds, March 2024.

[14496-12]	ISO/IEC 14496-12: Information technology — Coding of audio-visual objects; Part 12: ISO base media file format.

## =====  CHANGE (new clause) =====

## 5.X	Latency Measurement and Control

### 5.X.1	Description

Service Latency is known to be an important measure for the quality of experience associated with a service. Several data points on this matter have been collected by DVB and DASH-IF [9], [10], [90], and [DIF-WEBRTC]. Also, the BBC in Project Timbre [TIMBRE] identified latency as an important measure. In the Project Timbre work it was outlined that the latency of each audio segment was a key indicator of quality of service for the listener. However, while it is possible to measure the segment latency in the client app, as is being done in the project, it would also be helpful to be able to measure latency, of one kind or another, from the network side.The information may be exposed as an API in the network.

More particular, in segmented media streaming as for example assumed in 5G Media Streaming, (typically CMAF objects and a manifest such as DASH or HLS), the service provider wants to control the end-to-end latency, possibly also for synchronized playback across the clients, but also wants to measure the latency and report this information back to the network or service provider.

Latency is typically measured from glass-to-glass or measured from encoder to glass – for details refer to [9] and [DIF-WEBRTC]. Information on the latency may include

-	what the actual latency is,

-	if a desired target latency is met

-	the deviation from the target latency

-	The reason for the target latency: late arrival, network issues, user controlled, etc.

The network and/or service provider may use this information to

-	Measure QoE based on the latency for each client

-	Do some network improvements if the latency is not met, e.g. Content Steering, QoS, etc.

-	Aggregate the information across multiple/all users

### 5.X.2	Problem Statement and Collaboration scenarios

Figure 5.X.2-1 provides a basic architecture for latency measurement in a live service aligned with DASH-IF [10]. The production camera and/or the encoder embed timing information on the media that relates to when the content was produced and/or encoded, respectively, using a time sync server. After encoding the content is packaged, a manifest is generated, and it is distributed via a CDN. The Media client consumes the media, measures the latency it observed by also syncing to the time sync server and provides the latency information to a reporting server. The reporting server exposes information, that can be used in operations or for QoE measurements.

Figure 5.X.2-1 Basic Architecture for latency measurement in a live service aligned with DASH-IF [10]

Latency in a streaming service can be measured for example based on the description in the DASH-IF Low-Latency modes [10], clause 9.X.6.2.3. A summary is provided in the following:

-	The Producer Reference Time supplies times corresponding to the production or encoding of associated media. This information permits among others to (i) provide media clients with information to enable consumption and production to proceed at equivalent rates, thus avoiding possible buffer overflow or underflow, and (ii) enable measuring and potentially controlling the latency between the production of the media time and the playout.

-	The Producer Reference Time ('prft') as defined in ISO/IEC 14496-12 [144.96-12]. The information may be provided inband as part of the Segments in the ('prft'), in the MPD or both. In the context of the low-latency DASH service offerings, providing information in the MPD is strongly recommended, whereas providing inband information is left to the deployment. The packager can extract the information from CMAF segments into manifests.

-	The producer reference time permits the DASH client to measure and control the End-to-End Latency (EEL) or the Encoding+Distribution Latency (EDL), and permits the service provider to provide information to the client to control this value.

-	Either or both values, capture and encoding, can be expressed in the manifest and/or in the segments. It is also possible to create an application specific anchor for the times included in the media.

-	Clients synchronized to the same time sync server can now use th information in the media as well as the playback time to determine the presentation latency, for details see clause 9.X.5 of [10].

Different collaboration scenarios may be considered, depending on the placement of different functions shown in Figure 5.X.2.1-1. The following different deployment options are discussed:

Option 1:	The 5GMSd Application Provider runs all encoding and packaging related functions, and the 5GMSd AS only provides content hosting for the DASH Presentation as a CDN. Also, the reporting server is in the Application provider domain.

Option 2:	The 5GMSd System runs a time sync server that is trusted and can be accessed by the application provider as well as the media client. The time sync server may be in the 5GMSd AF or 5GMSd AS.

Option 3:	This option adds manifest generation to the 5GMSd AS, requiring that a function that converts inband timing information to manifest timing information. Time sync server may be internal or external, three sub-options are provided.

Option 4:	The 5GMSd System generates the media including encoding and provides a latency from ingest (distribution encoding) to the client. In this case the 5GMSd system controls the latency and does potential operational improvements in encoding, packaging and so on. Reporting may be done between Media Player and AS, or Media Session Handler and AF.

Option 5:	In this case, only the reporting is done with the 5GMSd system. Again may be done between Media Player and AS, or Media Session Handler and AF.

Table 5.X.2-1 summarizes the different deployment options on how the Latency measurement functions are mapped to the downlink media streaming functions.

Table 5.X.2-1 Possible deployment options to map Latency measurement functions
to downlink media streaming functions

| Latency Measurement Function | 5GMSd Application Provider | 5GMSd AS | 5GMSd AF | Media Player | Media Session Handler |
| --- | --- | --- | --- | --- | --- |
| Content Generation | 1,2,3,4,5 |  |  |  |  |
| Time Sync Server | 1,3a,5 | 2a, 3b, 4a | 2b, 3c, 4b |  |  |
| Encoder | 1,2,3,5 | 4 |  |  |  |
| Packager | 1,2,3,5 | 4 |  |  |  |
| Manifest Creator | 1,2,5 | 3,4 |  |  |  |
| Content Hosting |  | 1,2,3,4,5 |  |  |  |
| Media Client Playback |  |  |  | 1,2,3,4a,5a | 4b |
| Media Client Reporting |  |  |  | 1,2,3,4 | 5b |
| Reporting Server | 1,2,3 | 4a,5a | 4b,5b |  |  |

Based on the discussions, option 1 is basically out of scope of 5G Media Streaming. Option 3 may be considered a subset of option 4 and hence is not explicitly discussions. For the remaining options 2, 4 and 5, it s worthwhile to consider more detailed architecture mappings and call flows.

### 5.X.3	Architecture mappings

To be completed – refinements are needed.

### 5.X.4	High-level call flows

To be completed

### 5.X.5	Gap analysis and requirements

The following gaps are identified:

-	Provisioining of a Time Sync Service via M4d to the media client as well as via an exposure function to the Application service provider via M3d and M1d.

-	Addition to producer reference times to media segments and manifests and time synchronization between 5GMSd client and adder of reference times.

-	Implementation of latency measurement in 5GMSd client

-	Reporting of latency information via M4d and M13d, and possibly M5d.

-	Controlling the playback latency for a client

-	Exposing the latency measurement.

### 5.X.6	Candidate solutions

Editor’s Note

- The producer reference time may be

  - the capture time of the media sample

  - the encoding time of the media sample

  - Any other relevant time relevant for the network provider

- The Time Synchronization is accomplished by UTC Time Sync in DASH

- Producer reference time can be carried in Segments, Manifest or is implicit (see DASH-IF)

- Reporting of latency may be done in DASH metrics or CMCD

- Latency metric may be measured, but also other information such as latency deviation from target latency, reasons for latency and so on

- API for exposure may use NWDAF or any other defined API

### 5.X.7	Summary and conclusions

To be completed