3GPP TSG-SA WG4 Meeting #134	S4-251717

Dallas, US, 17 – 21 November 2025

Agenda item: 	9.6

Source: 	Qualcomm Germany

Title: 	[FS_AVFOPS_MED] Vision on Spec Development and Conformance

Document for	Information and Agreement

# Introduction

Based on the ongoing work on conformance for VOPS, this document provides a vision for improved spec development and conformance related to video in 3GPP.

# Current discussion in VOPS

Annex B of TS 26.265 has been initiated to introduce conformance-related information. Figure B.1-1 provides an overview of the conformance functionalities expected to be provided to support interoperable deployment of the operation points defined in this specification.

Figure B.1-1 Overview of conformance functionalities provided for this specification

Based on the overview in Figure B.1-1, the following functionalities are provided:

-	A set of video signals are provided that conform to one of the representation formats defined in clause 4.4.3 of this specification. In addition, a consistent annotation of the signals using the parameters defined in clause 4.4 are defined. For details refer to Annex B.2.1.

-	Workflows including reference software encoders with appropriate configuration are provided to encode the video signals to create a bitstream and potentially coded metadata that conform to the operation point. For details refer to Annex B.2.2.

-	A conformance validator is defined that permits to validate the bitstream (and possibly the coded metadata) to be conforming to a specific operation point defined in this specification. For details refer to Annex B.2.3.

-	A conforming decoder may be provided for certain operation points that provides a reference implementation to generate the adequate signals based on conforming bitstreams. The conforming decoder may provider information related to the output signal for example an MD5. For details refer to Annex B.2.4.

-	Bitstreams that have been validated are provided with collected information including the information of the used video signal, the encoder and its configuration, the report from the validator as well as the coded metadata. In addition, decoding related information may be provided. The bitstreams are referred to as conformance bitstream. For details refer to Annex B.2.5.

This tools and functionalities provided in this Annex may be used for different purposes to support interoperable deployments. Examples for the usage of the tools are provided in Annex B.3.

# Vision moving forward

In order to consolidate conformance, there is a clear benefit to move conformance already into the study phase to allow completion of the conformance in the same release as the normative work is proposed.

Based on the framework defined in TS 26.265, for new functionalities to be added, the following aspects would need to be addressed during the study:

1)	Define the video signals that are to be used. Along with the the following is needed:

-	Annotation for the video signals

-	Example video signals that follow this format

-	Covering different video signals to address the most relevant parameters.

2)	Bitstream requirements that are addressing the compressed representation

-	defining the requirements by reference and profiling existing video codec specifications

-	defining the details to map the signal parameters to the video bitstream and/or codec metadata

-	define the test assertions with the requirements. An example for writing requirements with test assertions is for example defined here: https://www.w3.org/TR/test-methodology/

3)	Generating the bitstreams following the considered specifications

-	preferably a software reference encoder that can use the video signal and provide a bitstream that follows the requirements defined in step 2

-	attaching the configuration of the

-	executing different encoding options to provide different relevant options for the specification, for example bitrates, random access points, other encoding options.

-	annotating the bitstreams with relevant information

4)	A conformance validator that maps the requirements defined 2) to a software that tests any bitstream for conformance against the specifications

-	mapping the defined requirements to software validation process

-	possibly providing the ability on the coverage of requirements that are defined in 2 to understand how well the provided bitstream in 3) cover the requirements

-	providing bitstreams generated in 3 with an annotation about the success of the conformance resulting in conformance bitstreams

5)	A conforming decoder is nice to have that is able to recover signals following the representation format, possibly with some additional information.

5)	A testing regime for a decoder under test that decodes a conforming bitstream is also nice to have and it should be documented, what observations one may have after decoding.

# Proposal

Based on the discussion, it is proposed that during the study phase, the aspects discussed in 3 are considered in order to support the normative phase. For example, if such conformance material is available, it may allow to test implementations under development to provide feedback on the specification requirements.