Source:	Fraunhofer IIS

Title:	Fraunhofer Listening Lab Report – IVAS Characterization

Agenda item:	7.5

Document for:	Approval

# Summary

The document presents a report on subjective testing performed as a part of 3GPP-SA4 IVAS Characterization. Fraunhofer IIS performed the experiments according to the procedures and test plan specified in permanent documents IVAS-7b (Processing Plan for Characterization Phase) and IVAS-8b (Test Plan for Characterization Phase). For part of the tests, deviations to the specifications described in the test plan were observed, they are not expected, however, to materially affect the results (see Section 7). Otherwise, no deviations from or exceptions to the listening test procedures and specifications described in the test plan have been observed or experienced.

Key Acronyms

CuT	Codec under Test

DCR	Degradation Category Rating

GAL	Global Analysis Laboratory

HL	Host Laboratory

LL	Listening Laboratory

MC	Material collection entity

# Introduction

Fraunhofer IIS performed 4 different experiments within the 3GPP SA4 IVAS Codec Characterization. The experiment name, methodology (as defined in ITU-T P.800  and ITU-R BS.1534 [2]) and language for each experiment is listed in Table 1.

Table 1: Allocation of experiments for Fraunhofer IIS

| Exp. | Methodology | Language |
| --- | --- | --- |
| P800-17 | P.SUPPL800 | GER |
| P800-19 | P.SUPPL800 | GER |
| BS1534-9 | ITU-R BS.1534 (7.1.4) | - |
| BS1534-16 | ITU-R BS.1534 (Binaural) | - |

# Test material

## P800-DCR

Fraunhofer IIS provided to MC the original mono sentences and in the German language in uncompressed WAVE format with 48kHz sampling rate and 16-bit resolution. The unprocessed speech samples provided conformed to the restrictions indicated in [1]. The samples were than processed by the HL with the room impulse responses and background noise selected from the MC. Additionally, Fraunhofer IIS provided to the MC preproduced material combining mono sentences with generic audio or music background.

All processed samples for the testing were provided from the HL through the exchange folder [5].

## BS.1534 (MUSHRA)

The test material for the BS.1534 (MUSHRA) was selected by the MC and the processed material was provided from the HL through the exchange folder [5].

# Listening Environment

## P800-DCR

The test was conducted in the Fraunhofer IIS premises in Erlangen. The P.800 DCR tests were conducted with a set of new Beyerdynamic DT-770 Pro 80 Ohm headphones. The software was an in-house software specifically developed for those tests, running on iPad Pro M4 touch-based devices.

## BS.1534 (MUSHRA) with headphones

The test was conducted in the Fraunhofer IIS premises in Erlangen. The BS.1534-16 test was conducted over headphones as defined in [4]. Tests were run in acoustically isolated rooms using STAX electrostatic headphones. Participants entered their ratings in the internally developed testing software, which logged all responses.

## BS.1534 (MUSHRA) with loudspeakers

The BS-1534-9 test was performed in an acoustically treated critical listening room in the Fraunhofer IIS premises in Erlangen [6] that conforms to the requirements of ITU-R BS.1116 [7]. Loudspeaker setups are calibrated according to [7] and listening room acoustics conform to [7]. The loudspeaker models are Geithain RL 903K and BASIS 14. The loudspeaker layouts and channel configurations were set up according to CICP19.

# Test instructions and presentation order

## P800-DCR

The instructions for the subjects were in German and are available in Annex A. The sample presentation order was as exchanged via email between MC and LLs on September 16, 2025. Note the separate discussion in Section 7.

## BS.1534 (MUSHRA)

The instructions for the subjects were in English and are available in Annex B. The sample presentation order was randomized by the testing software.

# Test Subjects

## P800-DCR

A total of 71 participants took part in the in-house test, recruited specifically for participating in listening tests. Of these, 39.43% were female and 60.57% were male. The participants‘ ages ranged from 14 to 80 years (Mean = 30.90, SD = 13.63). The results of 11 participants were excluded due to insufficient auditory capabilities as determined by audiometric assessment, inadequate proficiency in the German language, and random sampling procedures to obtain a final sample size of 60 participants. The final sample consisted of 60 participants comprised of 45% female and 55% male. Their ages ranged from 19 to 64 years (Mean = 29.56, SD = 11.67). The majority of participants were native German speakers, while the remainder demonstrated adequate proficiency in German.

## BS.1534 (MUSHRA)

For the BS.1534 MUSHRA experiments, assessors were recruited from the staff of Fraunhofer IIS. All listeners have a background in audio processing and high level of experience in MUSHRA listening tests. Subjects consisted of  16 male and 6 female. The average age was 36.7 years, the standard deviation was 7.98 years.

Assessors were to be excluded from the aggregated responses if he or she rated the hidden reference condition lower than a score of 90 or if he or she rated any anchor higher than the reference [2]. No assessors were disqualified based on these criteria.

# Raw data delivery

All raw voting data have been delivered to the GAL using the data delivery file provided by the GAL for all tests. Delivery was completed by the due date.

# Discussion of any problems encountered during testing and the solution used to address the problem

For the P800-DCR tests, P800-17 and P800-19, during the test setup, a typographical error in the configuration process caused Block 5 to be omitted from the test-software configuration. The verification step that checks whether configured items were valid passed, because it only examined items present in the configuration; the missing block went undetected. The root cause has been precisely identified as a configuration-copy error, with preliminary analysis indicating a chain of circumstantial events rather than a systemic flaw likely to recur in future tests. The omission reduced the stimulus count to 192 (5 × 36 + 12 preliminaries) instead of the intended 228 (6 × 36 + 12 preliminaries).

Continuous monitoring did not flag anomalies. Because the randomization was robust, statistical analyses remained within expected bounds, and the issue became evident only during data transfer into the Excel sheets used by the GAL. A targeted post-hoc repair, such as replaying Block 5, was not deemed feasible because the listeners were no longer available and blocks are valid only within a complete session. The only viable retest would be a full repetition of the tests, which was not possible within the characterization testing schedule, and given the plausibility of the results, a repetition was not warranted. Nevertheless, the results remain interpretable: the favourable statistical distribution of the randomization playlist means all expected effects are observable despite one-sixth of the votes missing. Therefore, we recommend retaining the results despite the missing block.

# References

- ITU-T P.800, “Methods for subjective determination of transmission quality”, 1996.

- ITU-R BS.1534, “Method for the subjective assessment of intermediate quality level of audio systems, 2015

- S4-251626, “IVAS Characterization Material Collection Entity Report”, VoiceAge Corporation

- S4-251565-IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase Version v.1.1.0

- S4-251663- “Host Lab and Cross-Check Lab Report for IVAS Characterization Tests” - Dolby Laboratories Inc., Ericsson LM, Fraunhofer IIS, Nokia, NTT, Orange, Philips, Qualcomm, VoiceAge Corporation

- Silzle et al., “Vision and Technique behind the New Studios and Listening Rooms of the Fraunhofer IIS Audio Laboratory”, AES 126th Convention, Munich, Germany, 2009 May 7–10

- ITU-R BS.1116, “Methods for the subjective assessment of small impairments”, 2015

# Annex A

Sample Instructions to Subjects in German as used by Fraunhofer IIS during the subjective P800-DCR listening tests performed during 3GPP SA4 IVAS selection phase:

Sie werden an einem P.800 DCR (Degradation Category Rating, zu Deutsch: Bewertung der Verschlechterungskategorie) Test teilnehmen, um Systeme für zukünftige immersive Telekommunikationsdienste mit räumlichem Audio zu bewerten.

Bitte lesen Sie die Anleitung sorgfältig durch, bevor Sie mit dem Test beginnen.

In diesem Experiment werden Sie Hörproben bewerten, wie sie in zukünftigen Telekommunikationsanwendungen mit immersivem Raumklang auftreten könnten. Das bedeutet, dass Sie verschiedene Schallquellen um sich herum lokalisieren können. So kann zum Beispiel ein Sprecher von links und ein zweiter von rechts wahrgenommen werden, oder ein Sprecher kann sich im Raum bewegen, usw.

In jedem Durchgang hören Sie zunächst eine Referenz. Im Anschluss folgt die Hörprobe, welche denselben Inhalt wie die Referenz enthält, aber möglicherweise durch ein Telekommunikationssystem beeinträchtigt wurde.

Diese Beeinträchtigung kann sowohl Verschlechterungen in der Klangqualität (z. B. durch zusätzliches Rauschen, Rauigkeit, Unterbrechungen oder andere Verzerrungen) als auch Unterschiede in der räumlichen Darstellung (z. B. Position der Schallquelle, Entfernung, räumliche Weite, Bewegung usw.) umfassen.

Ihre Aufgabe ist es, die Gesamtbeeinträchtigung der Hörprobe im Vergleich zur Referenz zu bewerten.

Sie sollten sich Referenz und Hörprobe innerhalb eines Versuchs sorgfältig anhören. Wählen Sie anschließend die Kategorie aus, die Ihren Gesamteindruck davon am besten beschreibt, wie stark die Hörprobe im Vergleich zur Referenz beeinträchtigt ist.

Die Hörprobe enthält gegenüber der Referenz: 5 = Keine Beeinträchtigung 4 = Geringe Beeinträchtigung 3 = MäßigeBeeinträchtigung 2 = Starke Beeinträchtigung 1 = Sehr starke Beeinträchtigung

Bitte besprechen Sie Ihre Bewertungen nicht mit anderen Personen, die ebenfalls an dem Experiment teilnehmen.

Wenn Sie Fragen haben, wenden Sie sich bitte an die Versuchsleitung.

Für diesen Test benötigen Sie verkabelte Stereo-Kopfhörer.

Bitte setzen Sie die Kopfhörer auf!

Achten Sie darauf, dass die linke und rechte Seite korrekt positioniert sind (Beschriftung: L = linkes Ohr, R = rechtes Ohr).

# Annex B

Instructions to expert subjects used by Fraunhofer IIS during the BS-1534 MUSHRA tests performed during the 3GPP SA4 IVAS characterization

- The test must be done in Room Hendrix (Room 2C4.11) using the STAX headphones.

- The test files are located in folder XXX

  - If the drive is not mounted, please use command+K and mount the above link

- There are two tests in the folder:

  - You must start with PRELIMINARY_BS1534_XX.ltg. This is a small warm-up test with 4 items to understand what you should expect from the official test.

  - Then proceed with BS1534_XX.ltg which is the official test

- The test must be done in one shot (!). This means that you may not start the test one day and continue it on another day. However, since it is indeed a long test, it is important to take a break in between if needed.

- We are testing binauralized spatial audio. When comparing it to the original input, it is important to assess both the spatial accuracy of the audio scene and the overall coding quality.

This instruction was given only for the BS.1534 test with headphones