3GPP TSG-SA4 Meeting #134	S4-251726

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.997 | CR | 0002 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Documentation of IVAS Characterization Test Results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS (Editor) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inclusion of IVAS Characterization Test Results |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclusion of IVAS Characterization Test Results |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IVAS Characterization Test Results are not documented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses 2, 7.2 (new), 9.2.1, 9.2.7 (new), 9.2.8 (new), 9.2.9 (new), 9.2.10 (new), 9.3.1, 9.3.8 (new), 9.3.9 (new), 9.3.10 (new), 9.3.11 (new), 9.3.12 (new), 9.3.13 (new), 9.3.14 (new), 9.4.1, 9.4.6 (new), 9.4.7 (new), 9.4.8 (new), 9.4.9 (new), 9.4.10 (new), 9.4.11 (new), 9.4.12 (new), 9.5.1, 9.5.6 (new), 9.5.6 (new), 9.5.8 (new), 9.5.9 (new), 9.5.10 (new), 9.5.11 (new), 9.6.1, 9.6.6 (new), 9.6.7 (new), 9.6.8 (new), 9.6.9 (new), 9.6.10 (new), 9.6.11 (new), 9.6.12 (new), 9.7.1, 9.7.3 (new), 9.7.4 (new), 9.7.5 (new), 9.7.6 (new), 9.7.7 (new), 9.8.1, 9.8.3 (new), 9.8.4 (new), 9.8.5 (new), 9.8.6 (new), 9.9.1, 9.9.3 (new), 9.10, 9.10.1 (new), 9.10.2 (new), 9.10.3 (new), 9.10.4 (new), 9.10.5 (new), 9.10.6 (new), 9.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services (IVAS); General overview".

[3]	3GPP TS 26.251: "Codec for Immersive Voice and Audio Services (IVAS); C code (fixed-point)".

[4]	3GPP TS 26.252: "Codec for Immersive Voice and Audio Services (IVAS); Test Sequences".

[5]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services (IVAS); Detailed Algorithmic Description including RTP payload format and SDP parameter definitions".

[6]	3GPP TS 26 254: "Codec for Immersive Voice and Audio Services (IVAS); Rendering".

[7]	3GPP TS 26.255: "Codec for Immersive Voice and Audio Services (IVAS); Error concealment of lost packets".

[8]	3GPP TS 26.256: "Codec for Immersive Voice and Audio Services (IVAS); Jitter buffer management".

[9]	3GPP TS 26.258: "Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)".

[10]	Recommendation ITU-T P.800: "Methods for subjective determination of transmission quality".

[11]	Supplement ITU-T P.Suppl29: "ITU-T P.800 – Use Cases".

[12]	Recommendation ITU-R BS.1534-3: "Method of the subjective assessment of intermediate quality level of audio systems".

[13]	ITU-T Handbook of subjective testing practical procedures, 2011.

[14]	Audio File Format Specifications: WAVE, https://www-mmsp.ece.mcgill.ca/Documents/AudioFormats/WAVE/WAVE.html.

[15]	AFsp Package https://www-mmsp.ece.mcgill.ca/Documents/Downloads/AFsp/.

[16]	IEEE Recommended Practice for Speech Quality Measurements, in IEEE Transactions on Audio and Electroacoustics, vol. 17, no. 3, pp. 225-246, September 1969, doi: 10.1109/TAU.1969.1162058.a.

[17]	Recommendation ITU-R BS.2051-3 (05/2022): Advanced sound system for programme production.[18]	3GPP TR 26.952: "Codec for Enhanced Voice Services (EVS); Performance characterization".

[19]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[20]	3GPP TR 26.996: "Immersive Audio for Split Rendering Scenarios; Performance characterization".

[21]	Recommendation ITU-T P.191 (03/2023): Software tools for speech and audio coding standardization.

[22]	Recommendation ITU-R BS.1770-4 (10/2015): Algorithms to measure audio programme loudness and true-peak audio level.

==============Next change==============

## 7.2	IVAS Characterization Phase Testing

### 7.2.1	General methodology

#### 7.2.1.1	Overview

The following test methodologies were used in the IVAS Characterization test:

- 	P.800 DCR [10] under consideration of P.Suppl29 [11];

-	BS.1534 [12];

-	P.800 ACR [10] under consideration of P.Suppl29 [11|;

-	A preference testing methodology for testing room acoustics synthesis.

The high-level configuration of experiments for envisaged methodologies is outlined below.

#### 7.2.1.2	P.800 DCR

-	Considerations of P.Suppl29 [11] shall be taken into account;

-	Test duration should not exceed 2 hours per listening panel. The typical value of voting period was used for estimation of test durations, but actual voting period is not specified;

-	Randomizations constructed under randomized blocks experimental design described in [13];

-	6 categories for each test. Categories are defined for each experiment separately;

-	6 samples/category (1 for each listening panel) plus 1 sample/category for preliminaries;

-	30 naïve listeners, 6 listening panels (5 listeners per panel), each panel with an independent randomization;

-	180 votes for each condition;

-	Total number of conditions for each experiment: 36;

-	Number of trials: number of test conditions x 6 talkers/categories = 216 trials;

-	P.800 experiments are performed with native listeners of the tested language.

#### 7.2.1.3	BS.1534

-	Number of items per experiment: 12;

-	14 experienced listeners;

-	Maximum total number of conditions: 8;

-	Number of anchor conditions: 2.

-	Direct;

-	3.5 kHz low-pass anchor.

The exact number of conditions may vary depending on actual experiment. Each BS.1534 experiment comprises a training phase in which the subjects familiarize themselves with the testing methodology and environment [12].

#### 7.2.1.4	P800 ACR

-	Considerations of P.Suppl29 [11] shall be taken into account as far as they can be interpreted for ACR test methodology;

-	Test duration should not exceed 2 hours per listening panel;

-	Randomizations constructed under randomized blocks experimental design described in [13];

-	6 sample categories for ACR test;

- 	6 samples/category (1 for each listening panel) plus 1 sample/category for preliminaries;

-	30 naïve listeners, 6 listening panels (5 listeners per panel), each panel with an independent randomization;

-	180 votes for each condition;

-	Total number of conditions for the ACR experiment: 60;

-	Number of trials: number of test conditions x 6 talkers/categories = 360 trials;

-	P.800 ACR experiment is performed with native listeners of the tested language-

Initially the experimenter should provide a written copy of the experiment instructions to the listeners. When the listeners have acknowledged that they understand the instructions, they will be presented with a practice session to rate the preliminary conditions. After the practice session has been completed, the experimenter should ask if there are any questions. After practice session all samples are played back in full quality (Spatial Fullband) to give listeners the feeling of the best possible quality. After listening through the material, actual listening test starts. There are comfort pauses after listening every 120 samples.

#### 7.2.1.5	Preference test methodology for testing room acoustics

For room acoustics testing preference test methodology is used. The listeners are requested to provide preference scoring based on quality rating.

- 4 different rooms;

- 4 audio items per room;

- 4 conditions per test: IVAS rendering with HRTF (reference), IVAS rendering with BRIR, IVAS rendering with synthetic reverb, rendering with BRIR (Python implementation);

- Total: 64 trials.

Each testing experiment is preceded with a training phase in which the listeners should familiarize themselves with the environment and methodology. Unipolar preference scale is used for rating.

### 7.2.2	Opinion Scales

#### 7.2.2.1	ITU-T P.800 DCR

The same opinion scale, instructions, and voting screen as in 7.1.2.1 were used.

#### 7.2.2.2	ITU-R BS.1534

For BS.1534 tests, the continuous quality scale (QCS) according to [12] is used. See 7.1.2.2 for details.

The following instructions for BS.1534 testing were provided in IVAS-8b:

Familiarization or training phase The first step in the listening tests is to become familiar with the testing process. This phase is called a training phase, and it precedes the formal evaluation phase. The purpose of the training phase is to allow you, as an evaluator, to achieve the following two objectives: - Part A: to become familiar with all the sound excerpts under test and their quality level ranges; and - Part B: to learn how to use the test equipment and the grading scale. In Part A of the training phase, you will be able to listen to all sound excerpts that have been selected for the tests in order to illustrate the whole range of possible qualities. The sound items, which you will listen to, will be more or less critical depending on the bit rate and other “conditions” used. Figure 3 shows the user interface. You may click on different buttons to listen to different sound excerpts including the reference excerpts. In this way you can learn to appreciate a range of different levels of quality for different programme items. The excerpts are grouped on the basis of common conditions. Three such groups are identified in this case. Each group includes four processed signals. In Part B of the training phase, you will learn to use the available playback and scoring equipment that will be used to evaluate the quality of the sound excerpts. During the training phase you should be able to learn how you, as an individual, interpret the audible impairments in terms of the grading scale. You should not discuss your personal interpretation of the scale with the other assessors at any time during the training phase. However, you are encouraged to explain artefacts to other assessors. No grades given during the training phase will be taken into account in the true tests. Blind grading phase The purpose of the blind grading phase is to invite you to assign your grades using the quality scale. Your grades should reflect your subjective judgement of the quality level for each of the sound excerpts presented to you. Each trial will contain 9 signals to be graded. Each of the items is approximately 10 s long. You should listen to the reference, anchor, and all the test conditions by clicking on the respective buttons. You may listen to the signals in any order, any number of times. Use the slider for each signal to indicate your opinion of its quality. When you are satisfied with your grading of all signals you should click on the “register scores” button at the bottom of the screen. You will use the following quality scale when assigning your grades. The grading scale is continuous from “excellent” to “bad”. A grade of 0 corresponds to the bottom of the “bad” category, while a grade of 100 corresponds to the top of the “excellent” category. In evaluating the sound excerpts, please note that you should not necessarily give a grade in the “bad” category to the sound excerpt with the lowest quality in the test. However, one or more excerpts must be given a grade of 100 because the unprocessed reference signal is included as one of the excerpts to be graded.

#### 7.2.2.3	ITU-T P800-ACR

For ITU-T P.800 ACR tests, the opinion scale as defined in Table ‎7.2-1 is used.

Table ‎7.2-1: Opinion scale for ITU-T P.800 ACR test

| Overall Quality | Scale |
| --- | --- |
| Excellent | 5 |
| Good | 4 |
| Fair | 3 |
| Poor | 2 |
| Bad | 1 |

The following instructions (translated properly to the listening labs language and be given to the listeners) are used. The instructions given to the listeners shall be provided for information in the listening lab report.

INSTRUCTIONS TO NAÏVE LISTENERS FOR P.800 ACR TEST In this experiment you will be evaluating systems that might be used for future immersive telecommunication services using spatial audio. Spatial audio means that you can locate various sound sources around yourself. For example, a first talker may appear to talk from the left-hand side and a second talker from the right-hand side, a talker can be moving, etc. In each trial, you will hear a test sample. The test sample is possibly impaired after it has passed through a telecommunication system. Your task is to evaluate the overall quality of the test sample, comprising both degradations in the sound quality (e.g., harshness / roughness, loss of bandwidth, unnatural speech, clicks or other distortions), and/or lack of quality in the spatial representation (e.g., unnatural sound source movement, spatial instability, lack of spatial width or other spatial problems). You should listen carefully the sample within a trial. When sample playback has finished, select the category that best describes your overall impression about the overall quality of the sample you just heard: 5 - Excellent 4 - Good 3 - Fair 2 - Poor 1 - Bad Note that the level of quality present in different test samples is expected to span the complete range of the rating scale during the experiment. Please do not discuss your opinions with other listeners participating in the experiment. If you have any questions, please ask the test administrator.

P.800 ACR Voting screen (after playback of sample pair)

Please rate the OVERALL QUALITY of the sample: 5 - Excellent 4 - Good 3 - Fair 2 - Poor 1 - Bad

#### 7.2.2.4	Preference test for room acoustics

A unipolar preference scale is used for rating.

The following instructions are provided to the listeners:

INSTRUCTIONS TO THE LISTENERS FOR THE IVAS ROOM ACOUSTICS SYNTHESIS EVALUATION TEST In this experiment you will be evaluating systems that might be used for future immersive telecommunication services using spatial audio. Spatial audio is an audio experience where you can perceive sound as if coming from the space around you, outside and around your head. Spatial audio can be enriched by synthetic reverberation and acoustic reflections that simulate the acoustics of a room, creating a more natural and immersive experience. For this test you will be wearing a VR headset and headphones. The VR scene used in the test allows for three degrees of freedom (3DoF). That means that you can change your position only by rotation, and not by moving around. Please familiarize yourself with the equipment during the training session and adjust it appropriately. It is highly recommended to perform the tests while sitting on a swivel chair. It is advised not to take this test while standing. Please take time to observe the scene around you and listen to all the stimuli representing different CuT (conditions under test). Note that it will take a couple of seconds for the playback to start. The CuT provided have been processed with different reverb pathways from the IVAS decoder. A stimulus rendered without room acoustics is provided as a reference. Please rank your CuT preference regarding room acoustic synthesis quality and matching to the visual environment shown, as compared to the reference without room acoustics. Please consider factors such as timbral quality, spatial depth, and spatial envelopment and how do these relate to the image presented.

### 7.2.3	Material

#### 7.2.3.1	Overview

All audio material is sampled at 48 kHz with Full Band (FB) content. The audio material was delivered to the HL as 16-bit little endian WAVE format files [14] following the naming convention provided in the IVAS Processing Plan for Characterization Phase (IVAS-7b). For multi-track audio, the audio tracks are ordered according to Table 5 of IVAS Processing Plan for Characterization Phase (IVAS-7b). Additionally, it should be verified that the audio material can be processed with the AFsp package tools [15].

The following categories of audio content was used in IVAS Characterization Test using P.800 DCR:

- Clean speech: Except for experiment evaluating 1 object input, each sample contains two (or more) different talkers in conversation scenario. The talkers transition from one to another as in natural conversation, possibly with partial overlap;

- Speech with background;

- Music and Mixed content;

- Generic audio.

The following category of audio content was used in IVAS Characterization Test using BS.1534:

- Generic audio – critical generic audio items including speech with and/or without background, music, mixed content.

Since it was expected that a significant amount of new material has to be checked and processed for the IVAS Codec Characterization test, test material proponents were encouraged to submit material identical to the Selection testing if possible (i.e. if format, test methodology and test lab match the setup of the Selection phase testing).

#### 7.2.3.2	Material for P.800 DCR testing

##### 7.2.3.2.1	Speech Material for P.800 DCR testing

Except for Music and mixed content categories, P.800 DCR test experiments have used artificially created immersive audio. LLs have provided clean speech mono audio samples. SA4 has provided scene descriptions and scripts to create the immersive audio.

The recording SNR was in accordance with P.800 at least 40 dB but preferably 50 dB or higher. The leading and trailing inactivity portions should be shorter than 20 ms. The reverberation time RT60 was in accordance with P.800 less than 500 ms, preferably below 200 ms. The length of the sentences typically correspond to the length of traditional Harvard sentences [16].

##### 7.2.3.2.2	Background Material for P.800 DCR testing

A mix-based approach using separate background recordings has been used. The minimum lengths of noise files were 80 s. The following guideline is applied to the noise types used:

-	Car noise is intended to test the performance of the codec under steady state background noise and should be recorded in a moving car. A constant speed between 80 km/h (50 mph) and 110 km/h (70 mph) is recommended. The make and model of the car should be reasonably common in the country of the recording. Typically, the windows of the car should be closed, and the radio turned off;

-	Office noise is intended to represent a typical office environment. This noise type should also contain typical office sounds, such as keyboard noise, computer fans, telephones ringing, printers, air conditioner, etc;

-	Street noise is intended to represent a typical street environment. It should contain unsteady traffic noise for example recorded at traffic lights where cars stop, human noise such as steps. It should not contain speech, but baby cries are allowed.

##### 7.2.3.2.3	Music and Mixed Content Material for P.800 DCR testing

Music and mixed content samples has contained meaningful contents, and the duration of each sample shall be approximately 8 and at least 7seconds. The following categories were used:

- Classical music;

- Modern instrumental music;

- Modern vocal music;

- Radio Jingle;

- Movie Trailer;

- Advertisement.

LLs have provided music and mixed content stereo samples for the stereo experiments. This means that LLs have provided 7 samples per category: 6 for evaluation and 1 for preliminaries. Music and mixed content audio samples for the other P.800 DCR experiments were collected and selected by MC, similarly as done for the Generic Audio Items Selection for BS.1534 experiments.

##### 7.2.3.2.4	Audio Material for 3- and 4-object categories in P.800 DCR testing

Audio material for 3- and 4-object categories have been collected and selected by MC, similarly as done for the Generic Audio Items Selection for BS.1534 experiments. The collected audio material shall consist of complete audio scenes falling in the following categories:

- Speech + effects (scene with 3 objects);

- Speech + music (scene with 3 objects);

- Music or effects (scene with 3 objects);

- Speech + effects (scene with 4 objects);

- Speech + music (scene with 4 objects);

- Music or effects (scene with 4 objects).

#### 7.2.3.3	Critical Generic Audio Items for BS.1534 testing

##### 7.2.3.3.1	Steps of Critical Test Item Selection

The following steps have been conducted:

- Call for test material according to the generic audio signal categories described below;

- MC has collected candidate material submitted in response to the call and selects a number of critical items to be used in the Characterization test;

- MC has selected a limited set of training items to be used in a training phase.

##### 7.2.3.3.2	Test Material

First, a call has been sent out for test material according to a number of generic audio signal categories as specified below. All 3GPP members were invited to submit test material to MC. The submitting organization has assigned the items to the below-mentioned audio signal categories. Then, MC has identify 12 critical items per experiment, plus four items for training, which are representative for assumed typical IVAS application scenarios.

Generic audio signal categories:

- Stereo – generic stereo audio signals with a focus on music categories:

-	Pop, with and/or without vocals

-	Classic, with and/or without vocals

-	Single instruments

-	A capella vocals, solo and/or choir

-	Mixed speech and music

-	Speech with and/or without background noise

-	Multi-Channel (5.1, 5.1+2, 5.1+4, 7.1 and 7.1+4) – generic channel-based audio signals from produced content:

-	Music including concerts with live audience

-	Film soundtracks with and/or without speech dialogue

-	Effects (e,g, nature, city/transport sounds)

-	Scene-Based Audio / MASA – generic immersive audio signals in the form of complex scenes, captured and/or produced content which may or may not include speech:

-	Nature sounds (e.g. forest, water, wind)

-	City sounds (e.g. traffic, bus, train)

-	Music including concerts with live audience

-	Babble-like sound (e.g. market, restaurant, conference)

-	Event/Sport-like sound

-	Conferencing scene with and/or without background noise/music

-	Object-Based Audio - Realistic immersive audio signals, e.g.:

-	Scenarios comprising voice, music, background objects.

-	Conversational scenarios of several talkers with or without background, with or without partial overtalk of no more than two talkers. Talkers may be moving around the scene at natural pace.

The length in time of the items was 10 s at a maximum.

MC has further maintained and reported to SA4 a list indicating the number of proposed items per submitting organization.

In case the submitted material was insufficient/inadequate to conduct the tests, MC has added the missing test items.

In order to streamline the work of the Material Collection entity, the BS.1534 Selection test samples were alse used for the BS.1534 Characterization tests as far as possible, especially for the following formats:

-	Stereo

-	Ambisonics FOA, HOA2, HOA3

-	Multi-channel 5.1, 7.1, 5.1+2, 5.1+4, 7.1+4 (7.1, 5.1+2, 5.1+4 to be derived from 7.1+4)

-	Objects: 1-4 objects

-	MASA 1-2 TCs  MASA material expected to be derived from FOA/HOA2 material

In order to facilitate this, the material proponents are asked to re-submit the identical material as for the Selection phase testing.

##### 7.2.3.3.3	Training material

Limited material has been used in the training phase in which the subjects familiarize themselves with the testing methodology and environment.

The training was conducted with four sound items. These items were identified by MC and may be re-used in the blind grading phase. The training phase was executed as a separate short BS.1534 session.

### 7.2.4	Listening Systems and Listening Environments

The IVAS Characterization Test has used the following listening systems:

-	Stereo headphones for binaural listening, e.g.:

-	Beyerdynamic DT 770 Pro for P.800 experiments

-	Sennheiser HD 650 for BS.1534 experiments

-	Loudspeaker listening system – 5.1, 5.1+2, 5.1+4, 7.1, 7.1+4 loudspeaker setup [17].

-	All P.800 tests are carried out via headphones.

-	VR setup for 3DoF binaural room acoustics synthesis listening comprising of:

-	A gaming-category PC running Unity game engine, capable to also run 3 IVAS decoder instances and a lightweight Python renderer in parallel,

-	A VR headset, e.g., Meta Quest or HTC Vive Pro,

-	Headphones requirements for binaural tests apply.

### 7.2.5	Subjective Experiments

#### 7.3.5.1	General Consideration of Experiments

The main goal of the IVAS Characterization Test is to evaluate the aspects of the IVAS codec that were not tested in the Selection phase and validate the fixed-point implementation. Some of the aspects have been tested in formal subjective evaluation while other aspects were evaluated informally. The following aspects have been tested in formal subjective experiments:

-	The IVAS fixed-point implementation and the interoperability between the floating-point implementation and the fixed-point implementation

-	The integrated IVAS renderer is used, rendering to the playout configuration.

-	Stereo; it may include binauralized samples (without head tracking).

-	Multi-channel configurations 5.1, 5.1+2, 5.1+4, 7.1, and 7.1+4

-	Objects (1-4 ISMs including metadata/rendering)

-	FOA, HOA2, HOA3

-	MASA 1 TC, 2 TC

-	Combined input formats:

-	Objects + MASA (OMASA), 1-4 ISMs

-	Objects + SBA (OSBA), 1-4 ISMs

-	JBM

-	Packet loss conditions derived from delay and error profiles

-	Binaural rendering configurations, e.g.

-	room effects,

-	head rotation,

-	6 degrees-of-freedom (DoF) and directivity

-	EVS-coded mono downmix of stereo input (13.2 and 24.4 kbps)

The subjective material in the P.800 DCR experiments comprising fixed-point validation accommodates all three input signal levels. Low and high signal levels are integrated in different audio samples within categories following Table 7.2-2.

Table 7.2-2: Assignment of input levels to audio samples within categories

| Samples | Input Level (in LKFS) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | cat 1 | cat 2 | cat 3 | cat 4 | cat 5 | cat 6 |
| s01 | -16 | -36 | -36 | -26 | -26 | -16 |
| s02 | -16 | -16 | -36 | -36 | -26 | -26 |
| s03 | -26 | -16 | -16 | -36 | -36 | -26 |
| s04 | -26 | -26 | -16 | -16 | -36 | -36 |
| s05 | -36 | -26 | -26 | -16 | -16 | -36 |
| s06 | -36 | -36 | -26 | -26 | -16 | -16 |
| s07 | -16 | -26 | -36 | -16 | -26 | -36 |

The subjective material in the BS.1534 experiments is level-adjusted to the nominal level of -26 dB LKFS.

Evaluation of different configurations of the IVAS codec require different testing methodologies, outlined in the following clauses and annexes.

#### 7.2.5.2	P.800 DCR listening test layout

The following layout is a generic layout. The actual layouts of P.800 DCR experiments may be slightly different. They are described in Annex D.

Table 7.2-3: P.800 DCR testing under clean-channel conditions incl. a comparison of fixed-point and floating-point code

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q = xx dB | - | - |
| c03 | MNRU Q = xx dB | - | - |
| c04 | MNRU Q = xx dB | - | - |
| c05 | MNRU Q = xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| c09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

#### 7.2.5.3	BS.1534 listening test layouts

The following layouts are generic layouts. The actual layouts of BS.1534 experiments may be slightly different. They are described in Annex D.

Table7.2-4: High Bitrate MUSHRA Tests

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | LP 3.5 kHz | - | - |
| c03 | EVS | 1x 64.0 | Off |
| c04 | EVS | 1x128.0 | Off |
| c05 | IVAS | 64.0 | Off |
| c06 | IVAS | 96.0 | Off |
| c07 | IVAS | 128.0 | Off |
| c08 | IVAS | 256.0 | Off |

Table 7.2-5: Low Bitrate MUSHRA Tests

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | LP 3.5 kHz | - | - |
| c03 | EVS | 1x 16.4 | Off |
| c04 | EVS | 1x 32.0 | Off |
| c05 | IVAS | 16.4 | Off |
| c06 | IVAS | 24.4 | Off |
| c07 | IVAS | 32.0 | Off |
| c08 | IVAS | 48.0 | Off |

#### 7.2.5.4	Allocation of experiments

The following tables show high-level overview of the experiments with allocation of experiments to LLs. Table 7.2-6 shows overview of P.800 DCR experiments with languages used for each experiment. Table 7.2-7 shows overview of BS.1534 experiments. Finally, Table 7.2-8 shows overview of experiments run with different methodologies.

Detail conditions for each subjective experiment are defined in Annex D.

Table 7.2-6: Allocation of P.800 DCR experiments to LLs and corresponding languages

| Exp | Objective | Input format | Source material | FX validation | DTX | FER | Bitrates [kb/s] | Language | LL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P800-1 | FX, RD | Stereo | All | Yes | Off | 0% | 13.2-128 | FR | VA |
| P800-2 | DTX, FE | Stereo | Al | Yes | On | 5% | 13.2-128 | JAP | NTT |
| P800-3 | FX, RD | FOA | All | Yes | Off | 0% | 13.2-256 | FR | Orange |
| P800-4 | DTX, FE | HOA2 | All | Yes | On | 5% | 16.4-384 | FR | Orange |
| P800-5 | FX, RD | HOA3 | All | Yes | Off | 0% | 32-512 | ENG | Dolby |
| P800-6 | FX, RD | MC 5.1, 7.1 | Clean speech, mixed/music | Yes | Off | 0% | 24.4-256 | MAN | Mesaqin |
| P800-7 | FX, RD | MC 5.1+4, 7.11+4 | Clean speech, mixed/music | Yes | Off | 0% | 32-384 | DAN | Force |
| P800-8 | FE | MC (mixed CICP) | Clean speech, mixed/music | Yes | Off | 5% | 16.4-384 | CZ | Mesaqin |
| P800-9 | FX, RD | 1-2 Objects | All | Yes | Off | 0% | 13.2-128 | MAN | Mesaqin |
| P800-10 | FX, RD | 3-4 Objects | speech+ effects, speech + music, music | Yes | Off | 0% | 24.4-192 | FR | VA |
| P800-11 | DTX, FE | 1-4 Objects | Clean speech, speech+ effects, speech + music, music | Yes | On | 5% | 24.4-192 | DAN | Force |
| P800-12 | FX, RD | MASA 1 TC | All | Yes | Off | 0% | 13.2-128 | MAN | Mesaqin |
| P800-13 | FX, RD | MASA 2 TC | All | Yes | Off | 0% | 13.2-128 | DAN | Force |
| P800-14 | DTX, FE | MASA 1,2 TC | All | Yes | On | 4, 8% | 13.2-128 | FIN | Nokia |
| P800-15 | FX, RD | OSBA, 1-2 Objects | All | Yes | Off | 0% | 32-512 | ENG | Dolby |
| P800-16 | FX, RD | OSBA, 3-4 Objects | All | Yes | Off | 0% | 32-512 | SWE | Ericsson |
| P800-17 | RD, FE | OSBA, 1-4 Objects | All | Yes | Off | 5% | 13.2-512 | GER | FhG |
| P800-18 | FX, RD | OMASA, 1-2 Objects | All | Yes | Off | 0% | 13.2-128 | SWE | Ericsson |
| P800-19 | FX, RD | OMASA, 3-4 Objects | All | Yes | Off | 0% | 13.2-128 | GER | FhG |
| P800-20 | RD, FE | OMASA, 1-4 Objects | All | Yes | Off | 5% | 13.2-512 | FIN | Nokia |
| P800-21 | JBM/FE | Stereo | All | Yes | On | I1.O1, I1.O2 | 24.4-96 | CZ | Mesaqin |
| P800-22 | JBM/FE | 1-2 Objects | All | Yes | Off | I1.O1, I1.O2 | 24.4-96 | DAN | Force |
| P800-23 | JBM/FE | FOA | All | Yes | Off | I1.O1, I1.O2 | 24.4-96 | DAN | Force |

-	JAP = Japanese

-	FR = French

-	GER = German

-	MAN = Mandarin

-	DAN = Danish

-	ENG = English

-	FIN = Finnish

-	SWE = Swedish

-	CZ = Czech

Table 7.2-7: Allocation of BS.1534 experiments to LLs

| Exp | Objective | Input format | Source material | Bitrates [kb/s] | Listening environment | LL |
| --- | --- | --- | --- | --- | --- | --- |
| BS1534-1 | Compare to EVS | Stereo | Generic audio | 16.4-48 | Headphones | Force |
| BS1534-2 | Compare to EVS | Stereo | Generic audio | 64-256 | Headphones | Huawei |
| BS1534-3 | Compare to EVS | FOA | Generic audio | 16.4-48 | Headphones | Huawei |
| BS1534-4 | Compare to EVS | FOA | Generic audio | 64-256 | Headphones | Panasonic/NTT |
| BS1534-5 | Compare to EVS | HOA3 | Generic audio | 64-256 | 7.1 + 4 | Dolby |
| BS1534-6 | Compare to EVS | 5.1 | Generic audio | 16.4-48 | 5.1 | Ericsson |
| BS1534-7 | Compare to EVS | 5.1, 7.1 | Generic audio | 64-256 | Headphones | Dolby |
| BS1534-8 | Compare to EVS | 5.1+2, 5.1+4 | Generic audio | 64-256 | Headphones | Nokia |
| BS1534-9 | Compare to EVS | 7.1+4 | Generic audio | 64-256 | 7.1 + 4 | FhG |
| BS1534-10 | Compare to EVS | 1-2 Objects | Generic audio | 64-256 | Headphones | Qualcomm |
| BS1534-11 | Compare to EVS | 3-4 Objects | Generic audio | 24.4-64 | Headphones | Dolby |
| BS1534-12 | Compare to EVS | 3-4 Objects | Generic audio | 64-256 | Headphones | Philips |
| BS1534-13 | Compare to EVS | MASA 1 TC | Generic audio | 16.4-48 | Headphones | Qualcomm |
| BS1534-14 | Compare to EVS | MASA 1 TC | Generic audio | 64-256 | Headphones | Dolby |
| BS1534-15 | Compare to EVS | MASA 2 TC | Generic audio | 64-256 | Headphones | Nokia |
| BS1534-16 | Compare to EVS | OSBA 1-4 Objects | Generic audio | 16.4-48 | Headphones | FhG |
| BS1534-17 | Compare to EVS | OSBA 1-4 Objects | Generic audio | 64-256 | Headphones | Dolby |
| BS1534-18 | Compare to EVS | OMASA 1-4 Objects | Generic audio | 64-256 | Headphones | Nokia |
| BS1534-19 | Stereo downmix | Stereo | Generic audio | 13.2, 24.2 | Headphones | Orange |
| BS1534-20 | 6-DoF and directivity | 4 Objects | Generic audio | 64, 512 | Headphones | Ericsson |

Table 7.2-8: Allocation of experiments using different methodologies

| Exp | Objective | Input format | Source material | Listening environment | Bitrates [kb/s] | LL |
| --- | --- | --- | --- | --- | --- | --- |
| ACR-1 | 16, 32, 48 kHz | Stereo, FOA, HOA3, MASA, ISM2 | All | Headphones | 13.2-256 | Nokia |
| ROOM-1 | Room acoustics | FOA | Generic audio | Headphones | 80 | Philips |
| ROOM-2 | Room acoustics | FOA | Generic audio | Headphones | 96 | Philips |
| ROOM-3 | Room acoustics | MC – 5.1 | Generic audio | Headphones | 96 | Philips |
| ROOM-4 | Room acoustics | MC – 5.1.2 | Generic audio | Headphones | 128 | Philips |

Legend:

-	Objective – main objectives of the experiment

-	FE (in the “Objective” column) – performance in noisy channel

-	DTX (in the “Objective” column) – performance in DTX on

-	FX (in the “Objective” column) – fixed-point validation

-	RD (in the “Objective” column) – rate-distortion curve

-	JBM (in the “Objective” column) – including Jitter Buffer Management

-	Tan (in the “Objective” column) – Tandeming

-	EVS (in the “Objective” column) – comparison to EVS

-	P800 – P.800 DCR test

-	ACR – P.800 ACR test

-	ROOM – room acoustics test

-	All – means clean speech, speech with background, mixed content, and music

==============Next change==============

### 9.2.1	Overview

In Selection phase, five experiments have been conducted to evaluate the performance of the IVAS codec with stereo content. While the experiments P800-1, P800-2 and P800-3 were conducted as P.800 DCR tests, the experiments BS1534-1a and BS1534-1b were conducted as BS.1534 tests. All experiments were conducted using headphone presentation.

-	Selection Experiment P800-1: Stereo clean speech under clean and impaired channel conditions, headphone presentation

-	Selection Experiment P800-2: Stereo speech + background under clean channel conditions, DTX off and on, headphone presentation

-	Selection Experiment P800-3: Stereo mixed and music under clean and impaired channel conditions, headphone presentation

-	Selection Experiment BS1534-1a: Stereo generic audio, 48 and 64 kbps, headphone presentation

-	Selection Experiment BS1534-1b: Stereo generic audio, 96 and 128 kbps, headphone presentation

In Characterization phase, five additional experiments have been conducted. While the experiments P800-1, P800-2 and P800-21 were conducted as P.800 DCR tests, the experiments BS1534-1 and BS1534-2 were conducted as BS.1534 tests.

-	Characterization Experiment P800-1: Stereo speech + background and mixed/music under clean channel conditions, headphone presentation

- 	Characterization Experiment P800-2: Stereo speech + background and mixed/music under impaired channel conditions, DTX on, headphone presentation

-	Characterization Experiment P800-21: Stereo speech + background and mixed/music under impaired channel conditions using JBM, DTX on, headphone presentation

-	Characterization Experiment BS1534-1: Stereo generic audio, 16.4 – 48 kbps, headphone presentation

-	Characterization Experiment BS1534-2: Stereo generic audio, 64 – 256 kbps, headphone presentation

==============Next change==============

### 9.2.7	Characterization Experiment P800-1 (Stereo, Speech and Mixed/Music, Clean Channel, Headphone Presentation)

### 9.2.8	Characterization Experiment P800-2 (Stereo, Speech and Mixed/Music, Impaired Channel, DTX on, Headphone Presentation)

### 9.2.9	Characterization Experiment P800-21 (Stereo, Speech and Mixed/Music, JBM, DTX on, Headphone Presentation)

### 9.2.10	Characterization Experiment BS1534-1 (Stereo, Generic Audio, 16.4 – 48 kbps, Headphone Presentation)

### 9.2.11	Characterization Experiment BS1534-2 (Stereo, Generic Audio, 64 – 256 kbps, Headphone Presentation)

==============Next change==============

### 9.3.1	Overview

In Selection phase, six experiments have been conducted to evaluate the performance of the IVAS codec with Ambisonics content. While the experiments P800-4 and P800-5 were conducted as P.800 DCR tests on FOA content, the experiments BS1534-4a, BS1534-4b, BS1534-5a and BS1534-5b were conducted as BS.1534 tests for different Ambisonics orders. The experiments P800-4, P800-5, BS1534-4a, BS1534-4b, BS1534-5a were conducted using headphone presentation, whereas experiment BS1534-5b was conducted using 7.1+4 loudspeaker presentation.

-	Selection Experiment P800-4: Clean speech, FOA under clean and impaired channel conditions, headphone presentation

-	Selection Experiment P800-5: Speech + background, FOA under clean channel conditions, DTX off and on, headphone presentation

-	Selection Experiment BS1534-4a: Generic audio, FOA input, HOA3 output, 96 kbps, 128 kbps and 160 kbps, headphone presentation

-	Selection Experiment BS1534-4b: Generic audio, HOA2 input, HOA3 output, 160 and 192 kbps, headphone presentation

-	Selection Experiment BS1534-5a: Generic audio, HOA3 input, HOA3 output, 192 and 256 kbps, headphone presentation

-	Selection Experiment BS1534-5b: Generic audio, HOA3 input, HOA3 output, 384 and 512 kbps, 7.1+4 loudspeaker presentation

In Characterization phase, seven additional experiments have been conducted. While the experiments P800-3, P800-4, P800-5 and P800-23 were conducted as P.800 DCR tests, the experiments BS1534-3, BS1534-4 and BS1534-5 were conducted as BS.1534 tests.

-	Characterization Experiment P800-3: Speech + background and mixed/music, FOA under clean channel conditions, headphone presentation

-	Characterization Experiment P800-4: Speech + background and mixed/music, HOA2 under impaired channel conditions, DTX off and on, headphone presentation

-	Characterization Experiment P800-5: Speech + background and mixed/music, HOA3 under clean channel conditions, headphone presentation

-	Characterization Experiment P800-23: Speech + background and mixed/music, FOA under impaired channel conditions using JBM, headphone presentation

- 	Characterization Experiment BS1534-3: Generic audio, FOA input, binaural output, 16.4 – 48 kbps, headphone presentation

-	Characterization Experiment BS1534-4: Generic audio, FOA input, binaural output, 64 – 256 kbps, headphone presentation

-	Characterization Experiment BS1534-5: Generic audio, HOA3 input, 7.1+4 output, 64 – 256 kbps, loudspeaker presentation

==============Next change==============

### 9.3.8	Characterization Experiment P800-3 (FOA, Speech + background and mixed/music, Headphone Presentation)

### 9.3.9	Characterization Experiment P800-4 (HOA2, Speech + background and mixed/music, Impaired Channel, DTX, Headphone Presentation)

### 9.3.10	Characterization Experiment P800-5 (HOA3, Speech + background and mixed/music, Headphone Presentation)

### 9.3.11	Characterization Experiment P800-23 (FOA, Speech + background and mixed/music, JBM, Headphone Presentation)

### 9.3.12	Selection Experiment BS1534-3 (FOA, Generic Audio, 16.4 - 48 kbps, Headphone Presentation)

### 9.3.13	Selection Experiment BS1534-4 (FOA, Generic Audio, 64 - 256 kbps, Headphone Presentation)

### 9.3.14	Selection Experiment BS1534-4 (HOA3, Generic Audio, 64 - 256 kbps, 7.1+4 Loudspeaker Presentation)

==============Next change==============

### 9.4.1	Overview

In Selection phase, four experiments have been conducted to evaluate the performance of the IVAS codec with object content. While the experiments P800-6 and P800-7 were conducted as P.800 DCR tests on 1 or 2 object content, the experiments BS1534-6a and BS1534-6b were conducted as BS.1534 test on 3 or 4 object content. All experiments were conducted using headphone presentation.

-	Selection Experiment P800-6: Clean speech, 1 object under clean and impaired channel conditions, DTX off and on, headphone presentation

-	Selection Experiment P800-7: Clean speech, 2 objects under clean and impaired channel conditions, DTX off and on, headphone presentation

-	Selection Experiment BS1534-6a: Generic audio, 3 objects, 48, 64 and 96 kbps, headphone presentation

-	Selection Experiment BS1534-6b: Generic audio, 4 objects, 96, 128 and 256 kbps, headphone presentation

In Characterization phase, seven additional experiments have been conducted. While the experiments P800-9, P800-10, P800-11 and P800-22 were conducted as P.800 DCR tests, the experiments BS1534-10, BS1534-11 and BS1534-12 were conducted as BS.1534 tests.

-	Characterization Experiment P800-9: Speech and mixed/music, 1-2 objects under clean channel conditions, headphone presentation

- 	Characterization Experiment P800-10: Speech and mixed/music, 3-4 objects under clean channel conditions, headphone presentation

- 	Characterization Experiment P800-11: Speech and mixed/music, 1-4 objects under impaired channel conditions, DTX on, headphone presentation

- 	Characterization Experiment P800-22: Speech and mixed/music, 1-2 objects under impaired channel conditions using JBM, headphone presentation

-	Characterization Experiment BS1534-10: Generic audio, 1-2 objects, 64 – 256 kbps, headphone presentation

- 	Characterization Experiment BS1534-11: Generic audio, 3-4 objects, 24.4 – 64 kbps, headphone presentation

-	Characterization Experiment BS1534-12: Generic audio, 3-4 objects, 64 – 256 kbps, headphone presentation

==============Next change==============

### 9.4.6	Characterization Experiment P800-9 (1-2 Objects, Speech and Mixed/Music, Headphone Presentation)

### 9.4.7	Characterization Experiment P800-10 (3-4 Objects, Speech and Mixed/Music, Headphone Presentation)

### 9.4.8	Characterization Experiment P800-10 (1-4 Objects, Speech and Mixed/Music, Impaired Channel, DTX on, Headphone Presentation)

### 9.4.9	Characterization Experiment P800-22 (1-2 Objects, Speech and Mixed/Music, JBM, Headphone Presentation)

### 9.4.10	Characterization Experiment BS1534-10 (1-2 Objects, Generic Audio, 64 – 256 kbps, Headphone Presentation)

### 9.4.11	Characterization Experiment BS1534-11 (3-4 Objects, Generic Audio, 24.4 – 64 kbps, Headphone Presentation)

### 9.4.12	Characterization Experiment BS1534-12 (3-4 Objects, Generic Audio, 64 – 256 kbps, Headphone Presentation)

==============Next change==============

### 9.5.1	Overview

In Selection phase, four experiments have been conducted to evaluate the performance of the IVAS codec wit MASA content. While the experiments P800-8 and P800-9 were conducted as P.800 DCR tests, the experiments BS1534-7a and BS1534-7b were conducted as BS.1534 tests. All experiments were conducted using headphone presentation.

-	Selection Experiment P800-8: Clean speech, MASA under clean and impaired channel conditions, headphone presentation

-	Selection Experiment P800-9: Speech + background, MASA under clean channel conditions, DTX off and on, headphone presentation

-	Selection Experiment BS1534-7a: Generic audio, MASA, 96 and 128 kbps, headphone presentation

-	Selection Experiment BS1534-7b: Generic audio, MASA, 192 and 256 kbps, headphone presentation

In Characterization phase, six additional experiments have been conducted. While the experiments P800-12, P800-13, P800-14 were conducted as P.800 DCR tests, the experiments BS1534-13, BS1534-14 and BS1534-15 were conducted as BS.1534 tests.

-	Characterization Experiment P800-12: Speech + background and mixed/music, MASA 1TC under clean channel conditions, headphone presentation

-	Characterization Experiment P800-13: Speech + background and mixed/music, MASA 2TC under clean channel conditions, headphone presentation

-	Characterization Experiment P800-14: Speech + background and mixed/music, MASA 1-2TC under impaired channel conditions, DTX on, headphone presentation

- 	Characterization Experiment BS1534-13: Generic audio, MASA 1TC, 16.4 – 48 kbps, headphone presentation

- 	Characterization Experiment BS1534-14: Generic audio, MASA 1TC, 64 – 256 kbps, headphone presentation

- 	Characterization Experiment BS1534-15: Generic audio, MASA 2TC, 64 – 256 kbps, headphone presentation

==============Next change==============

### 9.5.6	Characterization Experiment P800-12 (MASA 1TC, Speech and Mixed/Music, Headphone Presentation)

### 9.5.7	Characterization Experiment P800-13 (MASA 2TC, Speech and Mixed/Music, Headphone Presentation)

### 9.5.8	Characterization Experiment P800-14 (MASA 1-2TC, Speech and Mixed/Music, Impaired Channel, DTX on, Headphone Presentation)

### 9.5.9	Characterization Experiment BS1534-13 (MASA 1TC, Generic Audio, 16.4 – 48 kbps, Headphone Presentation)

### 9.5.10	Characterization Experiment BS1534-14 (MASA 1TC, Generic Audio, 64 – 256 kbps, Headphone Presentation)

### 9.5.11	Characterization Experiment BS1534-14 (MASA 2TC, Generic Audio, 64 – 256 kbps, Headphone Presentation)

==============Next change==============

### 9.6.1	Overview

In Selection phase, four experiments have been conducted to evaluate the performance of the IVAS codec with multi-channel content. All experiments were conducted as BS.1534 tests. While experiments BS1534-2a and BS1534-2b were conducted using 5.1 loudspeaker presentation, experiments BS1534-3a and BS1534-3b were conducted using 7.1+4 loudspeaker presentation.

-	Selection Experiment BS1534-2a: Generic audio, multi-channel 5.1, 64 and 96 kbps, loudspeaker 5.1 presentation

-	Selection Experiment BS1534-2b: Generic audio, multi-channel 5.1, 128 and 160 kbps, loudspeaker 5.1 presentation

-	Selection Experiment BS1534-3a: Generic audio, multi-channel 7.1.4, 128 and 160 kbps, loudspeaker 7.1+4 presentation

-	Selection Experiment BS1534-3b: Generic audio, multi-channel 7.1.4, 384 and 512 kbps, loudspeaker 7.1+4 presentation

In Characterization phase, seven additional experiments have been conducted. While the experiments P800-6, P800-7, P800-8 were conducted as P.800 DCR tests, the experiments BS1534-6, BS1534-7, BS1534-8 and BS1534-9 were conducted as BS.1534 tests.

-	Characterization Experiment P800-6: Speech + background and mixed/music, multi-channel 5.1 and 7.1, under clean channel conditions, headphone presentation

- 	Characterization Experiment P800-7: Speech + background and mixed/music, multi-channel 5.1+4 and 7.1+4, under clean channel conditions, headphone presentation

-	Characterization Experiment P800-8: Speech + background and mixed/music, mixed CICP layouts, under impaired channel conditions, headphone presentation

-	Characterization Experiment BS1534-6: Generic audio, multi-channel 5.1, 16.4 – 48 kbps, loudspeaker 5.1 presentation

- 	Characterization Experiment BS1534-7: Generic audio, multi-channel 5.1 and 7.1, 64 – 256 kbps, headphone presentation

- 	Characterization Experiment BS1534-8: Generic audio, multi-channel 5.1+2 and 5.1+4, 64 – 256 kbps, headphone presentation

-	Characterization Experiment BS1534-9: Generic audio, multi-channel 7.1+4, 64 – 256 kbps, loudspeaker 7.1+4 presentation

==============Next change==============

### 9.6.6	Characterization Experiment P800-6 (MC 5.1/7.1, Speech and Mixed/Music, Headphone Presentation)

### 9.6.7	Characterization Experiment P800-7 (MC 5.1+4/7.1+4, Speech and Mixed/Music, Headphone Presentation)

### 9.6.8	Characterization Experiment P800-8 (Mixed CICP, Speech and Mixed/Music, Headphone Presentation)

### 9.6.9	Characterization Experiment BS1534-6 (MC 5.1, Generic Audio, 16.4 – 48 kbps, Loudspeaker Presentation)

### 9.6.10	Characterization Experiment BS1534-7 (MC 5.1/7.1, Generic Audio, 64 – 256 kbps, Headphone Presentation)

### 9.6.11	Characterization Experiment BS1534-8 (MC 5.1+2/5.1+4, Generic Audio, 64 – 256 kbps, Headphone Presentation)

### 9.6.12	Characterization Experiment BS1534-9 (MC 7.1+4, Generic Audio, 64 – 256 kbps, Loudspeaker Presentation)

==============Next change==============

### 9.7.1	Overview

Provided as optional additional information for codec selection, two experiments have been conducted to evaluate the performance of the IVAS codec with the combination of combined objects and SBA content. All experiments were conducted as BS.1534 tests using headphone presentation.

-	Optional additional information experiment 1: Generic audio, combined objects and SBA, 256 kbps, headphone presentation

-	Optional additional information experiment 2: Generic audio, combined objects and SBA, 512 kbps, headphone presentation

In Characterization phase, five additional experiments have been conducted. While the experiments P800-15, P800-16, P800-17 were conducted as P.800 DCR tests, the experiments BS1534-16 and BS1534-17 were conducted as BS.1534 tests.

-	Characterization Experiment P800-15: Speech and mixed/music + background, 1-2 objects combined with SBA, under clean channel conditions, headphone presentation

-	Characterization Experiment P800-16: Speech and mixed/music + background, 3-4 objects combined with SBA, under clean channel conditions, headphone presentation

- 	Characterization Experiment P800-17: Speech and mixed/music + background, 1-4 objects combined with SBA, under impaired channel conditions, headphone presentation

-	Characterization Experiment BS1534-16: Generic audio, 1-4 objects combined with SBA, 16.4 – 48 kbps, headphone presentation

-	Characterization Experiment BS1534-17: Generic audio, 1-4 objects combined with SBA, 64 – 256 kbps, headphone presentation

==============Next change==============

### 9.7.3	Characterization Experiment P800-15 (OSBA 1-2 Objects, Speech and Mixed/Music + Background, Headphone Presentation)

### 9.7.4	Characterization Experiment P800-16 (OSBA 3-4 Objects, Speech and Mixed/Music + Background, Headphone Presentation)

### 9.7.5	Characterization Experiment P800-17 (OSBA 1-4 Objects, Speech and Mixed/Music + Background, Impaired Channel, Headphone Presentation)

### 9.7.6	Characterization Experiment BS1534-16 (OSBA 1-4 Objects, Generic Audio, 16.4 – 48 kbps, Headphone Presentation)

### 9.7.7	Characterization Experiment BS1534-17 (OSBA 1-4 Objects, Generic Audio, 16.4 – 48 kbps, Headphone Presentation)

==============Next change==============

### 9.8.1	Overview

Provided as optional additional information provided for codec selection, one experiments has been conducted to evaluate the performance of the IVAS codec with the combination of combined objects and MASA content. The experiment was conducted as BS.1534 test using headphone presentation.

-	Optional additional information experiment 3: Generic audio, combined objects and MASA, 48, 64, 80 and 128 kbps, headphone presentation

In Characterization phase, four additional experiments have been conducted. While the experiments P800-18, P800-19 and P800-20 were conducted as P.800 DCR tests, the experiment BS1534-18 was conducted as a BS.1534 test.

-	Characterization Experiment P800-18: Speech and mixed/music + background, 1-2 objects combined with MASA, under clean channel conditions, headphone presentation

-	Characterization Experiment P800-19: Speech and mixed/music + background, 3-4 objects combined with MASA, under clean channel conditions, headphone presentation

-	Characterization Experiment P800-20: Speech and mixed/music + background, 1-4 objects combined with MASA, under impaired channel conditions, headphone presentation

-	Characterization Experiment BS1534-18: Generic audio, 1-4 objects combined with MASA, 64 – 256 kbps, headphone presentation

==============Next change==============

### 9.8.3	Characterization Experiment P800-18 (OMASA 1-2 Objects, Speech and Mixed/Music + Background, Headphone Presentation)

### 9.8.4	Characterization Experiment P800-19 (OMASA 3-4 Objects, Speech and Mixed/Music + Background, Headphone Presentation)

### 9.8.5	Characterization Experiment P800-20 (OMASA 1-4 Objects, Speech and Mixed/Music + Background, Impaired Channel, Headphone Presentation)

### 9.8.6	Characterization Experiment BS1534-18 (OMASA 1-4 Objects, Generic Audio, 64 – 256 kbps, Headphone Presentation)

==============Next change==============

### 9.9.1	Overview

Provided as optional additional information provided for codec selection, one experiments has been conducted to evaluate the performance of the IVAS codec for downmixing stereo content to an EVS compatible mono downmix. The experiment was conducted as P.800 DCR test using headphone presentation.

-	Optional additional information experiment 4: Speech and music, stereo input, mono output, 13.2 and 24.4 kbps, headphone presentation

In Characterization phase, one additional experiment has been conducted. The experiment BS1534-19 was conducted as a BS.1534 test.

-	Characterization Experiment BS1534-19: Generic Audio, stereo input, mono output, 13.2 and 24.4 kbps, headphone presentation

==============Next change==============

### 9.9.3	Characterization Experiment BS1534-19 (Stereo Downmix, Generic Audio, 13.2 and 24.4 kbps, Headphone Presentation)

==============Next change==============

## 9.10	Rendering

### 9.10.1	Overview

In Characterization phase, five experiments have been conducted to evaluate the performance of the rendering functionality of the IVAS codec for various content. Experimemnt BS1534-20 was conducted using BS.1534 test methodology, whereas the experiments ROOM-1 – ROOM-4 were conducted using a dedicated preference test methodology for room acoustics.

-	Characterization Experiment BS1534-20: Generic Audio, 4 objects, 64 and 512 kbps, evaluating 6-DoF and directivity rendering, headphone presentation

-	Characteriaztion Experiment ROOM-1: Generic Audio, FOA, 80 kbps, evaluating binaural rendering with and without room acoustics, headphone presentation

-	Characteriaztion Experiment ROOM-2: Generic Audio, FOA, 96 kbps, evaluating binaural rendering with and without room acoustics, headphone presentation

-	Characteriaztion Experiment ROOM-3: Generic Audio, multi-channel 5.1, 96 kbps, evaluating binaural rendering with and without room acoustics, headphone presentation

-	Characteriaztion Experiment ROOM-4: Generic Audio, multi-channel 5.1+2, 128 kbps, evaluating binaural rendering with and without room acoustics, headphone presentation

### 9.10.2	Characterization Experiment BS1534-20 (4 Objects, Generic Audio, 64 and 512 kbps, 6-DoF and Directivity Rendering, Headphone Presentation)

### 9.10.3	Characterization Experiment ROOM-1 (FOA, Generic Audio, 80 kbps, Room Acoustic Rendering, Headphone Presentation)

### 9.10.4	Characterization Experiment ROOM-2 (FOA, Generic Audio, 96 kbps, Room Acoustic Rendering, Headphone Presentation)

### 9.10.5	Characterization Experiment ROOM-3 (MC 5.1, Generic Audio, 96 kbps, Room Acoustic Rendering, Headphone Presentation)

### 9.10.6	Characterization Experiment ROOM-4 (MC 5.1+2, Generic Audio, 128 kbps, Room Acoustic Rendering, Headphone Presentation)

==============Next change==============

## 9.12	Multi-Format

### 9.12.1	Overview

In Characterization phase, one experiment has been conducted to evaluate the performance of the IVAS codec for multiple formats. Experimemnt ACR-1 was conducted using P.800 ACR test methodology.

-	Characterization Experiment ACR-1: Speech and mixed/music, Stereo/FOA/HOA3/MASA 1TC/MASA 2TC/2 objects, under clean and impaired channel conditions, headphone presentation

### 9.12.2	Characterization Experiment ACR-1 (Multi-Format, Speech and Mixed/Music, Clean and Impaired Channel, Headphone Presentation)

==============End of change==============