3GPP TSG-SA4 Meeting #134	S4-251783

# Dallas, US 17th – 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.143 | CR | 008 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeME-MED] AMR and mp3 files for messaging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc., Google, Dolby |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeME-MED, TEI-19 |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | GSMA is progressing to refer to SA4 specifications with TS 26.143 as an anchor for RCS messaging. However, AMR storage format (RFC 4867) and MP3 are used by RCS messaging, which are not supported by TS 26.143. This is a gap in the specification if RCS is to refer to this TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added support for AMR storage format (RFC 4867) and mp3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RCS support stays incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1, 5.2.1, 5.5.2, 6.2.2, 6.2.3, 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Khronos glTF 2.0, glTF™ 2.0 Specification (khronos.org)

[3] 	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[4]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[5]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[6]	IETF Draft draft-ietf-mimi-content-01: "More Instant Messaging Interoperability (MIMI) message content", 	Rohan Mahy

[7]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[8]	Open Mobile alliance, "MMS Architecture" OMA-AD-MMS-V1_3-20110913-A.

[9]	Open Mobile alliance, "Multimedia Messaging Service Encapsulation Protocol" OMA-TS-MMS_ENC-V1_3-20110913-A.

[10]	GSMA "RCS Universal Profile Service Definition Document", Version 2.6, 19 December 2022.

[11]	GSMA PRD RCC.07 version 13.0 - "Rich Communication Suite - Advanced Communications Services and Client Specification" 19 December 2022.

[12]	IETF RFC 2046, "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[13]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects -Part 12: ISO base media file format".

[14]	ISO/IEC 23000-24:2023 Preliminary Draft of: Information technology — Multimedia application format (MPEG-A) — Part 24: Messaging Media Application Format (MeMAF) ".

NOTE: 	A preliminary draft of this standard is available as MDS23345_W03_N1082 here: https://www.mpeg.org/wp-content/uploads/mpeg_meetings/144_Hannover/w23345.zip

[15]	3GPP 23.140: "Multimedia Messaging Service (MMS); Functional Description; Stage 2".

[16]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[17]	"JPEG File Interchange Format", Version 1.02, September 1, 1992.

[18]	"Exchangeable image file format for digital still cameras: EXIF 2.2", Specification by the Japan Electronics and Information Technology Industries Association (JEITA), April 2002, URL: http://www.exif.org/

[19]	CompuServe Incorporated: "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA, 1987.

[20]	Compuserve Incorporated, Columbus, Ohio (1990): "Graphics Interchange Format (Version 89a)".

[21]	IETF RFC 2083: "PNG (Portable Networks Graphics) Specification version 1.0 ", T. Boutell, et. al., March 1997.

[22]	ISO/IEC 23000-22:2019 Information technology — Multimedia application format (MPEG-A) — Part 22: Multi-image application format (MIAF)

[23]	IETF RFC 2045, "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies", November 1996

[24]	ISO/IEC 23008-12:2019 Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 12: Image File Format

[25]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

[26]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)"

[27]	3GPP TS 26.245: "Transparent end-to-end packet switched streaming service (PSS); Timed text format"

[28]	ISO/IEC 14496-30: "Information technology - Coding of audio-visual objects - Part 30: Timed text and other visual overlays in ISO base media file format".

[29]	IETF RFC 2387, "The MIME Multipart/Related Content-type"

[30]	IETF RFC 6381, "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types"

[31]	3GPP TS 26.307, "Presentation Layer for 3GPP Services"

[32]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs"

[33]	IETF RFC 2077, "The Model Primary Content Type for Multipurpose Internet Mail Extensions"

[34]	3GPP TS 26.119, "Media Capabilities for Augmented Reality"

[35]	3GPP TS 26.265: "Media Delivery: Video Capabilities and Operation Points"

[36]	3GPP TR 26.857, "5G Media Service Enablers"

[37]	ISO/IEC 14496-14: "Information technology - Coding of audio-visual objects -Part 14: MP4 file format".

[xx]	IETF RFC 4867: "RTP Payload Formats for AMR and AMR-WB Audio Codings", E. Burger, H. Schulzrinne, June 2007.

[xy]	ISO/IEC 11172-3: "Information technology - Coding of moving pictures and associated audio for digital storage media at up to about 1,5 Mbit/s - Part 3: Audio"

# * * * Next Change * * *

## 5.1	Introduction

In order to guarantee a minimum support and compatibility between messaging capable terminals, MMBP Generators and MMBP Players in UEs supporting specific media content with associated media types.

The clause defines multimedia messaging body parts (MMBPs) for different media types as well as the associated media types.

According to the introduction in clause 4, MMBPs defined in this clause,

-	may be used as full body parts or sub-parts in message bodies.

-	may either be a single binary octet string, or they may consist of multiple parts. If the latter, the conceptual relationship introduced in clause 4.4 is defined that is mapped to container formats defined in clause 5.2.

Media Types and related capabilities defined in this specification for playback are provided in Table 5.1-1.

Table 5.1-1 Media Types and Capabilities defined in TS 26.143 for playback and decoding

| Media Type in the present TS | Capabilities defined in this specification | Clause | Media Type signalling example |
| --- | --- | --- | --- |
| Multipart MMBPs and Container Formats | 26143_CONTAINER_RFC2046_SINGLE 26143_CONTAINER_RFC2046_MIXED 26143_CONTAINER_RFC2046_ALTERNATIVE 26143_CONTAINER_RFC2046_PARALLEL 26143_CONTAINER_RFC2387_RELATED 26143_CONTAINER_MP4_3GP9 26143_CONTAINER_MP4_MP42 26143_CONTAINER_AMR-WB 26143_CONTAINER_AMR | 5.2.1 | Media type of subtype multipart/mixed multipart/alternative multipart/parallel multipart/related video/mp4, profile="3gp9" video/mp4, profile="mp42" audio/AMR-WB audio/AMR |
| Text | 26143_TEXT_PLAIN | 5.3.1 | text/plain |
| Speech | 26143_AUDIO_IVAS 26143_AUDIO_EVS 26143_AUDIO_AMR-WB 26143_AUDIO_AMR 26143_AUDIO_AMR-WB_FILE 26143_AUDIO_AMR_FILE | 5.5.1 | audio/mp4 audio/AMR-WB audio/AMR |
| Audio | 26143_AUDIO_IVAS 26143_AUDIO_XHE-AAC 26143_AUDIO_EAAC+ 26143_AUDIO_MP3 | 5.5.1 | audio/mp4 audio/mpeg |
| Image | 26143_IMG_JPEG 26143_IMG_HEIC 26143_IMG_GIF 26143_IMG_PNG | 5.4.1 | image/jpeg image/heic, profile="heic,MiHB" imageTypes="hvc1.2.4.L153.B0" image/gif image/png |
| Video | 26143_VIDEO_AVC-HD 26143_VIDEO_AVC-FullHD 26143_VIDEO_HEVC-HD 26143_VIDEO_HEVC-FullHD 26143_VIDEO_HEVC-UHD | 5.6.1 | video/mp4, profile="3gp9" codecs="avc1.640028" video/mp4, profile="3gp9" codecs="avc1.640029" video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" video/mp4, profile="3gp9" codecs="hvc1.1.2.L123.B0" video/mp4, profile="3gp9" codecs="hvc1.1.2.L153.B0" |
| Subtitles and Text | 26143_TT_3GPP 26143_TT_IMSC11 | 5.7.1 | text/mp4, profile="3gp9" codecs="tx3g" application/mp4, profile="3gp9" codecs="stpp.ttml.im2t" |
| 3d scenes and assets | 26143_SCENE_GLTF20 26143_SCENE_GLTF20_AR 26143_SCENE_GLTF20_GLB 26143_SCENE_GLTF20_GLB_AR | 5.8 | model/gltf+json model/gltf-binary |
| Presentation format | 26143_PRESENTATION_HTML5 | 5.9 | text/html |

Media Types and related capabilities defined in this specification for content generation are provided in Table 5.1-2.

Table 5.1-2 Media Types and Capabilities defined in TS 26.143 for generation

| Media Type in the present TS | Capabilities defined in this specification | Clause | Media Type signalling example |
| --- | --- | --- | --- |
| Multipart MMBPs and Container Formats | 26143_CONTAINER_RFC2046_SINGLE_GEN 26143_CONTAINER_RFC2046_MIXED_GEN 26143_CONTAINER_RFC2046_ALTERNATIVE_GEN 26143_CONTAINER_RFC2046_PARALLEL_GEN 26143_CONTAINER_MP4_3GP9_GEN 26143_CONTAINER_MP4_MP42_GEN 26143_CONTAINER_AMR-WB_GEN 26143_CONTAINER_AMR_GEN | 5.2.2 | Media type of subtype multipart/mixed multipart/alternative multipart/parallel video/mp4, profile="3gp9" video/mp4, profile="mp42" audio/AMR-WB audio/AMR |
| Text | 26143_TEXT_ENC_PLAIN | 5.3.1 | text/plain |
| Speech | 26143_AUDIO_ENC_IVAS 26143_AUDIO_ENC_EVS 26143_AUDIO_ENC_AMR-WB 26143_AUDIO_ENC_AMR 26143_AUDIO_ENC_AMR-WB 26143_AUDIO_ENC_AMR | 5.5.1 | audio/mp4 audio/AMR-WB audio/AMR |
| Audio | 26143_AUDIO_ENC_IVAS 26143_AUDIO_ENC_XHE-AAC 26143_AUDIO_ENC_EAAC+ 26143_AUDIO_ENC_MP3 | 5.5.1 | audio/mp4 audio/mpeg |
| Image | 26143_IMG_ENC_JPEG 26143_IMG_ENC_HEIC | 5.4.1 | image/jpeg image/heic, profile="heic,MiHB" itemTypes="hvc1.2.4.L153.B0" |
| Video | 26143_VIDEO_ENC_AVC-HD 26143_VIDEO_ENC_AVC-FullHD 26143_VIDEO_ENC_HEVC-HD 26143_VIDEO_ENC_HEVC-FullHD 26143_VIDEO_ENC_HEVC-UHD | 5.6.2 | video/mp4, profile="3gp9" codecs="avc1.640028" video/mp4, profile="3gp9" codecs="avc1.640029" video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" video/mp4, profile="3gp9" codecs="hvc1.1.2.L123.B0" video/mp4, profile="3gp9" codecs="hvc1.1.2.L153.B0" |
| Text | 26143_TT_ENC_3GPP | 5.7.2 | text/mp4, profile="3gp9" codecs="tx3g" |

# * * * Next Change * * *

### 5.2.1	Player and Decoding capabilities

The capability 26143_CONTAINER_RFC2046_SINGLE is defined as the capability of processing a body part of with a top-level media type as defined in IETF RFC 2046 [12] with one of the following top-level media types: text, audio, image, video, model, multipart, and application.

The capability 26143_CONTAINER_RFC2046_MIXED is defined as the capability of processing a body part of subtype multipart/mixed as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for mixed MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_MIXED shall be signalled with multipart/mixed as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2046_ALTERNATIVE is defined as the capability of processing a body part of subtype multipart/alternative as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for alternative MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_MIXED shall be signalled with multipart/alternative as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2046_PARALLEL is defined as the capability of processing a body part of subtype multipart/parallel as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_PARALLEL shall be signalled with multipart/parallel as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2387_RELATED is defined as the capability of processing a body part of subtype multipart/related as defined in IETF RFC 2387 [29] further restricted by the processing defined in clause 4.4 for related MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2387_RELATED shall be signalled with multipart/parallel as defined in IETF RFC 2387 [29] with the root MMBP either signalled with the start parameter, or if not present, the root MMBP being the first body part within the Multipart/Related body and the type of the root MMBP signalled with the type parameter.

The capability 26143_CONTAINER_MP4_3GP9 is defined as the capability of processing a body part conforming to a 3GP file Rel-9 basic profile as defined in TS 26.244 [26] identified by the brand '3gp9' and further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_MP4_3GP9 shall be signalled using a media type as defined IETF RFC 6381 [30] using for example video/mp4, profile="3gp9" or an equivalently compatible media type and shall use the codecs parameter to further provide information about the contained MMBPs.

The capability 26143_CONTAINER_MP4 is defined as the capability of processing a body part conforming to a file identified by the brand 'mp42' as defined in ISO/IEC 14496-14 [37] and further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_MP4 shall be signalled using a media type as defined IETF RFC 6381 [30] using for example video/mp4 or an equivalently compatible media type and shall use the codecs parameter to further provide information about the contained MMBPs.

The capability 26143_CONTAINER_AMR-WB is defined as the capability of processing a body part conforming to an AMR and AMR-WB Storage Format as specified in [xx] and further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_AMR-WB shall be signalled using a media type as defined IETF RFC 4867 [xx] using for example audio/AMR-WB or an equivalently compatible media type and shall use the codecs parameter to further provide information about the contained MMBPs.

The capability 26143_CONTAINER_AMR is defined as the capability of processing a body part conforming to an AMR and AMR-WB Storage Format as specified in [xx] and further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_AMR shall be signalled using a media type as defined IETF RFC 4867 [xx] using for example audio/AMR or an equivalently compatible media type and shall use the codecs parameter to further provide information about the contained MMBPs.

NOTE: 	This specification does not define mechanisms for referencing external content. This aspect is for further study.

# * * * Next Change * * *

### 5.5.1	Player and Decoding capabilities

The capability 26143_AUDIO_IVAS is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the IVAS decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.3.5.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'sivs' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_IVAS shall be signalled with audio/mp4, profiles="3gp9" codecs="sivs" or an equivalently compatible media type.

The capability 26143_AUDIO_EVS is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the EVS decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.4.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'sevs' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profiles="3gp9" codecs="sevs" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR-WB is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR-WB decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.3.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'sawb' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_AMR-WB shall be signalled with audio/mp4, profiles="3gp9" codecs="sawb" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR-WB_FILE is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR-WB decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.3.2 of TS 26.117 [5],

-	is contained in an AMR and AMR-WB Storage Format that conforms to the 26143_CONTAINER_AMR-WB capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_AMR-WB_FILE shall be signalled with audio/AMR-WB or an equivalently compatible media type.

The capability 26143_AUDIO_AMR is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'samr' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profiles="3gp9" codecs="samr" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR_FILE is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.3.2 of TS 26.117 [5],

-	is contained in an AMR and AMR-WB Storage Format that conforms to the 26143_CONTAINER_AMR capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_AMR_FILE shall be signalled with audio/AMR or an equivalently compatible media type.

The capability 26143_AUDIO_XHE-AAC is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the xHE-AAC stereo decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.4.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'mp4a.40.42',

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_XHE-AAC shall be signalled with audio/mp4, profiles="3gp9" codecs="mp4a.40.42" or an equivalently compatible media type.

The capability 26143_AUDIO_EAAC+ is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the eAAC+ decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.3.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'mp4a.40.29',

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EAAC+ shall be signalled with audio/mp4, profiles="3gp9" codecs="mp4a.40.29" or an equivalently compatible media type.

The capability 26143_AUDIO_MP3 is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the MP3 decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.3.x.2 of TS 26.117 [5],

In the context of this specification, the media type for files with this capability 26143_AUDIO_MP3 shall be signalled with audio/mpeg or an equivalently compatible media type.

# * * * Next Change * * *

### 5.5.2	MMBP Content Generator capabilities

The capability 26143_AUDIO_ENC_IVAS for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_IVAS,

-	the IVAS encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.3.5.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'sivs' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profiles="3gp9" codecs="sivs" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_EVS for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_EVS,

-	the EVS encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.4.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'sevs' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profiles="3gp9" codecs="sevs" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_AMR-WB for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_AMR-WB,

-	the AMR-WB encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.3.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'sawb' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profiles="3gp9" codecs="sawb" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_AMR-WB_FILE for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_AMR-WB_FILE,

-	the AMR-WB encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.3.3 of TS 26.117 [5],

-	the generation of an AMR and AMR-WB Storage Format that conforms to the 26143_CONTAINER_AMR-WB capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/AMR-WB or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_AMR for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_AMR,

-	the AMR encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'samr' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profiles="3gp9" codecs="samr" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_AMR_FILE for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_AMR_FILE,

-	the AMR encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.3.3 of TS 26.117 [5],

-	the generation of an AMR and AMR-WB Storage Format that conforms to the 26143_CONTAINER_AMR capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/AMR or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_XHE-AAC for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_XHE-AAC,

-	the xHE-AAC stereo encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.4.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'mp4a.40.42' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profiles="3gp9" codecs="mp4a.40.42" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_EAAC+ for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_EAAC+,

-	the eAAC+ encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.3.2.3 of TS 26.117 [5],

-	the capability to generate an ISO BMFF track that conforms with the requirements of the sample entry 'mp4a.40.29' as defined in TS 26.244 [26].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using audio/mp4, profiles="3gp9" codecs="mp4a.40.29" or an equivalently compatible media type.

The capability 26143_AUDIO_ENC_MP3 for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from an audio signal in real-time, such that the file can be played back by a player with the capability 26143_AUDIO_ENC_MP3,

-	the MP3 encoding capabilities as defined in clause 5.3 of TS 26.117 [5] and the sender requirements in clause 6.2.x.3 of TS 26.117 [5],

-	the provisioning of media type signalling with the generated file using audio/mpeg or an equivalently compatible media type.

# * * * Next Change * * *

### 6.2.2	Container Format

The following capabilities for the container format as defined in clause 5.2.1 shall be supported:

-	26143_CONTAINER_RFC2046_SINGLE

-	26143_CONTAINER_RFC2046_MIXED

-	26143_CONTAINER_RFC2046_ALTERNATIVE

-	26143_CONTAINER_RFC2387_RELATED

-	26143_CONTAINER_MP4_3GP9

-	26143_CONTAINER_MP4_MP42

-	26143_CONTAINER_AMR-WB

-	26143_CONTAINER_AMR

The following capabilities for the container format as defined in clause 5.2 should be supported:

-	26143_CONTAINER_RFC2046_PARALLEL

In addition, multipart/parallel shall not be present in an MMBP on the same level if a 3GP file Rel-9 basic profile as defined in TS 26.244 [26] identified by the brand '3gp9'is present and contains more than on track.

### 6.2.3	Media Types

The capability 26143_TEXT_PLAIN as defined in clause 5.3 shall be supported.

If still images are supported,

-	the 26143_IMG_JPEG capability as defined in clause 5.4.1 shall be supported,

-	the 26143_IMG_HEIC capability as defined in clause 5.4.1 should be supported.

If bitmap graphics are supported,

-	the 26143_IMG_GIF capability as defined in clause 5.4.1 should be supported.

-	the 26143_IMG_PNG capability as defined in clause 5.4.1 should be supported.

If the reception of audio or speech is supported, then the following applies:

-	the 26143_AUDIO_IVAS capability as defined in clause 5.5.1 should be supported.

-	the 26143_AUDIO_EVS capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_AMR-WB capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_AMR-WB_FILE capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_XHE-AAC capability as defined in clause 5.5.1 should be supported.

-	the 26143_AUDIO_AMR capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_AMR_FILE capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_EAAC+ capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_MP3 capability as defined in clause 5.5.1 shall be supported.

If the reception of video is supported, then the following applies:

-	the 26143_VIDEO_AVC-HD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-HD capability as defined in clause 5.6.1 should be supported.

If the reception of HD-HDR video is supported, then the following applies:

-	the 26143_VIDEO_AVC-FullHD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-FullHD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-UHD capability as defined in clause 5.6.1 should be supported.

If timed text is supported,

-	the 26143_TT_3GPP capability as defined in clause 5.7.1 shall be supported.

-	the 26143_TT_IMSC11 capability as defined in clause 5.7.1 should be supported.

If a processor for media type 'model' as defined in RFC2077 [33] is supported (i.e. a processor for 3D scenes and objects), then a processor for the media subtype 'model/gltf' should be supported. If a processor for the media subtype 'model/gltf' is supported,

-	the 26143_SCENE_GLTF20 capability and the 26143_SCENE_GLTF20_GLB capability as defined in clause 5.8 shall be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2.

-	and if the device is a device type as defined in TS 26.119 [34], clause 10, the 26143_SCENE_GLTF20_AR and the 26143_SCENE_GLTF20_GLB_AR capability as defined in clause 5.8 shall be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2.

If a processor for the media type text/html is supported, the 26143_PRESENTATION_HTML5 capability as defined in clause 5.9 should be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2. The media formats shall be restricted to the capabilities defined in this clause.

# * * * Next Change * * *

### 6.3.2	Container Format

The following capabilities for the container format as defined in clause 5.2.2 shall be supported:

-	26143_CONTAINER_RFC2046_SINGLE_GEN

-	26143_CONTAINER_RFC2046_MIXED_GEN

-	26143_CONTAINER_AMR-WB_GEN

-	26143_CONTAINER_AMR_GEN

In addition, at least one of the following two capabilities for the container format as defined in clause 5.2.2 shall be supported:

-	26143_CONTAINER_MP4_3GP9_GEN

-	26143_CONTAINER_MP4_MP42_GEN

The following capabilities for the container format as defined in clause 5.2 should be supported:

-	26143_CONTAINER_RFC2046_PARALLEL_GEN

-	26143_CONTAINER_RFC2046_ALTERNATIVE_GEN

### 6.3.3	Media Types

If the transmission of images is supported, then the following applies:

-	the 26143_IMAGE_ENC_JPEG capability as defined in clause 5.4.2 shall be supported.

-	the 26143_IMAGE_ENC_HEIC capability as defined in clause 5.4.2 should be supported.

If the transmission of speech is supported, then the following applies:

-	the 26143_AUDIO_ENC_IVAS capability as defined in clause 5.5.2 should be supported.

-	the 26143_AUDIO_ENC_EVS capability as defined in clause 5.5.2 shall be supported.

-	the 26143_AUDIO_ENC_AMR-WB capability as defined in clause 5.5.2 should be supported.

-	the 26143_AUDIO_ENC_AMR-WB_FILE capability as defined in clause 5.5.2 should be supported.

-	the 26143_AUDIO_ENC_AMR capability as defined in clause 5.5.2 may be supported.

-	the 26143_AUDIO_ENC_AMR_FILE capability as defined in clause 5.5.2 may be supported.

If the transmission of audio is supported, then the following applies:

-	the 26143_AUDIO_ENC_IVAS capability as defined in clause 5.5.2 should be supported.

-	the 26143_AUDIO_ENC_EAAC+ capability as defined in clause 5.5.2 shall be supported.

-	the 26143_AUDIO_ENC_XHE-AAC capability as defined in clause 5.5.2 should be supported.

-	the 26143_AUDIO_ENC_MP3 capability as defined in clause 5.5.2 shall be supported.

If the transmission of video is supported, then the following applies:

-	the 26143_VIDEO_ENC_HEVC-FullHD capability as defined in clause 5.6.2 shall be supported.

If the transmission of timed text is supported, then the following applies:

-	the 26143_TT_ENC_3GPP capability as defined in clause 5.7.2 shall be supported.

# * * * End of Change * * *