3GPP TSG-SA WG4 Meeting #134	                                           S4-251820

Dallas, US, 17 – 21 November 2025

Source:	Samsung

Title:	Way forward on Automatic Resumption in IMS Data Channel

Agenda item:	10.3

Document for:	Agreement

1. Introduction

At the SA4#132 meeting in May 2025, GSMA TSG IMSDCAS sent LS [1] to SA2 with copy to SA4 and CT1, requesting the feedback about the new use case called “Automatic Resumption” and its technical feasibility. When IMS DC application session is terminated by a network or an user, it enables application continuity by resuming at the proper point. It was agreed by most of GSMA IMSDCAS members and the details are to be described in the next version of GSMA PRD NG.123. Thus they are seeking for the technical solution for this use case.

However, SA2 replied that it was not considered in any of IMS data channel related works in Rel-18 and Rel-19 and they have no plan to study in Rel-20. Further, they asked SA4 to review and to consider leading the technical discussion under our scope [2]. This reply LS was received in SA4#133-e in July and it was postponed. Therefore, we need to build up our answer to SA2 as well as GSMA.

This input provides the way forwards how to handle this issue.

2. Potential Solutions based on technical discussion inside GSMA

To support the automatic resumption, GSMA has identified that it is required to have a persistent identifier that serves as the unique key into the database that stores the application context/state. In their original LS [1], they provided two alternative solutions to communicate the unique ID;

Figure 1: Potential solutions for peer-to-peer session in [1]

When the DC App session is dropped off, then the application will check for the availability of previously stored application state which matches to unique-id in local database. If available, DCMTSI client in terminal will retrieve the previous status and initiate the application to resume.

Observation 1) All the technical feasibility should be carefully examined. (e.g., generation or management of unique ID)

Observation 2) Impact of above solutions should be identified for technical feasibility, especially considering IANA dependency as well as existing 26.114.

In addition to peer-to-peer case above, GSMA had discussed the call transfer case by possibly extending SIP header. If the feature is supported, the UE sends SIP message (REFER request) to the new UE with a new subtype feature tag, indicating the availability of app state. Then the new UE downloads the application state and the call is transferred.

Observation 3) It should be studied whether the SIP/SDP extension would be the best solution for automatic resumption. In [1], GSMA also asked the feasibility of other method (i.e., using session cookies, or exchanging the state data as part of application or bootstrap data channel protocol)

3. Proposed way forward

It is proposed to agree the following way forward.

- Confirm whether SA4 would take any potential study on automatic resumption and send a reply LS to SA2 and GSMA.

NOTE) In case of no further work in SA4, better to suggest any other alternative way to GSMA.

- Confirm the technical feasibility of SIP/SDP extension provided by GSMA and investigate any other alternatives, if any

  - If the SIP/SDP extension turns out to be favorable, then make a draft the formal CR as well as reply LS

  - If more technical discussion is needed (or no consensus reached), draft a new WID to study the solutions. Note that work load in RTC SubWG should be carefully considered in this case.

## References

[1] Tdoc S4-250755, " LS to SA2 About Requirements Concerning Automatic Resume Where You have Left Off "

[2] Tdoc S4-251419, " Reply to LS on Automatic Resumption in IMS data channel"