Source:	Editor

Title:	DaCAS-2: Test methodologies and requirements v0.4

## Document for:     AGREEMENT

Agenda item:	7.6 - DaCAS

Revision history:

| Date | Meeting | Subject/Comment | Old | New |
| --- | --- | --- | --- | --- |
| 2025-05-22 | SA4#132 | Initial version including S4aA250021, S4-250967 | N/A | 0.1 |
| 2025-06-16 | Audio SWG AH Meetings Post SA4 #132 Telco on DaCAS | Addition of compensation method from S4Aa250054 and single source recording scenario from S4Aa250055, editorial updates | 0.1 | 0.2 |
| 2025-07-22 | SA4#133-e | Addition of the agreed content from S4-251375, S4-251466, S4-251472. | 0.2 | 0.3 |
| 2025-11-17 | Audio SWG AH Meetings Post SA4 #133-e Telco on DaCAS | Addition of agreed content from S4aA250107 | 0.3 | 0.4 |

# Scope

As outlined in the scope of the DaCAS WID, several points need to be considered:

- The test methodologies and requirements are based on relevant sending side terminal audio quality test methods defined in TS 26.260.

- The evaluation may be conducted on the databases processed by the example solution.

# Introduction

The Pdoc is structured according to the relative objectives that are in scope of DaCAS:

- Definition of minimum performance requirement/objective criteria for raw microphone signal performance and characteristics including, e.g., expected compensation of the microphone signals: e.g., directional response, SNR, frequency response.

- Evaluation of available immersive audio capture example solutions based on relevant sending side terminal audio quality test methods defined in TS 26.260.

- Verification and potential revision of the minimum performance requirement/objective criteria for raw microphone signal performance and characteristics based on how the example solutions perform under the audio quality test methods defined in TS 26.260.

- Potential alignment with TS 26.260 and 26.261.

- In addition, potential test methodologies for subjective tests and requirements are collected

# Requirements for raw and compensated microphone signals

## Raw microphone signals

### Requirements

[

Editor’s note: Decision needed whether to have normative or informative requirements/recommendations.

[Raw microphone input for DaCAS example solutions shall comply with the requirements specified in Table 1.]

Table 1 Raw microphone signal requirements

| Feature | Requirement | Recommendation |
| --- | --- | --- |
| Raw frequency response (excluding resonances) | TBD | Minimum captured frequency (-3dB point) should be below 100 Hz Frequency response above the minimum frequency and below the resonances should be as flat as possible. |
| Resonances | TBD | Should be above 8 kHz. |
| SNR | TBD | Should be at least 60 dB |
| Sensitivity | TBD | [Level should be below -25 dBFS (±1 dB) with 1 kHz sine signal @ 94 dB SPL] |
| Directivity | TBD | Recommended to have omnidirectional characteristics. Other directivity patterns are not generally excluded. Microphone directivity characteristics shall be documented clearly. |
| ADC bit depth | TBD | The ADC bit depth should be 24 bits. |

NOTE: It is expected that all the raw integrated microphones comply with the above requirements. In addition, it is favorable to have as similar characteristics as possible for all raw integrated microphones.

]

## Compensation on microphone signals

Any raw microphone compensation, including the approaches outlined in the section 3.2.2, are considered as optional in the development of example solutions. Example solution developers may selectively adopt the solutions outlined in the section 3.2.2 or any other solution based on specific application requirements. If other solutions are adopted, these shall be documented.

### Requirements

[

Editor’s note: Decision needed whether to have normative or informative requirements/recommendations.

Compensated microphone input for DaCAS example solutions shall comply with the requirements specified in Table 2.

Table 2 Compensated microphone signal requirements

| Feature | Requirement | Recommendation |
| --- | --- | --- |
| Compensated frequency response | When compensation processing is applied to the same signal as used for designing the compensation filters: - Compensated frequency response shall be within mask defined in Table 3 - Differences between compensated frequency responses shall be within mask defined in Table 4 | Compensated frequency response should be considered in the context of isotropic equalization target. |
| Phase properties | TBD | Compensation processing should compensate sound source direction independent phase differences between integrated microphone responses. |

Table 3 Compensated microphone sensitivity/frequency mask

|  | Required |  | Recommended |  |
| --- | --- | --- | --- | --- |
| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) |
| 100 | 4 |  | 2 |  |
| 200 | 4 | -4 | 2 | -2 |
| 5000 | 4 | -4 | 2 | -2 |
| 12500 | 4 | -6 | 2 | -3 |
| 16000 | 4 |  | 2 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |  |  |

Figure 1 Compensated microphone response masks

Table 4 Compensated microphone sensitivity/frequency difference mask

|  | Required |  | Recommended |  |
| --- | --- | --- | --- | --- |
| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) |
| 100 | 2 |  | 1 | -1 |
| 200 | 2 | -2 | 1 | -1 |
| 5000 | 2 | -2 | 1 | -1 |
| 12500 | 2 | -4 | 1 | -2 |
| 16000 | 2 |  | 1 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |  |  |

Figure 2 Compensated microphone response difference masks

]

### Compensation processing

[

#### Diffuse field based raw microphone compensation method

Following compensation method proposal is integrated from the input document [4].

##### Derivation of compensation filters

###### Recording environment and requirements

# The proposed method requires a quiet room with low reverberation, e.g. a listening room, and a surround loudspeaker layout of ideally 7.1.4, (or a known loudspeaker layout with locations towards the front, rear and sides, and at least some height differentiation). The signal to be recorded is a diffuse series of uncorrelated pink noise signals played out of the 7.0.4 loudspeakers, omitting the subwoofer due to differences in frequency response.

# The target device is placed at the center of the room, in a landscape orientation, and positioned such that the camera is pointing towards the center speaker, or front of the room. The source has successfully applied the approach with 10 seconds of noise recordings, along with 15 seconds of silence for estimation of the noise floor.

###### Gain matching between microphones

# By observing respective average frequency responses of the device microphone recorded signals, a broadband relative gain can be derived per channel, G(ch), to bring any outlier microphones in line with the rest. In principle this step may not be required, if the device microphones are closely matched.

###### Equalization estimation

# The device descriptions provided in [1] are sufficient to model theoretical impulse responses (including equalization and delay) from a particular direction to the microphone position on the device surface, without accounting for porting effects. This is sufficient, as including the port geometries increases the complexity of the required models, and measuring the port geometries would be quite difficult. Any small errors in measuring the port geometries could also have profound effects on the modelled output.

# Using the modelled impulse responses from the known loudspeaker directions to the device microphone positions, as derived from a model of the specified device geometry, a set of simulated device microphone signals may be created. These simulate the combined effect of the incoming diffuse noise field at the microphone positions on the surface of the device.

# By comparing the simulated device signals and real device recordings, an estimate of an equalization can be calculated to account for the port resonances. Given the room requirements stated above, the source assumes that the effects of the room are negligible compared to the effects of the device geometry. It is also assumed that the MEMS mics have a flat response.

# Details:

- Calculate a smoothed average device signal DFT log magnitude, dev(ch,f) and smoothed average simulated reference signal DFT log magnitude, sim(ch,f). Based on experience, the specific smoothing method is less important.

- Subtract the smoothed average device DFT from the smoothed average simulated reference DFT in the log magnitude domain, to obtain a set of equalization gains that model the port resonances only.

# port(ch,f) = sim(ch,f) – dev(ch,f)

- Convert the port spectrum to a banded spectrum, port(ch,b), e.g. ERB or similar, to reduce the number of stored parameters. The banded responses may be normalised to abide by some sensible rule, e.g. response of mic 1 is 0dB at 1kHz.

- Given the assumption of flat MEMS mic responses, the resonances of each microphone port can then be observed and modelled by hand from the resulting banded responses, port(ch,b), using simple filters. This creates a new parametric banded spectrum that accounts only for the observed resonances, EQ(ch,b). The final equalization coefficients are thus derived only from the observed microphone port resonances.

###### Noise floor estimation

# The silence recording from the target device is used to generate a banded per channel noise floor estimate for the target device, NF(ch,b).

##### Compensation processing

# The microphone compensation processing can be performed in the DFT domain, using the following steps:

- Convert the input DFT spectrum to the banded spectrum, in log magnitude domain, spec(ch,b).

- Calculate gains that compensate for the noise floor

# g(ch,b) = (spec(ch,b) – NF(ch,b))/(spec(ch,b) + eps).

- Expand the noise floor gains and the EQ gains to DFT bins, g(ch,f) and EQ(ch,f).

- Apply a combined compensation gain G(ch) * EQ(ch,f) * g(ch,f) to the input spectrum bins, to perform gain matching between microphones, equalization of the porting effects and noise floor.

Editor’s note: Clarification needed whether the applied gain is in linear or log scale.

Editor’s note: Status of this method is to be clarified.

]

[

#### IMPro method

Following compensation method proposal is integrated from the input document [6]. Further information about the IMPro measurement can be found from [7].

##### IMPro calculation

Integrated microphone pressure frequency response Hm is calculated by dividing the measured integrated microphone output signal's response Um with probe signal output signal response Up measured at the reference point at the sound port inlet according to following equation

Hm=HprobeUmUp,       [units/Pa]                                                          (1)

where is Hprobe is the calibration frequency response of the probe microphone [7]. Note the notation is slightly modified from [7] to clarify time domain and frequency domain representation.

##### Compensation filter target response

Let

- M: number of device microphones

- upit: pressure at ith microphone sound inlet where i=1,2,…,M

- hmit: impulse response of the integrated microphone response using IMPro

- fit: equalization filter for microphone signal compensation

- umit: raw microphone signal output for ith microphone

- yit: Compensated at ith microphone output

Each microphone captures a raw microphone signal

umit=upit*hmit

Compensated microphone output signal can be defined as follows

yi​t=umi​t*fi​t=upit*hmit*fi​t

The target equalization filter fit response compensates for integrated microphone response within the target frequency response mask such it corresponds to delayed pressure signal at the sound inlet without loss of information such that:

hmit*fit≈δt-τ, ∀i∈1,…,M

In frequency domain the compensation filter target frequency response can be defined as:

Fiω=e-jωτ​Hmi​ω,∀i                                                                                 (2)

within the range of target frequency response mask ωlow≤ ω≤ ωhigh

Microphone component manufacture data sheets specify commonly free field frequency response characteristics including sensitivity tolerances typically normalized at 1kHz. If target UE uses the same component type for all microphones, the transducer differences are quite well understood based on manufacturer component specification. Use of different types of microphones open additional questions on how different components operate under different environmental conditions. The designer should consider all different factors and try to minimize variations between different microphones.

##### Proposed integrated microphone calibration method

Integrated microphone array calibration is performed using following steps:

- Perform IMPro measurements for all device microphones [1]

  - Prepare UE software enabling raw microphone recording from all DUT microphones

  - Setup loudspeaker and device setup for measurement having at about 0.5-2m source distance

  - Prepare the measurement signal stimulus (sine sweep) and check the loudspeaker playback levels approximately 30dB above background noise level while avoiding signal clipping

  - Calibrate probe microphone(s) using a probe calibrator or calibrated measurement microphone

  - Perform IMPro measurement by measuring pressure signal at sound inlet using the probe microphone while DUT is recording all microphone signals

  - Time-align measured signals audio signals

  - Calculate integrated microphone pressure frequency response Hmi for each microphone i=1,2,…,M according to equation (1)

- Calculate compensation filter for the integrated microphone output

  - Measure IMPro frequency responses Hmi from reference sample device or average measurement results from sufficiently many devices for statistical relevance

  - Design linear equalization filters according to (2) such that responses align within

    - equalization filter frequency response mask and

    - equalization filter frequency response difference mask

- Implement equalization filter in UE software providing compensated microphone signal output

- Process recorded raw microphone signals umit with equalization filters fit to compensated microphone output signals yi​t

- Verify that compensated microphone output signal yi​t satisfies frequency response mask requirements

  - Check that all microphone signals are equalized to satisfy equalization filter frequency response mask ensuring target timbre for compensated microphone signals

  - Check that all microphone signals are equalized to satisfy equalization filter frequency response difference mask to ensure that spatial algorithms can assume that compensated microphone responses do not distort magnitude or phase of the spatial audio information present in acoustic signal at sound inlet ports

  - Check that output signal yi​t is effectively a delayed version of pressure signal at sound inlet with constant delay δt-τ, ∀i

]

# Test methodologies

Following content is based on the input documents [1], [2] and [5].

## Objective Test Methodologies for Immersive Capture Audio Systems

### Test conditions

### Recording setups and scenarios for tests

#### Single source scenario

To obtain suitable recordings for evaluating single source capture, following recording scenario is used. The same recordings can be used for assessing the loudness, frequency response, and directional information of the target device and example solution combinations.

Table 1 Recording scenario for single sound source evaluation scenario

| No | X (Single source objective test) |
| --- | --- |
| Sound source | High quality loudspeaker Recommended to use loudspeaker that complies with clause 4.0.2 in 3GPP TS 26.260 (including provisional values). |
| Source signal | British English single talk sequence according to the ITU-T P.501 |
| Source signal characteristics | Language: English Gender: Male, Female Range: 20Hz-20kHz Length: 35.4 s Level: -27 dB RMS Sample Rate: 48 kHz Bit depth: 16 bit |
| Sound source calibration | Loudness: Source signal playback calibrated to 75 dB SPL according to the clause 5.5.1 of 3GPP TS 26.260. Frequency response: The spectrum of the acoustic signal produced by the loudspeaker shall be equalized with a measurement microphone positioned on the main loudspeaker axis at 1 m from the loudspeaker membrane. The achieved equalized spectrum in 1/3rd octave bands, when measured in the test environment, shall be within ±1 dB from 100 Hz to 200 Hz and shall be within ±0.5 dB from 200 Hz to 20 kHz. |
| Acoustic environment | Anechoic chamber OR acoustically well treated room with short reverberation (room compliant e.g., with Clause 6.1. in ETSI TS 103 224, or with clause 8 in ITU-T BS.1116). Recommended environment is an anechoic chamber according to the clause 4.0.3 of 3GPP TS 26.260. |
| Detailed Positioning | Source distance: - For Hand-held and Headset devices 1 – 1.5 m - For table-mounted devices/table orientation of the device, source placement is according to the clause 5.4.2.5 for Spatial capture arrangement in 3GPP TS 26.260 Source direction: Loudspeaker diaphragm towards the UE Source angles: Azimuth angles: 0°, ±30°, ±60°, ±90° - For Hand-held and Headset devices elevation angle 0° - For table-mounted devices elevation angle 26.6° (see clause 5.4.2.5 in 3GPP TS 26.260) |
| Reference recording |  |
| Description/additional info | More information can be found from 3GPP TS 26.260 Applied sound source, acoustic environment, and mounting of the device shall be clearly documented. |

### Recording databases for objective evaluation

### Test methods

[

#### Delay

For example immersive audio solution, the delay of interest is the algorithmic delay, i.e., delay from inserting the input signal to example solution to obtaining the output signal (signal in IVAS input format). By assessing only the delay caused by the example solution, impact of testing inaccuracies and acoustic path can be mitigated. However, it is recognized, that there are many dependencies on the obtained delay of the example solution, e.g., platform where the example solution is run.

#### Loudness

For assessing the loudness of example solution/target device -combination in send direction, following procedure shall be used:

- The recordings according to the recording scenario [X (Single source objective test)]] defined in clause 4.1.2.1 for sound source direction ( = 0,  = 0) shall be made.

- The recorded audio signal is processed with the example solution under test.

- The loudness analysis is done according to clause 5.6.2 of TS 26.260

Editor’s note (pending discussion under ATIAS):

- Additionally ITU-R BS.1770/P.700 via potential Binaural rendering could be considered.

#### Frequency response

For assessing the frequency response of example solution/target device combination in send direction, following procedure shall be used:

- The recording according to the recording scenario [X (Single source objective test)]] defined in clause 4.1.2.1 for sound source direction ( = 0,  = 0) shall be made.

- The recorded audio is processed with the example solution under test.

- The frequency response shall be analyzed according to clause 5.6.3 in 3GPP TS 26.260.

#### Directional information

For assessing the directional information of example solutions, test methodology based on clause 5.6.4 of TS 26.260 [3] is used for Stereo, SBA and MASA formats. Assessment is done directly to the example solution output, excluding any assumptions on transmission.

Assessing a directional information of example solution/target device -combination in send direction, following procedure shall be used:

- The recordings according to the recording scenario [X (Single source objective test)] defined in clause 4.1.2.1 for all defined sound source directions shall be made.

- The recorded audio signals are processed with the example solution under test.

- The directional measurement and applied metric shall be computed as defined in clause 5.6.4 of 3GPP TS 26.260.

Editor’s note:

- For other formats than Stereo, SBA and MASA, intermediate rendering to one of the supported format via IVAS reference renderer could be considered.

]

## Subjective Test Methodologies for Immersive Capture Audio Systems

### Test conditions

### Recording setups and scenarios for test

### Recording database for tests

### Test methods

# Requirements

## Requirements for objective test methodologies

#### Delay

#### Loudness

#### Frequency response

#### Directional information

## Requirements for subject test methodologies

#### …

## References

- S4aA250021: “On objective performance evaluation”, Nokia

- S4-250957: “On performance evaluation”, Nokia

- 3GPP TS 26.260: “Objective test methodologies for the evaluation of immersive audio systems”, Release 18

- S4aA250054: “On raw microphone signal compensation”, Dolby Laboratories, Inc.

- S4aA250055: “Single source scenario for performance evaluation”, Nokia

- S4-251466: “Compensation of integrated microphone responses”, Nokia

- Cozens J., Hämäläinen M, Pekkarinen M, IMPro – Method for Integrated Microphone Pressure Frequency Response Measurement Using a Probe Microphone, AES 158th Convention, Warsaw, Poland 2025 May 22–24. https://aes2.org/publications/elibrary-page/?id=22907

Arvi Lintervo, Nokia