3GPP TSG-SA WG4 Meeting #134	S4-251901

# Dallas, Texas, USA, 17 – 21 November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR | 11 | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consistent use of terminology: base avatar model, avatar representation, and related modifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Canada |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED, TEI |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Use of inconsistent terminology. The terms “base avatar model” and “avatar representation” are defined in section 3 of the specification. However “base avatar” is used 69 times in the specification and refers either to the base avatar model or to an avatar representation depending on the clauses. The avatar representation creation is also missing in the avatar negotiation call flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change Base Avatar to Base Avatar Model or to Avatar Representation as needed and ensure consistency in all clauses except for clauses in Annex B which are addressed in CR S4-251902 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is unclear, and could lead to non-interoperable implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.3, 5.6, 6.4.2, 7.3, 7.4, 8.4, 9.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Animation data: Skeletal, blend shape set, and other animation-related information.

Animation stream: Timed animation data sequence used to animate the avatar representation.

AR data: Collection of information to be exchanged among participants in a call with AR experience. It includes AR media and AR metadata.

AR media: Media (e.g., audio, video, text or image) that will be rendered by the AR-MTSI client as an overlay over the user’s real perception. This includes traditional 2D media (e.g., a 2D audio stream rendered to be perceived by the user to originate from their left side) and 3D media (e.g., spatial audio and volumetric video).

AR metadata: Data that provides information on AR media and its rendering. This includes pose, spatial descriptions and scene descriptions.

AR-MTSI client: DCMTSI client supporting AR capabilities as defined by this specification.

AR MF: AR-MTSI client implemented by functionality included in the MF.

AR-MTSI client in terminal: An AR-MTSI client that is implemented in a terminal or UE. The term "AR-MTSI client in terminal" is used in this document when entities such as AR MF/MRF is excluded.

Asset: Independently accessible component of an avatar.

Avatar: Digital representation of a user.

Base avatar model: Personalized and animatable model of the user.

Avatar representation: Version of a base avatar model using a subset of assets,  selected for a particular avatar call.

Avatar call: IMS call that uses an avatar representation for at least one participant in the call.

Split rendering: The procedure in which a UE offloads some of the media processing related to rendering tasks to a media function as considered for network centric AR IMS session procedures in TS 23.228 [4]

* * * Second Change * * *

## 4.3	End-to-End Reference Architecture

The end-to-end architecture to support AR communication over IMS can be found in TS 23.228 Annex AC [4]. The following Figure 4.3.1 is a simplified version showing the media functions within the scope of this specification.

Figure 4.3.1: Generalized IMS DC Architecture to support AR communication

NOTE 1:	General control-related elements over Gm interface, such as SIP signalling (TS 24.229 [5]), fall outside the scope of this specification, albeit parts of the session setup handling and session control for AR conversational media at Gm reference point, such as the usage of SDP and setup and control of the individual media streams between clients, are defined in this specification.

NOTE 2:	DC Application Repository may be in external DN but can also be in operator domain. The DC Application Repository holds the application(s) that can be used in AR communication sessions and is out of scope of 3GPP.

AR Application Server (AR AS):

-	AR Application Server is responsible for AR service control related to AR communication, including AR session media control and AR media capability negotiation with the UE.

NOTE 3:	AR Application Server is a specific DC Application Server and is out of scope of 3GPP.

NOTE 4:	The UE can download the AR metadata from AR AS through application data channel.

NOTE 5: Whether DC AS used in an avatar call is an AR AS is FFS.

DCSF:

-	The DCSF receives event reports from the IMS AS, and decides whether AR communication service is allowed to be provided during the IMS session. Additionally, the DCSF interacts with the AR AS for DC resource control.

MF:

-	Support AR conversational service by providing transcoding for terminals with limited capabilities. Additionally, the MF may collect spatial and media descriptions from UEs and create scene descriptions for symmetrical AR call experiences.

-	Provide remote rendering for AR-MTSI clients in terminals with limited capabilities based on rendering negotiation. For remote rendering the AR-MTSI client provides AR metadata, e.g., pose data, as defined in clause 6 of this specification.

-	For sessions that use avatar call, it optionally provides avatar animation functionality to the AR-MTSI clients with limited avatar animation capabilities.

IMS AS:

-	The IMS AS receives the media control instructions from the DCSF and accordingly interacts with the UE for connecting the UE's audio/video media termination to the MF [4], and interacts with MF for data channel media resource management for AR media processing.

BAR:

-	Manages and provides access to a user’s base avatar models repository for the purpose of managing the assets (base avatar models, avatar representations and other assets such as accessories)

-  	Manages and provide access to a user’s avatar representation to be used during an IMS session with avatar call.

* * * Third Change * * *

## 5.6	Avatars

### 5.6.1	General

An AR-MTSI client in terminal supporting Avatar call services over IMS data channel shall support the Avatar Representation Format (ARF) as specified in [10] and shall have at least one base avatar model stored in the Base Avatar Repository (BAR) and at least one defined avatar representation in one of the two supported container formats defined in [10].

An AR-MTSI Rx client that supports avatars shall support the ARF container in both ISOBMFF and ZIP formats.

NOTE 1: The ARF specification is still under development. Some details may change.

The base avatar model shall comply with the ARF specification [10]. In addition, the ARF document as specified in clause 6 of [10] shall include the following information:

-	A list of the supported animations, which includes at least one animation type (e.g., face or landmark animation),

- 	At least one asset with at least one level of detail.

All data of relevant assets shall be contained in the ARF container of the base avatar model. All data of relevant assets selected for an avatar representation shall be contained in the ARF container of that avatar representation.

NOTE 1: Evaluation of MPEG-ARF is FFS. Interoperability aspects of ARF have not been fully evaluated and are FFS.

NOTE 2: URNs for specific animation frameworks can be defined in operator-specific profiles or through industry fora.

### 5.6.2	3D Avatar Format

#### 5.6.2.1	General

An AR-MTSI client that supports 3D avatars shall support the ARF base avatar model format as specified in [10] with the requirements in clause 5.6.1.

#### 5.6.2.2	3D Avatar simple profile

The container of a 3D base avatar model shall consist of the following mandatory components:

At least one Skeleton component as defined in [10] that defines the hierarchical joint structure for body animation, with support for at least partial humanoid joint configurations. Inverse Bind Matrices (IBMs) shall be provided for each joint of the skeleton.

At least one Skin component as defined in [10] that references both the skeleton and associated meshes to enable skeletal deformation.

3D mesh geometry data that conforms to the binary glTF (GLB) format version 2.0, with support for triangle-based topology.

Skinning weight data provided as dense tensors in the format specified in Annex E of [10] with a maximum of 4 joint influences per vertex.

Texture data components that conform to still image formats as defined in clause 5.5.

When facial animation is supported, at least one BlendshapeSet component as defined in [10] which should include:

A minimum of 50 blend shapes,

Shape key data as meshes in GLB format, restricted to vertex positions, polygon/face information, normals, and tangents.

Data items of the 3D avatar should signal no compression or protection schemes by default.

NOTE 1: Compression aspects of ARF are for FFS.

NOTE 2: Content protection aspects are for FFS

### 5.6.3	2D Avatar Format

#### 5.6.3.1	General

An AR-MTSI client that supports 2D avatars shall support the ARF base avatar model format as specified in [10] with the requirements specified in clause 5.6.1.

#### 5.6.2.2	2D Avatar simple profile

The container of a 2D avatar base model shall consist of the following mandatory components:

At least one 2D mesh representation that consists of a single planar mesh or quad suitable for texture mapping, conforming to the binary glTF (GLB) format.

At least one static image asset for the base avatar representation that conforms to still image formats as defined in clause 5.5.

At least one LandmarkSet component as defined in [10] for facial animation.

Data items of the 2D avatar base model should signal no compression or protection schemes.

Support for animation using voice-based animation through a pre-trained and fine- tuned model for the user may be supported.

* * * Fourth Change* * *

### 6.4.2	Integration in Scene Description

An AR-MTSI client supporting avatar calls with scene description shall support the integration of MPEG-ARF avatars as defined in Annex B of [10]. When avatar representations are used in a call with scene description, the avatar representation shall be integrated using the MPEG_node_avatar extension as specified in [7] and extended in Annex B of [10].

The scene description containing avatar nodes shall be exchanged over the data channel as described in [2] clause 6.2.10, using a stream with a stream ID in the range 1 to 1000. Each participant's avatar representation shall be associated with their own avatar node in the scene, identified through the node name, which shall be provided as part of the SDP through the "sd-nodes" attribute of the media session of the data channel that carries the scene description, with clear marking of the node as an avatar node as defined in clause 6.4.

The MPEG_node_avatar extension shall include:

The avatar type set to the URN defined in clause 4.2 of ISO/IEC 23090-39

The ARFContainer URL pointing to the avatar representation in the BAR

Animation stream definitions for blendshapes, joints, landmarks, or multiplexed animation data

An MF that supports avatar scene description shall support:

The capability to reference ARF containers for the avatar representations stored in the BAR through the MPEG_node_avatar extension

The capability to reference animation streams that are delivered over application data channels

The capability to synchronize avatar animations with other scene elements and RTP media streams

When scene description is used as the entry point for an avatar-enhanced call, the MF shall apply animation updates received from each participant to their respective avatar nodes, as negotiated by the SDP avatar-nodes attribute. The animation data shall be transmitted through the application data channel established for avatar animation.

* * * Fifth Change* * *

## 7.3	Avatar animation and rendering configuration

### 7.3.1	Avatar capability configuration

The AR-MTSI client in terminal shall indicate its support for avatar calls by including the “webrtc-datachannel” in the “+sip.sub-type” Contact header field.

A new Contact header field parameter, “+sip.3gpp-avatar-support” is used to indicate the level of support for Avatar calls. The possible values for the “3gpp-avatar-support” parameter are:

-	“avatar-capable”: indicates that the terminal is fully capable of receiving, animating and rendering of avatars.

- 	“avatar-assisted”: indicates that the UE has no support for animating or rendering an avatar

NOTE: The SIP register message for avatar calls is FFS. It may be needed for indicating which UEs require avatar assistance and which ones are avatar capable.

### 7.3.2	Network Animation and Rendering

When an AR-MTSI client initiates or receives a call with avatar media, the IMS Application Server (IMS AS) shall evaluate whether network-based avatar animation and rendering is required. The IMS AS forwards avatar-related INVITE requests to the Avatar-capable AR AS for capability assessment and media routing decisions.

The AR AS shall invoke network-based animation and rendering through an MF when:

the receiving MTSI client has not registered the "+sip.3gpp.avatar-support" feature tag,

the receiving MTSI client has registered the "+sip.3gpp.avatar-support" feature tag with “avatar-assisted” value,

the receiving MTSI client's registered capabilities indicate insufficient resources for avatar animation,

the offered animation frameworks are not supported by the receiving MTSI client.

When network animation and rendering is invoked, the sending AR-MTSI client shall use the negotiation processes with the AR AS to determine the avatar animation and rendering configuration. The exchange of the configuration information shall take place using an established application data channel. The application data channel established for animation and rendering negotiation of the avatar representation with SDP offer/answer between the sending AR-MTSI client in terminal and the MF with the sub-protocol “3gpp-avatar-negotiation” and continue to be used for animation and rendering re-negotiation until the end of the avatar call. In this case, the MF shall establish a video stream with the receiving MTSI client using standard video codecs as specified in [2].

When network animation and rendering is invoked, the AR AS shall allocate an MF capable of real-time avatar rendering and configure it with the appropriate rendering parameters based on the receiving UE's video capabilities. The IMS AS shall modify the SDP to route avatar animation data to the MF instead of the receiving UE, effectively inserting the MF into the media path between the sending and receiving MTSI clients.

The MF performing network-based avatar animation and rendering shall fetch the ARF container from the BAR using the reference provided in the SDP and load the avatar representation for rendering. The MF shall receive animation streams through the data channel from the sending AR-MTSI client and apply these animation samples to the avatar representation in real-time. The animated avatar representation shall be rendered as a 2D video stream or stereoscopic 3D video stream based on the receiving UE's capabilities.

For the receiving MTSI client, the avatar data channel media description shall be replaced with a video media description including standard video codec negotiation as specified in [2], and avatar-specific attributes that are not applicable to video streams shall be removed.

The MF should ensure lip-sync between avatar representation animation and associated audio streams.

* * *Sixth Change* * *

## 7.4	SDP Negotiation and Signaling of Avatars

### 7.4.1	General

The SDP is used for establishing the Avatar negotiation application data channel (ADC) between an AR-MTSI client and its remote peer (MF or AR-MTSI client).

### 7.4.2	SDP signalling

#### 7.4.2.1	Establishment of ADC for avatar negotiation

For avatar negotiation over an established IMS application data channel, the data channel sub-protocol shall be identified as “3gpp-avatar-negotiation”.

The data channel shall be ordered and reliable.

#### 7.4.2.2	Negotiation of fallback mechanism

An AR-MTSI client receiving an SDP offer for an avatar call shall:

1.	Examine the avatar capabilities in the offer, and

2.	If avatar rendering is supported, the AR-MTSI client shall:

a.	Fetch the avatar representation using the provided reference from the BAR via the MF

b.	Verify compatibility with at least one offered animation framework

c.	Accept the data channel for animation parameters

3.	If avatar rendering is not supported, the following process is used:

a.	Network-based rendering may be invoked as specified in 7.3.2, or

b.	Sender-centric rendering may be invoked, where the sending UE proceed with the rendering and the receiving UE receives a video stream representing the animated avatar, or

c.	A fallback mechanism to regular non-avatar video call may be invoked with necessary media renegotiation, or

d.	A fallback to audio only call may take place, or

e.	If none of the above is supported, the session may be rejected.

When avatar rendering is not supported at the receiving UE and network-based rendering is invoked by the IMS network, the IMS AS shall renegotiate the connection by sending a modified SDP offer by including a new “video” media “m=” line to the receiving MTSI client.

When avatar rendering is not supported at the receiving UE and network-based rendering is not invoked by the IMS network, then the IMS AS shall delete the application data channel media description for avatar animation data from the SDP answer. The SDP answer shall not include a media description (“m=” line) for the avatar animation negotiation data channel. Upon receiving this SDP answer, the sending MTSI client may invoke sender-centric rendering.

When avatar rendering is not supported at the receiving UE and sender-centric rendering is invoked, the sending MTSI client shall renegotiate the connection by sending a modified SDP offer by including a new “video” media “m=” line to the receiving MTSI client. The new “video” media “m=” line shall include “a=avatar” attribute line to indicate that this video stream is generated by animating the avatar model.

The following cases can happen:

1.	When avatar rendering is not supported at the receiving UE and sender-centric rendering is invoked, the receiving entity may accept the “video” media “m=” line with the associated attribute lines, indicating that the receiving UE accepts to receive a video of the animated avatar.

2.	The receiving entity may reject the avatar video by deleting the “a=avatar” attribute line from the SDP offer and sending the modified SDP answer to the sending UE, indicating that it wishes to receive a regular video call.

3.	Alternatively, the receiving UE may also remove the “video” media “m=” line from the SDP answer. This may indicate that the receiver wishes to have an audio only call or

4.	The receiving UE may also remove all media “m=” lines from the SDP answer to reject the communication.

In cases 2 and 3, the sending UE may renegotiate the call with a regular video call by adding a new “video” media “m=” line without including the “a=avatar” attribute line.

An SDP offer may contain multiple “m=” lines, each corresponding to one of the above cases 1 and 2. The receiver may accept either the “video” media “m=” line that contains the “a=avatar” attribute line or the “video” media “m=” line that does not contain the “a=avatar” attribute line in the SDP answer.

When an avatar video call is negotiated between a sending UE and a receiving UE and the sending UE or the receiving UE (whoever acts as an invoker) decides to send only regular video at a later point in time in the Avatar call session, then the invoker shall renegotiate the session by sending a modified SDP offer that includes a new “video” media “m=” line and it shall not contain the “a=avatar” attribute line. The invoker shall remove the old “video” media “m=” line with “a=avatar” attribute line from the modified SDP offer.

When a regular video call is negotiated between a sending UE and a receiving UE and the sending UE or the receiving UE (whoever acts as an invoker) decides to switch to an avatar video call at a later point in time in the Avatar call session, then the invoker shall renegotiate the session by sending a modified SDP offer that includes a new “video” media “m=” line and it shall contain the “a=avatar” attribute line in the corresponding “video” media “m=” line. In this case, the invoker shall remove the old “video” media “m=” line from the modified SDP offer. The other UE may accept the modified SDP offer or may reject it.

#### 7.4.2.3	Usage of the label attribute and label parameter

AR-MTSI clients supporting avatar calls shall use the "label" attribute by including it under the corresponding media streams (for AV media), or the “label” parameter by including it under the DCMAP attribute for corresponding data channel m-lines, in order to describe the media data to be delivered. These labels and their values should be the same in both the SDP offer and answer, and are used for descriptive purposes during media re-negotiation after avatar negotiation.

The label values defined for avatar calls are as follows:

a=label:3gpp-avatar-proc-{process}

as a label attribute for RTP media streams:

a=dcmap:0 label="3gpp-avatar-proc-{process}"

as a label parameter under the DCMAP attribute for data channel streams.

-	process: the name of the process in the avatar workflow (see clause 7.1 of TR 26.813 [9]) which outputs the data to be delivered using the media stream or data channel under which this label attribute or parameter is included. Defined values (avatar processes) include:

-	"cp": the data transported is the output of the capture posture process which captures the user’s posture for creation of an animation stream; this output is typically either captured video, sensor data, or both.

-	"gc": the data transported is the output of the generate command process which generates animation stream data; this output is typically blendshape data or joint data.

-	"aa": the data transported is the output of the animateAvatar process which animates a 3D base avatar using animation stream data; this output is typically a file or patch containing the animated 3D base avatar data.

-	"ms": the data transported is the output of the manageScene process which either creates or updates the scene description for an avatar call; this output is typically a glTF file in the form of JSON or a JSON PATCH.

-	"rs": the data transported is the output of the renderScene process which renders the avatar (or scene description including any avatars); this output is typically 2D video.

Avatar workflow processes:

-	Capture posture: the process of capturing the data representing the user’s posture which is used as input data to generate animation stream data.

-	Generate command: the process of generating animation stream data.

-	Animate avatar representation: the process of animating a 3D  avatar representation using animation stream data.

-	Manage scene: the process of creating or updating a scene description for an avatar call.

-	Render scene: the process of rendering the animated avatar representation.

If avatar animation capability negotiation has already been completed (animation mode decided), the AR-MTSI client supporting avatar calls shall include the label attribute or label parameter with the suitable value under each corresponding m-line of the SDP offer/answer, according to the required RTP stream(s) and ADC(s), respectively, which need to be established as defined in Figure A.2.4-1. E.g. if network rendering mode has been decided, the AR-MTSI client shall include the label attribute with value "a=label:3gpp-avatar-proc-rs" under the video m-line media stream intended to transport the rendered avatar to the receiver client.

### 7.4.3	Avatar negotiation over ADC

#### 7.4.3.1	Avatar negotiation protocol

Messages sent over the ADC for avatar negotiation shall use the AR metadata message format defined in clause 6.2.

The message type for avatar negotiation messages shall be set to "urn:3gpp:avatar:v1:negotiation". The payload of the messages shall be as defined in Table 7.4-1.

Table 7.4-1: Payload format of the avatar negotiation message type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| subtype | number | 1..1 | An identifier of the subtype of the message payload. The allowed values are listed in Table 7.4-2. |
| content | object | 1..1 | A URN that identifies the message type. |

Table 7.4-2 defines all supported sub-types of the avatar negotiation message type:

Table 7.4-2: Subtypes of avatar negotiation messages

| Value | Message Name |
| --- | --- |
| 1 | REQUEST |
| 2 | ACCEPT |
| 3 | REJECT |
| 4 | ACK |

The REQUEST is sent by the UE or by the MF when acting as the negotiation coordinator.

The ACCEPT and REJECT messages are sent by the UE or MF to indicate that the received REQUEST is accepted or rejected.

The ACK is issued by the DC-AS or MF when it decides to perform animation and rendering on behalf of the receiver.

#### 7.4.3.2	Message Formats

The following tables describe the message formats exchanged for avatar negotiation.

The REQUEST message payload is defined in Table 7.4-3.

Table 7.4-3: REQUEST payload

| Name |  | Type |  | Cardinality |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| avatarProfile |  | string |  | 1..1 |  | Indicates the avatar profile as defined in clause 5.6. NOTE: allowed values for the avatarProfile are FFS. |  |
| arfContainer |  | object |  | 1..1 |  | Reference to ARF container of the Avatar representation that is offered for the call. The container shall be retrievable from the BAR. |  |
| animationSourceData |  | array(URI) |  | 1..1 |  | Provides a list of supported animation data for the avatar. The following values are allowed: A subset of the AnimationFrameworks as declared in the ARF container of the base avatar Audio with the URN "urn:3gpp:avatar:v1:animation:audio" Video with the URN "urn:3gpp:avatar:v1:animation:video" At least one animation source data shall be included. |  |
| renderingLocation |  | array(string) |  | 1..1 |  | Indicates the possible location(s) for animating and rendering of the offered avatar. The possible values are: “sender”, “receiver” or “mf”. An MF may add “mf” in the offered list if it is capable of animating and rendering the offered avatar. |  |
| animationFallback |  | array(URI) |  | 0..1 |  | A list of URNs indicating the supported animation data to be used as a fallback for animating the avatar when the selected animation stream stops. If the animationFallback is an RTP stream, the MID value of the fallback stream shall be sent as part of the urn:3gpp:avatar:v1:animation:stopped message over ADC. |  |

The ACCEPT message payload is defined in Table 7.4-4

Table 7.4-4: ACCEPT payload

| Name |  | Type |  | Cardinality |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| renderingLocation |  | string |  | 1..1 |  | Indicates the selected location for the rendering and animation of the offered avatar. If the location is set to “mf”, it has to be confirmed by the transmission of an ACK message from the MF to the receiver. |  |
| animationSourceData |  | array(URI) |  | 1..1 |  | Includes the selected subset of the animation frameworks that the receiver prefers to receive for performing the avatar animation. If an audio or a video source is used, the receiver or MF shall use one of the audio or video stream from the IMS session to animate the avatar. |  |
| animationFallback |  | URI |  | 0..1 |  | Selected URN from the values offered in the animationFallback field of the REQUEST message to be used as a fallback for animating the avatar when the selected animation stream stops. If the animationFallback is an RTP stream, the MID value of the fallback stream shall be sent as part of the urn:3gpp:avatar:v1:animation:stopped message over ADC. |  |

If a fallback stream is indicated in the animationFallback field of the ACCEPT message and the animation fallback is an RTP stream, then the SDP media description of the fallback RTP stream shall include a MID value.

The ACK message payload is defined in Table 7.4-5.

Table 7.4-5: ACK payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| selectedRenderingLocation | string | 1..1 | Indicates the selected rendering location as a final decision. |

The REJECT message payload is defined in Table 7.4-6.

Table 7.4-6: REJECT payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| code | string | 1..1 | Machine-readable error code. NOTE 1: If the REJECT message needs to be associated with a particular REQUEST is FFS. NOTE 2: Definition of Error codes is FFS. |
| detail | string | 0..1 | Human-readable diagnostic text. |

The arfContainer typer is defined in Table 7.4-7.

Table 7.4-7: Component: arfContainer

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| uri | string | 1..1 | Authorization-bound reference to the avatar representation in BAR or an operator repository. Typical schemes: bar://… or https://…. |
| auth | object | 0..1 | Ephemeral authorization for dereferencing URI (e.g., bearer token); present when required by BAR policy. |

The auth object of the arfContainer is defined in Table 7.4-8.

Table 7.4-8: Sub-component: arfContainer.auth

| Name |  | Type |  | Cardinality |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| scheme |  | string |  | 1..1 |  | Authentication scheme; "bearer" is supported. NOTE: Other HTTP defined authentication schemes can be added and is FFS. |  |
| token |  | string |  | 1..1 |  | Opaque credential used to access and download the ARF container. |  |
| expiresAt |  | number |  | 0..1 |  | Expiry time in milliseconds since epoch for the token. |  |

* * * Seventh Change* * *

## 8.4	Data Channel Transport of Avatar Data

### 8.4.1	General

An AR-MTSI client that supports avatars shall use the avatar ADC for the transport of avatar control and animation data.

The SDP attribute a=3gpp_armetadata_types shall enumerate all avatar-related message URNs that the endpoint is prepared to send or receive on that ADC.

### 8.4.2	Transport of Avatar Representation

When an avatar representation or selected assets are delivered in-session over a data channel, the sender and receiver shall establish a second application data channel whose sub-protocol is "http".

The http channel shall be reliable and in-order.

### 8.4.3	Transport of Animation Streams

The format of the avatar animation streams is defined in clause 6.3.2.

* * * eighth Change* * *

## 9.2	Avatar-related QoE

### 9.2.1	Timing Information for Avatar Animation and Rendering

In avatar calls, compared to traditional remote or split rendering for AR, one important parameter to estimate the user quality of experience is the posture to render to photon time delay, defined as the time duration between the posture-capture-time or animation-data-generation-time and the actual-display-time.  The calculation or measurement of the timestamps related to this delay is dependent on the entity performing the avatar animation and rendering, which is also influenced by the latencies involved in generating and delivering the animation data required at the corresponding entity.

The posture-capture-time (T10) is measured in the sending UE, as the time when the sender UE user’s pose is captured in order to generate the animation data.

The animation-data-generation-time (T11) is measured either in the sending UE or the MF, as the time when the animation data is generated from the source data.

The avatar-animation-time (T12) is measured, depending on the animation mode, either in the sending UE, MF or receiving UE, as the time when the avatar representation is animated and rendered.

The actual-display-time (T2) is measured in the receiving UE, as the time when the rendered avatar representation is displayed to the user.

These timestamps may be delivered to each corresponding entity via feedback messages in order to facilitate better quality of experience. Better QoE may be provided to the user either through an adjustment in pose correction (in the receiving UE), or by other means such as the re-negotiation of a more suitable entity for animation data generation and/or avatar animation and rendering.

* * * Ninth Change* * *

# A.2	Avatar Call Flows

## A.2.1	General Avatar Call Flow

Figure A.2.1-1: IMS Avatar Delivery and Animation Flow

Z. Base Avatar model and Avatar Representation Generation Before Call Setup

The base avatar model and derived avatar representations are generated before step A (Call Setup and Capability Negotiation). These may be uploaded to the BAR by using the Avatar management interface defined in Annex B.

A. Call Setup and Capability Negotiation

A.1 An audio/video session is established between UE1 and UE2 and parameters of the session are negotiated.

The list of Avatar ID(s) and/or Avatar Representations is downloaded to the UE by the following options:

Pre-configured in the UE: The Avatar ID List and/or Avatar Representations is provisioned or downloaded to the UE before a data channel for avatar call is setup.,

Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR when the associated Avatar call application is downloaded and transferred from the DC AS to the DCSF and downloaded to UE through bootstrap data channel (see details in Annex AC 11.3.1 in TS 23.228) [4].

Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR and downloaded to the UE through an application data channel.

NOTE 1: Further details of avatar selection and negotiation are defined in clause A.2.3.

B. Scene Description Retrieval

The MF and the participating UEs retrieve the scene description. The scene description may be shared by the MF with the UEs, in case of a shared experience, or the UEs may have their own scene descriptions.

C. Scene Description Update

A scene update trigger occurs, e.g., if an object is added to or removed from a scene or if spatial information is updated. The update trigger may originate from the MF itself or the UEs. The UEs may update their scene descriptions independently or the MF may generate an updated scene description and share it with the UEs.

NOTE 2:	The steps B and C are not needed for 2D avatar.

D.1. Avatar Acquisition

D.1.1: The MF loads the avatar representation for UE1 from BAR.

D.2. Avatar Delivery

Alternative #1: Sender-centric

D.2a.1: The MF delivers the avatar representation of UE 2 to UE1 through data channel.

Alternative #2: Receiver-centric

D.2b.1: The MF delivers the avatar representation of UE1 to UE2 through data channel.

D.3. Animation Data Generation

Based on the capability negotiation result in step A, the UE or network may generate animation data.

Alternative #1: UE centric animation data generation

D.3a.1: The UE1 generates the animation data based on the source data (e.g., audio, video, text) or using an XR runtime. The animation data may be transformed from the source data (e.g., from audio to text), or the same as the source data.

D.3a.2: UE1 delivers the animation data to the entity actuating avatar animation through data channel. The animating entity may be the MF or UE2.

Alternative #2: Network centric animation data generation

D.3b.1: UE1 sends source data for animation data generation to the MF over RTP (audio, video, text) or a data channel (text).

D.3b.2: The MF processes the received source data to generate animation data during the session. The animation data may be transformed from the source data (e.g., from audio to text, video to motion data), or the same as the source data.

D.3b.3: The MF delivers animation data over a data channel to the UE2 animating the avatar representation. If network-centric avatar animation is used, this step will be skipped. The animation data may be delivered to UE1 as well.

D.4. Avatar Animation

Based on the capability negotiation result in step A, the UE or network may animate the avatar.

Alternative #1a: Sender-centric avatar animation

[Optional] D.4a.1: UE2 delivers its pose information to UE1 for viewer-dependent avatar animation and rendering.

D.4a.2: UE1 animates and renders the avatar representation using animation data. The animation data is generated by UE1 in step D.3a.1.1

D.4a.3: UE1 delivers the animated and rendered avatar to UE2. The animated and rendered avatar may be delivered as a 2D video through RTP.

D.4a.4: UE2 corrects the rendered video (for latency compensation) from UE1 before displaying as rendered avatar.

D.4a.5: UE2 delivers a report of timing information, including its actual display time to UE1 for the monitoring of the UE1 centric rendering service.

Alternative #1b: Receiver-centric avatar animation

D.4b.1: UE2 animates and renders the avatar representation using animation data. The animation data may be generated by the MF, following steps D.3b.1 to D.3b.2 and received by UE2 in step D.3b.3 or it may be generated by UE1 in step D.3a.1 and received by UE2 in step D.3a.2.

D.4b.2: UE2 delivers a report of timing information, including its actual display time to UE1 (and MF) for the monitoring of the UE1-centric rendering service.

Alternative #1c: Network-centric avatar animation

D.4c.1: The MF animates and renders UE1’s avatar representation using animation data. The animation data may be generated by the MF, following step D.3b.1 and D.3b.2 or it may be received from UE1 following steps D.3a.1 and D.3a.2.

D.4c.2: The MF delivers the animated and rendered avatar to the UEs. In the figure, delivery to UE2 is shown as an example. The animated and rendered avatar may be delivered as a 2D video through RTP.

D.4c.3: UE2 corrects the rendered video (for latency compensation) from the MF before displaying as the rendered avatar.

D.4c.4: UE2 delivers a report of timing information, including its actual display time to UE1 (and MF) for the monitoring of the network-centric rendering service.

NOTE 3: Rendering is not needed for 2D avatars.

NOTE 4: Network centric modes include both MF rendering mode and DC AS rendering mode. Alternatively, the DC AS can also perform the above functions if the DC AS rendering mode is chosen. This specification currently only considers MF-based rendering mode.

### A.2.2	Avatar Management Call Flow

Figure A.2.2-1: Avatar management call flow via IMS network for registering and uploading base avatar model, avatar representation, and associated assets

Figure A.2.2-1 depicts the call flow procedure for registering and uploading a user’s base avatar and associated assets. Note: registration corresponds to the creation process of the desired resources in the BAR.

The main steps in the call flow are as follows:

1.	The UE creates the base avatar model

2. 	Base avatar model registration (the use of a base Avatar model ID assigned by the BAR is required for the secure upload of the base avatar by the UE):

2.1	The UE sends registration request to the MF/DC AS via application data channel to request the registration of its base avatar model.

2.2	The DC AS forwards the registration request to the BAR.

2.3	The BAR assigns a unique base Avatar model ID and URL for the base avatar model of the UE according to the UE identifier known via the DC AS.

NOTE: How the BAR obtains and maps user identifiers to  base Avatar model IDs is FFS.

2.4	The BAR sends a registration response containing the base Avatar model ID and URL for the registered base avatar model to the UE via the MF/DC AS.

3.	The UE associates the assigned base Avatar model ID to the Base Avatar model data created from step 1.

4.	The UE uploads the base avatar model data to the BAR via the MF using the application data channel.

6. 	Adding new assets to the base avatar model repository:

6.1	The UE creates or obtains a new asset for adding to the base avatar model repository.

6.2	The UE sends an asset registration request for the new asset to the BAR via the MF using the application data channel.

6.3	The BAR creates a new Asset ID for the new asset and associates it to the base Avatar model ID of the base avatar model.

6.5	The BAR sends an asset registration response containing the  Asset ID to the UE via the MF.

6.6	The UE uploads the asset data to the BAR via the MF.

7.	Steps 6.1 to 6.7 are repeated for the registration of additional assets.

8. 	The UE creates an avatar representations from the base avatar model.

9. 	Avatar representation registration:

9.1 The UE sends registration request to the MF/DC AS via application data channel to request the registration of the avatar representation based on the provided description.

9.2 The DC AS forwards the registration request to the BAR.

9.3	The BAR assigns a unique Avatar ID and URL for the avatar representation according to the UE identifier known via the DC AS.

9.4 The BAR updates the associated information corresponding to the Avatar ID.

9.5 The BAR sends a registration response containing the Avatar ID and URL for the registered avatar representation to the UE via the MF/DC AS.

### A.2.3	Avatar Selection and Negotiation Call Flow

For an avatar call over the IMS data channel, the avatar ID list (a list of the avatar representations available for the user in the BAR) is obtained by the UE using one of following options:

-	Pre-configured in the UE: The Avatar ID List and/or Avatar Representations are provisioned or downloaded to the UE before any session for the avatar call is established.

-	Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR when the associated Avatar call application is downloaded and transferred from the DC AS to the DCSF and downloaded to UE through the bootstrap data channel.

-	Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR and downloaded to the UE through an application data channel.

Three avatar animation modes are defined for avatar call over the IMS data channel:

-	Sender-centric: the sender UE animates and renders its avatar representation before sending it to the receiving UE as 2D video.

-	Receiver-centric: the receiving UE animates and renders the sender UE’s avatar representation.

-	Network-centric: the MF animates and renders the sender UE’s avatar representation before sending it to the receiving UE as 2D video.

The decision of which avatar animation mode to use for avatar call is dependent on the outcome of the capability and Avatar Animation Negotiation procedure via an application data channel, as detailed in Figure A.2.3-1.

A.2.3-1: Avatar selection and negotiation call flow

The avatar animation negotiation procedure is based on the avatar type (2D or 3D) and the capability information of the sender/receiver UEs and the MF. The capability information includes the animation data type(s) (e.g., text, expression data, and motion signals for joints) supported by either UEs or the MF. For network centric mode, after avatar animation negotiation, the IMS AS instructs the MF to download UE1’s avatar representation from the BAR, generate animation data from the source data received from UE1, and animate UE1’s avatar representation using the animation data received from UE1 or generated by the MF itself.

A.1.0: (optional) An Avatar ID List is pre-downloaded, or pre-configured in UE1.

NOTE 1: 	Step A.1.0 is optional; in this step the Avatar ID List is provisioned or downloaded to the UE before any session for the avatar call is setup. The UE and the BAR may interact by means out of the scope of 3GPP.

A.1.1: An IMS session is established between UE1 and UE2, and a bootstrap data channel is established between UE1, the MF, and the DC AS.

A.1.2:	The Avatar ID List and the Avatar call App are downloaded to UE1 via the BDC (see details in AC 11.3.1 in TS 23.228 [4]).

A.1.3:	A P2A2P application data channel for Avatar Animation Negotiation is established between UE1, the MF DC AS, and UE2. If the Avatar ID List has not been downloaded in either steps A.1.0 or A.1.2, it may optionally be requested via this ADC.

NOTE 2:	TS 23.228 [4] only defines MF allocation during the initial IMS session procedure and not during the IMS session modification procedure. Whether and how a suitable MF with avatar capabilities can be re-allocated after avatar negotiation is FFS.

A.1.4.1. (optional): If an Avatar ID list is not obtained in A.1.0 or A.1.2, UE1 may send an Avatar ID List request to the DC AS via MF; the DC AS then sends the request to the BAR.

A.1.4.2 (optional) If the BAR receives an Avatar ID List request, the BAR (generates and) sends the Avatar ID List (i.e., the list of avatar representations) to UE1 via the DC AS and MF

NOTE 3:	Steps A.1.4.1 and A.1.4.2 are optional. Whether and which user identity(ies) should be used by the user of the sending UE (UE1) and/or the receiving UE (UE2) for the download of the avatar representations in the case of a receiving UE centric rendering mode will be decided by SA WG3 and the procedure will be aligned with SA WG3’s decision.

A.1.5: UE1 selects an avatar representation to be used for the avatar call using the list of available avatar representations known to UE1 via the Avatar ID List.

A.1.6:	Avatar animation negotiation takes place via the established P2A2P ADC.

A.1.6.1:	UE1 sends an avatar animation negotiation request using the ADC through the MF to the DC AS. The message carries parameters which may include: an avatar ID associated with the selected avatar representation selected in step A.1.4, animation data types (e.g., text, expression data, or motion signals for joints) supported by UE1, and related rendering requirements or capability information.

A.1.6.2: (optional) To facilitate the negotiation of the rendering mode, the DC AS may interact with the current serving MF to check the MF capability through the DC1, DC2, and DC3/DC4 interfaces The MF sends a response with its avatar capability information to the DC AS that enable the DC AS to make further decisions. For example, to determine whether network-centric rendering is supported and which rendering mode (MF or DC AS) to use.

NOTE 4: Whether the interaction between the DC AS and the MF is supported needs confirmation from SA2. In particular, the MF response with its supported capabilities to the DC AS.

A.1.6.2:	(optional) Through an established P2A application data channel, MF/DC AS sends a capability negotiation request to UE2. The message may include the same information as in described in A.1.5.1.

A.1.6.3:	(optional) UE2 sends a capability negotiation response to the MF/DC AS. The message carries the capability negotiation result related to UE2’s preference.

A.1.6.4:	The DC AS gets the avatar type (2D or 3D) associated with the avatar ID from avatar representation retrieved from BAR or to be generated by the MF and confirms the avatar animation negotiation result based on the avatar type and the capabilities supported by UE1, the MF, and UE2. The capability negotiation result includes the rendering mode and animation method (e.g., by audio, text, or expression data and motion signals for joints).

A.1.6.5:	The DC AS sends the capability negotiation response to UE1 through the MF. The message carries the capability negotiation result.

### A.2.4	ADC and RTP Stream Establishment for Avatar Call

As a result of the avatar selection and negotiation procedure as described in clause A.2.3, depending on the configuration negotiated, certain application data channels and/or RTP streams are established between different endpoints (UE1, MF, UE2) for the delivery of different avatar data components as shown in Figure A.2.1-1. These include (where necessary) the delivery of scene description, avatar representation, animation source data, animation data, pose information and feedback information.

The decision of which avatar animation mode to use for avatar call is dependent on the outcome of the capability and Avatar Animation Negotiation procedure, as detailed in Figure A.2.3-1.

A.2.4-1: ADC(s) and RTP stream establishment call flow

Avatar selection and avatar animation mode negotiation takes place as described in clause A.2.3. Depending on the configuration negotiated, delivery channels are established between certain endpoints as shown in Figure A.2.4-1.

A.1.7:	Depending on the avatar negotiation result for the selected avatar representation, media re-negotiation (via SDP) takes place in order to establish the necessary ADCs and RTP streams to deliver the various avatar data. In the case where media processing in the network is required but not supported by the allocated MF, an additional MF supporting such processing may be allocated. Avatar data may include: scene description, avatar representation, animation data, pose information, and feedback information.

Sending UE Centric:

A.1.7S.1:	P2A ADC (UE1) is established for base avatar delivery (Figure A.2.1-1 step D.2a.1).

A.1.7S.2:	P2A2P ADC is established for scene description retrieval and update (Figure A.2.1-1 steps B and C).

A.1.7S.3:	P2P ADC is established for pose information and/or feedback information respectively (Figure A.2.1-1 steps D.4a.1 and D.4a.5).

Receiving UE Centric:

A.1.7R.1:	P2A ADC (UE2) is established for base avatar delivery (Figure A.2.1-1 step D.2b.1).

A.1.7R.2:	P2A2P ADC is established for scene description retrieval and update (Figure A.2.1-1 steps B and C).

alt#1a Animation Data Generation in sending UE

A.1.7R.3a:	P2P ADC is established for avatar animation data delivery (Figure A.2.1-1 step D.3a.2).

alt#1b Animation Data Generation in Network

A.1.7R.3b:	RTP stream is established for animation source data delivery (Figure A.2.1-1 step D.3b.1).

A.1.7R.4b:	P2A ADC (UE2) is established for avatar animation data delivery (Figure A.2.1-1 step D.3b.2).

A.1.7R.5: P2P ADC is established for feedback information (Figure A.2.1-1 step D.4b.2)

Network Centric:

A.1.7N.1: P2A2P ADC is established for scene description retrieval and update (Figure A.2.1-1 steps B and C).

alt#1a Animation Data Generation in Network

A.1.7N.2a: RTP stream is established for animation source data delivery (Figure A.2.1-1 step D.3b.1).

alt#1b Animation Data Generation in sending UE

A.1.7N.2b: P2A ADC is established for avatar animation data delivery (Figure A.2.1-1 step D.3a.2).

A.1.7N.3: P2P ADC is established for feedback information (Figure A.2.1-1 step D.4c.4)

* * * End of Changes* * *