3GPP TSG-SA WG4 Meeting #134	S4-251992

# Dallas USA, 11 – 17 November, 2025

Source:	InterDigital Canada

Title:	[FS_DCTC_eQoS_MED] real-time communication for conversational XR

Spec:	3GPP TR 26.823 v0.0.1

Agenda item:	10.6

Document for:	Agreement

1. Introduction

The Study on dynamically changing traffic characteristics and usage of enhanced QoS support in 5GS for media applications and services examines various use cases and scenarios representative of real-world media usage, such as real-time communication, streaming, short-form video download and media upstream for AI inferencing. For each scenario, corresponding test setups will be developed and documented. The primary objective is to identify how traffic patterns evolve dynamically and to explore the usage, benefits and deployment aspects of related QoS features available in the 5G System.

This contribution focuses on real-time communication for conversational XR.

2. Reason for Change

Clause 5.2 on real-time communication for conversational XR is currently empty.

This contribution documents the real-time communication for conversational XR service scenario that may exhibit dynamically changing traffic characteristics, and describes a first set of QoE and QoS criteria and requirements.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.823 v0.0.1.

* * * First Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 26.822: "Study of 5G RTP protocol phase 2".

[5]	3GPP TR 23.700-70: "Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2".

[6]	3GPP TR 26.925: Technical Specification Group Services and System Aspects; Typical traffic characteristics of media services on 3GPP networks

[7]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[8] 3GPP TR 26.819: “Study on Spatial Computing for Augmented Reality (AR) Services”.

[9] 3GPP TR 22.870: “Study on 6G Use Cases and Service Requirements”.

[10] 3GPP TR 26.812: “Study on QoE Metrics for AR/MR Services”.

[11] 3GPP TR 26.928: “Technical Specification Group Services and System Aspects; Extended Reality (XR) in 5G”.

[12] 3GPP TS 26.119: “Technical Specification Group Services and System Aspects; Device Media Capabilities for Augmented Reality Services”.

* * * Second Change * * * all new

## 5.2	Real-Time Communication for Conversational XR

### 5.2.1	Description

Real-time communication between two or more users may be augmented by an eXtended Reality (XR) scene shared by all the participants. A user may be represented by his (2D or 3D) avatar. A user may have several XR devices (e.g., XR glasses/headset, immersive audio headset, haptics devices) for a multi-modal immersive experience.

Each participant may interact with the virtual objects composing the XR scene. The main XR Scene Manager, located in the Media Function (MF) of an Application Server (AS) is responsible to maintain the XR scene up to date for all the participants during the communication.

The participants of a XR real-time communication may be

- all remote and are represented by their avatars inside a common virtual 3D environment (i.e., full Virtual Reality – VR scenario),

- all local and the common XR scene is inserted and viewed into the conference room using Augmented Reality (AR) technology,

- either local or remote. In this hybrid configuration, the common XR scene is inserted into the conference room and viewed by the local participants using AR devices. The remote participants are represented by their avatars.

Several use cases are describing such kind of interactive immersive XR real-time communication, in TR 26.928 [11] or in TR 22.870 [9]

-	section 5.3 of TR 26.928: Core use case on real-time XR sharing, where an interactive XR experience is shared by several users. A spatial computing server is used for spatial maps, spatial anchors and location & positioning services to the AR devices for AR experience. This core use case summarizes and integrates real-time 3D Communication, AR guided assistant at remote location (industrial services), real-time communication with the shop assistant, AR animated avatar calls and 5G Shared Spatial Data use cases.

-	section 9.5 of TR 22.870: Use case on seamless immersive reality in education, where the immersive classroom may be local (all students are physically co-located and learn with virtual objects), hybrid (with both physically co-located as well as remote participants), or fully immersive (where both students and instructors are virtually present).

-	section 9.7 of TR 22.870: Use case on multiple application media synchronization, where the need of tighter time synchronization between different media is required for critical immersive communications such as 3D remotely controlled repairs or surgery. In such immersive communication, the users may be equipped with multiple devices for multiple media components (e.g. haptic device for pressure, VR glasses for video, wireless headphones for audio) and each device receives traffic for the corresponding media component from the networks.

-	section 9.12 of TR 22.870: Use case on personalized interactive immersive guided tour, where each visitor of the group may have a personalized experience (e.g., different types of XR devices such as lightweight AR glasses, tablets/phones, headset or haptics devices) during the real-time communication with a remote touristic guide represented by an avatar. The insertion of virtual objects (e.g., 2D or 3D video streams, overlaid text, picture) are linked to the real surrounding objects they discover during the guided tour. These virtual objects are properly inserted by ensuring that each visitor of the group has a good point of view of the virtual content thanks to XR Spatial Computing service. The visitor group’s immersion and the related user’s QoE are ensured thanks to the spatial and time synchronizations between the rendering the different media (e.g. haptics, video, audio).

The Real-Time Communication for Conversational XR may exhibit different dynamically changing uplink and downlink traffic characteristics:

- in the uplink: in addition to periodic XR traffic (e.g., pose information, gestures, eye tracking, voice input), information of the real environment (e.g., image capture, video sequence, sensor data such as depth information) may be sent either pseudo-periodically (the rate may depend on the user’s mobility), or based on event/user’s location as input to a XR Spatial Computing Service

- in the downlink: in addition to periodic XR traffic (e.g., rendered frame for remote rendering or state information of the XR scene for local rendering, voice), other media flows (e.g., haptics, 2D or immersive video, audio) may be transmitted, paused or resumed based on user’s interaction or location. In that case, round trip delay and time synchronization between all the downlink media flows need to be controlled to ensure a suitable user’s QoE.

### 5.2.2	Typical implementation and end-to-end procedure

The following sample scenario and end-to-end procedure for Real-Time Communication for Conversational XR is provided, derived from the description of section 5.2.1. Both local and remote rendering cases are considered.

-	A group of users are willing to participate to an interactive collaborative AR experience with the support of a remote expert. The session takes place in a dynamic-changing real environment area. Some of users wears lightweight AR glasses with no local rendering capability and the others wear XR headsets with local rendering capability. Some also wear haptics and immersive audio devices for a full immersive experience.

-	When the AR experience starts, the interactive XR scene composed of virtual objects and media streams (e.g. video, immersive audio) and the avatar of the remote expert are seamless inserted in the real environment at low latency.

-	During the AR experience, information of the user’s real environment may be sent pseudo-periodically (e.g., the rate may depend on the user’s mobility) or event-based (e.g., related to the location of the group of users) to a XR Spatial Computing Service for determining the adequate low-latency insertion of virtual objects in the dynamic-changing real environment (i.e., a virtual object needs to be positioned to be seen by each user of the group without any obstacle and by ensuring user’s safety)

-	When a user interacts with the remote expert or with some virtual objects, he (or all the users) may receive new media stream (e.g., haptics feedback or audio stream) which needs to be timely synchronized with the other media streams to ensure a good user’s QoE

-	An interaction of a user or the remote expert may impact a media stream (e.g., start/pause/resume/suppression of a video). For user with local rendering AR device, this interaction dynamically impacts the traffic of the related downlink flow.

A typical implementation of this end-to-end procedure is provided in Figure 5.2.2-1

Figure 5.2.2-1: Typical implementation of Real-Time Communication for Conversational XR

A simplified implementation (Figure 5.2.2-2) may be used for evaluation. The uplink and downlink traffic may be composed of:

- in the uplink:

  - a periodic XR data traffic (e.g., pose information, gestures, eye tracking, voice input),

  - a pseudo-periodic or event-based data traffic (e.g., image capture, video sequence, sensor data such as depth information)

- in the downlink:

  - Several periodic or event-based media traffic (e.g., rendered frame for remote rendering or state information of the XR scene for local rendering, voice, haptics)

    - Round trip delay and time synchronization between all the downlink media flows need to be controlled to ensure a suitable user’s QoE.

Figure 5.2.2-2: Simplified implementation of Real-Time Communication for Conversational XR

### 5.2.3	Typical QoS and QoE criteria

Real-time services require low end-to-end latency to ensure a suitable user’s QoE for interactive experiences.

A pose-to-render-to-photon delay QoE metric, derived from the motion-to-photon latency, for which the rendering is performed remotely, is defined in the TR 26.928 [11]. A threshold value of less than 20-50ms is defined for system having pose correction.

The following thresholds values are defined in the clause 4.2.2 of TR 26.928 [11] for the round-trip interaction delay: lower than 50ms (ultra-low latency), lower than 100ms (low latency), and lower than 200ms (moderate latency).

These latency QoE metrics and their related Observation Points (OPs) in the XR Baseline Client have been specified in clause 11 of TS 26.119 [12].

The time synchronization QoE criteria between the haptics and other media have been addressed in clause 7.4.2.2 of TR 26.925 [6].

QoE metrics have been defined in the TR 26.812 [10] for AR/MR services. The following QoE delay metrics are particularly relevant for Real-Time Communication for Conversational XR requiring the insertion of virtual objects in the user’s real environment:

- Trackable Creation Delay (TCD), which indicates the delay between the time of the trackable creation request and the time when the related XR space is created,

- Trackable-Detection-to-Render-to-Photon (TDRP), which indicates the delay between the time of the trackable pose request leading to the detection of the trackable and the time when the virtual content is displayed in the user’s real environment

* * * End of Changes* * * *