3GPP TSG-SA4 Meeting #134	S4-252012

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0026 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Guidelines on PDU Set size and data burst size indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN-2 sent the reply LS R2-2504811/S4-251434 states that for the purpose of scheduling efficiency the more accurate data burst size and PDU set size is preferred. However for XR applications, scheduling delay is also important, because it is part of the end-to-end delay. Over-provisioning was a candidate solution to the PDU Set size inaccuracy issue, as documented in TR26.822. Over-provisioning can be done by the RTP sender, UPF or the RAN, but it is best done by the RTP sender for two reasons: (1) RTP sender knows whether low latency is important for the application and whether the end-to-end delay needs to be reduced; (2) the RTP sender behavior is within the scope of SA4, while the UPF’s and RAN’s behavior is not and yet SA2 and RAN2 have not adopted any guidelines on over-provisioning of the PDU Set size and/or data burst size. The CR to TS 26.522, S4-251527, which was agreed at the SA4 133-e meeting, didn’t accurately reflect the reply LS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add guidelines to reduce delay by over-provisioning in the PDU Set size and the Data Burst size indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The guidelines lacks support for over-provisioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.6.3, 4.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2: fixed cover page, removed “end-to-end delay” in the proposed texts. Rev 1: this document is to correct TS 26.522 v19.2.0. Orginal: S4aR250149 is to correct TS26.522 v19.1.0. |  |  |  |  |  |  |  |  |

==============First change==============

#### 4.2.6.3	PDU Set Size Field

The PDU Set Size field may be present in the RTP HE for PDU Set marking if appropriately enabled for an RTP sender as per clause 4.2.5. In case the PDU Set Size is enabled the application shall express the PDU Set Size in bytes as per the PSSize semantics defined in clause 4.2.4.

The PDU Set Size value of a PDU Set should be determined by the RTP sender based on the RTP payload corresponding to the PDU Set, transmission path MTU Size, or alternatively, maximum RTP SDU size, and network IP transport configuration.

A more accurate PDU Set Size indication is preferred. There may be some practical limits in indicating the PDU Set size accurately by the sender for use at the receiver due to network operations unknown to the RTP sender; in practice, a limited accuracy within a margin (of a few percents) can still be acceptable. The RTP sender may indicate a PDU Set Size that is a few percent larger than it observes.

==============Next change==============

### 4.5.6	Guidelines for signalling dynamically changing traffic characteristics

It is recommended that the first several RTP packets and the last few packets contain the dynamically changing traffic characteristics signalling. In addition, some additional RTP packets may contain the RTP HE for dynamically changing traffic characteristics.

The RTP sender/application may decide on how frequently to add the RTP HE for dynamically changing traffic characterstics based on different factors such as estimated packet losses or other network conditions. The RTP HE for dynamically changing traffic characteristics are consumed by the core network, i.e., the UPF, as defined in 3GPP TS 23.501 [12], clause 5.37.10.

For data burst size indication, the best possible estimate or calculation is preferred. Guidelines in clause 4.2.6.3 for PDU Set Size calculation can also be used for computing the data burst size in a comparable way with similar accuracy requirements. In practice, an error margin up to a few percent at the receiver is recommended to enable benefits of utilizing this value in the 5G System. The RTP sender may indicate a data burst size that is a few percent larger than it observes.

==============End of change==============

==============First change==============

<Contents of the first change, including the text of the whole of the affected clause.>

==============Next change==============

<Contents of the next change, including the text of the whole of the affected clause. Insert separator if there are more than one additional changes>

==============End of change==============