3GPP TSG-SA WG4 Meeting SA4 #134 	S4-252059 is revision of S4-241889r01

17th- 23rd November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0033 | rev | 0 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### [FS_AMD_Ph2] Baseline procedure for establishment of a multi-access uplink media streaming session using ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This present contribution relates to 5G Media Streaming (5GMS) systems defined in 3GPP TS 26.501 and related specifications (for example TR 26.804, TR 26.802). In particular, this contribution addresses “Network Assistance procedures” specified for media delivery sessions. This contribution applies to 3GPP SA4 work on “Study on Advanced Media Delivery Phase 2- FS_AMD_MED_Ph2”, the 3GPP Rel 20 study item that focusses on advanced media delivery architectures. The phase 2 of this study has just been approved in SA#109 and the work has begun in SA4. Several new Work Taks (WTs) have been recently added to the study. Among others, one key issue is extracted below: - WT#8: Multi-access media delivery phase 2: Clause 5.18.7 of TR 26.804 lists topics for further study on multi-access media delivery, including the impact on splitting M4 media flows when transported as GBR QoS flows; implications of the ATSSS architecture for UE multipath management, dynamic policy, network assistance, and network slicing procedures; and alignment with the study on media delivery from multiple service locations/endpoints. Additionally, based on discussion with 5G-MAG, the impact of multi-access media delivery for uplink streaming is to be studied. It is proposed to add the proposed content to the latest draft of TR 26.804 v19.1.0 under clause 5.18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.804 on “Clause 5.18 Mulit-access Media delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

##### 5.18.1.3.1	Background Specification for ATSSS architecture

Clause 5.32 of TS 23.501 [23] describes ATSSS (Access Traffic Steering, Switching, and Splitting) an optional feature supported by the UE and 5G Core network for multi-access. Some of the key principles this feature defines that are relevant for our study are:

1.	The ATSSS feature enables a Multi-Access PDU Connectivity Service allowing for the exchange of PDUs between the UE and a Data Network by simultaneously using one 3GPP access network and one non-3GPP access network via two independent N3/N9 tunnels between a PDU Session Anchor UPF (PSA UPF) and the RAN/AN.

NOTE 1:	The limits on the number and type of access network refer to Release 18 and may differ in subsequent releases.

2.	The Multi-Access PDU Connectivity Service is facilitated by a Multi-Access PDU (MA PDU) Session that may have User Plane resources on two access networks. In the context of the generalised media delivery architecture specified in TS 26.501 [15]:

-	If conveyed over an MA PDU Session, the application flow between the Media Session Handler and the Media AF (e.g., 5GMS AF) at reference point M5 may use two different access networks.

-	If conveyed over an MA PDU Session, the application flow between the Media Access Client (e.g., Media Player or Media Streamer) and the Media AS (e.g., 5GMS AS) at reference point M4 may use two different access networks.

3.	The UE is supplied with policy rules ("ATSSS rules") by the network for deciding how to distribute uplink traffic across multiple access networks. Similarly, the UPF anchor is supplied with policy rules ("N4 rules") by the network for deciding how to distribute downlink traffic across the two N3/N9 tunnels and the two access networks. The network entity configuring ATSSS rules and N4 rules is the SMF. The SMF may map PCC rules from the PCF to create these ATSSS and N4 rules.

The Policy and Charging Configuration (PCC) rules generated by the ATSSS-enabled Policy Control Function (PCF) for each Service Data Flow (SDF) type are translated by the ATSSS-enabled Session Management Function (SMF) into N4 rules for the User Plane Function (UPF). These rules include routing and Quality of Service (QoS) parameters, such as Forwarding Action Rules (FARs) and Packet Detection Rules (PDRs), as well as dedicated Multi-Access Rules (MARs) and ATSSS rules. The SMF also delivers these ATSSS rules to the User Equipment (UE) via the Access and Mobility Management Function (AMF), along with the UE Route Selection Policy (URSP) rules, which are transmitted over the NAS interface at reference point N1.

The ATSSS functions in the UE are responsible for managing the uplink traffic (i.e., from the UE to the DN), while the ATSSS functions in the UPF are responsible for managing the downlink traffic, by enabling a combination of MPTCP (higher layer) and/or lower layer ATSSS-LL multi-link technology.

4.	The UE indicates its support for ATSSS (steering functionalities and steering modes) in the PDU Session Establishment Request that is sent to request a new MA PDU Session.

5.	If the UE requests a network slice instance, the same S-NSSAI is allowed to span both access networks.

NOTE 2:	Support for QoS when PDUs are conveyed over a PDU Session belonging to a network slice that spans non-3GPP access network is unknown.

6.	For QoS support, the same 5G QoS model used for conventional PDU Sessions also applies to MA PDU Sessions, i.e. QoS Flow is the finest granularity of QoS differentiation. However, QoS Flow is access-agnostic: the same network QoS applies to each of the different access network comprising the MA PDU Session, i.e. the same QoS is available across two different paths in different access networks. The network (SMF) may provide QoS rules to the UE via one access network that are used for both the 3GPP access network and non-3GPP access network.

-	In the context of the generalised media delivery architecture, application flows at reference point M5 and/or M4 using a MA PDU Session may have similar network QoS as when they are transmitted via the 3GPP access network alone.

NOTE 3:	Support for PDU Session QoS when PDUs are conveyed over a non-3GPP access network is unknown.

7.	The network may provide Measurement Assistance Information to the UE and/or UPF to assist them in determining which measurements (packet round-trip time measurements, packet loss rate measurements) are to be performed before deciding how to distribute traffic across the two access networks.

8.	The ATSSS rules provided to the UE by the network contain information about the type of steering to be used to distribute traffic across multiple access networks. This allows traffic to be steered, switched and split across multiple access networks. From clause 5.32.8 of TS 23.501 [23], the supported steering mechanism defined in this release are:

-	Higher-layer MPTCP (Multipath TCP) functionality – The UPF provides MPTCP proxy functionality. Corresponding MPTCP functionality in the UE may communicate with the MPTCP proxy in the UPF to distribute and aggregate traffic across multiple access networks.

-	Higher-layer MPQUIC (Multipath-enabled QUIC) functionality – The UPF provides MPQUIC proxy functionality. The corresponding MPQUIC functionality in the UE may communicate with the MPQUIC proxy in the UPF to distribute and aggregate traffic across multiple access networks.

-	ATSSS-LL (ATSSS Low-Layer) functionality – The UPF allows steering, switching, and splitting of traffic across two access networks based on information from the IP layer and below.

9.	The ATSSS rules provided to the UE by the network indicate which steering mode is to be applied to matching traffic for each Service Data Flow (SDF). The steering mode determines how the matching traffic is to be distributed across 3GPP and non-3GPP access networks. Supported steering modes in Release 18 include:

-	Active-Standby: Used to steer matching SDF packets onto one access network (the "Active access") when this is available, and onto another (the "Standby access") when the Active access is unavailable.

-	Smallest Delay: Matching SDF packets are steered to the access network with smallest packet round-trip time.

-	Load-Balancing: Used to split the delivery of SDF packets between both the access networks if both of them are available.

-	Priority-based: Used to steer SDF packets onto an access network with a higher priority.

-	Redundant: Used to duplicate SDF packets on both access networks if both of them are available.

Figure 5.18.1.3-1 illustrates the traffic steering mechanisms defined in this release by TS 23.501 [23] and their respective functionalities.

Figure 5.18.1.3-1: Traffic steering mechanisms and their functionalities
in an illustrative UE model specified in TS 23.501 [23]

For access traffic steering, switching, and splitting procedures, the UE may be provided with up to five different IP addresses by the network:

-	one IP address/prefix for the Multi-Access PDU session (allocated regardless of type of steering functionality).

-	two IP addresses/prefixes, one bound to each access network, called the "MPTCP link-specific multipath" addresses (if UE and network agree on using MPTCP steering functionality).

-	two IP addresses/prefixes, one bound to each access network, called the "MPQUIC link-specific multipath" addresses (if UE and network agree on using MPQUIC steering functionality).

NOTE 1:	The MPTCP link-specific multipath addresses and the MPQUIC link-specific multipath addresses may not be routable via N6.

NOTE 2:	The "MPTCP link-specific multipath" addresses/prefixes can be the same as the "MPQUIC link-specific multipath" addresses/prefixes.

To support the operation of media delivery services specified in TS 26.501 [15], TS 26.506 [171], and TS 26.502 [135] with multi-access, there is a need to first document clear potential issues to split, steer, and switch the M4 application flows of these media delivery services based on methods specified in ATSSS architecture.

## 2nd Change

#### 5.18.4.3	Multi-Access media delivery for uplink direction using ATSSS

This clause describes multi-access media delivery in the uplink direction using Access Traffic Steering, Switching and Splitting (ATSSS). From clause 5.32 of TS 23.501 [23] and clause 4.22.3 of TS 23.502 [24], the ATSSS support is enabled by dedicated user plane functions distributed between the UE and the UPF, and new policy rules at the control plane required to enforce traffic management on the two links by enabling a Multi-Access (MA) PDU layer.

A “Multi Access (MA) PDU Request” in the uplink NAS Transport message indicates to the 5G Core that it is required to establish a new MA PDU Session and to apply the ATSSS-LL functionality, or the MPTCP functionality (or both) to implement a multi-access PDU session. The decision of whether to use Multipath-TCP (MPTCP) or ATSSS-LL for a specific Service Data Flow (SDF) is made by the network's Policy and Charging Rules Function (PCRF) and provisioned in the User Equipment (UE) via the ATSSS rules. The PCRF selects the appropriate Steering Functionality (MPTCP or ATSSS-LL) and Steering Mode based on the capabilities of the MA PDU Session and operator policies to control how traffic is managed across different access networks.

A Multi-access PDU Session may be set up in one of three different ways for uplink media steaming:

1.	The UE may set up a single-access PDU Session over one access network and then register over another access network and request a multi-access PDU Session to be set up using both the access networks.

2.	The UE may indicate its capability for ATSSS and request the setting up of a multi-access PDU Session to begin with.

3.	The UE may request to set up a single-Access PDU Session, but the network may transparently set up a multi-access PDU Session instead.

For simplicity, the first option above is used to illustrate the high-level call flow for uplink 5G Media Streaming with multi-access media delivery. Figure 5.18.4.2-1 shows a high-level call flow for an uplink 5G Media Streaming session over a multi-access PDU Session that uses two different access networks: a 3GPP access and a non-3GPP access. Differences from the baseline call flow for uplink media streaming in clause 6.3.2 of TS 26.501 [26501] are highlighted in blue.

Assumptions:

-	The 5GMS Client is unaware of the UE ATSSS steering functionality.

-	The uplink 5G Media Streaming session is set up over 3GPP access first before the UE switches to a multi-access PDU Session to use both the access networks.

Figure 5.18.4.3-1: Baseline procedure for 5G Uplink Media Streaming session
with multi-access media delivery

### The steps are as follows:

Step 1 to 7: Single Access (initial state): UE app requests to start uplink streaming. The Media Session Handler coordinates with 5G core network (AF → PCF → SMF) to establish a PDU session and apply any ATSSS/PCF policies. The Media Streamer completes media session setup with the 5GMSu AS and streaming begins on a single access. Baseline single-access operation including resource provisioning, QoS flows set-up, and media paths are provisioned and established before multi-access is considered.

1:	Start uplink streaming using single access.

2:	PDU session establishment request for uplink transport session using single access.

3:	SMF retrieves ATSSS policy from PCF.

4:	PDU session establishment response for uplink transport session using single access.

5:	PDU session establishment request for uplink media streaming session using single access.

6:	PDU session establishment for uplink media streaming using single access.

7:	Uplink  media streaming using single access.

Steps 8 to 9: Access availability detection: The UE must detect candidate extra access before activating multi-acces to improve throughput/resilience. The Modem / OS continuously monitors access technologies (e.g. Wi-Fi, 5G NR). When a new access becomes available (e.g. Wi-Fi association completed, IP address assigned), the OS notifies upper layers via events (netlink, Android ConnectivityManager, or similar). The UE Modem/OS provides an Access Availability Report to the Media Streamer.

8:	Event: new access discovered (Wi-Fi association complete).

9:	Notify: AccessAvailable (accessId=Wi-Fi, IP ready).

10:	Evaluate: check eligibility for media streaming.

Steps 11: Local decision and reporting to Media Session Handler: The Media Streamer sends detailed per-access metrics (capabilities) to the Media Session Handler so media control can decide. The Media Streamer evaluates whether the new access is eligible for media traffic (e.g., same PDU session or via ATSSS policy). The Media Streamer sends a MultiAccessDetected notification to the Media Session Handler, including: Access ID and type (e.g., Wi-Fi / 5G), QoS capability or path metrics (latency, throughput, loss rate), Possible IP endpoints. MSH needs per-access info (e.g., throughput, latency) to choose whether to activate MA and how to split/steer streams for non-GBR flows or GBR flows.

11:	Notify: MultiAccessDetected(accessId=Wi-Fi, QoS, latency, loss, throughput)

Steps 12 to 13: MSH triggers multi-access activation: The Media Session Handler informs the 5GMSu-Aware Application and simultaneously contacts the 5GMSu AF (via the media session handling API at reference point M5u) to request authorization/assistance for MA activation. The MSH analyzes the event and decides whether to activate MA operation. It notifies the 5GMSu-Aware Application about multi-access availability. In parallel, it sends a “Activate MA” request to the Media AF to align control plane and policy configuration. Control plane coordination is needed: The 5GMSu AF can consult PCF and decide on ATSSS rules, delivery boosts, or QoS changes.

12:	Inform: additional access detected, preparing MA activation

13:	POST /events MultiAccessDetected (M5u API)

Steps 14 to 18: Media AF & network provisioning: The 5GMSu AF asks the PCF to permit multi-access; the PCF/SMF allocate per-access QoS flows, steering rules, and any PCC templates; the 5GMSu AF returns activation parameters to the Media Session Handler. The 5GMSu AF then contacts the PCF/SMF to:

-	Authorize dual access usage (ATSSS rules or PCC policy),

-	Allocate new QoS flows for the added access path,

-	Return updated transport / steering instructions to the Media Session Handler.

-	The PCF/SMF sends a confirmation upon acceptable conditions and informs the 5GMSu AF.

The 5G Core must authorise resource usage per operator policy; QoS flows must be created/mapped per access for correct scheduling.

14:	Request: authorize dual access (update ATSSS / PCC policy).

15:	Allocate QoS Flows for new access (e.g. QFI for Wi-Fi).

16:	Confirm: QoS rules provisioned.

17:	Ack: multi-access allowed / parameters.

18:	Response: activation authorized, steering rules.

Steps 19 to 22. Establish new subflow: The Media Session Handler instructs the Media Streamer to open a transport subflow on the new access (MP-TCP add_addr or MP-QUIC create PATH). Modem/OS returns socket ready, the Media Streamer confirms this to the Media Session Handler. The Media Streamer establishes a new subflow (e.g. MP-TCP, MP-QUIC) for the newly detected access based on the confirmation received from the Media Session Handler. At the same time, the Media Streamer also requests to the modem to open a socket to the newly detected access. As the modem confirms this, and once the new subflow is ready, the Media Streamer notifies the Media Session Handler that multi-access is active. Only after the transport subflow is up can traffic be sent/received on that access.

19:	Command: establish subflow for accessId=Wi-Fi (MP-TCP add_addr / MP-QUIC path).

20:	Request: open socket/interface binding for Wi-Fi.

21:	Response: socket ready, interface active.

22:	Notify: new path established (Wi-Fi).

Steps 23 to 24. Update transport policy: The Media Session Handler decides split ratios/steering policy and updates the Media Streamer and/or the transport. The 5GMSu-Aware Application is informed that multi-access is active. The Media Session Handler updates scheduling / path selection rules (e.g. round-robin, split, weighted load). It informs the 5GMSu-Aware Application that multi-access is activated and that the media streaming session is now using both accesses. The application can now adjust its encoder settings (e.g., send enhancement layers over Wi-Fi) to take advantage of this, or the User Interface can reflect improved connectivity.

23:	Update scheduling policy (split / aggregation).

24:	Notify: Multi-access active (5G + Wi-Fi).

## End of change