3GPP TSG-SA WG4 Meeting SA4 #134 	S4-252060 is revision of S4-241890r06

17th- 23rd November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0034 | rev | 0 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### [FS_AMD_Ph2] Network assistance and Delivery boost for media streaming session using multi-access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This present contribution relates to 5G Media Streaming (5GMS) systems defined in 3GPP TS 26.501 and related specifications (for example TR 26.804, TR 26.802). In particular, this contribution addresses “Network Assistance procedures” specified for media delivery sessions. This contribution applies to 3GPP SA4 work on “Study on Advanced Media Delivery Phase 2- FS_AMD_MED_Ph2”, the 3GPP Rel 20 study item that focusses on advanced media delivery architectures. The phase 2 of this study has just been approved in SA#109 and the work has begun in SA4. Several new Work Taks (WTs) have been recently added to the study. Among others, one key issue is extracted below, this is the target of this contribution. - WT#8: Multi-access media delivery phase 2: Clause 5.18.7 of TR 26.804 lists topics for further study on multi-access media delivery, including the impact on splitting M4 media flows when transported as GBR QoS flows; implications of the ATSSS architecture for UE multipath management, dynamic policy, network assistance, and network slicing procedures; and alignment with the study on media delivery from multiple service locations/endpoints. Additionally, based on discussion with 5G-MAG, the impact of multi-access media delivery for uplink streaming is to be studied. The proposed solution allows the UE application (App) or UE client to connect to multiple access paths to positively improve Quality of Experience (QoE) while leveraging the “network assistance” feature. This takes into account dynamic session conditions and is especially tailored for media streaming or content delivery between an Application Server (AS) and the UE, in the uplink direction. It is proposed to add the proposed content to the latest draft of TR 26.804 v19.1.0 under clause 5.18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.804 on “Clause 5.18 Mulit-access Media delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change (All new text)

##### 5.18.5.2.6	Access-specific Network Assistance and Delivery Boost extensions to support multi-access

In this section we study access-specific Network Assistance extensions to the 5GMS control plane. These extensions enable a 5GMS Client and AF to perform per-access QoS interrogation and manipulation during ongoing multi-access media delivery sessions. This allows to optimize per-access bitrate and path selection to maximize end-user media delivery quality subject to dynamic network assistance and ATSSS constraints.

The following call flow illustrates the steps in this Candidate Solution for extending the AF-based Network Assistance feature for  media delivery to include multi-access (MA) functionality via the ATSSS process.

Figure 5.18.4.4.2-1: Procedures for access-specific Network Assistance and Delivery Boost extensions to multi-access

In the initial phase, a media streaming session is established over a single access network:

1.	Provisioning: The UE is provisioned with configuration data for the media service, including media access information, network assistance endpoints, etc.

2.	Acquire Service Access Information: The UE client retrieves access info for media services (e.g., via Mb or Ms interface).

3.	Start media delivery using single access network: The UE initiates streaming using a single network interface (e.g., Wi-Fi or cellular).

4–7.	Establish  transport session for  media delivery session: Transport and streaming sessions are set up using a single access path.  Media data is transmitted using a single access path.

8.	QoE metrics collection and reporting: The application layer collects QoE metrics and reports them. Quality of Experience (QoE) metrics are monitored between the application and application server.

Multi-Access is activated for the media streaming session:

9.	Detect multiple access networks: The UE detects the presence of multiple access networks (e.g., 4G and Wi-Fi available).

10–11.	Request to activate multi-access: The UE modem informs the application about the presence of multi-access, and the application requests the Media Session Handler to request activation of multi-access mode.

12.	Activate multi-access: The Media Session Handler, in collaboration with the Operating System (OS) and modem, enables multi-access, allowing simultaneous usage of multiple paths.

13.	Multi-access activated notification: The Media Session Handler confirms the app that the Confirmation of activation of multi-access and the application could now attach to the multiple access technologies in order to improve its QoE.

16.	Start  media delivery with MA PON (Primary Operation Node): The App reaches the media access function (media streamer) in order for the media streaming to begin over multiple access links for improved performance.

Network Assistance is activated:

17.	Request to Activate Network Assistance: The app requests assistance from the network in order to improve bit rate recommendation (throughput estimation) for each access type it is attaached. The identification of access type by the Media AF, non-3GPP and 3GPP is as per clause 6.1.of TS 23.503. The identification of same access types, for instance same 3GPP access types is FFS.  .

18.	Create Network Assistance Session (M11): The Media Access Function reaches the MSH so that a Network Assistance session for network assistance is initiated.

17–18.	Estimate recommended bit rate: The Media Session Handler now sends the list access types information to the Media AF. This is possible, thanks to steps 9-11, where the modem informs the app about the possible access points and the app informs this to the MSH. The Media AF calculates recommended bitrate parameters for each access type (e.g., non 3GPP e.g.Wi-Fi vs 3GPP e.g.5G NR) and reports this to the Media Session Handler. Alternatively, the MSH can receive the access type information from the Media AF as per clause 6.1 of TS 23.503. This information is known to the Media AF by the “Access network information reporting” as per clause 6.6.1 of TS 23.503.

19.	Network Assistance Session request and response:The Media Access Function requests session setup and receives bitrate recommendations (e.g., 5 Mbps for LTE, 8 Mbps for Wi-Fi).

20.	Set initial bit rate: The Media Access Function chooses an optimal bit rate and operational point based on recommendations.

21-22. Establish media delivery using multi-access: Media delivery starts using multi-access with network assistance.

The 5GMS Client subscribes to receive bit rate recommendations:

23.	 Multi-access PDU Session continues: Session continues with network assistance active.

24.	Subscribe to bit rate recommendations: The Media Session Handler subscribes to receive ongoing bit rate recommendations from the Media AF as network conditions change.

25.	Bit rate update notification: The Media AF sends updated bit rate recommendations (e.g., due to congestion or throughput improvement) to the Media Session Handler.

26–27.	Media Access Function adapts bit rate: This information is shared by the Media Session Handler with the Media Access Function via the M7 interface. The Media Access Function adjusts encoding rate or representation according to new bit rate.

28.	Continue streaming: Media streaming session continues with the adapted bit rate recommended by the Media AF.

The 5GMSu Client requests a delivery boost:

29. UE detects a sudden drop in QoE: The Media-Aware Application detects degraded QoE (e.g., high latency, jitter, or packet loss). This can be detected by comparing the ongoing session QoE with the historic QoE metrics collected in step 8, using for instance QMC framework as per TS 28.405.

30. Request to activate delivery boost: The Media-Aware Application requests a delivery boost to the Media Access Function. The Media Client uses this function to indicate to the network that a temporary boost, i.e., a temporary increase of network throughput for this client is needed, for example in order to prevent the  media streaming buffer in the Media Access Function from overflowing.

31.	Request delivery boost: The Media Access Function sends a request to the Media Session Handler at reference point M11 to activate a temporary performance boost.

32.	Detecting sudden drop: The Media Session Handler confirms QoE drop detection event. The Media Session Handler also detects which access typet needs a delivery boost, based on access availability information, thanks to steps 9–11.

33–34.	Invoke and respond to delivery boost: The Media Session Handler requests the Media AF to give a temporary boost to the specific access point requiring this. The Delivery Boost API is triggered; the network temporarily enhances QoS or bandwidth on that particular access point.

35.	Notify boost result: TheMedia Access Function receives confirmation that boost has been applied from the Media Session Handler at reference point M11.

36.	Media Streamer adjusts  bit rate: The Media Access Function increases bit rate and/or encoding quality level temporarily based on the above received information.

Finally, the media streaming session is terminated:

39.	End Network Assistance Session: When media streaming completes, the UE application ends the Network Assistance Session.

40.	Destroy Network Assistance Session: The Media AF releases network resources.

41.	Continue without Network Assistance: If still active, the session continues normally without Network Assistance or ends fully.

## End of change