3GPP TSG-SA4 Meeting #87 	S4-160303

# Sophia Antipolis, France, 25-29 January 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.916 | pCR |  | rev |  | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | More on Codec Compatibility 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_SETA_S4 |  |  |  |  |  |  | Date: |  | 2016-01-29 |
| Category: |  |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The Codec Compatibility Chapter is still not complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | More Codec Compatibility details added or some corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Codec Compatibility rules not complete in the TR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11 Codec Compatibility |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This document uses the "Bottom up Configuration principle" to explain the Codec compatibility relations. This is, however, not meant to mandate the EVS Bottom up Configuration principle. This decison is left for other fora. |  |  |  |  |  |  |  |  |

1rst Change

# 11	Codec Compatibility

===> snip <===

## 11.4	Transcoding Less Operation

Transcoding Free Operation (TrFO) is of key importance to many voice service aspects. High Definition Voice services (HD Voice) is an important example (although – strictly speaking – transcoding may occur also in some HD Voice calls, see below). It is important that the Codecs used at both ends of the communication are TrFO-compatible (TFO-compatible) to achieve best possible quality, as transcoding always degrades quality.

In its simplest form Codec 1, left of the MGW and Codec 2, right of the MGW, are identical. The MGW detects this and connects both links without transcoding. It is, however, not strictly required that both Codecs are identical to avoid Transcoding. It is sufficient that both Codecs are TrFO-compatible. Table 11.4-1 lists TrFO-compatible 3GPP Codecs.

Table 11.4-1: Examples of TLCI-compatible 3GPP Codecs.

| Codec 2 Codec 1 | GSM EFR | AMR (7) | AMR (0,2,4,7) | AMR-WB (0,1,2) and AMR-wB() | AMR-WB or EVS-IO Bottom up Configurations | EVS Bottom up Configurations | EVS single audio bandwidth Configuration (NB / WB / SWB / FB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GSM EFR | TLCI | SID-Con |  |  |  |  |  |
| AMR(7) | SID-Con | TLCI |  |  |  |  |  |
| AMR (0,2,4,7) |  |  | TLCI |  |  |  |  |
| AMR-WB (0,1,2) and AMR-WB() |  |  |  | TLCI | TLCI by Mode-ctrl | TLCI by Mode-ctrl | (TLCI via AMR-WB IO) |
| AMR-WB or EVS-IO Bottom up Configurations |  |  |  | TLCIby Mode-ctrl | TLCI | Mode-ctrl CMR-IO ≤ 8 | (TLCI via AMR-WB IO) |
| EVS Bottom up Configurations |  |  |  | TLCI by Mode-ctrl | TLCI by Mode-ctrl | TLCI |  |
| EVS single audio bandwidth Configuration (NB / WB / SWB / FB) |  |  |  | (TLCI via AMR-WB IO) | (TLCI via AMR-WB IO) |  | TLCI only to itself (NB<=>NB etc.) and via AMR-WB IO |

The diagonal "upper-left to lower-right" of Table 11.4-1 shows "TLCI" in all squares: Codec 1 and Codec 2 are identical or TLCI-compatible by Mode Control. Empty squares indicate: transcoding is required.

The green marked area indicates the TLCI-compatibility section of the EVS Bottom up Configurations in combination with all the EVS AMR-WB IO Configurations that include all lower modes up to their maximum. If TLCI is put in brackets (TLCI) in some squares, then additional conditions exist for compatibility. For example, an EVS single-audio bandwidth Configuration may include the EVS AMR-WB IO () Configuration and then this is TLCI-compatible to AMR-WB (0,1,2)  or any other EVS AMR-WB IO Configuration that include all lower modes up to their maximum or any AMR-WB Configuration that include all lower modes up to their maximum.

### The AMR-WB (0,1,2) is explicitly listed, although it belongs to the class of AMR-WB Configurations that include all lower modes up to their maximum, because it is an important and well-known CS-Codec.

### 11.4.1	GSM_EFR and AMR (mode-set=7)

GSM_EFR and AMR (mode-set=7) are "nearly" TrFO-compatible: the Speech frames are compatible, i.e. a GSM_EFR encoded frame can be decoded by AMR and an AMR (7) encoded frame can be decoded by GSM_EFR. The SID frames of both are, however, different and a "SID Conversion" (SID-Con) is needed. The term "SID Transcoding" is not used here, as the conversion is done without full decoding/encoding. SID frames describe the background noise in speech pauses and a small deviation in background noise is typically not perceivable by end-users, so GSM_EFR and AMR (7) are called TrFO-compatible. GSM_EFR and AMR (7) play still an important, although decreasing role in many GERAN and UTRAN networks.

### 11.4.1.1 Additional Conditions for TLCI-Compatibility for GSM_EFR

GSM_EFR and AMR (7) are single-rate-single-band Codecs. They are specified in all parameters in a way that they are always TrFO-compatible. DTX may be switched ON/OFF in the encoding side in 2G networks, but the decoding side and all network elements in the path are able to handle DTX. In 3G networks, DTX in uplink is always mandatory for AMR (7).

### 11.4.2	AMR

Important Codecs are AMR(0,2,4,7), AMR(0,2,4), AMR(0,2) and AMR(0). Not all of these Codecs are listed explicitly in Table 11.4-1 to keep the table readable. Please note that these four Codecs should be kept formally as four different Codecs: same Codec Type, but different Codec Configurations. They are all TrFO-compatible under the important assumption that the Rate Control rules are strictly followed by all terminals and all nodes in the voice path! For details, see chapter 9.

Example: Codec 1 == HR_AMR(0,2,4)  ----- Codec 2 == AMR(0,2,4,7) ----- Codec 3 == UMTS_AMR2(0,2)/SF=256.

This cascade of a GERAN----Core----UTRAN call is transcoding free for the two AMR-modes 0 (4.75) and 2 (5.90). Rate Control end-to-end (CMR ≤ 2) ensures that the maximum Rate is 5.90, i.e. mode=2. If one of the partners would not comply to AMR Rate Control rules, then transcoding would have to be included with lower voice quality than AMR(5.90) end-to-end. Otherwise one side of the call could end in "silence", e.g. if the GERAN side sends with AMR(4) the UTRAN side could not receive this and would go muting. Even worse: the AMR-SID frames, sent in speech pauses, would be able to pass and be decoded: the UTRAN side would not be totally silent, but background noise and some speech clips could be heard.

The term "SF=256" denotes here the WCDMA Spreading Factor 256 and SF=128 the WCDMA Spreading Factor 128.

Another important AMR Configuration is AMR (mode-set=0,2,5,7). This is not TrFO-compatible to AMR (mode-set=0,2,4,7), but has otherwise similar properties.

### 11.4.2.1 Additional Conditions for TLCI-Compatibility for AMR

The most used, recommended AMR Configuration, AMR (mode-set=0,2,4,7), is a multi-rate-single-band Codec. It is deployed in GERAN and UTRAN and in MTSI with different options for the transport of mode-control-commands and the rate switching. Not all these options areTLCI-compatible. It is therefore important to obey some rules. It is not recommended to deploy the AMR in another way, as there is no obvious advantage. These rules and additional conditions are the following.

GERAN allows changing the rate in media-sending and media-receiving direction every second speech frame, i.e. every 40ms, but not in between. Changing the rate in between, at the wrong phase, causes a severe decoding error and a substantial, potentially catastrophic quality loss. It is indispensable that every remote partner that wants to be TLCI-compatible with GERAN obeys this additional condition. Typically, rate changes occur far less often than every 40ms and this  additional condition is de facto no disadvantage, but not obeying it causes either the need for transcoding or severe quality degradations.

The Codec Type UMTS_AMR allows the encoder changing the rate every frame. UMTS_AMR is therefore not TLCI-compatible to GERAN and not recommended for any use today.

The Codec Type UMTS_AMR2 obeys this additional condition and is therefore the recommended multi-rate Codec Type in UTRAN.
MTSI deploys the AMR, but allows a multitude of options for rate switching. The SDP Parameters "mode-change-capability" and "mode-change-period" (see TS 26.114 and RFC 4867) allow negotiating this GERAN-specific condition. Because it is typically unknown at call setup (or handover, or re-negotiation), where a call is routed to and which access is used at the remote end, it is recommended to always set mode-change-capability=2 in the SDP Offer. TS 26.114 table 6.1 mandates this. If that is not included in the SDP Offer or SDP Answer towards a CS Network with GERAN (and UTRAN), then it is unavoidable to insert transcoding. The safest way it to include mode-change-capability=2 also in every SDP Answer.

GERAN mandates an AMR Encoder switching the rate only one step up or down. This second  additional condition was intended for optimal channel decoding at the radio receiver side (most likelihood decoding in case of bad radio channels). Every GERAN mobile obeys this rule in uplink. In downlink, however, it is necessary to accept also other changes, because handover or other events may change the rate unpredictably. In good radio conditions, this is no problem and therefore this second additional condition is less stringent. Nevertheless, it is recommended to obey it by every AMR Encoder. The SDP Parameter "mode-change-neighbor" (see RFC 4867) allows negotiating this additional condition. If this is not achieved, then the call may continue without Transcoding: the degradation to be expected is less severe than transcoding and far less severe than mode switching in the wrong phase.

CS Networks are sending one speech frame in one RTP packet or one Iu PDU Type 0, typically one every 20ms. Several alternative transport solutions exist due to history and development of the standard, like AoTDM and AoIP, but these are simply selected depending on the version of the control protocol. This guarantees minimal transport delay and simple interworking.

RCF 4867, however, allows a multitude of packing options, e.g. packing of multiple speech frames into one RTP packet in order to reduce the number of packets per second and to reduce the packet overhead. This increases the speech path delay. RCF 4867 allows also sending a speech frame redundantly several times in several consecutive RTP packets in order to reduce the rate of lost frames. This increases the speech path delay, too. Other options are octet-aligned or bandwidth-efficient packing, or inclusion of CRC, or robust sorting. 
These different transport conditions are, however, no problem for TLCI-compatibility! If different packing methods are deployed along a speech path, then MGWs are inserted to re-pack and, if necessary deploy buffering, but they do not need transcoding, as long as both codecs at both sides of the MGW are TLCI-compatible with respect to the other rules.

GERAN transports the Codec Mode Request (CMR) endlessly repeated in every second speech frame, i.e. every 40ms. Always the "active Codec Mode Restriction" is sent; there is no "neutral" CMR-value defined. This little overhead (2 bit every 40ms) guarantees that the CMR-status is always clear and transmission errors are quickly healed, rate changes are achieved as fast as possible.
In UTRAN and the CS Core Network, Rate Control Commands are transported totally different: only on demand, i.e. only when a Codec Mode Restriction changes. It is important that the receiver of such a Rate Control Command on demand remembers always the latest received one. It needs a Rate Control Command Status-memory. MGWs in the CS Core Network terminating GERAN translate these different signalling means. That is no severe problem as long as no transmission errors occur. In case of transmission errors (e.g. loss of an on-demand Rate Control Command), it takes quite a while, until the error is detected and corrected. This is, however, not judged as a TLCI-compatibility problem.

In MTSI every RTP packet includes a CMR-bit-field. It was for long time not clearly mandated that this CMR-bit-field contains always the active Codec Mode Restriction, as mandated in GERAN. A CMR-code-point "CMR=15" was defined with ambiguous meaning, leading to severe interworking issues. Now (since 2015) this is clarified: CMR=15 has exactly the same meaning as "the active Codec Mode Restriction is equal to the maximum mode of the selected mode-set". An important consequence of this clarification has to be obeyed for the case that two different AMR mode-sets are selected at both sides of a MGW, e.g. AMR (mode-set=0,2,4) <==> MGW <==> AMR (mode-set=0,2,4,7). Such a situation can occur during a call, e.g. due to eSRVCC or handover. If the MGW receives CMR=15 on the left side, then it has to (!) translate this to CMR=4 on the right side. If the MGW does not perform this correctly, then the call may go into muting on the left side! If the MGW receives CMR=15 on the right side, equivalent to CMR=7, then it would be good (!) to translate this to CMR=4 on the left side. CMR=7 is - strictly speaking - outside the selected mode-set on the left side, but it's clear and tolerable for many receivers. Some receivers, however, ignore any CMR outside their mode-set, because RFC 4867 recommends this. This may cause interworking issues. CMR=15 on the left side is possible, but not recommendable due to this ambiguity and existing legacy equipment using CMR=15 in different ways. 
This Mode-Control additional condition is not a severe TLCI-compatibility problem, but MGWs have to obey it, otherwise transcoding would be required, or the call fails (muting), with no gain and only higher costs and quality degradation.

Some implementations have been observed in the past that did not obey a received CMR or Rate Control Command from the remote side, because they did not observe any local reason to restrict the rate. This behaviour is clearly outside the AMR standard and causes call failure (muting).

The 2G network always supports DTX in uplink and downlink, but may enable/disable DTX on the encoding side. In most 2G networks, DTX is enabled in uplink and supported in downlink, but may be disabled in downlink inside network-located transcoders. DTX works well end-to-end in both directions in case of TLCI. 3G networks always enable DTX in the encoding side (at least in UEs). The decoding side and all network elements in the path are required to be able to handle DTX, i.e. transport and decode SID frames.

### 11.4.3	AMR-WB and EVS-IO

The AMR-WB and the EVS AMR-WB IO are compatible Codec Types with 9 modes and bit rates each. The EVS AMR-WB IO is an integral part of the EVS Codec.

AMR-WB(0,1,2) is deployed world-wide in UTRAN as UMTS_AMR-WB(0,1,2)/SF=128 and in GERAN as FR_AMR-WB(0,1,2).

In VoLTE  (MTSI) the higher modes of AMR-WB are deployed, too, notably the highest mode 8 (23.85). AMR-WB (mode-set=8) is not TLCI-compatible to any other AMR-WB Configuration, because it does not fulfil the Maximum Rate Control principle. In order to allow TLCI-Interworking between GERAN, UTRAN and VoLTE, an AMR-WB Configuration has to be  used, that includes all lower modes up to its maximum, i.e. at least mode-set=0 or better, i.e. mode-set=0,1,2 is to be included in all Codecs in the path. It is recommended to deploy AMR-WB(), i.e. the AMR-WB with all 9 modes in VoLTE (MTSI), i.e. the full AMR-WB Configuration. All other AMR-WB Configurations, which include all lower modes up to their maximum, could also be used and TLCI would always be guaranteed.

A VoLTE<=>VoLTE call may use all 9 modes of AMR-WB () or EVS AMR-WB IO ().

A VoLTE <=> CS call with AMR-WB () <=> AMR-WB (0,1,2) may use the three lower modes, disallowing the higher modes by Maximum Rate Control: end-to-end Rate Control takes care that no mode higher than 2 is allowed: CMR≤2. Essential is, that the VoLTE-UE (any MTSI-client) follows the Rate Control commands strictly and as fast as possible. An important rule for Codec Negotiation is set in RFC 4867: "If an SDP Offer is received without a mode-set, then the Selected Codec may contain any mode-set, or no mode-set". The SDP Offer without mode-set is called "Open Offer" in TS 26.114. The SDP Answer without mode-set is called "Open Answer".

AMR-WB(0,1,2) in end-to-end TLCI is better than AMR-WB(0,1,2) plus transcoding to AMR-WB(8).

Interworking between any AMR-WB mode-set and any EVS-IO mode-set is always transcoding free, if the mode-set are compatible, i.e. include all lower modes in common up to a certain maximum common mode.

### 11.4.3.1 Additional Conditions for TLCI-Compatibility for AMR-WB and EVS-IO

### The same additional conditions as for AMR apply also for AMR-WB and EVS AMR-WB IO.

### 11.4.4	EVS

The most recent 3GPP Codec is the Codec for Enhanced Voice Services (EVS). EVS supports four different audio bandwidths (NB, WB, SWB and FB) and a wide range of bit rates (5.6-VBR, 7.2 up to 128 kbps). The AMR-WB is included within the EVS as "EVS AMR-WB IO", in short EVS-IO in this document. EVS-CMR supports seamless transitions between all EVS Primary modes and between EVS Primary and EVS-IO modes during the call, as well as commanding EVS-VBR and EVS-CA modes of operation. Again, as for AMR and AMR-WB, all Codecs in the speech path have to follow the EVS-CMR rules strictly.

As stated above: All EVS Bottom up Configurations are TLCI-compatible to each other and using EVS Bottom up Configurations in SIP/SDP negotiation guarantees always best possible interworking.

A call may be setup with different, TLCI-compatible EVS Bottom up Configurations in the path, reflecting different preferences or limitation of the involved UEs and different interworking operators. An important example is 
EVS (br=5.9-13.2; bw=nb-swb) in CS-UTRAN, EVS (br=5.9-24.4; bw=nb-fb) in CS-CN and EVS (br=5.9-128; bw=nb-fb) in IMS. The common "Framework Bottom up Configuration" in this example is EVS (br=5.9-13.2; bw=nb-swb) and guarantees end-to-end TLCI-compatibility. Mode Control by EVS-CMR during the call may adapt the active EVS Configurations in both directions of the speech path to changing transport conditions by limiting maximum bit rate (and possibly audio bandwidth). 
In case CS-UTRAN is (temporarily or locally) overloaded, it could downgrade to EVS (br=5.9-8; bw=nb-wb). 
In case CS-UTRAN is free of load, it could upgrade to EVS (br=5.9-24.4; bw=nb-fb) for best possible quality.
In VoLTE<=>VoLTE calls the full bit rate and bandwidth could be used.

A call may also be setup with different, TLCI-compatible single audio bandwidth EVS Configurations in the path, reflecting different preferences or limitation of the involved UEs and different interworking operators. An important example is EVS (br=9.6-13.2; bw=swb) in CS and EVS (br=9.6-128; bw=swb) in IMS. The resulting common "Framework Configuration" is EVS (br=9.6-13.2; bw=swb) and guarantees end-to-end TLCI-compatibility. Mode Control by EVS-CMR during the call may adapt the active EVS Configurations in both directions of the speech path to changing transport conditions by limiting maximum bit rate (keeping the maximum audio bandwidth).
In case CS-UTRAN is (temporarily or locally) overloaded, rate limitation to 9.6 is allowed.).
In case CS-UTRAN is free of load, the rate limitation is lifted to 13.2.
In VoLTE<=>VoLTE calls the full bit rate and swb could be used.

Every EVS implementation includes the EVS AMR-WB IO. Every offered and selected EVS Configuration has to include a parallel EVS-IO Configuration. EVS Primary mode Configurations are strictly speaking not TLCI-compatible to EVS-IO Configurations. However, the EVS Codec allows by design a seamless (i.e. inaudible) transition between both modes of operation. This is important for interworking between EVS and AMR-WB, especially when mid-call modifications occur, like eSRVCC, or other handovers or when mid-call services are invoked. A call may be setup end-to-end with any EVS Configuration and a seamless transition to EVS AMR-WB IO (0,1,2) allows continuation, without transcoding, after a remote eSRVCC to AMR-WB (0,1,2).

### 11.4.4.1 Additional Conditions for TLCI-Compatibility for EVS

All EVS-Bottom up Configurations are TLCI-compatible to each other. An Initial SIP Offer with an EVS Bottom up Configuration should always find a remote partner that can accept it, unless the other partner explicitly reject EVS Bottom up Configurations. The SDP response may include a smaller EVS Bottom up Configuration.

Within such an offered and selected EVS Bottom up Configuration, some single-audio bandwidth EVS Configurations may be "emulated", as long as all additional conditions allow this. Including an EVS-IO Bottom up Configuration in the SIP/SDP negotiation guarantees also TLCI-compatibility to AMR-WB (0,1,2), provided the additional conditions as for AMR-WB are obeyed.

All single audio bandwidth Configurations sharing the same lowest bit rate limit are TLCI-compatible to each other. Similarly to EVS-Bottom up configurations, TLCI-compatibility to AMR-WB(0,1,2) is guaranteed, provided the additional conditions as for AMR-WB are obeyed.

As long as Bottom up Configurations are not widely accepted, an Initial SIP/SDP Offer should not only include punctured EVS Configurations and not only Bottom up Configurations, because the answerer may not be able to accept it.

As for AMR and AMR-WB, the RTP packing rules allow a multitude of options. If different RTP packing options are used on both sides of a MGW, then the MGW repacks, but transcoding is not required. In CS Networks, it is mandatory to send one speech frame in one packet.

## 11.5	Transcoding Free Operation at call setup

======> snip <======

## 11.6	Transcoding Free Operation after Handover

End of Changes