3GPP TSG SA4 Meeting #88 	S4-160542

# Memphis, USA, 2016-04-18

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.103 | CR | 0044 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing Redundancy in EVS Configurations for CS Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVSoCS |  |  |  |  |  |  | Date: |  | 2016-04-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | The relevant specifications for EVSoCS were completed in Rel-13, with some minor points left open for further completion. One item is related to an agreement at the closing plenary of SA4#87 to remove a paragraph from the LS to CT/RAN groups. The removed paragraph addressed why SA4 could not decide on a default network configuration. Note that SA4 made no decision on a default network configuration although CT/RAN groups explicitly adviced SA4 to choose one default network configuration, namely Set 1 (5.9-13.2). This agreement was based on the intention expressed at the closing plenary of SA4#87 to remove Set 3 (9.6-13.2) under the condition that Set 3 (9.6-13.2) can be fully emulated from Set 1 (5.9-13.2). Set 3 (9.6-13.2) can be emulated with no system impact out of Set 1 (5.9-13.2) so Set 3 is redundant and is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EVS Configuration Set 3 is redundant and is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary increased implementation complexity due to support of an additional redundant configuration (Set 3) in EVSoCS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start change 1 ***

## 5.7A	EVS Codec Type (UMTS_EVS)

The EVS Codec type [25]…[38] is applied in UTRAN and the CS core network, with the Codec IDentification (CoID) code as follows: 
UMTS_EVS_CoID	:= 0x0000.1110.

The EVS Codec Type can be used in conversational speech telephony services in a number of different configurations. These allowed configurations for CS networks are defined in Table 5.7A-1.

The combination of the EVS Codec Type with one of these allowed configurations for CS networks is abbreviated with "EVS (set x)", e.g. EVS (set 1).

Table 5.7A-1: Allowed Configurations for the EVS Codec Type

| Configuration→ (Config-EVS-Code or Set) ↓Codec Rate (kbps) | 0 | 1 | 2 |  |
| --- | --- | --- | --- | --- |
| Typical SF on downlink | 256 | 128 | 64 |  |
| EVS Primary 24.4 |  |  | nb-fb |  |
| EVS Primary 16.4 |  |  | nb-fb |  |
| EVS Primary 13.2 |  | nb-swb | nb-swb |  |
| EVS Primary 9.6 |  | nb-swb | nb-swb |  |
| EVS Primary 8 | nb-wb | nb-wb | nb-wb |  |
| EVS Primary 7.2 | nb-wb | nb-wb | nb-wb |  |
| EVS Primary 5.9VBR (NOTE 2) | nb-wb | nb-wb | nb-wb |  |
| EVS Primary SID | nb-wb | nb-swb | nb-fb |  |
| EVS AMR-WB IO 12.65 |  | wb | wb |  |
| EVS AMR-WB IO 8.85 |  | wb | wb |  |
| EVS AMR-WB IO 6.60 | wb | wb | wb |  |
| EVS AMR-WB IO SID (NOTE 3) | wb | wb | wb |  |
| NOTE 1: Every Speech or SID payload field is complemented by appending the 7-bit EVS-CMR field for Maximum Rate and Bandwidth Control. For details, see TS 26.454 [38]. NOTE 2: From a transmission point of view, the EVS Primary 5.9 VBR mode of operation includes three bit rates of 2.8, 7.2 and 8.0 kb/s to achieve the 5.9 kb/s average bit rate in active speech segments. Together with EVS Primary SID in speech pauses and the EVS-CMR appended in every Speech and SID frame, the total average net bit rate, assuming 40% speech pause and 60% active speech is in the order of 4.4 kb/s. |  |  |  |  |

Table 5.7A-2 defines the coding of the "Single Codec" information element for the EVS Codec Type in a BICC-based CS core network.

Table 5.7A-2: Coding of "Single Codec" for the EVS Codec Type

| Octet | Parameter | MSB 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 m | Single Codec | Single Codec (see ITU-T Q.765.5) |  |  |  |  |  |  |  |
| 2 m | Length Indication | 4, 5 |  |  |  |  |  |  |  |
| 3 m | Compat. Info | Compatibility Information |  |  |  |  |  |  |  |
| 4 m | OID | ETSI OID (See ITU-T Q.765.5 [6]) |  |  |  |  |  |  |  |
| 5 m | CoID | UMTS_EVS_CoID |  |  |  |  |  |  |  |
| 6 m | Config-EVS | (spare) | (spare) | (spare) | (spare) | Config-EVS-Code |  |  |  |

Config-EVS-code has the following allowed values: 0, 1, 2. All other values are reserved for future use.

Mandatory for UTRAN terminals supporting UMTS_EVS:
If an UTRAN terminal offers Codec Type UMTS_EVS in the Supported Codec List Information Element, then all allowed EVS Configurations for CS networks shall be supported by this UTRAN terminal. The UTRAN terminal shall always apply Discontinuous Transmission (DTX) in uplink direction and shall accept in downlink direction all received EVS Codec Modes and DTX.

NOTE:	Mandatory EVS configurations for UTRAN and CS core network infrastructures for UMTS_EVS are for further study.NOTE 2:	The rules for interworking on user plane between the allowed EVS Configurations for CS networks and the other EVS Configurations (e.g. for MTSI) are defined in TS 26.454 [38]. Also the rules for Maximum Rate and Bandwidth Control in CS networks are defined in TS 26.454 [38].

*** End change 1 ***