3GPP TSG-SA4 Meeting #89 	S4-160641

# Kista, Sweden, 27. Jun - 1. Jul 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.454 | CR | xxxx | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 7 Uu Interface User Plane for EVSoCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM; |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVSoCS |  |  |  |  |  |  | Date: |  | 2016-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - It is currently unspecified that the UE shall always use the highest possible rate and audio bandwidth. This mandatory requirement is added in order to guarantee best possible quality for the remote end and in order to prevent egoistic usage of low rate modes, e.g. to save battery power. - A clarification is necessary for the Transport Layer implementation within the UE to prevent blocking the higher EVS rates during a short transition period from a higher rate to a lower rate, if this rate reduction is caused by the UE autonomous mode or an RNC command. Note: the problem is not existent if the rate reduction is commaned by the DL EVS-CMR. - Some remaining [] brackets are removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that the highest possible rate is mandatory. Clarify the Transport Layer implementation. Remove the []. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unwanted behaviour of UEs may occur in the market, leading to reduced quality perception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Begin of first change

# 7	Uu Interface User Plane (UE-RAN)

## 7.1	General

The interface between the EVS Encoder and Decoder within the UE and the Radio Access Network within the UE is an UE-internal interface and is not detailed in the present document. The mapping of Speech, SID and EVS-CMR bits corresponds to the mapping described in clause 6 for the Iu interface.

The UE shall decode every Speech or SID frame received in downlink direction, regardless of the EVS mode of operation in uplink.

The UE shall always apply in uplink the "Source Controlled Operation" (SCR), also called "Discontinuous Transmission" (DTX).

If the UE uses the EVS AMR-WB IO mode of operation to encode speech for the uplink direction, then it shall restrict changes of the Codec Mode to every other frame (40 ms grid) and to neighbouring modes.

The UE shall always use the highest rate of the set of available rates that complies with the latest received DL EVS CMR and its major operation mode.

NOTE: 	If the control logic for the UE autonomous rate and/or an RNC command modifies the set of available rates, then the EVS-Encoder is informed and decreases or increases its encoding rate as soon as possible. It takes a short while, until the speech frames, sent to the Transport Layer, comply with the modified set of available rates. The Transport Layer of the UE allows the Encoder to adapt and does not block currently used higher rates for the next [60ms] after the information about the highest allowed rate has been sent to the Encoder.

## 7.2	Determination of the local EVS encoding mode

Table 7.2-1: Local EVS encoding mode and Interworking

| # | Item | Interworking | Motivation/comment/discussion |
| --- | --- | --- | --- |
| 1 | Set of available rates | The UE shall confine the EVS encoder operation to modes, whose rates are within the Set of available rates. | The UE may otherwise not be able to send. |
| 2 | DL EVS-CMR | The EVS-UE shall take into account a DL EVS-CMR as soon as possible, at maximum within 60 ms after reception on the radio interface. The UE shall follow the received DL EVS CMR, unless it is in conflict with the Set of available rates, in which case the CMR conflict resolution applies. | Shortest possible reaction time is required for optimal adaptation speed. Typically, the UE uses the highest rate and bandwidth of the DL EVS-CMR for encoding in uplink. |
| 3 | CMR conflict resolution | If the received DL EVS-CMR is outside the Set of available rates, then the rate requested by DL EVS-CMR shall be changed to the closest rate supported by the Set of available rates that complies with the Major operation mode of the DL EVS-CMR. That is: if DL EVS-CMR corresponds to an EVS AMR-WB IO rate, then the requested rate is changed to an allowed EVS AMR-WB IO rate; if DL EVS-CMR corresponds to an EVS Primary mode, then the requested rate is changed to an allowed EVS Primary rate and bandwidth. | A change of the Major operation mode (EVS primary <=> EVS AMR-WB IO) can only be initiated through DL EVS-CMR via the CN, but not through another change of the Set of available rates. Otherwise, such a change of the major operation mode from EVS AMR-WB IO to EVS primary would result in an interworking failure, if the remote UE is an AMR-WB-only phone. A change from EVS primary to EVS AMR-WB IO would result in non-optimal audio quality, if the remote UE is an EVS phone. The MSC guarantees through GBR that at least the lowest AMR-WB IO mode and at least the lowest EVS Primary mode of the EVS Configuration for UE remain selectable. The RNC behaviour and GBR setting is subject to operator policy. The UE reduces/increases the requested rate to the closest rate of the requested Major operation Mode, supported by the Set of available rates. If the requested Major operation mode equals the EVS primary mode, then the rate is modified within the set of rates of 24.4, 16.4, 13.2 (non-CA), 9.6, 8, 7.2, as far as these are within the EVS Configuration for UE. If requested Major operation mode equals the EVS AMR-WB IO mode, the rate is modified within the set of rates of 12.65, 8.85 and 6.60, as far as these are within the EVS Configuration for UE. |
| 4 | UE autonomous rate | The control logic for the UE autonomous rate shall guarantee that at least the lowest AMR-WB IO mode and at least the lowest EVS Primary mode of the EVS Configuration for UE are included in the Set of available rates. In addition, this control logic shall guarantee that the Set of available Rates at least includes the lowest EVS Primary rate supporting the audio bandwidth requested by the DL EVS-CMR (after mapping the DL EVS-CMR into the EVS Configuration for UE). | Otherwise, the result might be interworking failure, if the remote UE is an AMR-WB only phone, or the result may be non-optimal audio quality. |
| 5 | EVS Channel Aware mode (EVS-CA) | The EVS-CA mode shall solely be selected through DL EVS-CMR. The UE shall not enter or leave the EVS-CA based on RNC command or its autonomous mode. | The remote UE may request and the CN may select or deselect the EVS CA mode, e.g. based on the perceived transmission channel condition, see also clause 11. |
| 6 | EVS VBR mode (EVS-VBR) | The EVS-VBR mode shall solely be selected through DL EVS-CMR. The UE shall not enter or leave the EVS VBR based on RNC command or its autonomous mode. | The use of the EVS-VBR mode requires that the upper limit of the Set of available rates is greater or equal to 8 kbps. A prerequisite for ensuring the EVS VBR mode is a GBR of at least 8 kbps. |

End of changes