3GPP TSG-SA4 Meeting #90	S4-161009

Ljubljana, Slovenia, September 5–9, 2016

Source:	Qualcomm, Incorporated

Title:	Test plan proposal for conducting delay and speech quality over the headset jack.

Document for:	Discussion and agreement

Agenda Item:	10.7

[Editor’s Note]: This contribution proposes that a new draft TR is initiated containing a test plan for conducting delay and speech quality over the headset electrical interface. It is further proposed that an initial draft is populated with the text below, to be edited and progressed as needed at subsequent meetings.

# Scope

This Technical Report provides guidance on testing of speech quality and delay through a UE headset electrical interface, including both analog and digital headset interfaces. The Technical Report includes test set-up configurations, measurement scenarios and range of results that can be expected in measurements.  It is envisioned that this Technical Report will serve as a reference for UE vendors and Mobile Network Operators wishing to conduct such tests.

# Introduction

Testing of speech transmission quality and delay through the UE headset electrical interface is a common practice in the industry for troubleshoot and benchmark of network and UE performance. The UE headset electrical interface provides a convenient access point for testing that removes acoustic interface related aspects from the measurement. Test equipment (TE) solutions designed to connect to the UE headset electrical interface are commercially available and target both “drive-test” as well as lab environment testing applications. However, the lack of a standardized test methodology causes difficulties in interpreting results between different TEs, leading to possibly misleading conclusions on both network and UE performance.

This Technical Report attempts to address this problem by providing a unified test procedure methodology.

# Test Configurations

3.1 Setup for Terminals

The general test setup configuration and access to the UE is described in Figure 1.

Figure 1 – Test Configuration for headset electrical interface

The preferred way of testing is the connection of a UE to the system simulator with exact defined settings and access points. The test sequences are fed in either electrically using a reference codec or using the direct signal processing approach.

The system simulator shall simulate the access network and core network including the speech encoding/decoding specified for the test (e.g. AMR, AMR-WB or EVS) but excluding further transcoding beyond linear PCM, see Figure 1.

The output of the test system connected to the interface in Send of the headset connector must be DC resistant. The output impedance shall be between 1 Ω and 10 kΩ. The dynamic range of the test system shall be consistent with (or exceed) the level range provided by headset microphones.

The input of the test system connected to the receiving interfaces of the headset connectors shall have an input impedance of 32 Ω. The dynamic range shall be consistent with (or exceed) the output level range provided by the electrical output of digital mobile terminals' headset outputs.

The common ground impedance (between sending and receiving sides) for the test system shall be ≤ 0.05 Ω.

Unless specified otherwise for the respective test, the radio conditions on the air interface shall have a block error rate of 0% and the jitter in the IP transport for MTSI-based speech shall be ≤ 1 ms.

In case of MTSI-based speech, the reference client shall allow to synchronize to the clock of the device under test and include a de-jitter buffer to equalize possible jitter in the signal received from the UE.

When operating with synchronized clock, the de-jitter buffer shall be a static de-jitter buffer and the jitter buffer management shall not compensate for clock drift. The reference client shall not lose or discard packets, shall not trigger retransmission, and shall not use error concealment or time-warping. The initial jitter buffer size (filling level) shall be higher than the maximum expected network jitter and the maximum jitter buffer size shall be at least twice the initial size. During jitter buffer reset, the de-jitter buffer shall be emptied/filled to the initial buffer size. In case of buffer over- or underruns, the reference client shall give a warning and it shall be reported.

NOTE 1: A static de-jitter buffer is a first-in-first-out (FIFO) buffer which at the beginning buffers packets until a given initial buffer size is reached. Due to changing network delays the filling level of the de-jitter buffer can change, but the sum of network delay and jitter buffer delay is constant (as opposed to an adaptive jitter buffer management). The filling level of the de-jitter buffer represents the de-jitter buffer delay.

For measurements with unsynchronized clock e.g. the measurement of clock drift, jitter buffer over- and underruns cannot be avoided due to the unsynchronized clocks. Under the assumption of jitter-free condition the initial jitter buffer size (filling level) shall be chosen such that the maximum clock drift can be compensated without any loss of packets for a given time. For the measurement of clock drifts the jitter buffer size should be chosen such that for clock drift of up to 100ppm no loss of packets due to buffer over- or under-run shall occur for a sequence of 160s.

Unless stated otherwise, for LTE connections, the system simulator shall be configured for FDD operation, with a default or dedicated bearer and reference measurement channel scheduling that provides enough resource block allocation for transmitting a full speech packet within a transmission time interval of 1ms. No HARQ re-transmissions shall occur. TDD operation, TTI bundling, connected DRX and other forms of scheduling (e.g. SPS) are for further study.

The test setup has to ensure proper clock synchronization of the test equipment to the UE. Clock drift shall be negligible and packet loss shall not occur during the test.

NOTE 2: Any clock drift may result in improper delay calculation or in wrong positioning of the analysis window

## 3.2	Setup of the electrical interfaces

#### 3.2.1	Codec approach and specification

In this approach, a codec is used to convert the digital input/output bit-stream of the system simulator to the equivalent analogue values. With this approach a system simulator simulating the radio link to the terminal under controlled and error-free conditions is required, unless otherwise specified for the respective test. The system simulator has to be equipped with a high-quality codec with characteristics as close as possible to ideal.

Definition of 0 dBr point:

D/A converter -	a Digital Test Sequence (DTS) representing the codec equivalent of an analogue sinusoidal signal with an RMS value of 3,14 dB below the maximum full-load capacity of the codec shall generate 0 dBm across a 600 ohm load;

A/D converter -	a 0 dBm signal generated from a 600 ohm source shall give the digital test sequence (DTS) representing the codec equivalent of an analogue sinusoidal signal with an RMS value of 3,14 dB below the maximum full-load capacity of the codec.

Narrowband telephony testing

For testing of a GSM, 3G or LTE terminal supporting narrowband telephony, the system simulator shall use the AMR speech codec as defined in the 3GPP TS 26 series of specifications, at the source coding bit-rate of 12,2 kbit/s, unless otherwise specified.

Wideband telephony testing

For testing of a GSM, 3G or LTE terminal supporting wideband telephony, the system simulator shall use the AMR-WB speech codec as defined in 3GPP TS 26 series of specifications, at the source coding bit-rate of 12,65 kbit/s, unless otherwise specified.

Super-wideband telephony testing

For testing of a LTE terminal supporting super-wideband telephony, the system simulator shall use the EVS speech codec as defined in 3GPP TS 26 series of specifications at 32 kHz sampling rate, mono, at the source coding bit-rate of 24,4 kbit/s, unless otherwise specified. The system simulator should support all operation modes of EVS speech codec, the complete EVS RTP payload format, and all SDP attributes as specified in TS 26.445 Annex A; it should be possible to control all the SDP attributes declared and negotiated by the system simulator.

Fullband telephony testing

For testing of a LTE terminal supporting fullband telephony, the system simulator shall use the EVS speech codec as defined in 3GPP TS 26 series of specifications at 48 kHz sampling rate, mono, at the source coding bit-rate of 24,4 kbit/s, unless otherwise specified.

### 3.2.2	 Direct digital processing approach

In this approach, the digital input/output bit-stream of the terminal connected through the radio link to the system simulator is operated upon directly.

Narrowband telephony testing

For testing of a GSM, 3G or LTE terminal supporting narrowband telephony, the system simulator shall use the AMR speech codec as defined in the 3GPP TS 26 series of specifications, at the source coding bit-rate of 12,2 kbit/s, unless otherwise specified.

Wideband telephony testing

For testing of a GSM, 3G or LTE terminal supporting wideband telephony, the system simulator shall use the AMR-WB speech codec as defined in the 3GPP TS 26 series of specifications, at the source coding bit rate of 12,65 kbit/s, unless otherwise specified.

Super-wideband telephony testing

For testing of a LTE terminal supporting super-wideband telephony, the system simulator shall use the EVS speech codec as defined in 3GPP TS 26 series of specifications in super-wideband mode, at 32 kHz sampling rate, mono, at the source coding bit-rate of 24,4 kbit/s, unless otherwise specified. The system simulator should support all operation modes of EVS speech codec, the complete EVS RTP payload format, and all SDP attributes as specified in TS 26.445 Annex A; it should be possible to control all the SDP attributes declared and negotiated by the system simulator.

Fullband telephony testing

For testing of a LTE terminal supporting fullband telephony, the system simulator shall use the EVS speech codec as defined in 3GPP TS 26 series of specifications in fullband mode, at 48 kHz sampling rate, mono, at the source coding bit-rate of 24,4 kbit/s.

## 3.3	Accuracy of test equipment

Unless specified otherwise, the accuracy of measurements made by test equipment shall exceed the requirements defined in table 1a.

Table 1a: Test equipment measurement accuracy

| Item | Accuracy |
| --- | --- |
| Electrical Signal Power | ± 0,2 dB for levels  -50 dBm |
|  | ± 0,4 dB for levels < -50 dBm |
| Time | ± 5% |
| Frequency | ± 0,2% |

Unless specified otherwise, the accuracy of the signals generated by the test equipment shall exceed the requirements defined in table 1b.

Table 1b: Test equipment signal generation accuracy

| Item | Accuracy |
| --- | --- |
| Electrical excitation levels | ± 0,4 dB (see note 1) |
| Frequency generation | ± 2% (see note 2) |
| NOTE 1: Across the whole frequency range. NOTE 2: When measuring sampled systems, it is advisable to avoid measuring at sub-multiples of the sampling frequency. There is a tolerance of ± 2% on the generated frequencies, which may be used to avoid this problem, except for 4 kHz where only the -2% tolerance may be used. |  |

The measurements’ results shall be corrected for the measured deviations from the nominal level.

The sound level measurement equipment shall conform to IEC 60651 Type 1.

## 3.4	Test signals

Unless stated otherwise, appropriate test signals for GSM/3G/LTE acoustic tests are generally described and defined in ITU-T Recommendation P.501 [22].

More information can be found in the test procedures described below.

For testing the narrowband telephony service provided by the UE, the test signal used shall be band limited between 100 Hz and 4 kHz with a bandpass filter providing a minimum of 24 dB/oct. filter roll-off, when feeding into the receiving direction.

For testing the wideband telephony service provided by the UE, the test signal used shall be band limited between 100 Hz and 8 kHz with a bandpass filter providing a minimum of 24 dB/oct. filter roll-off, when feeding into the receiving direction.

For testing the super-wideband telephony service provided by the UE, the test signal used shall be band limited between 50 Hz and 16 kHz with a bandpass filter providing a minimum of 24 dB/oct. filter roll-off, when feeding into the receiving direction.

For testing the fullband telephony service provided by the UE, the test signal used shall be band limited between 20 Hz and 20 kHz with a bandpass filter providing a minimum of 24 dB/oct. filter roll-off, when feeding into the receiving direction.

The test signal levels are referred to the average level of the (band limited in receiving direction) test signal, averaged over the complete test sequence, unless specified otherwise. For real speech, the test signal levels are referred to the ITU-T P.56 [37] active speech level of the (band limited in receiving direction) test signal, calculated over the complete test sequence.

# Test Scenarios

The test scenarios cover a number of typical use scenarios for IMS based speech services over LTE. The scenarios are described as follows:

- Scenario 1: Impairments at the physical layer (RF Impairments)

- Scenario 2: Impairments at the internet layer (IP Impairments)

- Scenario 3: Operation with different modes/bit-rates

- Scenario 4: Switching bit-rates / codec modes during a call

- Scenario 5: Codec change initiated by remote UE

- Scenario 6: Intra-RAT Handover (e.g. LTE to LTE)

- Scenario 7: Inter-RAT Handover (e.g. LTE to UMTS SRVCC)

- Scenario 8: Operation with DRX

- Scenario 9: Simultaneous data download/upload during a voice call

[Editor’s Note]: Further inputs are necessary to populate the test methodology for each scenario:

- Specific speech material used

- Delay and MOS-LQO calculation methods

- Number of measurement repeats and statistical processing of the data

- Time-lines for the measurement (e.g. at what point should a handover be performed, how long should it last, etc.)

## 4.1  Scenario 1:   Impairments at the physical layer

This test scenario simulates situations where less than ideal RF operating conditions exist for the UE.  The purpose of the test is to verify that the UE operates as expected under those situations.

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall be able to implement the RF fading profiles from TS 36.XXX. At a minimum, the EPA5, EPA7 and EVA70 fading profiles should be supported.

Measurements are performed both in the send and receive directions.

## 4.2  Scenario 2:   Impairments at the internet layer  (IP impairments)

This test scenario simulates situations where less than ideal EPC operating conditions exist for the UE.  The purpose of the test is to verify that the UE (and more specifically, its jitter buffer management) operate as expected under those situations.

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall be able to introduce RTP packet delay and loss impairments. At a minimum, the system simulator shall be able to introduce RTP packet delays according to Gaussian and uniform distributions.

Measurements for this scenario are performed in the receive direction only.

## 4.3  Scenario 3:   Operation with different modes / bit-rates

This test scenario simulates operation of the UE at different bit-rates. The purpose of the test is to verify that the UE has implemented the speech encoder / decoder appropriately.

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall be able to configure the bit-rate / operation mode of the speech codec being used in the call. Different bit rates / operation modes are tested and MOS-LQO scores are compared to expected values.

Measurements for this scenario are performed in the send and receive directions.

## 4.4  Scenario 4:   Switching bit-rates / operating modes during a call

This test scenario simulates the situation where bit-rates / operating modes of a codec are switched during a call. The purpose of the test is to verify that the UE has implemented the speech encoder / decoder appropriately and that the speech quality after a bit-rate / mode-switch is as expected.

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall be able to initiate a codec rate change during a call. The codec rate change is initiated and completed during a speech pause. The MOS-LQO scores for the speech material after the codec rate change are compared to expected values.

Measurements for this scenario are performed in the send and receive directions.

## 4.5  Scenario 5:   Codec change initiated by remote UE

This test scenario simulates the situation where a codec change is initiated by a remote UE, e.g. when the remote UE uses SRVCC to handover between an LTE area with wideband speech coverage and an UMTS area with narrowband speech coverage only. The purpose of the test is to verify that the UE can successfully perform the codec change requested and that the speech quality after the codec rate change

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall be able to initiate a codec rate change during a call. The codec rate change is initiated and completed during a speech pause. The MOS-LQO scores for the speech material after the codec rate change are compared to expected values.

Measurements for this scenario are performed in the send and receive directions.

## 4.6  Scenario 6:   Intra-RAT Handover

This test scenario simulates the situation where a UE performs a handover within a single radio access technology. The purpose of the test is to verify that no speech quality degradation exists after the handover is completed.

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall be able to initiate an intra-RAT handover. The handover is initiated and completed during a speech pause. The MOS-LQO scores for the speech material before and after the handover are compared.

Measurements for this scenario are performed in the send and receive directions.

## 4.7  Scenario 7:   Inter-RAT Handover

This test scenario simulates the situation where a UE performs a handover within two different radio access technologies. The purpose of the test is to verify that no speech quality degradation exists after the handover is completed.

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall be able to initiate an inter-RAT handover from LTE to UMTS (SRVCC). The handover is initiated and completed during a speech pause. The MOS-LQO scores for the speech material before and after the handover are compared.

Measurements for this scenario are performed in the send and receive directions.

## 4.8  Scenario 8:   Operation with DRX

This test scenario simulates the situation where a UE operates with discontinuous reception technology (DRX). The purpose of the test is to verify that no speech quality degradation exists due to the presence of DRX.

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall support DRX operation. The MOS-LQO scores for the speech material with and without DRX are compared.

Measurements for this scenario are performed in the send and receive directions.

## 4.9  Scenario 9:   Voice call with simultaneous data upload/download

This test scenario simulates the situation where a UE is performing an upload/download during a voice call. The purpose of the test is to verify that no speech quality degradation exists due to the upload/download operation

The UE is setup in a call with the system simulator. The system simulator used for this test scenario shall be able to upload/download data from/to the network. The MOS-LQO scores for the speech material with and without the data transfer are compared.

Measurements for this scenario are performed in the send and receive directions.

Table 1 - Test scenarios for calls initiated as AMR-WB

|  | Scenario | Initial NW | End NW | Scheduling | Initial codec | End codec | Initial rate | End rate | Data (UL/DL) | AWGN | SNR | Profile | IP Jitter  | IP Packet Loss | Delay | MOS-LQOswb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TX / RX | RF impairment | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | -70dBm | 10 | EPA 5 | none | none |  |  |
| TX / RX | RF impairment | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | -65dBm | 5 | EVA 70 | none | none |  |  |
| TX / RX | RF impairment | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | -85dBm | 20 | EVA 7 | none | none |  |  |
| RX | IP impairment | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | - | - | - | none | none |  |  |
| RX | IP impairment | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | - | - | - | 10ms | 1% |  |  |
| RX | IP impairment | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | - | - | - | 20ms | 2% |  |  |
| RX | IP impairment | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | - | - | - | 40ms | 3% |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 6.6 | 6.6 | - | - | - | - | none | none |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 8.85 | 8.85 | - | - | - | - | none | none |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | - | - | - | none | none |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 14.25 | 14.25 | - | - | - | - | none | none |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 15.85 | 15.85 | - | - | - | - | none | none |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 18.25 | 18.25 | - | - | - | - | none | none |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 19.85 | 19.85 | - | - | - | - | none | none |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 23.05 | 23.05 | - | - | - | - | none | none |  |  |
| TX / RX | Codec | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 23.85 | 23.85 | - | - | - | - | none | none |  |  |
| TX / RX | Codec rate change | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 23.85 | 12.65 | - | - | - | - | none | none |  |  |
| TX / RX | Remote UE SRVCC | LTE 1 | LTE 1 | RMC | AMR-WB | AMR | 12.65 | 12.2 | - | - | - | - | none | none |  |  |
| TX / RX | LTE Handover | LTE 1 | LTE 2 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | - | - | - | - | none | none |  |  |
| TX / RX | LTE->UMTS SRVCC | LTE 1 | UMTS 1 | RMC | AMR-WB | AMR | 12.65 | 12.2 | - | - | - | - | none | none |  |  |
| RX | DRX 20ms | LTE 1 | LTE 1 | DRX 20ms | AMR-WB | AMR-WB | 12.65 | 12.65 | - | - | - | - | none | none |  |  |
| RX | DRX 40ms | LTE 1 | LTE 1 | DRX 40ms | AMR-WB | AMR-WB | 12.65 | 12.65 | - | - | - | - | none | none |  |  |
| RX | Simultaneous data | LTE 1 | LTE 1 | RMC | AMR-WB | AMR-WB | 12.65 | 12.65 | 1 Mbps | - | - | - | none | none |  |  |