3GPP TSG-SA4 Meeting #90	S4-161008

Ljubljana, Slovenia, September 5–9, 2016

Source:	Fraunhofer HHI

Title:	VR video systems

Document for:	Discussion and agreement

Agenda Item:	11.6

# Introduction

One of the objectives of the Study Item FS_VR (Virtual Reality) as agreed in S4-160165 is as follows:

- Analysing the different technologies and equipment in place that provide a Virtual Reality experience.

This document provides an overview of the existing VR video systems.

# Video systems

2.1. Production and delivery

The following figure shows the basic workflow for production and delivery of VR video.

Capture:

| Camera systems | Examples |
| --- | --- |
| Mirror-based: - Avoids the parallax problem by providing a common optical center behind the mirrors - Parallax-free zone from 1m to infinity with an overlap of about 10 pixels | HHI OmniCam 360 |
| Segmented stereo: - Several stereo pairs in a star arrangement - Small overlap to enable stitching | Samsung Beyond, Next VR, VOKE VR |
| Stereo by extreme overlap: - Fisheye or wide-angle lenses - Each image sensor split into a left and right section to stitch left and right panoramas | Nokia OZO, Jaunt NEO, HHI OmniStar3D |
| Light field cameras: - Depth-based rendering to generate intermediate views (to reduce the number of cameras) - Enables parallax-free rendering - Need very accurate depth maps and sophisticated processing | Lytro Immerge |

Projection:

-     FFS, e.g pending outcome of MPEG Omnidirectional Media Application Format (OMAF) activity [X]

Encoding/Decoding:

- General video decoder requirements specified in TS 26.234 [Y] and TS 26.346 [Z]

- FFS: motivation for higher levels?

Packaging:

- Tile-based (overlapping or non-overlapping)

- Non-tile-based (complete scene)

Delivery

- Partial scene

- Complete scene

- Region adaptive bitrate, quality or resolution

Rendering:

- FFS: Requires support for selected projection methods above, if any.

# Proposal

It is proposed to include section 2 into the section 4.2 of TR 26.918 on Virtual Reality.

# References

# [X] MPEG#115 (Geneva), w16189, WD on ISO/IEC 23000-20 Omnidirectional Media Application Format

[Y] 3GPP TS 26.234 : Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs. V13.2.9, June 2016

[Z] 3GPP TS 26.346 : Technical specification group services and system aspects; multimedia broadcast/multicast service (MBMS); Protocols and codecs. V13.5.0, June 2016