Source:	Samsung Electronics Co., Ltd.

Title:	VR FoV Streaming

Document for:	discussion and agreement

Agenda Item:	17.8

# Introduction

3GPP SA4 is working on a study item on Virtual Reality and is addressing use cases on streaming of 360 video.

In this contribution, we discuss the use case of 360 video streaming over unicast and the support for Field-of-View streaming to improve the end-to-end video quality and reduce the bandwidth requirements.

# Discussion of 360 Video Streaming

The 360 video content may be available at the PSS server as an ISOBMFF compatible file. Sufficient internal or external metadata is available to inform the PSS server that this content is 360 video and not simply regular content. For instance, MPEG is currently working on the definition of the VR metadata for storage as part of ISOBMFF files. The PSS client must be aware of the nature of the VR content to be able to render it correctly. However, the PSS server does not necessarily need to take any particular precautions to be able to serve VR content.

When no particular measures are taken from the PSS server to support VR streaming, the bandwidth requirements will typically be very high ranging from 15 to 20 Mbps to serve a 4K view of the whole 360 video scene at an acceptable frame rate (usually either 30 or 60 fps).

At the rendering side, the PSS client must be aware of the exact characteristics of the video, such as the projection format, the packing format, the used geometry and others. The following figure depicts stereo perspective projection on a 360 video that uses a sphere geometry.

The stereo camera represents a virtual camera that is mimicking the human eyes. In this example, the 360 video is warped around the sphere geometry and the area that is shown to the user corresponds to the perspective projection of that sphere onto the far plane of the perspective camera. The camera is also configured with the horizontal and vertical field of views of the HMD that will be used for rendering as well as any pixel aspect ratio and lens distortion correction considerations.

Depending on these parameters, it becomes obvious that the actual rendered video will only cover a portion of the whole scene, which provides room for optimizing the bandwidth usage.

As part of this study, the following aspects need to be addressed:

- Identify the metadata that is necessary to identify the content as VR content and enable its rendering. This should be done by referencing the appropriate MPEG tools for this purpose.

- Study different ways of optimizing streaming and hybrid delivery of VR content, potentially using FoV streaming techniques

# FoV Streaming Gap Analysis and Working Assumptions

Delivering 360 video content is very delay sensitive and bandwidth extensive in order to enable the immersive experience that this type of content is meant to convey to the end users. However, designing for these requirements might result in an advert behavior, especially with the dynamic environment of Internet streaming. The user experience may be prone to frequent playback interruptions for re-buffering. At the same time, most of the content of the video will not be visible to the user depending on the current direction where the user is looking as well as on characteristics of the Head-Mounted Display (HMD). As a consequence, the use of a tiling mechanism to partition the original video into smaller tiles and then distributing the minimum number of tiles that cover the current user’s viewport, has been considered extensively.

In such a solution, the player at the HMD is required to react to user head movements by determining the new viewport and then calculating the required tiles to cover that viewport. Due to the network and DASH delays, this cannot happen fast enough to address the motion-to-photons latency requirement of VR. Consequently, the full scene of the video must always be available at the renderer to enable correct rendering.

The following table depicts the possible approaches to achieve FoV streaming:

|  | 1 video decoder | 2 or more video decoders |
| --- | --- | --- |
| SNR optimization | HEVC tiling with lower QP for ROI. Sender must be able to switch tiles or make multiple encodings available each with a different ROI. | Separate tiles taken from same 360 video with same resolution but different QP encoding (e.g. both base view and tiles are taken from 4K video) |
| Resolution optimization | Frame packing to arrange ROI in high resolution with low resolution base view. | Separate tiles taken from higher resolution video (e.g. tiles from 8k video and base view in HD format) |

The task of determining the correct tiles that need to be received is quite complex and depends on the following parameter:

- Geometry parameters: the geometry may typically be a sphere with a certain radius and is positioned in the center of the scene.

- Rendering camera parameters: the rendering undergoes a projection that is simulated by a stereo camera that mimics the human eyes. The position of the rendering cameras, their direction and field of view have a significant impact on the resulting displayed area.

- HMD: the HMD requires adjustment based on the field of view, the horizontal and vertical resolution of the display, and the lens distortion correction.

- Partitioning: the areas that every tile covers in the scene.

The decision on what video stream(s) to send or receive based on the current HMD viewport is taken either by the client based on appropriate description of the content alternatives or by the server based on feedback on the current viewport and the HMD characteristics. Appropriate measures to enable at least one of these alternatives need to be defined.

For the rendering purposes, the graphics shaders need to be configured appropriately to be able to do the correct mappings of the polygons to the texture videos. As part of the scene description in HTML5, WebGL API has been recommended. WebGL vertex and fragment shaders need to be configured properly based on descriptive information from the video metadata.

# Proposal

We proposed to agree the contents of sections 3 and 4 to the TR on Virtual Reality.

# References

[1]	3GPP SP-160084, FS_VR: Study on Virtual Reality