3GPP TSG-SA WG4 Meeting #92			S4-170015

Tallin, Estonia, 23 - 27 January 2017	Agenda Item: 5.3

3GPP TSG-RAN WG2 Meeting #96			R2-168917

Reno, Nevada, USA, 14 – 18 November 2016

Title:	LS on the progress of QoE Measurement Collection for Streaming to RAN3, SA4, SA5 and CT1

Response to:

Release:	Rel-14

Work Item:	UMTS_QMC_Streaming

Source:	TSG RAN2

To:	TSG RAN3, TSG SA4, TSG SA5, TSG CT1

Cc:

Contact Person:

#### Name:	Yue Ma

Tel. Number:	+86 18601102532

E-mail Address:	may18@chinaunicom.cn

Attachments:

1. Overall Description:

At RAN2#95b and RAN2#96 meetings, RAN2 discussed the WI QoE Measurement Collection for streaming services and RAN2 has made the following agreements:

- RAN2 agreed to apply QMC (QoE Measurement Collection) solutions only in CELL_DCH state;

- RAN2 agreed that both signalling based QMC initiation and management based QMC initiation are allowed, from RAN2 point of view;

- RAN2 agreed to introduce container for QoE configuration and reporting, the content/format of the container is referred to SA4 definition (i.e. TS 26.247); the container is a generic one which could be named as Non-Access Statum Container for data configuration and data reporting separately, detailed naming could be revisited during stage 3 CR discussion phase;

- It is RAN2’s common understanding for the moment that the maximum size of the reporting container is 60, 000 bytes and the maximum size of the configuration container is 6,000 bytes, RAN2 agree to take such size as a tentative working assumption;

- RAN2 agreed to use measurement control and measurement report message to carry the container for the purpose of QoE configuration and reporting; we could continue to think and investigate the feasibility of using direct transfer message;

- RAN2 agreed to introduce new UE capability of supporting the QoE Measurement Collection for Streaming.

2. Actions:

To TSG RAN3:

RAN2 kindly asks RAN3 to take the above information into account in their work on the WI QoE Measurement Collection for steaming services.

To TSG SA4:

RAN2 kindly asks SA4 to consider and check the maximum size of the containers (i.e. configuration container and reporting container).

To TSG SA5:

RAN2 kindly asks SA5 to take the above information into account and check whether any modifications would be required to the SA5 specifications.

To TSG CT1:

RAN2 kindly asks CT1 to take the above information into account and check whether any CT1 specification impacts are foreseen regarding the interaction between Access Stratum layer and upper layer.

3. Date of Next TSG RAN2 Meetings:

TSG RAN2 Meeting #97 		13 – 17 Feb 2017	Athens, Greece

TSG RAN2 Meeting #97bis 	3 – 8 Apr 2017		US, US