TSG SA4#93 meeting	Tdoc S4-170256

24-28 April, Busan, Republic of Korea	Agenda Item: 5.3

3GPP TSG-RAN WG2 Meeting #97			R2-1702098

Athens, Greece, 13 – 17 February 2017

Title:	LS on the progress of QoE Measurement Collection for Streaming to RAN3, SA4, SA5 and CT1

Response to:

Release:	Rel-14

Work Item:	UMTS_QMC_Streaming

Source:	TSG RAN2

To:	TSG RAN3, TSG SA4, TSG SA5, TSG CT1

Cc:

Contact Person:

#### Name:	Cecilia Eklöf

Tel. Number:	+47 76 3353243

E-mail Address:	cecilia.eklof@ericsson.com

Attachments:

1. Overall Description:

At RAN2#97 meeting, RAN2 discussed the WI QoE Measurement Collection for streaming services and made the following agreements:

-	The configuration of QoE measurements will be done in the RRC message Measurement Control transmitted on SRB2 (normal priority Signalling Radio Bearer) and the report will be sent in the RRC message Measurement Report transmitted on SRB4 (low priority Signalling Radio Bearer).

-	The RAN behaviour when concurrent requests are initiated by OAM and CN is the same as for MDT, up to network implementation.

-	When the UE moves out of the area requested by CN or OAM, QoE measurements should be stopped. The QoE measurements are active when the requested cells are part of the active set.

-	QoE measurement will continue at SRNS relocation.

-	The measurements initiated by Measurement Control are suspended when leaving CELL_DCH but staying in connected mode and resumed when entering CELL_DCH again.

-	The measurement initiated by Measurement Control continue at soft handover and HS serving cell change, if not explicitly released by the RNC.

-	The new names of the containers are “Container for application layer measurement configuration” and “Container for application layer measurement reporting” instead of “Non-Access Stratum Container for data configuration and data reporting”.

-	It is RAN2’s understanding that the configuration file and the result file are compressed by upper layer which is out of RAN2 scope.

-	It is RAN2’s assumption that the maximum file size for configuration container is 1,000 bytes and for the report container 8,000 bytes.

-	It is FFS whether location information, e.g. measured cells where the QoE measurement was performed, should be included in the RRC message which contains the QoE container report.

-	It is FFS whether QoE measurement type needs to be included in the RRC message.

-	It is FFS whether events or messages in RAN can trigger the sending of the result file.

It is RAN2’s understanding that those FFS will not impact conclusion of the WI.

2. Actions:

To TSG RAN3:

RAN2 kindly asks RAN3 to take the above information into account in their work on the WI QoE Measurement Collection for steaming services.

To TSG SA4:

RAN2 kindly asks SA4 to consider and check the maximum size of the containers (i.e. configuration container and reporting container).

To TSG SA5:

RAN2 kindly asks SA5 to take the above information into account in their work on the WI QoE Measurement Collection for steaming services. RAN2 would also like to understand if location information, e.g. measured cells where the QoE measurement was performed, should be reported?

To TSG CT1:

RAN2 kindly asks CT1 to take the above information into account and check whether any CT1 specification impacts are foreseen regarding the interaction between Access Stratum layer and upper layer.

3. Date of Next TSG RAN2 Meetings:

TSG RAN2 Meeting #97bis 	3 – 7 Apr 2017		Spokane, US

TSG RAN2 Meeting #98 		15 – 19 May 2017	Hangzhou, China