TSG SA4#96 meeting	Tdoc S4-171152

# 13-17 November 2017, Albuquerque, New Mexico, USA	Agenda Item: 6.2

# SA WG2 Meeting #123	S2-178049

# 23-27 October 2017, Ljubljana, Slovenia

Title:		LS on default values for 5GS QoS averaging window for standardised 5QIs

Reply to:	-

Release:	Release 15

Work Item:	5GS_Ph1

Source:	SA WG2

To:		SA WG4, RAN WG2, SA WG6

Cc:		CT WG1

Contact person:	Haris Zisimopoulos

harisz@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

At SA2#122bis (August 2017), SA2 added the "Averaging Window" to the list of the 5G QoS characteristics in TS 23.501 subclause 5.7.3, and specified that:

The Averaging window is defined only for GBR QoS flows. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. (R)AN, UPF, UE). The averaging window is signalled with 5QIs, and if it is not received a standardized default value will apply (for standardized 5QIs the value in the QoS characteristics Table 5.7.4-1 applies).

At SA2#123 the following default values for "Averaging Window" for the standardised 5QIs were identified:

| 5QI Value | Resource Type | Priority Level | Packet Delay Budget | Packet Error Rate | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms | 10-2 | 2000 ms | Conversational Voice |
| 2 |  | 40 | 150 ms | 10-3 | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms | 10-3 | 3000 ms | Real Time Gaming, V2X messages |
| 4 |  | 50 | 300 ms | 10-6 | 3000 ms | Non-Conversational Video (Buffered Streaming) |
| 65 |  | 7 | 75 ms | 10-2 | 2000 ms | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 66 |  | 20 | 100 ms | 10-2 | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 75 |  | 25 | 50 ms | 10-2 | 3000 ms | V2X messages |
| 5 | Non-GBR | 10 | 100 ms | 10-6 | N/A | IMS Signalling |
| 6 |  | 60 | 300 ms | 10-6 | N/A | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  | 70 | 100 ms | 10-3 | N/A | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 80 | 300 ms | 10-6 | N/A | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file |
| 9 |  | 90 |  |  | N/A | sharing, progressive video, etc.) |
| 69 |  | 5 | 60 ms | 10-6 | N/A | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 70 |  | 55 | 200 ms | 10-6 |  | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 |  | 65 | 50 ms | 10-2 | N/A | V2X messages |
|  |  |  |  |  | N/A |  |

# 2	Actions

To SA4, RAN2, SA6

ACTION: 	3GPP SA WG2 kindly asks SA4, RAN2 and SA6 to provide feedback for the values for "Averaging Window" for standardised 5QIs

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting 124	27 November- 1 December 2017	Reno, USA

TSG SA WG2 Meeting 125	22 January- 26 January 2018		Gothenburg, Sweden