Source:	Qualcomm, Incorporated

Title:	Definition of Equivalent Spatial Domain Representation

## Document for:	Agreement

## Agenda Item:	10.6

# Introduction

At 3GPP TSG SA WG4 it was requested that a definition, or reference, to the term Equivalent Spatial Domain Representation be provided (see comments in [1]). This contribution provides a definition of the term.

# Definition of Equivalent Spatial Domain Representation

The equivalent spatial domain representation of a Nthorder soundfield representation ct is obtained by rendering ct to O virtual loudspeaker signals wjt, 1≤j≤O , with O=(N+1)2. The respective virtual loudspeaker positions are expressed by means of a spherical coordinate system, where each position lies on the unit sphere, i.e., a radius of 1. Hence, the positions can be equivalently expressed by order-dependent directions Ωj(N)=(θj(N),ϕj(N)),  1≤j≤O, where θj(N) and ϕj(N) denote the inclinations and azimuths, respectively. These directions are defined according to [1].

The rendering of ct into the equivalent spatial domain can be formulated as a matrix multiplication

wt=Ψ(N,N)-1⋅ct,

where ⋅-1 denotes the inversion.

The matrix Ψ(N,N) of order N  with respect to the order-dependent directions Ωj(N) is defined by

ΨN,N:=S1NS2N…SON

with

Sj(N):=S00Ωj(N)S-1-1Ωj(N)S-10Ωj(N)S-11Ωj(N)S-2-2Ωj(N) …SNNΩj(N)T

Where Snm∙ represents the real valued spherical harmonics of the order n and degree m.

The matrix Ψ(N,N) is invertible so that the HOA representation ct can be converted back from the equivalent spatial domain by

ct=Ψ(N,N) ⋅wt⋅

## Proposal

The source proposes to include the contents of this contribution to TS 26.260.

4. 	References

| [1] | Editor, 26.260 v.0.0.3 Objective Test Methodologies for the Evaluation of Immersive Audio Systems, Albuquerque, New Mexico, USA: 3GPP TSG SA4, 2017. |
| --- | --- |
| [2] | J. Fliege und U. Maier, „A two-stage approach for computing cubature formulae for the sphere,“ Dortmund University, 1999. |