Source:	Qualcomm, Incorporated, Fraunhofer IIS, ORANGE

Title:	Proposal for Submission Information for VRStream audio

## Document for:	Agreement

## Agenda Item:	16.8

# Introduction

This document proposes a set of requirements for Submission information, starting from a table originally presented in [1].

# Submission Information for VRStream audio

| Submission information |  |
| --- | --- |
| Codec and Reference Renderer Description | The following documentation of the audio media profile shall be provided: - Definition/specification of the bitstream, decoding and reference rendering process – as reference or by a full text documentation - Constraints applicable for streaming, file storage and configurations changes - Constraints (if any) on the number and configuration of encoded audio input channels that can be supported at the decoder. - Reference Renderer latency - Complexity of Decoder and Renderer - If applicable, default set of HRTF used for binaural rendering |
| Interfaces | The following interfaces shall be described - Interfaces for file storage and streaming - Interfaces for audio output to binaural headphones and loudspeakers - Interfaces for head tracking - Common Renderer API (TBD) - External Renderer API - API to specify an external set of HRTFs used for binaural rendering |
| Signalling | The following signalling for PSS and MBMS based download and streaming shall be described - Signalling for file-based download delivery - Signalling for DASH delivery |
| Audio Quality Characterization Test Results | The following characterization test shall be conducted and results provided and documented: Codec Quality Characterization (Tests 1 and 2) - ITU-R BS.1534-3 - Evaluates Basic Audio Quality - Test 1 - Using Reference Renderer for Reference and Degraded conditions - Over loudspeakers - Test 2 - Optional - Using Common Informative Binaural Renderer for Reference and Degraded conditions - Over headphones - All test materials for all proponents are according to corresponding LiQuImAS test plan. Reference Binaural Renderer Quality Characterization (Test 3) - 3GPP TS.26.260 (LiQuImAS) - Rendering Test (TBD in LiQuImAS) - Over headphones with head tracking - Characterize media profile performance with Reference Renderer and optionally but strongly recommended media profile performance with Common Informative Binaural Renderer. - Evaluates the following attributes: - Spatial Quality - Artefacts - Timbre Quality - Overall Quality - All test materials for all proponents are according to corresponding LiQuImAS test plan. |

## Proposal

It is proposed to agree on these submission requirements.

# References

[1] S4-AHVIC114, On VRStream Audio Submission, Fraunhofer IIS.