Source:	Qualcomm Incorporated

Title:	On capture formats for IVAS

Document for:	Discussion and Agreement

Agenda Item:	7.5

- Introduction

Higher Order Ambisonics (HOA) signals can carry immersive scene-based audio content with a scalable accuracy which depends on the HOA order. The HOA order N determines the number of HOA signals M which is expressed with M=(N+1)2 for a three-dimensional sound field. For instance, a 2nd order HOA soundfield requires 9 HOA signals and a 4th order soundfield requires 25 signals. The required amount of processing instructions and buffer memory for such a number of audio signals may pose a burden to the IVAS codec. Moreover, the audio architecture of at least one major state-of-the-art mobile phone operating system limits the number of concurrent audio input channels to eight which is not even sufficient to support 2nd order HOA.

To circumvent these audio bus limitations, to minimize overall system complexity, and to allow for a lean, yet HOA-capable, IVAS codec, the source proposes the support of the HOA Transport Format (HTF) within the IVAS design constraints [1].

- Problem Statement

Capturing of immersive audio can be achieved with a variety of microphone arrangements. As part of the capturing process usually a microphone processing step is needed to convert the captured microphone signals into the HOA format. This processing step depends on a-priori knowledge about the microphone arrangements (microphone position, orientation, directivity…) and is often calibrate to the device-specific form factor (see Fig. 1). Because of this device-dependency, the standardization of such front-end processing is less beneficial.

With ongoing advances in signal processing, the previous restriction that the number of microphones determines the maximum achievable HOA order could be lifted. Consequently, with a-priori knowledge, a low number of microphones can be converted into a higher order scene-based audio format (e.g., 8 microphones converted to a 16 channel 3rd order HOA signal).

Figure 1 - Examples of live-captured HOA input to IVAS

Because the captured signals have to go through a device-specific HOA conversion pre-processing, it would be preferable to transform them into a format that is optimal to the following processing stages. On handheld devices the conversion to a conventional HOA format is less efficient because of the need to transmit (N+1)2 audio signals to the IVAS codec.

To minimize overall system processing complexity, memory requirement, and latency, the source recommends the application of a front-end processing that creates the so-called HOA Transport Format (HTF) which can be efficiently transmitted via IVAS, see Fig. 2.

Figure 2 - Examples of live-captured HTF input to IVAS

- HTF capture for mobile devices

The HTF consists of a low number of audio signals with associated time-variant metadata. These audio signals represent sound elements within the scene {si}. Their spatial properties are parameterized in vector form {Vi} in the associated time-variant metadata, see Figure 3. Compared to converting the microphone signals directly into HOA signals, the HTF conversion reduces the number of required audio channels from (N+1)2 to a usually much lower number K, while preserving the perceived quality of the sound field.

Figure 3 - Examples for different spatial beam representations that can be expressed by V-vectors

At a later stage, the HTF can be converted into HOA by:

With K being the number of the HTF audio signals, and n being the frame number.

  - HTF Time-Variant Metadata

| Parameter | Description | Range | Bits/Transport Channel |
| --- | --- | --- | --- |
| HtfV[k] | HTF directional vector with (N+1)2 elements for the k-th HTF transport channel | [-1.0 … 1.0] | Depending on (N+1)2 |
| Priority[k] | Priority of the k-th transport channel. (Priority[k]=0 is the transport channel with the highest priority). | [0 … K-1] | Depending on the total number of transport channels |

- Proposal

The source proposes to consider HTF in the design constraints [1] as a spatial audio format that comprises audio channels plus spatial metadata.

| Audio Formats | The IVAS codec shall support the following [input] formats: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. - Spatial audio, up to 8 input channels and spatial metadata defined by HTF and [TBD]. [Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.] - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] |
| --- | --- |

References

[1]	S4-180265 - IVAS Design Constraints (IVAS-4) v0.0.3, SA4#97 output document