3GPP TSG-SA WG4 Meeting #97	S4-180582

# Kista, Sweden, April 9th – 13th 2018										   revision of S4-180356

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.131 | CR | 0075 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of requirements and objectives for SWB and FB terminals |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HEAD acoustics GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SPAN |  |  |  |  |  | Date: |  |  | 2018-04-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Performance requirements and objectives in the presence of ambient noise for SWB and FB terminals are not defined or are in a provisional state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Performance objectives and requirements are provisionally added based on the input from the SPAN work item. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance requirements and objectives for SWB and FB terminals remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10.2, 7.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

******** START OF MODIFIED CLAUSE ********

### 7.10.1	General

For sending, in handset mode, the UE shall reduce the ambient noise picked up by the microphone(s) without significantly degrading the quality of the speech signal.

### 7.10.2	Connections with handset UE

The UE shall comply with the following requirements:

S-MOS-LQOfb

-	The average of S-MOS-LQOfb scores across all test conditions shall be ≥ [3.6…3.9]

-	As a performance objective, the average of the S-MOS-LQOfb scores across all test conditions should be ≥ [3.9...4.2]

N-MOS-LQOfb

-	The average of the N-MOS-LQOfb scores across all test conditions shall be ≥ [2.6…2.9]

-	As a performance objective, the average of N-MOS-LQOfb scores across all test conditions should be ≥ [3.0…3.9]

G-MOS-LQOfb

-	No requirement.

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

NOTE 1: Values in brackets are provisional for an intermediate-term evaluation period. They may be updated and are expected to be finalized in the near future.

NOTE 2: Due to missing round-robin-tests investigating the inter-lab-reproducibility, a budget for S-MOS and N-MOS is included in the performance requirements. This budget might be decreased or removed in case of corresponding studies are available in the future.

### 7.10.3	Connections with Handheld hands-free UE

It is recommended that the UE meets the following performance objectives:

S-MOS-LQOfb

-	The average of S-MOS-LQOfb scores across all five test conditions should be ≥ [3.3…3.4]

N-MOS-LQOfb

-	The average of N-MOS-LQOfb scores across all test conditions should be ≥ [2.2…2.6]

G-MOS-LQOfb

-	No performance objective.

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

NOTE 1: Values in brackets are provisional for an intermediate-term evaluation period. They may be updated and are expected to be finalized in the near future.

NOTE 2: Due to missing round-robin-tests investigating the inter-lab-reproducibility, a budget for S-MOS and N-MOS is included in the performance requirements. This budget might be decreased or removed in case of corresponding studies are available in the future.

******** END OF MODIFIED CLAUSE ********