3GPP TSG-SA WG4 Meeting #99 	S4-180785

# Rome, Italy, July 09-13, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.118 | CR | CRNum | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of VRStream audio profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRStream |  |  |  |  |  |  | Date: |  | 2018-07-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Audio profiles required for VRStream |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds 3D audio profile based on existing 3GPP eAAC+ audio codec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No audio for VRStream service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< BEGINNING OF MODIFIED CLAUSE >>>

# 6	Audio Enablers

## 6.2	Media Profiles

### 6.2.2	Spatial Advanced Audio Coding Extension (spAACe) Audio Profile

#### 6.2.2.1	General

The Spatial Advanced Audio Coding Extension (spAACe) audio profile extends the application of the 3GPP TS 26.401 Enhanced aacPlus General Audio Codec (which normatively references to AAC) to support 3D audio including Channel-based, Object-based and Scene-based audio and/or a combination of these formats.

The audio profile additionally supports a topology that maintains backward compatibility with legacy devices supporting AAC based stereo decoding.

#### 6.2.2.2 	Bit Stream and Decoding Specification

The spAACe bit-stream structure is described in:

-	Annex C: Syntax of spAACe Audio Stream (spAACeAS); and

-	Annex D:AAC frame and syntax of raw_data_block()

##### 6.2.2.2.1	Channel-based audio signal processing chain and Metadata

The spAACe audio profile supports Channel-based audio content containing up to 16 channels, including e.g. stereo, 5.1, 5.1.2 and 7.1.4 formats. The signal processing chain for Channel-based audio content is depicted in Figure 1.

Figure 1 – Channel workflow

With reference to the Channel-based audio signal processing chain in Figure 1,

-	C denotes the total number of channels (e.g., C=12 for a 7.1+4 layout). The ordering of the audio streams in C are the ones described in the CICP index, but can be extended to another arbitrary speaker layout configuration.

-	The Legacy Path provides support for legacy devices supporting AAC-based stereo decoders (specified e.g. in TS 26.401). The Legacy Path carries a stereo mix of the original channel content. The stereo mixing takes C channels in input and outputs stereo signals.

-	The Extended Path provides full immersive audio support. In the Extended Path, a set of AAC-based Core Codecs is used to encode, decode and deliver C channels.

-	The output of the Extended Path is fed to a loudspeaker or binaural renderer.

##### 6.2.2.2.2	Object-based audio signal processing chain and Metadata

The spAACe audio profile supports Object-based audio content that is ideally suited for object-based content with less than or equal to M objects (M=16 for Level 1).

Figure 2 – Object workflow

With reference to the Object-based audio signal processing chainin Figure 2,

-	M denotes the total number of objects and R indicates the total number of loudspeakers in the target playback layout, with the constraints on M defined in Clause 6.2.2.3

-	In the Legacy Path, the M Objects are mixed to produce a stereo version of the original content.

-	The output of the Extended Path is the set of M objects and their corresponding Metadata reconstructed after the Core Codec and Metadata Codec.

-	An optional Object Renderer (which can perform loudspeaker rendering, binaural rendering, or a combination thereof) takes the M objects and their metadata and produces the appropriate feeds for the playback device.

The spAACe audio profile supports Metadata for Object-based audio according to ITU-R Recommendation BS.2076-1 (ADM), which includes, e.g. in Section 5.4.3.3:

-	Position

-	Gain

-	Spread

6.2.2.2

.3	Unified Scene-based audio and mixed content signal processing chain and Metadata

The spAACe audio profile supports a Unified Scene-based audio signal processing chain. Besides support for pure Scene-based audio content, the Unified Scene-based audio signal processing chain can also be used to transport channel and object-based audio in a scene-based representation. By converting the objects and channels to a unified scene-based audio format (e.g., by means of appropriate channels and objects to HOA transforms), the signal processing chain in Figure 3 can be used for the delivery of channel- and object-based immersive audio content. This signal processing chain is particularly useful if many channel and/or objects must be delivered with low target bitrates.

Figure 3 – The unified Scene-based audio and mixed content signal processing chain

The unified Scene-based audio and mixed content signal processing chain is shown in Figure 3.

-	N denotes the order of the HOA content and (N+1)2 indicates the number of HOA signals.

-	The Higher Order Ambisonics Transport Format (HTF) Encoder outputs T HTF Transport Channels and the HTF Metadata.

-	The Spatial Transform block takes the T transport channels as input and outputs a) two channels containing the stereo rendered version of the original HOA content which is fed to the Legacy Path and, b) a set of T–2 modified transport channels which are fed to the Extended Path.

-	In the Extended Path, the T–2 transport channels in output from the Core Codec and the Left/Right signals from the Legacy Path are combined in the Inverse Spatial Transform block to reconstruct the TT transport channels. The transport channels are fed in the HTF Decoder together with the HTF Metadata and the output of the HTF Decoder is the reconstructed (N+1)2 HOA signals.

-	An HOA Renderer (which can render to loudspeakers, binaural, or a combination thereof) takes the reconstructed (N+1)2 HOA signals and produces the appropriate feeds for the reproduction device. The HOA Renderer can be e.g. the Reference Renderer specified in Clause 6.2.2.5.

#### 6.2.2.3	Constraints

The number and configuration of encoded audio input channels that can be supported at the decoder and the number and configuration of output loudspeaker channels that can be supported by the renderer are defined in the following table. These numbers are different for the production and emission profiles.

| Level | Max. Sampling rate | Max. no. of core coded channels in compressed data stream | Max. no. of decoder processed core channels | Max. no. of loudspeaker output channels | Max. no. of decoded objects | Max. HOA order |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 48000 | 32 | 16 | 12 | 16 | 6 |
| 2 | 48000 | 56 | 28 | 12 | 28 | 6 |

#### 6.2.2.4	Complexity

Complexity numbers for spAACe Level 1 are  based of Table 4 in ISO/IEC 23008-3 and Table 1.4 of ISO/IEC 14496-3;2009. As such, for spAACe Level 1 the following worst-case processing power in MOPS are estimated:

-	Core decoding:

-	Up to 16 times the complexity of eAAC mono decoding (for 16 mono streams), approximately 48 MOPS.

-	or up to 8 times complexity of eAAC stereo decoding (for 8 stereo strams), approximately 48 MOPS.

-	Spatial decoding and HOA rendering:

-	Up to 36 MOPS for 6th order HOA and 12 speaker layout rendering.

-	Dynamic Range Control:

-	24 MOPS.

-	Binaural renderer:

-	See Clause 6.2.2.5.2 for information on the Reference Binaural Renderer complexity.

#### 6.2.2.5	Reference Rendering

##### 6.2.2.5.1	General

The Binaural Reference Renderer for the spAACe audio profile is the Common Informative Binaural Renderer described in Clause 4.5.1.2. The Loudspeaker Reference Renderer for the spAACe audio profile for objects and channels is the VBAP, as described in https://github.com/polarch/Vector-Base-Amplitude-Panning.

The Loudspeaker Reference Renderer for HOA is the ISO/IEC 23008-3:2018. Implementation of the Reference Renderer is optional and an External Rendering API is also provided in Clause 6.2.2.6.

##### 6.2.2.5.2	Complexity

An estimated complexity for the CIBR Reference Renderer is provided in Table 1 for a 3rd order HOA configuration.

Table 1 – Estimated Complexity of CIBR for 3rd order HOA configuration

| CIBR portion | Note | Operation | Formula | Operations per frame | Total MOPS | % of Total MOPS |
| --- | --- | --- | --- | --- | --- | --- |
| ESD-to-HOA | ESD-to-HOA and rotation operation could be optimized into one process | matrix multiply | =B*C*C | 262144 |  |  |
|  |  |  |  |  | 12.288 | 18.6 |
| HOA Rotation |  | sparse matrix multiply | =B*(3*3 + 5*5 + 7*7 +...) | 34816 |  |  |
|  |  |  |  |  | 1.632 | 2.5 |
| Binauralization |  | FFT | =C*F*K*log2(K) | 901120 |  |  |
|  | could be considered as one MAC operation | complex multiplication | =G*P*C*ceil((K+1)/2) | 32800 |  |  |
|  |  | complex addition | =2*G*P*C*ceil((K+1)/2) | 65600 |  |  |
|  |  | IFFT | =2*F*K*log2(K) | 112640 |  |  |
|  |  |  |  |  | 52.1325 | 78.8 |
| adding non-diegetic |  | addition | =2*B | 2048 |  |  |
|  |  |  |  |  | 0.096 | 0.1 |
| SUM |  |  |  |  | 66.1485 | 100 |

Table 2 - Description of symbols and default values for CIBR 3rd order HOA configuration

| Symbol | Default value | Description |
| --- | --- | --- |
| C | 16 | Number of HOA coefficients |
| N | 256 | Length of an HRTF in samples |
| B | 1024 | Processing blocksize in samples |
| P | 1 | Number of partitions of the HRIRs (given by ceil(N/B)) |
| K | 2048 | Transform length in samples used for the real-to-complex FFT (typically) |
| G | 2 | estimated additional complexity factor for complex addition/multiply |
| F | 2.5 | estimated additional complexity factor for the FFT |

##### 6.2.2.5.3	Reference Renderer Motion to Sound Latency (rM2S)

Introduction

The Reference Renderer Motion to Sound Latency (rM2S) is defined as the time difference between:

-	tmotion_in. The instant in time when a motion event is received by the Reference Renderer,

-	trend_out. The instant in time when the last sample of the audio frame, corresponding to the head position defined by the motion event at Tmotion_in, is rendered by the Reference Renderer algorithm.

The rM2S is 27.8ms as assessed with the method described below.

NOTE: In comparison to the motion-to-sound latency, rM2S latency excludes the implementation specific head-tracker motion to quaternion output latency and sound card playback buffer.

Reference Renderer Motion to Sound Latency Measurement

To determine the rM2S, an rM2S Measurement System was implemented in the Max/MSP platform. Figure 1 demonstrates a high-level signal flow diagram of the rM2S measurement system.

Figure 1

The rM2S Measurement System implements two instances of the system under test (SUT), named Binaural Renderer 1 (BR1) and Binaural Renderer 2 (BR2). The measurement procedure begins with a “BANG” message (consistent with the Max/MSP signal flow paradigm). At the onset of the “BANG” message, BR1 and BR2 begin playing a binaural white noise test signal located directly in front of the listener at 0⁰ azimuth, 0⁰ elevation. The onset also causes a quaternion input change, immediately rotating BR1 and BR2 by 90⁰ clockwise and 90⁰ counter-clockwise, respectively. The rotations position the sound source directly to the right of the listener for BR1, and directly to the left of the listener for BR2. After 10ms, two record blocks are triggered to start recording the binaural output of BR1 and BR2. After 1000ms, at time tmotion_in, BR2 is rotated by 180⁰ counter-clockwise, thus positioning the sound source directly to the right of the listener. Finally, at tmotion_in + 1000ms, playback is halted for both BR1 and BR2.

Figure 2 shows a sample recording of BR2 for one run of the measurement system. Because recording begins exactly one second before BR2’s 180⁰ rotation, the Reference Renderer Motion to Sound Latency is visually apparent as the time delay between one second (marked with the black dashed line) and the increase in gain of BR2’s right channel.

Figure 2

By subtracting BR2’s right channel recording from BR1’s right channel recording, the exact Reference Renderer Motion to Sound Latency can be measured as the number of samples between tmotion_in (i.e.. sample 48001 with a sampling rate of 48kHz) and the point at which the difference signal becomes zero at trend_out. Figure 3 illustrates overlaid BR1 and BR2 right channel recordings, as well as their difference signal.

Figure 3

Results

In this section, Reference Renderer Motion to Sound Latency results are presented for a real-time Max/MSP implementation of the CIBR system. The measurement procedure is executed with Max/MSP, and Reference Renderer Motion to Sound Latency calculations are automated in MATLAB. The Max/MSP implementation was realized on a Lenovo Thinkpad™ T460 laptop with an Intel Core™ i5-6300U CPU, 2 Cores, 4 Logical Processors.

100 total measurements were run, of which 13 were discarded due to alignment errors in the Max/MSP recorded signals. Such errors shifted the BR2 recording by a small number of samples, thus leading to a BR1/BR2 difference signal with no or premature convergence to 0. Figure 4 demonstrates a successfully automated M2S measurement.

Figure 5 displays each of the 87 remaining M2S latency measurements, as well as a mean M2S latency of 23.95ms. All CIBR M2S latency measurements fall within the range of 1332 and 1335 samples. Or, equivalently, between 27.75ms and 27.8125ms at a sample rate of 48kHz. However, it should be noted that the sound field rotation begins around 300 samples after the one second mark (as seen in Figure 4).

The sound field rotation is then interpolated over approximately one buffer of size 1024 samples. Therefore, the reported M2S latency values, which are calculated when this interpolation is complete, are an upper bound on the most extreme rotation scenario (e.g. a listener instantaneous rotating their head 180⁰).

##### 6.2.2.5.4	Default set of HRTFs

The default set of HRTFs for the Reference Renderer is the Diffuse-Field Compensated Neumann KU100 binaural head, corresponding to Subject 2 of the SADIE database available at: https://www.york.ac.uk/sadie-project/database_old.html.

##### 6.2.2.5.5	Interfaces for head tracking

Introduction

This section defines the syntax and format of scene displacement data for real-time sound field rotation in a Max/MSP implementation of the CIBR. HOA sound field rotation is accomplished by the ambiX_rotator VST plugin. The following information is agnostic to head tracking hardware and the head displacement data delivery method. Head displacement data reported by the head tracking device must be transformed and formatted to conform to the following scene displacement data format prior to reaching the CIBR.

Definition of the interface for scene-displacement data

The CIBR provides an interface for scene displacement, given by three Euler angles:

-	‘yaw’ is rotation around the z axis

-	‘pitch’ is rotation around the y axis

-	‘roll’ is rotation around the x axis

Scene displacement data is defined with a right-handed coordinate system using the following direction of the axes:

-	x axis pointing straight ahead

-	y axis pointing to the left

-	z axis pointing straight up

The CIBR implementation accepts Euler angle values between 0 and 1, corresponding to angular displacement between -180⁰ and 180⁰. Angular displacement in degrees is scaled to the accepted data range with the formula:

θ[0,1]=θ[-180, 180]+180360

Positive angular displacements (formatted displacement values greater than 0.5) correspond to rotation about each axis as indicated below. For this definition, clockwise and counter-clockwise are determined by looking directly along the axis of rotation, towards the origin of the coordinate system.

-	Yaw displacement value of over 0.5 corresponds to clockwise rotation about the z axis.

-	Pitch displacement value of over 0.5 corresponds to counter-clockwise rotation about the y axis.

-	Roll displacement value of over 0.5 corresponds to clockwise rotation about the x axis.

The order of sound field rotation may be set as either Yaw-Pitch-Roll or Roll-Pitch-Yaw within the ambiX_rotator VST.

Syntax of scene displacement interface

The first inlet of the CIBR Max/MSP object consumes scene displacement data as a stream of messages. Each message is a two-item list with the format [rotation_axis rotation_angle]. The rotation_axis parameter is defined as follows:

| rotation_axis value | Axis of rotation |
| --- | --- |
| 1 | Yaw (z axis) |
| 2 | Pitch (y axis) |
| 3 | Roll (x axis) |

The rotation_angle value is a float value in the range [0.0 1.0] that produces sound field rotation as described in the previous section.

#### 6.2.2.6	External Rendering

##### 6.2.2.6.1	Loudspeaker Renderer Interface

In Annex C, the spAACeConfig() syntax describes the target loudspeaker layout SpeakerConfig3d() that is defined in ISO/IEC 23008-3.

The Application Programming Interface for local loudspeaker setup and rendering is described in Section 17.3 of ISO/IEC 23008-3. Syntax for LoudspeakerRendering() in Table 248 defines the signaling of the local loudspeaker setup and target layout.

##### 6.2.2.6.2	Binaural Renderer Interface

The Application Programming Interfaces for the binaural renderer are as follows:

- for channel-based content, the outputs are the audio channels and the target loudspeaker layout information is contained in SpeakerConfig3d().

- for object-based content, the outpus are the audio objects and related metadata.

- for HOA content, the output is the set of (N+1)2 HOA signals (in SN3D normalization and ACN channel ordering) where N is the HOA order.

The binaural renderer takes any of the outputs specified above (channels, objects, and HOA) and performs the filtering of the output with the Head-Related Transfer Functions (HRTFs) or Binaural Room Impulse Responses (BRIRs). The HRTFs and BRIRs are specified using the SOFA format [SOFA].

[SOFA] AES STANDARD, AES69-2015: AES standard for file exchange - Spatial acoustic data file format http://www.aes.org/publications/standards/search.cfm?docID=99

#### 6.2.2.7	Loudness normalization and dynamic range control (DRC)

In Annex C, spAACeAS descrbes a way to use the DRC defined in ISO/IEC 23003-4. The interface for loudness normalization and dynamic range control is fully specified in ISO/IEC 23003-4.

<<< END OF MODIFIED CLAUSE >>>

<<< BEGINNING OF MODIFIED CLAUSE >>>

## 6.3	ISO BMFF Integration

### 6.3.1 Spatial AAC Extension (spAACe) Audio Profile

#### 6.3.1.1	General

Spatial AAC Extension (spAACe) elementary streams may be generated in one of the following three ways:

- as compatible enhanced AAC elementary stream following Annex B of the bitstream format which is then encapsulated into track conforming to the ‘mp4a’ sample entry ('mp4a') as defined in ISO/IEC 14496-14 [X]. For details on the track format, refer to clause 6.3.X.2.

- as a mode for which the two tracks are generated,

  - a base track that is compatible enhanced AAC elementary stream which is then encapsulated into track conforming to the ‘mp4a’ sample entry ('mp4a') as defined in ISO/IEC 14496-14 [X].

  - a track that conforms to a new sample entry spAACe that is a dependent track of the base track.

- a new track format that conforms to a new sample entry spAACe as a single track.

Note: This initial specification focuses on 1) to ensure consistency for legacy codecs, but mode 2 and/or 3 that rely on a new sample entry are under development.

#### 6.3.1.2	AAC Tracks with spAACe Extensions

3GP VR Tracks conforming to the spAACe media profile used in the context of the specification shall conform to the 3GP File Format [7] with the following further requirements:

-	The elementary stream shall follow the bitstream format in Annex B and conform to the spAACe constraints.

-	At least one sample entry type of each sample entry of the track shall be equal to 'mp4a'.

-	The Track Header Box ('tkhd') shall obey the following constraints:

- flags = 0x000007

- layer = 0

- volume = 0x0100

- matrix = {0x00010000, 0, 0, 0, 0x00010000, 0, 0, 0, 0x40000000} // unity matrix

- width = 0

- height = 0

- duration = 0

-	The Sound Media Header ('smhd') shall obey the following constraints:

-	The value of the balance field shall be set to '0'.

-	The Sample Description Box ('stsd') shall obey the following constraints:

-	the SampleDescriptionBox contains the AudioSampleEntry Box or AudioSampleEntryV1 Box.

-	For all audio media profiles in CMAF, the value of the samplesize parameter in the AudioSampleEntry box defined in ISO/IEC 14496-12 shall be set to 16.

-	Each AudioSampleEntry or AudioSampleEntryV1 shall contain a “(codingnamespecific)Box” containing codec-specific information.

If 3GP VR Tracks conforming to the constraints of this media profile and spAACe extensions are included in the bitstream, the '3spC' ISO brand should be set as a compatible_brand in the File Type Box ('ftyp').

## 6.4	DASH Integration

### 6.4.1 Spatial AAC Extension (spAACe) Audio Profile

#### 6.4.1.1 General

Spatial AAC Extension (spAACe) Extension streams may be added to DASH Periods in one of the following three ways:

- As a single Adaptation Set for which all Representations conform to the AAC Tracks as defined in clause 6.3.X.2. This is defined in clause 6.4.X.2.

- As a case for which two Adaptation Sets are generated,

  - a complementary Adaptation Set that is compatible to enhanced AAC.

  - a Preselection extension such that the Adaptation Set contains tracks that need to be combined with one Representation of the complementary Adaptation Set.

- As a single Adaptation Set for which all Representations conform to that conforms to a new sample entry spAACe.

Note: This initial specification focuses on 1) to ensure consistency for legacy codecs, but mode 2 and/or 3 that rely on a new sample entry are under development.

#### 6.4.1.2	AAC Adaptation Sets with spAACe Extensions

##### 6.4.1.2.1	Additional Restrictions for DASH Representations

If a VR Track conforming to this media profile is included in a DASH Representation, the following additional constraints shall apply:

-	The Media Header Box ('mdhd') shall obey the following constraints:

-	The value of the duration field shall be set to '0'.

-	The value of the duration field in the Movie Header Box ('mvhd') shall be set to a value of '0'

-	The Track Header Box ('tkhd') shall obey the following constraints:

-	The value of the duration field shall be set to '0'.

-	Movie Fragment Header Boxes ('mfhd') shall contain sequence_number values that are sequentially numbered starting with the number 1 and incrementing by +1, sequenced by movie fragment storage and presentation order.

-	Any Segment Index Box ('sidx'), if present, shall obey the additional constraints:

-	the timescale field shall have the same value as the timescale field in the Media Header Box ('mdhd') within the same track; and

-	the reference_ID field shall be set to the track_ID of the ISO Media track as defined in the Track Header Box ('tkhd').

##### 6.4.1.2.2	Adaptation Set Constraints

For an audio Adaptation Set, the following constraints apply:

-	Each Representation shall conform to the AAC Tracks with spAACe Extensions format defined in clause 6.3.1.2 and the general constraints in clause 6.4.1.2.1

-	The @codecs parameter shall be present on Adaptation Set level and shall signal the maximum required capability to decode any Representation in the Adaptation Set. The @codecs parameter should be signalled on the representation level if different from the one on Adaptation Set level.

-	Random Access Points shall be signalled by @startsWithSAP set to 1 or 2.

-	The presence of  spAACe extensions should be be signalled. If signalled,

-	a Supplemental Descriptor shall be used to signal the value by setting the @schemeIdURI attribute to urn:3GPP:org:spAACe and the value shall be set to ambisonis order.

If all Representations in an Adaptation Set conform to the requirements in clause 6.3.1.2 and the Adaptation Set conforms to the requirements in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Operation Point by using "urn:3GPP:vrstream:mp:spAACe ". The @profiles parameter s be present to signal the constraints for the Adaptation Set.

<<< END OF MODIFIED CLAUSE >>>>

<<< BEGINNING OF MODIFIED CLAUSE >>>

Annex C (normative): Syntax of spAACe audio stream (spAACeAS)

This appendix defines a self-contained stream format to transport spAACe data.

Core decoding such as single channel element (SCE), channel pair element (CPE), and LFE decoding is described in [1].

HOA decoding is described in [2].

DRC is described in [3].

Other decoding functions such as object decoding is described in [3]. We followed the low complexity profile constraints.

[1] ISO/IEC 14496-3, Information technology - Coding of audio-visual objects - Part 3: Audio.

[2] ETSI TS 103 589, Higher Order Ambisonics (HOA) Transport Format.

[3] ISO/IEC 23003-4, Information technology -- MPEG audio technologies -- Part 4: Dynamic Range Control.

[4] ISO/IEC 23008-3:2018, Information technology — High efficiency coding and media delivery in heterogeneous environments - Part 3: 3D audio.

Table A.1 — Syntax of spAACeAudioStream()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| spAACeAudioStream() |  |  |
| { |  |  |
| while (bitsAvailable() != 0) { |  |  |
| spAACeAudioStreamPacket(); |  |  |
| } |  |  |
| } |  |  |

Table A.2 — Syntax of spAACeAudioStreamPacket()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| spAACeAudioStreamPacket() |  |  |
| { |  |  |
| SPAACEASPacketType = escapedValue(3,8,8); | 3,11,19 | uimsbf |
| SPAACEASPacketLength = escapedValue(11,24,24); | 11,35,59 | uimsbf |
| SPAACEASPacketLabel = escapedValue(2,8,32); | 2,10,42 | uimsbf |
| SPAACEASPacketPayload(SPAACEASPacketType); |  |  |
| } |  |  |
| NOTE: With the given bit allocation, SPAACEASPacketPayload() is always byte-aligned |  |  |

Table A.2.1 — Semantics of spAACeAudioStreamPacket()

| SPAACEASPacketType | This element specifies the payload type which is defined below. SPAACEASPacketType; value. PACTYP_SYNC; 0. PACTYP_SPAACECFG; 1. PACTYP_SPAACEFRAME; 2. PACTYP_AUDIOSCENEINFO; 3. /* reserved */; 4-5. PACTYP_FILLDATA; 6. PACTYP_SYNCGAP; 7. PACTYP_MARKER; 8. PACTYP_CRC16; 9. PACTYP_CRC32; 10. PACTYP_GLOBAL_CRC16; 11. PACTYP_GLOBAL_CRC32; 12. PACTYP_USERINTERACTION; 13. PACTYP_LOUDNESS_DRC; 14. PACTYP_BUFFERINFO; 15. PACTYP_AUDIOTRUNCATION; 16. /* reserved */; 17-. |
| --- | --- |
| SPAACEASPacketLabel | This element provides an indication of which packets belong together. |
| SPAACEASPacketLength | This element indicates the length of the spAACeAudioStreamPacket(). |

Table A.3 — Syntax of SPAACEASPacketPayload()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| SPAACEASPacketPayload(SPAACEASPacketType) |  |  |
| { |  |  |
| switch (SPAACEASPacketType) { |  |  |
| case PACTYP_SYNC: |  |  |
| 0xB4; /* syncword*/ | 12 | uimsbf |
| break; |  |  |
| case PACTYP_SPAACECFG: |  |  |
| spAACeConfig(); |  |  |
| break; |  |  |
| case PACTYP_SPAACEFRAME: |  |  |
| spAACeFrame(); |  |  |
| break; |  |  |
| case PACTYP_AUDIOSCENEINFO: |  |  |
| mae_AudioSceneInfo(); |  |  |
| break; |  |  |
| case PACTYP_FILLDATA: |  |  |
| for (i=0; i< SPAACEASPacketLength; i++) { |  |  |
| spAACeAS_fill_data_byte(i); | 8 | bslbf |
| } |  |  |
| break; |  |  |
| case PACTYP_SYNCGAP: |  |  |
| syncSpacingLength = escapedValue(16,24,24); | 16,40,64 | uimsbf |
| break; |  |  |
| case PACTYP_MARKER: |  |  |
| for (i=0; i< SPAACEASPacketLength; i++) { |  |  |
| spAACeMarker(i); | 8 | bslbf |
| } |  |  |
| break; |  |  |
| case PACTYP_CRC16: |  |  |
| spAACeASParity16Data; | 16 | bslbf |
| break; |  |  |
| case PACTYP_CRC32: |  |  |
| spAACeASParity32Data; | 32 | bslbf |
| break; |  |  |
| case PACTYP_GLOBAL_CRC16: |  |  |
| numProtectedPackets; | 6 | bslbf |
| spAACeASParity16Data; | 16 | bslbf |
| break; |  |  |
| case PACTYP_ GLOBAL_CRC32: |  |  |
| numProtectedPackets; | 6 | bslbf |
| spAACeASParity32Data; | 32 | bslbf |
| break; |  |  |
| case PACTYP_USERINTERACTION: |  |  |
| spAACeElementInteraction(); |  |  |
| break; |  |  |
| case PACTYP_LOUDNESS_DRC: |  |  |
| uniDrcInterface (); |  |  |
| break; |  |  |
| case PACTYP_BUFFERINFO: |  |  |
| if (spAACeAS_buffer_fullness_present) | 1 | uimsbf |
| spAACeAS_buffer_fullness = escapedValue(15,24,32); | 15,39,71 | uimsbf |
| } |  |  |
| break; |  |  |
| case PACTYP_AUDIOTRUNCATION: |  |  |
| if (spAACeAudioTruncPresent) { spAACeTruncFrom; spAACeTruncSamples; } | 1 1 13 | uimsbf uimsbf uimsbf |
| break; |  |  |
| } |  |  |
| ByteAlign(); |  |  |
| } |  |  |

Table A.3.1 — Semantics of SPAACEASPacketPayload()

| spAACeConfig() | A spAACe audio config which is defined in Table A.4. |
| --- | --- |
| spAACeFrame() | A spAACe audio payload which is defined in Table A.9. |
| mae_AudioSceneInfo() | The set of metadata consists of descriptive metadata, restrictive metadata, positional metadata, and structural metadata. An MPEG-H 3D audio scene information structure as defined in 15.2 of ISO/IEC 23008-3. |
| spAACeAS_fill_data_byte(i) | 8-bit data elements |
| syncSpacingLength | the length in Bytes between the last two SPAACEASPacketType PACTYP_SYNC |
| spAACeMarker(i) | This element indicates a marker event. marker_byte (i); Meaning. 0x01; Configuration Change Marker. 0x02; Random Access / Immediate Playout Marker. 0x03; Program Boundary Marker. the other values; /*reserved */. |
| spAACeElementInteraction () | Identical to MPEG-H 3D audio element interaction structure as defined in 17.7.4. |
| uniDrcInterface () | defined in ISO/IEC 23003-4. |
| spAACeAS_buffer_fullness_present | a bit signaling the presence of spAACeAS_buffer_fullness |
| spAACeAS_buffer_fullness | This element indicates the state of the bit reservoir |
| spAACeAudioTruncPresent | If 1 the truncation message is active. |
| spAACeTruncFrom | if 0 truncate samples from the end, if 1 truncate samples from the beginning. |
| spAACeTruncSamples | number of samples to truncate |
| ByteAlign() | fill bits to achieve byte alignment |

Table A.4 — Syntax of spAACeConfig()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| spAACeConfig() |  |  |
| { |  |  |
| referenceLayout = CICPspeakerLayoutIdx; | 6 | uimsbf |
| SpAACeSignals3d(); |  |  |
| spAACeDecoderConfig(); if (spAACeConfigExtensionPresent) { spAACeDecoderConfigExtension(); | 1 | uimsbf |
| } } |  |  |

Table A.4.1 — Semantics of spAACeConfig ()

| CICPspeakerLayoutIdx | ChannelConfiguration value. Shall be as defined in ISO/IEC 23001-8. |
| --- | --- |
| SpAACeSignals3d() | Signal configuration defined in Table A.5 |
| spAACeDecoderConfig() | Decoder configuration defined in Table A.6 |
| spAACeConfigExtensionPresent | If 1, it indicates the presence of extended configuration. |
| spAACeDecoderConfigExtension() | It indicates the extended configuration function defined in Table A.4.2. |

Table A.4.2 — Syntax of spAACeDecoderConfigExtension()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| spAACeDecoderConfigExtension() |  |  |
| { |  |  |
| numConfigExtensions = escapedValue(2,4,8) + 1; |  |  |
| for (idx=0; idx<numConfigExtensions; idx++) { |  |  |
| spAACeConfigExtType[idx] = escapedValue(4,8,16); |  |  |
| spAACeConfigExtLength[idx] = escapedValue(4,8,16); |  |  |
| switch (spAACeConfigExtType[idx]) { |  |  |
| case ID_SPAACE_CONFIG_EXT_FILL: |  |  |
| while (spAACeConfigExtLength[idx]--) { |  |  |
| fill_byte[i]; | 8 | uimsbf |
| } |  |  |
| break; |  |  |
| case ID_SPAACE_CONFIG_EXT_LOUDNESS_INFO: |  |  |
| spAACeLoudnessInfoSet(); |  |  |
| break; |  |  |
| case ID_SPAACE_CONFIG_EXT_AUDIOSCENE_INFO: |  |  |
| mae_AudioSceneInfo(); |  |  |
| break; |  |  |
| case ID_SPAACE_CONFIG_EXT_HOA_MATRIX: |  |  |
| HoaRenderingMatrixSet(); |  |  |
| break; |  |  |
| case ID_SPAACE_CONFIG_EXT_SIG_GROUP_INFO: |  |  |
| for ( grp = 0; grp < bsNumSignalGroups + 1 ; grp++ ) { groupPriority[grp]; fixedPosition[grp]; } | 3 1 | uimsbf uimsbf |
| break; |  |  |
| default: |  |  |
| while (spAACeConfigExtLength[idx]--) { |  |  |
| tmp; | 8 | uimsbf |
| } |  |  |
| break; |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table A.4.3 — Semantics of spAACeDecoderConfigExtension()

| numConfigExtensions | The number of extended configurations |
| --- | --- |
| spAACeConfigExtType | This element specifies the extended config type which is defined below. spAACeConfigExtType; value. ID_SPAACE_CONFIG_EXT_FILL; 0. ID_SPAACE_CONFIG_EXT_LOUDNESS_INFO; 1. ID_SPAACE_CONFIG_EXT_AUDIOSCENE_INFO; 2. ID_SPAACE_CONFIG_EXT_HOA_MATRIX; 3. ID_SPAACE_CONFIG_EXT_SIG_GROUP_INFO; 4. /* reserved */; the others. |
| spAACeLoudnessInfoSet() | The content of mpegh3daLoudnessInfoSet() is listed in Table 125 of ISO/IEC 23008-3. It is defined in ISO/IEC 23003-4. |
| mae_AudioSceneInfo() | The set of metadata consists of descriptive metadata, restrictive metadata, positional metadata, and structural metadata. An MPEG-H 3D audio scene information structure as defined in 15.2 of ISO/IEC 23008-3. |
| HoaRenderingMatrixSet() | An HOA rendering matrix structure as defined in Table 32 of ISO/IEC 23008-3. |
| groupPriority | If groups are discarded, the groups with lowest priority should be discarded first. |
| fixedPosition | This field defines if the positions of the members of a group shall be updated in the context of processing of tracking data. |

Table A.5 — Syntax of SpAACeSignals3d()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| SpAACeSignals3d() |  |  |
| { |  |  |
| numAudioChannels = 0; |  |  |
| numAudioObjects = 0; |  |  |
| numHOATransportChannels = 0; |  |  |
| bsNumSignalGroups; | 5 | uimsbf |
| for ( grp = 0; grp < bsNumSignalGroups + 1 ; grp++ ) { |  |  |
| signal_groupID[grp] = grp; |  |  |
| signalGroupType[grp]; | 3 | bslbf |
| bsNumberOfSignals[grp] = escapedValue(5, 8, 16); |  |  |
| if ( SignalGroupType[grp] == SignalGroupTypeChannels ) { |  |  |
| numAudioChannels += bsNumberOfSignals[grp] + 1; |  |  |
| } |  |  |
| if ( SignalGroupType[grp] == SignalGroupTypeObject ) { |  |  |
| numAudioObjects += bsNumberOfSignals[grp] + 1; |  |  |
| } |  |  |
| if ( SignalGroupType[grp] == SignalGroupTypeHOA ) { |  |  |
| if (DefaultHoaRenderingMatrix[grp]) { HoaRenderingMatrixId[grp]; } | 1 7 | uimsbf uimsbf |
| numHOATransportChannels += bsNumberOfSignals[grp] + 1; |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table A.5.1 — Semantics of SpAACeSignals3d()

| bsNumSignalGroups | It defines the number of signal groups that are present in the bitstream. |
| --- | --- |
| signal_groupID | It indicates an ID to the signal groups. |
| signalGroupType | It indicates a signal group type. SignalGroupTypeChannels; 0; It contains channel signals. SignalGroupTypeObject; 1; It contains object signals. 2; /* reserved */. SignalGroupTypeHOA; 3; It contains HOA signals. 4-; /* reserved */. |
| AnyHoaRenderingMatrix [grp] | It defines whether any rendering matrix can be used (DefaultHoaRenderingMatrix[grp]=0) or a transmitted rendering matrix shall be used (DefaultHoaRenderingMatrix[grp]=1) for this audio group. |
| HoaRenderingMatrixId[grp] | It indicates which HOA rendering matrix shall be used for this audio group. All transmitted HoaRenderingMatrixId’s are available in Table 32 of ISO/IEC 23008-3. |

Table A.6 — Syntax of spAACeDecoderConfig()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| spAACeDecoderConfig() |  |  |
| { |  |  |
| numElements = escapedValue(4,8,16) + 1; |  |  |
| for (elemIdx=0; elemIdx<numElements; ++elemIdx) { |  |  |
| spAACeElementType[elemIdx]; inLegacyPath[elemIdx]; if (inLegacyPath){ legacyPathChannelIdx[elemIdx]; } | 4 1 5 | Uimsbf Uimsbf Uimsbf |
| switch (spAACeElementType[elemIdx]) { |  |  |
| case ID_SPAACE_SCE: |  |  |
| case ID_SPAACE_CPE: |  |  |
| case ID_SPAACE_LFE: |  |  |
| program_config_element(); break; |  |  |
| case ID_SPAACE_EXT: |  |  |
| spAACeExtElementConfig(); break; default: break; } |  |  |
| } |  |  |
| } |  |  |

Table A.6.1 — Semantics of spAACeDecoderConfig()

| spAACeElementType | It defines bit stream extensions types. (compatible with Table 4.85 of ISO-IEC-14496-3;2009 ID_SPAACE_SCE; 0; It contains single channel element.. ID_SPAACE_CPE; 1; It contains channel pair elements.. ID_SPAACE_CCE; 2; It contains coupling channel element.. ID_SPAACE_LFE; 3; It contains LFE.. 4; /* reserved */. 5; /* reserved */. ID_SPAACE_FIL; 6; It contains fill element.. 7; /* reserved */. ID_SPAACE_EXT; 8; It contains extension payload.. 9-15; /* reserved */. |
| --- | --- |
| inLegacyPath | Set to 1 if the payload for the element is carried in the legacy AAC path as described in Annex B. |
| legacyPathChannelIdx | Index of the legacy path channel that is used for carrying the payload for this element. |
| program_config_element() | Configuration for SCE, CPE and LFE. This function is defined in Table 4.2 of ISO/IEC 14496-3. |
| spAACeExtElementConfig() | Configuration for extension elements defined in Table A.7 |

Table A.7 — Syntax of spAACeExtElementConfig()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| spAACeExtElementConfig() |  |  |
| { |  |  |
| SpAACeExtElementType = escapedValue(4, 8, 16); |  |  |
| SpAACeExtElementConfigLength = escapedValue(4, 8, 16); |  |  |
| switch (SpAACeExtElementType) { |  |  |
| case ID_EXT_ELE_FILL: |  |  |
| /* No configuration element */ |  |  |
| break; |  |  |
| case ID_EXT_ELE_AUDIOPREROLL: |  |  |
| /* No configuration element */ |  |  |
| break; |  |  |
| case ID_EXT_ELE_UNI_DRC: |  |  |
| spAACeUniDrcConfig(); |  |  |
| break; |  |  |
| case ID_EXT_ELE_OBJ_METADATA: |  |  |
| ObjectMetadataConfig(); |  |  |
| break; |  |  |
| case ID_EXT_ELE_HOA: |  |  |
| HOAConfig_SN3D(); |  |  |
| break; |  |  |
| case ID_EXT_ELE_ENHANCED_OBJ_METADATA: |  |  |
| EnhancedObjectMetadataConfig(); |  |  |
| break; |  |  |
| default: | NOTE |  |
| while (SpAACeExtElementConfigLength--) { |  |  |
| tmp; | 8 | uimsbf |
| } |  |  |
| break; |  |  |
| } |  |  |
| } |  |  |
| NOTE: The default entry for the SpAACeExtElementType is used for unknown extElementTypes so that legacy decoders can cope with future extensions. |  |  |

Table A.7.1 — Semantics of spAACeExtElementConfig()

| SpAACeExtElementType | This element specifies the extension element type which is defined below. SPAACEASPacketType; value. ID_EXT_ELE_FILL; 0. ID_EXT_ELE_AUDIOPREROLL; 1. ID_EXT_ELE_UNI_DRC; 2. ID_EXT_ELE_OBJ_METADATA; 3. ID_EXT_ELE_HOA; 4. ID_EXT_ELE_ENHANCED_OBJ_METADATA; 5. /* reserved */; 6-. |
| --- | --- |
| SpAACeExtElementConfigLength | This element defines the length of spAACeExtElementConfig in bytes. |
| spAACeUniDrcConfig() | DRC configuration defined in Table 126 of ISO/IEC 23008-3. |
| ObjectMetadataConfig() | Object metadata configuration defined in Table 133 of ISO/IEC 23008-3 (lowDelayMetadataCoding shall be set to 1). |
| HOAConfig_SN3D() | HOA configuration defined Table A.7.2. It is a modified version of Table 8 of ETSI TS 103 589. |
| EnhancedObjectMetadataConfig() | Enhanced object metadata configuration defined in Table 144 of ISO/IEC 23008-3. |

Table A.7.2 — Syntax of HOAConfig_SN3D()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| HOAConfig_SN3D() { NumOfHoaCoeffs_E = (HoaOrder_E + 1 )^2; IsScreenRelative_E; HOADecoderConfig_SN3D(NumOfTransportChannels); if (RecorrIdx == 2) { bitDepth; if (RecorrMtxSizeFlag) { RecorrMtx_numRow = rowIdx + 1; RecorrMtx_numCol = colIdx + 1; } else { RecorrMtx_numRow = MinAmbHoaOrder_E; RecorrMtx_numCol = MinAmbHoaOrder_E; } MaxValueRecorrMtx; for (i=0; i< RecorrMtx_numRow; i++){ for (j=0; j< RecorrMtx_numCol; j++){ RecorrMtx(i,j) = tmpValueRecorrMtx/MaxValueRecorrMtx; } } } elseif (RecorrIdx == 3) { BeamAzimuth = 0; BeamElevation = 0; StereoSpread; BeamCharacter = BeamCharacterIdx/15; InPhasePostprocessingFlag; if (hasAngleOffset) { BeamAzimuth += azimuthAngleOffset; BeamElevation += elevationAngleOffset; } RecorrMtx_numRow = 4; RecorrMtx_numCol = 4; RecorrMtx = RecorrMtxGeneration(); } } | 5 1 3 4 1 4 4 bitDepth+1 bitDepth+1 4 4 1 1 8 5 | uimsbf uimsbf uimsbf uimsbf uimsbf uimsbf uimsbf uimsbf bslbf uimsbf uimsbf uimsbf uimsbf uimsbf uimsbf |

Table A.7.3 — Semantics of HOAConfig_SN3D()

| HoaOrder_E | This element determines the HOA order of the coded signal. |
| --- | --- |
| NumOfHoaCoeffs_E | This element determines the number of HOA coefficients of the coded HOA representation, which is equal to the number of HOA coefficients to be reconstructed. |
| IsScreenRelative_E | This element indicates if the HOA representation shall be rendered with respect to the reproduction screen size as described in ISO/IEC 23008-3, clause 18.4. 0: not screen related 1: screen related |
| RecorrIdx | This element indicates which decorrelation matrix is applied to recorrelate the ambience signals. 0: identity matrix 1: the matrix define in ISO/IEC 23008-3 2: transmitted recorrelation matrix (method #1) 3: transmitted recorrelation matrix (method #2) 4-7: reserved |
| HOADecoderConfig_SN3D() | This function is defined in Table 9 of ETSI TS 103 589. |
| bitDepth | This element contains information about the coded bit depth. |
| RecorrMtxSizeFlag | If this value is 1, RecorrMtx_numRow and RecorrMtx_numCol are transmitted. Otherwise, the default value of MinNumOfCoeffsForAmbHOA_E is used. |
| MinNumOfCoeffsForAmbHOA_E | This element determines the minimum number of HOA coefficients used for the coding of the ambient HOA. It is defined in Table 9 of ETSI TS 103 589. |
| RecorrMtx_numRow | This element contains information about the number of rows of a recorrection matrix. |
| RecorrMtx_numCol | This element contains information about the number of columns of a recorrection matrix. |
| MaxValueRecorrMtx | It defines the maximum value of RecorrMtx. |
| RecorrMtx(i,j) | This element indicates an i-th row and j-th column of recorrelation matrix. This matrix is applied to ambient channels for (1) recorrelation and/or (2) backward compatible decoding with eAAC of [1]. |
| BeamAzimuth | It defines the azimuth angle for the center of the stereo beams. |
| BeamElevation | It defines the elevation angle for the center of the stereo beams. |
| StereoSpread | It defines the spread angle between the stereo beams (30~120 degree). |
| BeamCharacter | Its value range is between 0.0 and 1.0. |
| InPhasePostprocessingFlag | If this flag is set, in-phase post-processing is performed on the stereo beams. |
| hasAngleOffset | If this flag is set, BeamAzimuth and BeamElevation will be modified by azimuthAngleOffset (-180 ~ 180 degree) and elevationAngleOffset (-90 ~ 90 degree). |
| RecorrMtxGeneration() | This function generates RecorrMtx based on BeamAzimuth, BeamElevation, StereoSpread, BeamCharacter, InPhasePostprocessingFlag. |

Table A.8 — Syntax of spAACeFrame()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| spAACeFrame() |  |  |
| { |  |  |
| for (elemIdx= 0; elemIdx<numElements; ++elemIdx) { |  |  |
| if (spAACeElementType[elemIdx] !=ID_SPAACE_EXT){ |  |  |
| elementLength; | 16 | Uimsbf |
| } |  |  |
| switch(spAACeElementType[elemIdx]) { |  |  |
| case ID_SPAACE_SCE: if (inLegacyPath[elemIdx]) { SpAACe_legacy_single_channel_element(); } else { | Note 2 |  |
| SpAACe_single_channel_element(); } break; | elementLength, Note 1 |  |
| case ID_SPAACE_CPE: if (inLegacyPath[elemIdx]) { SpAACe_legacy_channel_pair_element(); } else { | Note 2 |  |
| SpAACe_channel_pair_element(); } break; | elementLength, Note 1 |  |
| case ID_SPAACE_LFE: if (inLegacyPath[elemIdx]) { SpAACe_legacy_lfe_channel_element(); } else{ | Note 2 |  |
| SpAACe_lfe_channel_element(); | elementLength, Note 1 |  |
| } break; case ID_SPAACE_CCE: if (inLegacyPath[elemIdx]) { SpAACe_legacy_channel_coupling_element(); } else{ | Note 2 |  |
| SpAACe_channel_coupling_element(); } break; | elementLength, Note 1 |  |
| case ID_FIL: if (inLegacyPath[elemIdx]) { SpAACe_legacy_fill_element(); } else{ | Note 2 |  |
| SpAACe_fill_element(); } | elementLength, Note 1 |  |
| case ID_SPAACE_EXT: |  |  |
| spAACeExtElement(); break; } } | elementLength, Note 1 |  |
| } |  |  |
| NOTE 1: If present, elementLength represents the length of the corresponding element it refers to in number of bits. Note 2: These elements are located outside the SpAACeFrame and present in the legacy path of the decoding process as described in Annex B. legacyPathChannelIdx[elemIdx] should be selected from legacy path. |  |  |

Table A.8.1 — Semantics of spAACeFrame()

| spAACeElementType | As defined in Table A.6.1 |
| --- | --- |
| SpAACe_legacy_single_channel_element() | single_channel_element() from legacy path as described in Table 4.4 in ISO-IEC-14496-3;2009. |
| SpAACe_single_channel_element() | single_channel_element() as described in Table 4.4 in ISO-IEC-14496-3;2009 |
| SpAACe_legacy_channel_pair_element() | channel_pair_element() from legacy path as described in Table 4.5 in ISO-IEC-14496-3;2009.. |
| SpAACe_channel_pair_element() | channel_pair_element() as described in Table 4.5 in ISO-IEC-14496-3;2009 |
| SpAACe_legacy_lfe_ channel_element() | lfe_ channel_element()from legacy path as described in Table 4.9 in ISO-IEC-14496-3;2009.. |
| SpAACe_lfe_ channel_element() | lfe_channel_element() as described in Table 4.9 in ISO-IEC-14496-3;2009 |
| SpAACe_legacy_channel_coupling_element() | channel_coupling_element()from legacy path as described in Table 4.8 in ISO-IEC-14496-3;2009.. |
| SpAACe_channel_coupling_element() | channel_coupling_element() as described in Table 4.8 in ISO-IEC-14496-3;2009 |
| SpAACe_legacy_fill_element() | fill_element()from legacy path as described in Table 4.11 in ISO-IEC-14496-3;2009. |
| SpAACe_fill_element() | fill_element() as described in Table 4.11 in ISO-IEC-14496-3;2009 |
| spAACeExtElement() | Core data for extension elements. This function is identical to mpegh3daExtElement() which is described in Table 45 of ISO/IEC 23008-3. |

<<< END OF MODIFIED CLAUSE >>>

<<< BEGGINING OF MODIFIED CLAUSE >>>

Annex D (normative): AAC frame and syntax of raw_data_block()

SpAACe Audio Stream bits are partitioned into a sequence of byte aligned data chunks with a maximum size of 256 bytes. Each partition is then embedded as a separate fill_element within the raw_data_block of the AAC bitstream to maintain backwards compatibility with the legacy AAC format. SpAACe Audio Stream bits embedded in one raw_data_block of the legacy AAC bitstream include all the SpAACe payload bits belonging to that time slot (corresponding to the legacy AAC frame).

This appendix describes the parsing of the AAC bitstream format to extract these data chunks and form one contiguous block of SpAACe Audio Stream bits.

The overview and syntax of ADTS frame is provided in Annex 1.A of ISO_IEC_14496-3;2009. Refer to tables 1.A.4 to 1.A.11.

Syntax of raw_data_block() is explained in Table 4.3 of ISO_IEC_14496-3;2009.

single_channel_element() and channel_pair_element() are used to carry the mono and stereo channels in the legacy path. Syntax is described in Table 4.4 and Table 4.5 of ISO_IEC_14496-3;2009. Any number of these elements from the legacy path might be used in the SpAACe decoding process as described in Table A.8.

A sequence of Fill_elements are used to carry the SpAACe Audio Stream. Fill_element syntax is described in Table 4.11 of ISO_IEC_14496-3;2009. We define a new extension type to carry the SpAACe data bytes.

Syntax of extension_payload() is updated by adding one more extension_type as follows.

Table B.1 — Syntax of extension_payload()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| extension_payload(cnt){ |  |  |
| extension_type; | 4 | Uimsbf |
| align = 4; |  |  |
| switch( extension_type ) { |
| case EXT_DYNAMIC_RANGE: |  |  |
| return dynamic_range_info(); |  |  |
| case EXT_DYNAMIC_RANGE: |
| return dynamic_range_info(); |
| case EXT_SAC_DATA: |
| return sac_extension_data(cnt); |  |  |
| case EXT_SBR_DATA: |  |  |
| return sbr_extension_data(id_aac, 0); |  |  |
| case EXT_SBR_DATA_CRC: |  |  |
| return sbr_extension_data(id_aac, 1); |  |  |
| case EXT_FILL_DATA: fill_nibble; /* must be ‘0000’ */ for (i=0; i<cnt-1; i++) { fill_byte[i]; /* must be ‘10100101’ */ 8 } return cnt; case EXT_DATA_ELEMENT: data_element_version; switch( data_element_version ) { case ANC_DATA: loopCounter = 0; dataElementLength = 0; do { dataElementLengthPart; dataElementLength += dataElementLengthPart; loopCounter++; } while (dataElementLengthPart == 255); for (i=0; i<dataElementLength; i++) { data_element_byte[i]; } return (dataElementLength+loopCounter+1); default: align = 0; } case EXT_SPAACE_DATA: return SpAACe_data(cnt); case EXT_FIL: default: for (i=0; i<8*(cnt-1)+align; i++) { other_bits[i]; } return cnt; } | 4 8 4 8 8 1 | Uimsbf Uimsbf Uimsbf Uimsbf Uimsbf Uimsbf |

Table B.2 — Syntax of SpAACe_data()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| SpAACe_data(cnt) { if (SizeOfDataBytes>0){ NumSpAACeFillElements = NumSpAACeFillElements+1; } for (i=0; i< cnt-1; i++) { SpAACeDataByte[i]; } return (cnt); } | 8 | bslbf |

Table B.3 — Additional extension_type definition in Table 4.121 of ISO_IEC_14496-3;2009

| EXT_SPAACE_DATA | ‘0101’ | SpAACe Payload |
| --- | --- | --- |

The aggregated SpAACeDataByte[] buffers from one raw_data_block forms the spAACeAudioStream().

<<< END OF MODIFIED CLAUSE >>>

## References

HOA Transport Format (HTF)

[1] Higher Order Ambisonics (HOA) Transport Format, ETSI TS 103 589 V1.1.1.

3GPP eAACplus

[2] Enhanced aacPlus general audio codec, Encoder specification, Advanced Audio Coding (AAC) part. TS 26.403 V14.0.0.

[3] Enhanced aacPlus general audio codec; Floating-point ANSI-C code TS 26.410 V14.0.0.

Rendering

[4] 3GPP Virtual Reality profiles for streaming applications, 3GPP TS 26.118 V1.0.0.

File format

[5] 3GPP TS 26.244 V14.1.0 Technical Specification Group Services and System Aspects; Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP).

[6] ISO/IEC 14496-3 Information technology; Coding of audio-visual objects; Part 3: Audio

[7] ISO/IEC 14496-12 Information technology -- Coding of audio-visual objects -- Part 12: ISO base media file format