Source:	Xperi

Title:	Verification of CIBR configuration for FOA

## Document for:	Discussion

## Agenda Item:	10.5.2

- Summary

This contribution presents the verification of the CIBR configuration for FOA conversion. In particular, we compare the performance of the conversion to FOA with ESD4 as specified by 3gpp (one virtual speaker in the front and two in the back), ESD4-F (two virtual speakers in the front and one in the back) and ESD16. The test results suggest that rendering objects to ESD16, converting to HOA and then truncating to FOA could lead to a higher quality rendering process in the CIBR configuration.

- FOA rendering in CIBR

2.1 Test Design

The rendering to the ESD domain for the CIBR configuration [1] has been specified as

with rendering to ESD16 for HOA and rendering to ESD4 for FOA. The locations of the virtual speakers for ESD16 and ESD4 have been provided in [3, Section 5.1], and the conversion matrices specified in [4].

When examining this configuration, it seemed unclear to us why we would render objects to ESD4, as opposed to always render to ESD16 and then truncate to FOA in the ambisonics domain. We decided to verify our belief that for objects, rendering to ESD16 and then converting to FOA in the HOA domain would result in better performance. The reference for such “better performance” is, naturally, playback of objects rendered directly to the playout system: 7.4.1 loudspeakers in this test.

In the performed test, we have also added an updated ESD4 configuration, ESD4-F, which rotates the original ESD4 virtual speaker locations (from [3]) by 180 degrees: bottom speakers updated to +/-60 and 180 degrees (instead of 0 and +/- 120 degrees). This rotation results in two virtual speakers in the front and one in the back (instead of one in the front and two in the back). This rotated configuration, ESD4-F, should preserve the spatial left-right frontal image, unlike the original ESD4 setup which collapses it.

There is no audio compression in this test; the test only assesses the renderer processing configuration.

2.2 Test Material

Test materials included in the test were all the objects-based content from the 3GPP VRStream characterization tests.

| Track | Content Type |
| --- | --- |
| 8Obj_Music+Bird | Object-based (8 Objects) |
| Fork | Object-based (10 objects) |
| 8Obj_reservoir | Object-based (8 Objects) |
| CICP19_Festival | Mixed (Channels-7.1.4 + 2 Objects) |
| JammJam | Object-based (12 Objects) |
| Spoon | Object-based (12 Objects) |

Table 1: Test Items

2.3 Listening Environment

The tests were performed in a BS.1116-3 compliant listening lab in XPERI’s Calabasas office over a 7.1.4 speaker layout.

2.4 Listening Panel

The listening panel for this test consisted of seven XPERI/DTS employees (2 Female, 5 Male), all experienced in critical listening tests. All listeners passed post screening as specified in ITU-R BS.1534-3 section 4.1.2.

2.5 Results

All presented results include average scores and 95% confidence intervals (t-distribution).

Figure 1: Results per System

Figure 6: Results per System and Test Item

- Conclusions

The performed tests indicate the following:

- Rendering objects to ESD16 and then truncating to FOA in the ambisonics domain performs better than rendering to ESD4; the former performed about 10 MUSHRA points higher and was statistically better than the latter

- There is not as much difference between ESD4 and ESD4-F as we thought there might be (at least for this test material)

The presented results suggest that rendering to ESD16, converting to HOA, and then truncating to FOA could lead to a higher quality rendering process in the CIBR configuration.

- References

[1] 3GPP TS 26.259 “Subjective test methodologies for the evaluation of immersive audio systems”

[2] ITU-R BS.1534-3 “Method for the subjective assessment of intermediate quality levels of coding systems”

[3] TR 26.861 “On the validity of CIBR baseline testing for VR Stream”

[4] AHVIC-157 “ESD Conversion Matrices for VRStream Audio Profile Characterization”