Source:	NTT and Orange

Title:	Test plan based on trial test results of stereo downmix

Document for:	Discussion

Agenda Item:	2.4

Introduction

In the 126th SA4 meeting in January 2024, the characterization test plan was proposed [4].

There were some concerns

- OPUS has many optional modes and could be tested on other occasions.

- Listeners may be confused if stereo and mono output are combined.

- The number of conditions is too small for one experiment, and one may consider combining several types of audio content in one longer experiment. Listeners, however, may be confused if clean speech, noisy speech, and music are randomly combined.

This contribution provides trial test results of stereo downmix for interoperable EVS recently conducted in NTT labs in order to make further discussions on the test plan.

Conditions of trail listening test

Conditions are shown in Table 1, 2,3, 4 and 5. These conditions are similar to those proposed in [4] (copied to the Annex (Table A1- Table A5)) except for some items, which are shown in red characters.

Table 1 Comparison of conditions between the proposed test and trial test at NTT

|  | Proposed in [5] | Trial test at NTT |
| --- | --- | --- |
| Number of Codecs | 16 | 18 |
| Number of categories | 6 | 6 |
| Number of panels | 6 | 3 |
| Number of listeners per panel | 5 | 8 |
| Number of total listeners | 30 | 24 |
| Number of input items per listener | 6 | 10 # |
| Number of votes per listener | 288 (16*6*3) | 540 (18*10*3) |

# 10 items are assigned to each panel from 30 items (6 categories and 5 pairs of talkers).

Note that

- Only two modes of bit rates (13.2 and 24.4 kbps) were tested.

- For output signals, stereo and mono signals were mixed in the same randomization.

- For input signals, clean speech, noisy speech, and music items were mixed in the same randomization.

- Newly merged FIX_947 updates were checked.

- In the P.800-1 at the selection test [6], the number of votes per listener was 204 (34*1*6)

Table 2: Factors and conditions for Experiment P800-dmx

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | 2 | 13.2, 24.4 kbps with DTX off at 0% FER |
| DTX | 0 | DTX off |
| Input Level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 1 | 0%, |
| Codec references |  |  |
| Codec references | 5 | IVAS stereo (stereo output) EVS dual mono + static dmx, static dmx + EVS static dmx + OPUS IVAS stereo 24.4 kbps mono output + EVS |
| Operating modes | 2 | 13.2, 24.4 kbps with DTX off at 0% FER |
| DTX | 0 | DTX off |
| Input Level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 1 | 0% |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=16, 22 28 dB α = 0.1, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool [27] |
| Audio sampling frequency/bandwidth | 1 | 32 kHz/maximum available audio bandwidth up to SWB |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and three female) |
| Number of categories | 6 | 6 Different environments and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 24 | Naïve listeners |
| Randomizations | 3 | Six panels of 4 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | １ | Japanese |

Table 3: Test conditions for Experiment P800-dmx,
clean speech, noisy speech, and music

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=28 dB | - | - |  |
| c03 | MNRU Q=22 dB | - | - |  |
| c04 | MNRU Q=16 dB | - | - |  |
| c05 | ESDRU α=0.7 | - | - |  |
| c06 | ESDRU α=0.1 | - | - |  |
| c07 | IVAS stereo | 13.2 | No errors |  |
| C08 | IVAS stereo | 24.4 | No errors |  |
| C09 | dual mono EVS@7.2 kbps + static dmx | 2x7.2 | No errors |  |
| c10 | Dual mono EVS@13.2 kbps + static dmx | 2x13.2 | No errors |  |
| c11 | Static dmx + OPUS @13.2 kbps | 13.2 | No errors |  |
| c12 | Static dmx + OPUS @ 24.4 kbps | 24.4 | No errors |  |
| c13 | Static dmx + EVS @13.2 kbps | 13.2 | No errors |  |
| c14 | Static dmx + EVS @24.4 kbps | 24.4 | No errors |  |
| c15 | FIX 947 CuT Stereo dmx + EVS | 13.2 | No errors |  |
| c16 | FIX 947 CuT Stereo dmx + EVS | 24.4 | No errors |  |
| c17 | Previous trunk CuT Stereo dmx + EVS | 13.2 | No errors | NWT c09 |
| c18 | Previous trunk CuT Stereo dmx + EVS | 24.4 | No errors | NWT c10 |

- Table 4: Content type categories and scene definitions (Exp P800-dmx: clean speech)

| Category |  | Room(1 | Reverb | Microphone Setup | Level [dB] | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 |  | small | anechoic | A-B (100cm) | -26 | 1 | Max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 2 |  | large | anechoic | A-B (150cm) | -26 | -1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 3 |  | small | anechoic | M-S | -26 | -1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 4 |  | small | echoic | A-B (100cm) | -26 | -1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 5 |  | large | echoic | A-B (150cm) | -26 | 1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 6 |  | large | echoic | Binaural | -26 | -1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |

- Table 5: Content type categories and scene definitions (Exp P800-dmx: noisy speech)

| Category | Room(1 | Reverb | Microphone Setup | Level [dB] | Background | SNR In dB | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | -26 | car | 15 | 1 | Max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 2 | large | anechoic | A-B (150cm) | -26 | office | 15 | -1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 3 | small | anechoic | M-S | -26 | street | 15 | -1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 4 | small | echoic | A-B (100cm) | -26 | street | 15 | -1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 5 | large | echoic | A-B (150cm) | -26 | office | 15 | 1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |
| cat 6 | large | echoic | Binaural | -26 | car | 15 | -1 | max [available up to SWB] | Random | depending on the panel,1 or 2 items are assigned from 5 pairs of talkers |

Notes:

(1 The specific room characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences.

(2 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(3 The talker positions are part of the scene definition of the different categories. They should be chosen in a random way from the available set of SRIRs for the used room, making sure that there is a good coverage of different possible positions. Different random selections should be made for the different listener panels. The details will be specified in the IVAS processing plan IVAS-7b.

(4 All sentences by the 6 talkers shall be unique.

Observations

We observed the following from the test results.

- The quality of the EVS encoded with the stereo (dynamic) downmix fulfilled the performance requirement defined in [5]

- The quality of the EVS encoded with the stereo (dynamic) downmix was better or not worse than that of the EVS encoded with the stereo (static) downmix.

- Newly merged Fix_947 has no degradation on average for various conditions.

- The quality of OPUS is lower than that of EVS.

- We have no serious problems if stereo and mono output are combined.

- We have no serious problems if clean speech, noisy speech, and music are combined.

Summary

Sources propose:

- The quality of OPUS could be tested on other occasions.

- We can combine stereo and mono output.

- We can combine clean speech, noisy speech, and music for input samples, but can conduct three separate tests.

- References

[1]	S4aA230127-IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase, Version v.0.1.2

[2]	ITU-T P.800 – Use cases, P Suppl. 29 (01/2023)

[3]	S4aA-230086: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.1

[4]	S4-230350: Characterization test plan for stereo downmix

[5]	S4-231049 IVAS-3 Performance Requirements 1.1.0

[6]	S4-230761-IVAS-8a_TestPlan_v.0.8.4

Fig. 1 Average scores of P.800 combining clean speech, music and noisy speech

ANNEX

The test plan proposed in S4-230350: (Characterization test plan for stereo downmix.)

Table A1: Factors and conditions for Experiment P800-dmx

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | 2 | 13.2, 24.4 kbps with DTX off at 0% FER |
| DTX | 0 | DTX off |
| Input Level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 1 | 0%, |
| Codec references |  |  |
| Codec references | 5 | IVAS stereo (stereo output) EVS dual mono + static dmx, static dmx + EVS IVAS stereo 24.4 kbps mono output + EVS |
| Operating modes | 2 | 13.2, 24.4 kbps with DTX off at 0% FER |
| DTX | 0 | DTX off |
| Input Level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 1 | 0% |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=16, 22 28 dB α = 0.1, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool |
| Audio sampling frequency/bandwidth | 1 | 32 kHz/maximum available audio bandwidth up to SWB |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and three female) |
| Number of categories | 6 | 6 Different environments and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | Six panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | １ | [tbd] |

Table A2: Preliminaries for Experiment P800-dmx

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c11 | clean | Static dmx + EVS | 13.2 | No errors |
| 2 | c02 | noisy | MNRU Q=28 dB | - | - |
| 3 | c05 | music | ESDRU α = 0.7 | - | - |
| 4 | c08 | music | IVAS stereo | 24.4 | No errors |
| 5 | c07 | clean | IVAS stereo | 13.2 | No errors |
| 6 | c14 | noisy | Stereo dmx +EVS | 13.2 | No errors |
| 7 | c12 | noisy | Static dmx+EVS | 24.4 | No errors |
| 8 | c01 | clean | Reference | - | - |
| 9 | c09 | clean | Dual EVS + static dmx | 2x7.2 | No errors |
| 10 | c04 | music | MNRU Q=16 dB | - | - |
| 11 | c14 | noisy | IVAS stereo monoout+ EVS | 24.4 | No errors |
| 12 | c16 | music | Stereo dmx + EVS | 24.4 | No errors |

Table A3: Test conditions for Experiment P800-dmx,
clean speech, noisy speech, and music

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=28 dB | - | - |  |
| c03 | MNRU Q=22 dB | - | - |  |
| c04 | MNRU Q=16 dB | - | - |  |
| c05 | ESDRU α=0.7 | - | - |  |
| c06 | ESDRU α=0.1 | - | - |  |
| c07 | IVAS stereo | 13.2 | No errors |  |
| c08 | IVAS stereo | 24.4 | No errors |  |
| c09 | Dual mono EVS@7.2 kbps + static dmx | 2x7.2 | No errors |  |
| c10 | Dual mono EVS@13.2 kbps + static dmx | 2x13.2 | No errors |  |
| c11 | Static dmx + EVS @13.2 kbps | 13.2 | No errors |  |
| c12 | Static dmx + EVS @24.4 kbps | 24.4 | No errors |  |
| c13 | IVAS stereo 24 kbps mono-out+ EVS @13.2 kbps | 13.2 | No errors |  |
| c14 | IVAS stereo 24 kbps mono-out+ EVS @24.4 kbps | 24.4 | No errors |  |
| c15 | CuT Stereo dmx + EVS | 13.2 | No errors | NWT c09 |
| c16 | CuT Stereo dmx + EVS | 24.4 | No errors | NWT c10 |

- Table A4: Content type categories and scene definitions (Exp P800-dmx: clean speech)

| Category |  | Room(1 | Reverb | Microphone Setup | Level [dB] | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 |  | small | anechoic | A-B (100cm) | -26 | 1 | Max [available up to SWB] | Random | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 |  | large | anechoic | A-B (150cm) | -26 | -1 | max [available up to SWB] | Random | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 |  | small | anechoic | M-S | -26 | -1 | max [available up to SWB] | Random | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 |  | small | echoic | A-B (100cm) | -26 | -1 | max [available up to SWB] | Random | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 |  | large | echoic | A-B (150cm) | -26 | 1 | max [available up to SWB] | Random | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 |  | large | echoic | Binaural | -26 | -1 | max [available up to SWB] | Random | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

- Table A5: Content type categories and scene definitions (Exp P800-dmx: noisy speech)

| Category | Room(1 | Reverb | Microphone Setup | Level [dB] | Background | SNR In dB | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | -26 | car | 15 | 1 | Max [available up to SWB] | Random | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | anechoic | A-B (150cm) | -26 | office | 15 | -1 | max [available up to SWB] | Random | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | anechoic | M-S | -26 | street | 15 | -1 | max [available up to SWB] | Random | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | small | echoic | A-B (100cm) | -26 | street | 15 | -1 | max [available up to SWB] | Random | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | large | echoic | A-B (150cm) | -26 | office | 15 | 1 | max [available up to SWB] | Random | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | large | echoic | Binaural | -26 | car | 15 | -1 | max [available up to SWB] | Random | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |