# Multi-SDO PI-KPI-KQI Definitions

# Document number:	S5eCPM00XX

# Source:	TEOCO (TM Forum)

# Title:		M-SDO PI-KPI-KQI Definitions

# Meeting date:	2013-06-19

# 1- Objective

# Reviewing the definitions of Performance Indicator (PI), Key Performance Indicator (KPI) and Key Quality Indicator(KQI) in draft of NGCOR PM Requirements Document (V1.16)

# 2- References

- Requirements for Performance Management for Converged Networks, Version 1.16

# 3- Detailed proposal

### Definitions in version 1.16

| Performance Indicator (PI) | Performance related data produced by Network Elements of wireline and wireless network. See lowest level of Figure 4. | TM Forum GB 917 Release 2.5 |
| --- | --- | --- |
| Key Performance Indicator (KPI ) | KPI provides a measurement of a specific aspect of the performance of a service resource (network or non-network) or group of service resources of the same type. A KPI is restricted to a specific resource type. | TM Forum GB923 V3.0 |
| Key Quality Indicator (KQI) | KQI provides a measurement of a specific aspect of the performance of the product, product components (services) or service elements and draw their data from a number of sources including the KPIs. | TM Forum GB923 V3.0 |

Some observations:

- PI is defined by the producing system and not by the monitored resources.

- TM Forum GB923 is focused on service resources.

### 3GPP 32.410 Definition

| Key Performance Indicator | 3GPP 32.410 | Key Performance Indicators (KPI) emanate from strategic goals of enterprise, and cascade through the entire organization. KPIs are specified through definition and measurement of key parameters of input/output of internal network system and/or maintenance & operation progress of an enterprise. KPIs are primary metrics to evaluate process performance as indicators of quantitative management, and to measure progress towards the enterprise's goals. |
| --- | --- | --- |

### Alternative approach for PI & KPI Definition

| Performance Indicator (PI) | A Performance Indicator provides a measurement of a specific aspect of the performance of network or non-network resources of the same type. A PI is restricted to a specific resource type. PIs are usually produced by Network Elements of wireline and wireless networks. |  |
| --- | --- | --- |
| Key Performance Indicator (KPI ) | KPIs are primary PIs that evaluate process performance as indicators of quantitative management, and measure progress towards an enterprise's goals. |  |

Notes:

- It may be required to distinguish between “Collected KPIs” and “Derived KPIs”

### Option 1: KPI Definition - by the Monitoring system

### Option 2: KPI Definition – by Indicator Importance/Role

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"