# Multi-SDO Project on Converged Management PM Interface Definitions

# Document number:	S5eCPM0098

# Source:	TEOCO (TM Forum)

# Title:		KPI Properties

# Meeting date:	2014-07-15

# Objective

# The recent draft of PM2 document – version 0.6 details a list of properties for measurements with some references of KPI properties. This contribution suggests some required properties to support KPIs.

# References

# Following is a list of references:

- S5eCPM0095, PM2: Metadefinitions for Measurements and KPIs V0.6

# Detailed Proposal

# First, it is assumed that most properties can serve both measurements and KPIs, in particular: name, description, unit, type, monitoredObjectClassList and technologyList.

## KPI additional properties

Based on the existing properties in 3GPP and TM Forum, it is suggested to add the following properties to support KPIs:

| Converged Property name | Description | Legal values |
| --- | --- | --- |
| KPIIndication | This property indicates whether the measurement is a collected counter or a KPI. | Counter, KPI |
| logicalKPI Formula | The logical formula should describe what the KPI formula is in logical way. The description of the formula is given in a written textual format without any measurement or counter names. E.g. a success rate KPI’s logical formula is the successful event divided by all event. | String |
| physicalKPIFormula | This field should contain the KPI formula description using the defined counter names. | String |
| measurementNames | Measurement names used for the KPI | String (an optional property) |

# Recommendation

# It is proposed to include the suggested properties in the converged list of PM2.

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"