Fixed Mobile Convergence (FMC)

Converged Performance Management

PM2: Metadefinitions for Measurements and KPIs

Version 0.20 (draft)

Source: M-SDO Converged PM project

Editor: Christian Toche (Huawei Technologies)

Table of Contents

1	Scope	4

2	References	4

3	Definitions and abbreviations	4

3.1	Definitions	4

3.2	Abbreviations	5

4	Requirements	6

5	Definition of Performance Measurement Metadata	6

Annex A (Informative): Counters/KPIs properties	10

A1	3GPP Performance Metadata	10

A2	TM Forum Performance Metadata	10

A3	Proposed JWG Measurement Profile	11

# 1	Scope

This document defines the metadata for Performance Management (PM) performance measurements (KPIs/counters). The definition would enforce consistency in description/definition of all standardized performance measurements (counters/KPIs) by various SDOs and organizations (or Multi-SDO) involved in the Management of Converged Networks [1,2]. This consistency would facilitate encoding and decoding of performance measurements (counters/KPIs) exchanged (via management protocols) and/or stored in (database or files).

The metadata related to PM operations/notifications is not part of this PM measurement metadata.

This specification takes into account the recommendations from the Fixed Mobile Convergence (FMC) model repertoire [2] developed by the Multi-SDO JWG on Model Alignment.

# 2	References

[1]	TM Forum TR 166 Information Model Federation Concepts and Principles (http://collab.tmforum.org/sf/go/doc13634?nav=1)

[2]	3GPP TR 28.821 Fixed Mobile Convergence (FMC) Model Repertoire

[3]	IETF RFC 2578 Structure of Management Information Version 2 (SMIv2)

[4]	3GPP TS 32.401 Performance Management (PM); Concept and requirements

[5]	PM8 Description of CollectionMethodTypes, Measurement DataTypes and List of Formulas

[6]	Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM)

# 3	Definitions and abbreviations

For the purposes of this document, the following definitions, symbols and abbreviations apply. For definitions, symbols and abbreviations not found here. See also Fixed Mobile Convergence (FMC) Model Repertoire [2] and Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) [6].

## 3.1	Definitions

Measurement/Indicator: A Performance attribute, a type of a parameter that is measured on a measured object Class.

Granularity period: The sampling rate of Performance Measurements (aka Performance Indicators). This is the rate of producing the measurements as determined by the Producing Application (aka the Performance server).

Reporting Period: – The rate (frequency) of collecting Performance Measurements as determined by the Consuming Application, (aka the Performance client). The Collection Resolution is an integral multiple of the corresponding granularity.

## 3.2	Abbreviations

CC	Cumulative Counter

DER	Discrete Event Register

FNIM	Federated Network Information Model

FMC	Fixed Mobile Convergence

HHO	 Hard Handover

IETF	Internet Engineering Task Force

IoT	Internet of Things

JWG	Joint Working Group

KPI	Key Performance Indicator

PM	Performance Management

RAB	 Radio Access Bearer

SDO	Standards Development Organization (e.g., 3GPP, TM Forum)

SI		Status Inspection

SMS	 Short Message Service

TBA	To Be Added

TBC	To Be Checked

TBD	To Be Defined

# 4	Requirements

Metadata is defined as data providing information about aspects of other data. Metadata can be exchanged in protocols, and can be stored and managed in a database, often called a Metadata registry or Metadata repository. Metadata can be defined in machine-readable syntax.

The PM performance measurement metadata is the properties of the performance measurements (counters/KPIs) including their associations to other performance measurements (counters/KPIs). The number of properties and the level of the details in the property definition depend on the metadata intended usage.

The purpose for defining the PM performance measurement metadata is to enforce a consistency in description/definition of all standardized counters/KPIs. This consistency facilitates encoding, decoding, interpreting and in general, processing of counters/KPIs defined by Multi-SDOs involved in the Management of Converged Networks.

# 5	Definition of Performance Measurement Metadata

The following table provides the PM performance measurement metadata; i.e., the definition of the properties.

When defining a new measurement and when this new measurement is similar to a measurement already defined in another standard, then this standard should be referenced and only the missing properties should be defined in addition.

Note: The following shortcuts are used in the table. Gen= GenericMeasurement, Counter=CounterMeasurement, KPI=KPIMeasurement.

| Converged Property name | Description | Legal values | Qualifier |
| --- | --- | --- | --- |
| Gen::name | This property captures the SDO agnostic measurement name (same name among several SDOs). The name does not carry the measurement family name. | String | Mandatory |
| Gen::sdoAliasNameList | This property captures the list of SDO specific measurement names. The measurement name convention is SDO specific. | String | Optional |
| Gen::dataType | This property identifies the type of the value of the measurement, such as Integer, Float, or derived value types like a fixed array of values (Vector) etc. | Integer, Float Refer to [5] for details of possible derived value types. | Mandatory |
| Gen::unit | This property defines the unit by which the measurement is measured. For example: seconds, Celsius, Kelvin, Ampere, Volt, Watt, dB, kBit/s, rate per second, etc. | String | Mandatory |
| Gen::dataRange | This property specifies the allowed range of values for a measurement. | String | Optional |
| Gen::family | This property identifies the measurement family (e.g. HHO, RAB, SMS). Note that this family may also be used for measurement administration purpose. Note: when defining a new family name, consult the family names defined by other SDOs so that all the names are unique. It is not allowed to define a new family with the same semantics of an already existing family. | String | Mandatory |
| Gen::monitoredObjectClassList | This property indicates the list of <<InformationObjectClass>> names to which this measurement is related (e.g. UtranCell, SgsnFunction). | String | Mandatory |
| Gen::documentation | This property contains a textual description of this modelled element. Should refer (to enable traceability) to a specific requirement. | String | Mandatory |
| Gen::sourceStandard | This property indentifies the referenced standard. All the other properties (e.g., collectionMethodType) of this measurement must refer to the referenced standard if defined there. In case the definition of this measurement conflicts with the referenced definition, the referenced definition takes precedence. | String | Mandatory |
| Gen::augmentationInformation | This property captures measurement information which is not performance information but can be used to augment the performance Measurement data. This property may be specified at the counter level or at group of counters level. | This property can have different types depending on the information content relevant in a specification (e.g., Integer, Real, String, set of Name Value pairs, enumeration etc.). The specification of this property must include the exact type of this information. Examples Integer : Number of IoT respondents Real: Temperature String : Geolocation Enumeration Type: Operation Status | Conditional Mandatory (This property must be specified along with the specific information type to be used legally a) if this information is necessary for understanding of the related performance information and/or b) if it disambiguates the related performance information. |
| Counter::collectionMethodType | This property indicates the five forms in which the measurement data can be captured. - COUNTER This literal defines a Collection Method using a variable whose value can only be incremented and which is reset at the beginning of each granularity period to well defined value (usually “0”). In 3GPP TS 32.401 [4], it is called Cumulative counter (CC). See [5] for detail description. - CUMULATIVE This literal defines a Collection Method using a variable whose value can only be incremented and which wraps around on reaching maximum possible value that the variable can attain. In IETF RFC 2578 [3] it is called Counter32/Counter64. See [5] for detail description. - GAUGE This literal defines a Collection Method using a dynamic variable whose value can be incremented or decremented, and which may be reset at the beginning of a granularity period or recording interval ( multiple consecutive granularity periods). See 3GPP TS 32.401 [4] and [5] for detail description. - DISCRETE_EVENT This literal defines a collection method (using a set of appropriate data containers) where data related to a particular event type is captured and every nth event is registered, where n ≥ 1. The captured information is reset at the beginning of each granularity period. In 3GPP TS 32.401 [4], it is called Discrete Event Registration (DER). Refer to [5] for detail description. - STATUS_INSPECTION This literal defines a Collection Method that uses a mechanism for high frequency samplings of internal counters at predefined rates and captured using appropriate data type. The captured information is reset at the beginning of each granularity period. See 3GPP TS 32.401 [4] and [5] for detail description. The differences between these Collection Methods and guidelines for usage of the same is explained in [5]. | COUNTER, CUMULATIVE, GAUGE, DISCRETE_EVENT, STATUS_INSPECTION | Mandatory |
| Counter::condition | This property indicates the condition which causes the measurement result data to be updated. This will be defined for example by identifying protocol related trigger events for starting and stopping measurement processes, or updating the current measurement result value. | String | Conditional Mandatory (When counter::CollectionMethodType is COUNTER or CUMULATIVE) |
| Counter::collectionFormula | This property indicates the formula that may be applied to whole or part of information collected (and optionally processed) using any of the specified Collection Method Types (Counter::collectionMethodType). These formulas are standard statistical functions which reduce a set of values to single indicative value based on type of function applied. Relationship to other Properties: If the Counter::collectionMethodType takes either of the values DER or SI, then: If the property Counter::processingProperty is specified then the specified processing should be carried out on the collected information and the Counter::collectionFormula should be applied to the processed information. If the property Counter::processingProperty is not specified then the Counter::collectionFormula should be applied on the collected information. If the Counter::collectionMethodType does not take either of the values DER or SI, then: the Counter::collectionFormula should be applied on the collected information. See [5] for the description of the collectionFormulas. | SUM, COUNT, MIN, MAX, AVERAGE, FIRST, LAST, PRODUCT, STDEV_SAMPLE, STDEV_POPULATION, VARIANCE_SAMPLE, VARIANCE_POPULATION, MEDIAN, MODE_SNGL, LARGE, SMALL, PERCENTILE_INC, QUARTILE_INC, PERCENTILE_EXC, QUARTILE_EXC OTHER | Conditional Mandatory (When multiple values are collected) |
| Counter::collectionFormulaParameter | This property indicates the parameter in case of LARGE, SMALL, PERCENTILE formulae (Counter::Formula). This parameter will indicate different type of value based on formula type. Relationship to other Properties: |

This property is applicable in case Counter::collectionFormula is specified and that property may take one of the three values LARGE, SMALL, PERCENTILE. In all other cases this is not relevant. This is used during processing of collected information using any of the above formulas.

| Refer to [5] for details of its usage in different formulas. | Integer |  | Conditional Mandatory (When Counter::collectionFormula is: LARGE, SMALL, PERCENTILE) |
| --- | --- | --- | --- |
| Counter::otherCollectionFormula | This property indicates the formula of a measurement/Counter where the value of Counter::collectionFormula is OTHER. Relationship to other Properties: This property is applicable in case Counter::collectionFormula is specified and that property takes the value OTHER. This property is not applicable in all other cases. (I.e., this is used to process collected information or processed information when Counter::processingProperty is specified and Counter::collectionMethodType takes the value DER or SI. |  | String | Conditional Mandatory (When Counter::collectionFormula is “OTHER”) |  |
| Counter::samplingInterval | This property indicates the sampling interval of a collection method type in the case where the collection is carried out by sampling the target at fixed intervals during a granularity period. Relationship to other Properties: This property is applied if Counter::collectionMethodType takes the values STATUS_INSPECTION or GAUGE and is used during collection of measurement information. It is not relevant for all other Counter::collectionMethodType values. |  | Integer | Conditional Mandatory (When counter::CollectionMethodType is STATUS_INSPECTION or GAUGE and the collection is carried out by sampling it in fixed time intervals) |  |
| Counter:samplingRate | This property indicates the sampling of every Nth event of a specified event type to be observed of those occurring in a granularity period. Relationship to other Properties: This property is applicable when the Counter::collectionMethodType takes the value DER and is used during collection of measurement information. |  | Integer; Used in case of DER Collection Method | Conditional Mandatory (When counter::CollectionMethodType is DER and the collection is carried out by sampling the Nth event) |  |
| Counter::processingProperty | This property describes the processing to be carried out on the data collected using a Collection Method. This is valid for DER and SI Collection Methods. Relationship to other Properties: In case this property is specified, the indicated processing should be carried out on the data collected by the Counter::collectionMethodType. The specified formula (Counter::collectionFormula) should then be applied on the processed data. In case this property is not specified, no processing needs to be carried out on the data collected by the Counter::collectionMethodType. The specified formula (Counter::collectionFormula) should be applied directly on the information collected by Counter::collectionMethodType. | String | Conditional Mandatory (When counter::CollectionMethodType is DER or STATUS_INSPECTION and when pre-processing needs to be carried out before applying counter::collectionFormula) |
| KPI::formulaDescription | This property contains a textual description of the KPI::formulaSpecification. |  | String | Mandatory |  |
| KPI::formulaSpecification | This property contains the formula. |  | String | Mandatory |  |

# Annex A: Measurement Profile

The converged metadata defined in the previous section are defined in a UML profile to use them as templates for defining measurement instances.

Figure 1: Measurement Profile

The profile is associated to the Class metaclass on a case by case basis. This allows a model designer to define individual measurement instances. See an example below:

Figure 2: Example Counter Measurement Definition

Figure 3: Example KPI Measurement Definition

Counter: SuccEstabPSSetupTimeMax (UMTS)

| Property | Value |
| --- | --- |
| Gen::name | SuccEstabPSSetupTimeMax |
| Gen::sdoAliasNameList |  |
| Gen::dataType | INTEGER |
| Gen::unit | Milliseconds |
| Gen::dataRange |  |
| Gen::family | UMTS.RAB |
| Gen::monitoredObjectClassList | RNC |
| Gen::documentation | This counter shows the maximum time for establishing a successful setup of a PS session in a time window. see TS 25.413 [5]. |
| Gen::sourceStandard | 3GPP |
| Gen::augmentationInformation |  |
| Counter::collectionMethodType | GAUGE |
| Counter::condition |  |
| Counter::collectionFormula | MAX |
| Counter::collectionFormulaParameter |  |
| Counter::otherCollectionFormula |  |
| Counter::samplingInterval |  |
| Counter:samplingRate |  |
| Counter::processingProperty | The processing takes the time intervals for each successful RAB establishment between the receipt by the RNC of a RANAP "RAB ASSIGNMENT REQUEST" message to establish a RAB for PS domain, and the first corresponding (based on RAB ID) transmission by the RNC of a RANAP "RAB ASSIGNMENT RESPONSE" |

KPI: E-UTRAN Cell Availability (LTE)

| Property | Value |
| --- | --- |
| Gen::name | EUTRANCellAvailability |
| Gen::sdoAliasNameList |  |
| Gen::dataType | Float |
| Gen::unit | Percentage |
| Gen::dataRange | 0..100 |
| Gen::family | Availability |
| Gen::monitoredObjectClassList | E-UTRAN Cell |
| Gen::documentation | A KPI that shows the Availability of E-UTRAN Cell. See TS 25.450 [6]. |
| Gen::sourceStandard | 3GPP |
| Gen::augmentationInformation |  |
| KPI::formulaDescription | The KPI calculates the percentage of the time when the E-UTRAN cell is available from the overall measured time window. |
| KPI::formulaSpecification | ((sum())/) * 100 |

3GPP is a trade mark of ETSI