3GPP TSG-SA5 (Telecom Management)	S5eHNB0021

3GPPSA5-Virtual Meeting on HeNB

- Source:	Verizon/ALU

- Title:	System Parameters for HeNB

- Document for:	Discussion and Approval

Agenda Item:

# 1	Decision/action requested

The group is asked to discuss the proposed high level system parameter, and introduce the agreed part into the draft TS 32.592 for HeNB

# 2	References

[1] 3GPP TS 32.582

[2] 3GPP TS 32.592

[3] TR-196

# 3	Rationale

The Home ENodeB is expected to be a complex entity when compared to a typical home router. The Home eNodeB being an entity in a subscriber’s home is expected to provide atleast the same level of reliability and service availability as a device typically found in a networked home. (E.g. a Broadband home router, DSL modem etc). Thus, from a service provider’s point of view, the Home eNodeB entity deployment requires the remote management of the device beyond the typical configuration and black box treatment. In other words, a service provider expects the remote management of a Home eNodeB be comparable to currently deployed managed networked devices in homes.

The typical Home eNodeB is expected to have multiple sub-systems that work together. This document proposes that the level of detail in the parameter categories include a generic system and sub-system level information that can be used by service providers.

Typical FemtoCells have multiple HW/SW subsystems and SW stacks on the HeNodeB. Service providers are particularly interested in information at a sub-system level of home networked devices. Hence the parameters in the info model should provide the ability for an insight into the system and subsystems of a Home eNodeB.

The system parameters described below are provided in the configuration section and are mainly read-only parameters that are very useful to service providers when the state/health of a particular Home eNodeB device in under investigation. These parameters provide a single point snapshot of the device without the overhead of  log file and fault correlation (i.e back end processing).

Start of Section

### 6.1.4	System

System Level Parameter types

| Parameter Type | Description | Notes |
| --- | --- | --- |
| Software Information | Information of all software images loaded/installed | n/a |
| Operational informatio | System operation information and history including information such as the current mode of operation e.g. diag mode, debug mode, release mode (read only), reset history, | n/a |
| LED status | Information related to user side display of LED entities including string description of LED label(if applicable) (read only) | n/a |
| Interface information | Information related to the sub-system interfaces e.g. Processor <-> modem interface, modem<->radio interface etc | n/a |
| System flows | Information of all system flows(running and historical) e.g signalling, traffic, security, PDCP flows etc. | n/a |

### System Level Parameters

| Name | Type | RW | Description | Object Default |
| --- | --- | --- | --- | --- |
| .SoftwareInfo | object |  | This object contains parameters related to all software loaded and installed |  |
| FirmwareInfo | String | R | Information of all firmware including version |  |
| OSInfo | String | R | Information related to Operating Systems and versions |  |
| Driver_Apps | String | R | Information related to any drivers and applications (if applicable) & version |  |
| UpgradeInfo | String | R | Information related to versions of software upgraded and rolled back with timestamps separated by commas |  |
| .OperationalInfo | object |  | This object contains parameters related to runjing configuration of the system |  |
| Mode | String | R | Information that describes the operation mode (e.g. diagnostic, normal, error) |  |
| OperationStartupTime | unsignedInt | R | Time in seconds from bootup until the system is considered operational |  |
| RebootHistory | String | R | Information listing the timestamp information related to past system reboots. |  |
| CriticalSystemsWatermark | String | R | Information listing which sub-systems are at or above a critical watermark. |  |
| .<i>.LEDInfo | Object |  | Information related to LED entities |  |
| Description | String | R | Description of LED function |  |
| State | Enum {solid, blinking,off} | R | Describes the current state of LED |  |
| .<i>.InterfaceInfo | Object |  | Information related to interfaces |  |
| Description | String | R | Description of interface |  |
| State | Enum {normal, loaded,idle,error,critical,non-existent} | R | State of the interface |  |
| .<i>.SystemFlows | Object |  | Information related to high level flows |  |
| Decription | String | R | Description of system flow |  |
| State | Enum {normal, loaded,idle,error,critical,non-existent} | R | State of the system flow |  |
| History | String | R | History of this system flow (if applicable) (for e.g. a tunnel that will have a history entry indicating when and how often the tunnel was toggled) |  |

End of Section