| 3GPP SA5 - TM Forum Model Alignment Study |  | Document: S5vTMFa193 |  |
| --- | --- | --- | --- |
| Meeting date: October/17-19, 2011 |  |  |  |
| Author: | Bernd Zeuner, Deutsche Telekom, b.zeuner@telekom.de |  |  |
| Title: | DT comments on UML Repertoire V1.2 (S5vTMFa174) - Revision 1 |  |  |
| Document for: | Review & Agreement |  |  |

1	Decision/action requested

Review & Agreement

2	References

[1]		S5vTMFa174 “Converged Management of Fixed/Mobile Networks – UML Repertoire Version 1.2”

3	Current status

This contribution provides comments from DT on the UML Repertoire V1.2 contained in S5vTMFa174 to be discussed during the October/17-19, 2011 meeting. The comments are highlighted by revision marks.

Main changes:

- Figures added to section on basic model elements (5.1)

- Labels removed from associations

- Required property list and meta model figure for attributes added (5.1.1.1)

- Composite aggregation association relationship added (5.1.10)

- Provide a definition for the predefined data types (5.3.3)

- Minor editorial updates

Changes made during the Nanjing meeting marked by this colour.

=================

3GPPSA5-TM Forum Model Alignment meetings 	S5vTMFa174

Source:	Editor (Ericsson)

Title:	Model Repertoire (for Converged Management of Fixed/Mobile Networks)

Document for:	Approval

1.   	Decision/action requested

This V1.2 is output of 3GPPSA5-TM Forum Model alignment F2F Vienna meeting, August 29-31, 2011.

2. 	References

None

3.		Current status

The following is a stand alone document that captures the current status and agreement of the characteristics of the Model Repertoire discussed in the Joint 3GPP/TM Forum model alignment project.

=================

Converged Management of Fixed/Mobile Networks –

Model Repertoire Version 1.2

Source: Joint 3GPP/TM Forum model alignment project

Editor: Edwin Tse (Ericsson)

Disclaimer: Final agreement on this document cannot occur until “Governance / Terms of Reference / Working Procedures” agreements between the involved organizations (including 3GPP & TM Forum) have been established.

Table of Contents

1        Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Requirements	6

5	Model Elements and Notations	7

5.1	Basic model elements	7

5.1.1	Attribute	7

5.1.2	Aggregation relationship	8

5.1.3	Association relationship	8

5.1.4	Generalization relationship	9

5.1.5	Dependency relationship	10

5.1.6	Comment	10

5.1.7	Multiplicity, a.k.a. cardinality used in relationship	10

5.1.8	Rolename	11

5.1.9	Xor constraint	11

5.2	Stereotype	12

5.2.1	<<ProxyClass>>	12

5.2.2	<<InformationObjectClass>>	13

5.2.3	<<names>>	13

5.2.4	<<dataType>>	14

5.2.5	<<enumeration>>	15

5.3	Others	16

5.3.1	Association class	16

5.3.2	Abstract class	17

5.3.3	Predefined data type	17

6.	Qualifier	17

A.1	First Example	19

A.2	Second Example	20

Table of Figures

Figure 1: Attribute notation	7

Figure 2: Aggregation relationship notation	8

Figure 3: Bidirectional association and name notation	9

Figure 4: Non navigational association and name notation	9

Figure 5: Unidirectional association and name notation	9

Figure 6: Generalization relationship notation	9

Figure 7: Dependency relationship notation	10

Figure 8: Comment notation	10

Figure 9: Cardinality notation	11

Figure 10: Rolename notation	11

Figure 11: {xor} notation	12

Figure 12: <<ProxyClass>> notation	13

Figure 13: <<InformationObjectClass>> notation	13

Figure 14: <<names>> notation	14

Figure 15: <<dataType>> notations	14

Figure 16: Usage example of <<dataType>>	15

Figure 17: <<enumeration>> notation	15

Figure 18: Association class notation	16

Figure 19: Abstract class notation	17

Figure 20: <<ProxyClass>> Notation Example A.1	19

Figure 21: <<ProxyClass>> Notation Example A.2	20

# Scope

The JWG on Model Alignment work has chosen UML to capture behaviour of systems/entities under management.

UML provides a rich set of concepts, notations and model elements to model distributive systems. Usage of all UML notations and model elements is not necessary for the purpose of JWG Model Alignment work. This paper documents the necessary and sufficient set of UML notations and model elements, including the ones built by the UML extension mechanism <<stereotype>>, for use by JWG Model Alignment work. Collectively, this set of notations and model elements is called the FNM (developed by the Converged Management of Fixed/Mobile Networks project) modelling repertoire.

JWG Model Alignment specifications shall employ the UML notation and model elements of this repertoire.  In the course of the JWG Model Alignment work, JWG Model Alignment group may modify (add, delete, modify) UML notation and model elements of this repertoire when necessary.

# References

[1]	OMG Unified Modelling Language (OMG UML), Infrastructure, Version 2.3.

[2]	OMG Unified Modelling Language (OMG UML), Superstructure, Version 2.3.

[3]	3GPP TS 32.300: 3rd Generation Partnership Projects; Technical Specification Group Services and System Aspects; Telecommunication management; Configuration Management (CM); Name convention for Managed Objects.

[4]	3GPP TS 23.002: 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network architecture.

# Definitions and abbreviations

## Definitions

Distinguished Name: See 3GPP TS 32.300 [3].

Naming attribute: See 3GPP TS 32.300 [3].

Lower Camel Case: It is the practice of writing compound words in which the words are joined without spaces. Initial letter of all except the first word shall be capitalized.  A word can be represented in abbreviated form. The abbreviation must be documented.  Examples: ‘mNIdentity’ and ‘minorDetails’ are the LCC for “managed node identity” and “minor details” respectively.

Upper Camel Case: It is the Lower Camel Case except that the first letter is capitalised.  Examples: ‘MNIdentity’ and ‘MinorDetails’ are the UCC for “managed node identity” and “minor details” respectively.

## Abbreviations

CM				Configuration Management

DN	Distinguished Name

FNM	Federated Network Model

IOC	Information Object Class

IRP	Integration Reference Point

JWG	(3GPP/TM Forum) Joint Working Group

LCC	Lower Camel Case

NRM	Network Resource Model

OMG	Object Management Group

UCC	Upper Camel Case

UML	Unified Modelling Language (OMG)

# Requirements

The UML notations and model elements captured in this repertoire shall be used to model behaviours of the systems/entities specified by the JWG Model Alignment work such as the Umbrella model of the FNM discussed in Converged Management of Fixed/Mobile Network project.

# Model Elements and Notations

## Xxxx

Note that the graphical notation in this document is only used to represent particular model elements. Although the graphical notation is a correct representation of the model element, it may not be a valid representation of a UML class diagram.

The examples used in this document are for illustration purposes only and may or may not exist in specifications.

UML properties not described in this document shall not be used in specifications based on this repertoire.

## Basic model elements

UML has defined a number of basic model elements. This sub-clause lists the subset selected for use in specifications based on this repertoire. The semantics of these selected basic model elements are defined in [1].

For each basic model element listed, there are three parts. The first part contains its description. The second part contains its graphical notation examples and the third part contains the rule, if any, recommended for labelling or naming it.

The graphical notation has the following characteristics:

- OMG defines the graphical notation of a UML class having up to five compartments (Name, Attribute, Operation, Signal, Structure -  see section ?? of [2]). This repertoire only allows the use of the name and attribute compartments. The Operation compartment may be present but is always empty.

- Classes may or may not have attributes.

- The graphical notation of a class may show an empty attribute compartment (i.e. the second compartment) even if the class has attributes.

- The attribute compartment shall show all attributes of the classes involved in an inheritance relationship.

- The visibility symbol shall not appear along with the class attribute.

### Attribute

#### Description

- It is a typed element representing a property of a class. See 10.2.5 Property of [1].

- An element that is typed implies that the element can only refer to a constrained set of values.

- See 10.1.4 Type of [1] for more information on type.

- See 5.2.4 and 5.3.3 for predefined data types and user-defined data types that can apply type information to an element.

Attributes shall have the following properties:

- Attribute Documentation
Contains a textual description of the attribute.
Should refer (to enable traceability) to the specific requirement.

- Ordered
For a multi-valued multiplicity; this specifies whether the values in an instantiation of this attribute are sequentially ordered. (See section 7.3.44 and its Table 7.1 of [?])
Legal values: true,false; default value =  false.

- Unique
For a multi-valued multiplicity, this specifies whether the values in an instantiation of this attribute are unique (i.e., no duplicate attribute values are allowed). (See section 7.3.44 and its Table 7.1 of [?])
Legal values: true,false; default value = true.

- Read Only
If true, the attribute can only be read, and not written by the manager; default value is false. (See section ?? of [?]).

- Type
Refers to a predefined or user defined data type. (See section ?? of [?]).

- Default Value
Provides the value that the attribute has to start with in case the value is not provided during creation or already defined because of a system state. (See section ?? of [?]).

- Multiplicity
Defines the number of values the attribute can simultaneously have. (See section ?? of [?]).

- Invariant
Identifies whether the value of the attribute can be changed; default value is false.

- Value Range
Identifies the allowed values the attribute can have. (See section ?? of [?]).

- Attribute Notifications
Identifies whether a notification has to be sent in case of a value change.

- Support Qualifier
Legal values: optional,mandatory,conditionalMandatory,conditionalOptional,conditional; default value =  mandatory.

Figure ?: Attribute meta model

#### Example

This example shows three attributes, i.e., a, b and c, listed in the attribute (the second) compartment of the class Xyz.

Figure 1: Attribute notation

#### Name style

An attribute has a name. Lower Camel Case (LCC) shall be used for it.

### Aggregation association relationship

#### Description

It shows a class as a part of or subordinate to another class.

An aggregation is a special type of association in which objects are assembled or configured together to create a more complex object. Aggregation protects the integrity of an assembly of objects by defining a single point of control called aggregate, in the object that represents the assembly.
An aggregation shall contain a description of its use.

See 7.3.2 AggregationKind (from Kernel) of [2].

#### Example

A hollow diamond attached to the end of a relationship is used to indicate an aggregation. The diamond is attached to the class that is the aggregate. The aggregation association shall have an indication of cardinality at each end of the relationship (see 5.1.7).

Figure 2: Aggregation relationship notation

#### Name style

It has no name so there is no name style.

### Association relationship

#### Description

It shows a relationship between two classes and describes the reasons for the relationship and the rules that might govern that relationship.

It has ends. Its end, the association end(s), specifies the role that the object at one end of a relationship performs. Each end of a relationship has properties that specify the role (see 5.1.8), multiplicity (see 5.1.7), visibility and navigability (see the arrow symbol used in 5.1.3.2) and may have constraints. Note that visibility would not be used (see bullet 5 of 5.1).

See 7.3.3 Association of [2].

Three examples below show a binary association between two model elements. The association can include the possibility of relating a model element to itself. The first example (Figure 3) shows a bi-directional navigational association in that each model element has a pointer to the other. The second example (Figure 4) shows a bi-directional non-navigational association in that each model element does not have a pointer to the other; i.e., such associations are just illustrative. The third example (Figure 5) shows a unidirectional association (shown with an open arrow at the target model element end) in that only the source model element has a pointer to the target model element and not vice-versa.

#### Example

An association shall have an indication of cardinality (not shown in the examples below; see 5.1.7) and shall have an indication of the role name (see 5.1.8).

The model element involved in an association is said to be “playing a role” in that association. The role has a name such as +theAClass in the examples below. See 5.1.8. Note that the "+" character in front of the role name, indicating the visibility, is ignored.

Figure 3: Bidirectional association relationship notation

If the UML graphical representation can not indicate bidirectional associations (shown by arrows at both ends); i.e., does not distinguish between navigable at both ends and non-navigable at both ends, role names must not be provided in the non-navigable case.

Figure 4: Non navigational association relationship notation

Figure 5: Unidirectional association relationship notation

#### Name style

A role name shall use the LCC style.

### Generalization relationship

#### Description

It indicates a relationship in which one class (the child) inherits from another class (the parent).

See 7.3.20 Generalization of [2].

#### Example

This example shows a generalization relationship between a more general model element (the IRPAgent) and a more specific model element (the IRPAgentVendorA) that is fully consistent with the first element and that adds additional information.

Figure 6: Generalization relationship notation

#### Name style

It has no name so there is no name style.

### Dependency relationship

#### Description

“A dependency is a relationship that signifies that a single or a set of model elements requires other model elements for their specification or implementation. This means that the complete semantics of the depending elements is either semantically or structurally dependent on the definition of the supplier element(s)...“, an extract from 7.3.12 Dependency of [2].

#### Example

This example shows that the BClass instances have a semantic relationship with the AClass instances. It indicates a situation in which a change to the target element (the AClass in the example) will require a change to the source element (the BClass in the example) in the dependency.

Figure 7: Dependency relationship notation

#### Name style

A dependency relationship has no name so there is no name style.

### Comment

#### Description

- A comment is a textual annotation that can be attached to a set of elements.

- See 7.3.9 Comment (from Kernel) from [2].

#### Example

This example shows a comment, as a rectangle with a "bent corner" in the upper right corner. It contains text. It appears on a particular diagram and may be attached to zero or more modelling elements by dashed lines.

Figure 8: Comment notation

#### Name style

It has no name so there is no name style.

### Multiplicity, a.k.a. cardinality in relationships

#### Description

- “A multiplicity is a definition of an inclusive interval of non-negative integers beginning with a lower bound and ending with a (possibly infinite) upper bound. A multiplicity element embeds this information to specify the allowable cardinalities for an instantiation of this element…“, an extract from 7.3.32 MultiplicityElement of [2].

#### Example

This example shows a multiplicity attached to the end of an association path. The meaning of this multiplicity is one to many. One Network instance is associated with zero, one or more SubNetwork instances. Other valid examples can show the “many to many” relationship.

The cardinality zero is not used to indicate the IOC’s so-called “transient state” characteristic. For example, it is not used to indicate that the instance is not yet created but it is in the process of being created. The cardinality zero will not be used to indicate this characteristic since such characteristic is considered inherent in all IOCs. All IOCs defined are considered to have such inherent “transient state” characteristics.

Note that the use of “0..*”, "0..n" or ‘*’ means “zero to many”. The use of ‘*’ is recommended.

Note that there is no default value for cardinality. If cardinality one is intended, then ‘1’ shall be used in the graphical notation (as shown in the example below).

Figure 9: Cardinality notation

#### Name style

It has no name so there is no name style.

### Role

#### Description

- See 3.43.2.6 of [1].

- Roles at navigable ends of relationships become attributes in the source class of the relationship. Therefore roles have the same behaviour as attributes. See section 5.1.1.

#### Example

This example shows that a Person (say instance John) is associated with a Company (say whose DN is “Company=XYZ”). We navigate the association by using the opposite association-end such that John’s Person.theCompany would hold the DN, i.e. "Company=XYZ".

Figure 10: Role notation

#### Name style

A role has a name. Use noun for the name. The name style follows the attribute name style; see section 5.1.1.3.
Corresponding naming rules for the representation of the role in the protocol specific specification need to be added.

### Xor constraint

#### Description

“A Constraint represents additional semantic information attached to the constrained elements. A constraint is an assertion that indicates a restriction that must be satisfied by a correct design of the system. The constrained elements are those elements required to evaluate the constraint specification…“, an extract from 7.3.10 Constraint (from Kernel) of [2].

For a constraint that applies to two elements such as two associations, the constraint shall be shown as a dashed line between the elements labeled by the constraint string (in braces). The constraint string, in this case, is xor.

Constraints attached to Comment and attached to an attribute are TBD.

#### Example

This example shows an Account (e.g. account 0960) that is associated with a Person (e.g. John Smith) or a Corporation (e.g. ABC Inc).

Figure 11: {xor} notation

#### Name style

It has no name so there is no name style.

### Composite aggregation association relationship

#### Description

A composite aggregation association is a strong form of aggregation that requires a part instance be included in at most one composite at a time. If a composite is deleted, all of its parts are deleted as well.

A composite aggregation shall contain a description of its use.

See 7.3.3 Association (from Kernel) of [2].

#### Example

A filled diamond attached to the end of a relationship is used to indicate a composite aggregation. The diamond is attached to the class that is the composite. The composition association shall have an indication of cardinality at each end of the relationship (see 5.1.7).
A composite aggregation association can e.g., be used to model an optional set of attributes of the composite class.

Figure 2: Composite aggregation relationship notation

#### Name style

It has no name so there is no name style.

## Stereotype

Sub-clause 5.1 listed the UML defined basic model elements. UML defined a stereotype concept allowing the specification of user-defined model elements.

This sub-clause lists all allowable stereotypes for this Repertoire.

For each stereotype model element listed, there are three parts. The first part contains its description. The second part contains its graphical notation examples and the third part contains the rule, if any, recommended for labelling or naming it.

### <<ProxyClass>>

#### Description

It is a form or template representing a number of <<InformationObjectClass>>. It encapsulates attributes, links, methods (or operations), and interactions that are present in the represented <<InformationObjectClass>>.

The semantics of a <<ProxyClass>> is that all behaviour of the <<ProxyClass>> is present in the represented <<InformationObjectClass>>. Since this class is simply a representation of other classes, this class cannot define its own behaviour other than those already defined by the represented <<InformationObjectClass>>.

A particular <<InformationObjectClass>> can be represented by zero, one or more <<ProxyClass>>. For example, the ManagedElement <<InformationObjectClass>> can have MonitoredEntity <<ProxyClass>> and ManagedEntity <<ProxyClass>>.

The attributes of the <<ProxyClass>> are accessible by the source entity that has an association with the <<ProxyClass>>.

#### Example

This shows a <<ProxyClass>> named MonitoredEntity. It represents all NRM <<InformationObjectClass>> (e.g. GgsnFunction <<InformationObjectClass>>) whose instances are being monitored for alarm conditions.

Note that MonitoredEntity does not define any attribute. The attributes are already defined by all NRM <<InformationObjectClass>>.

Figure 12: <<ProxyClass>> notation

See Annex A for more examples that use <<ProxyClass>>.

#### Name style

For <<ProxyClass>> name, use the same style as <<InformationObjectClass>> (see 5.2.2).

### <<InformationObjectClass>>

#### Description

Editor’s Note: There is differences between this stereotype definition and the normal (e.g. OMG-defined) UML class definition.  Will decide if we a) would use normal class definition with implicit differences or b) use this stereotype.

The <<InformationObjectClass>> is identical to UML class except that it does not include/define methods or operations.

A UML class represents a capability or concept within the system being modelled. Classes have data structure and behaviour and relationships to other elements.

See more on UML class in 10.2.1 of [1].

#### Example

This example shows an RncFunction <<InformationObjectClass>>.

Figure 13: <<InformationObjectClass>> notation

#### Name style

TBD but need to mention that the name cannot end with an underscore (less it would conflict with names for abstract class).

### <<names>>

#### Description

It specifies a unidirectional composition. The target instance is uniquely identifiable, within the namespace of the source entity, among all other targeted instances of the same target class and among other targeted instances of other classes that have the same <<names>> composition with the source.

The source class and target class shall each has its own naming attribute.

The composition aggregation association relationship is used as the act of name containment provides a semantic of a whole-part relationship between the domain and the named elements that are contained, even if only by name. From the management perspective access to the part is through the whole. Multiplicity shall be indicated at both ends of the relationship.

A target instance can not have multiple <<names>> with multiple sources, i.e. a target instance can not participate in or belong to multiple namespaces.

#### Example

This shows that all instances of MscFunction are uniquely identifiable within a ManagedElement instance's namespace.

Figure 14: <<names>> notation

#### Name style

It has no name so there is no name style.

### <<dataType>>

#### Description

It represents the general notion of being a data type (i.e. a type whose instances are identified only by their values) whose definition is defined by user (e.g. specification authors).

This Repertoire uses two kinds of data types: predefined data types and user-defined data types. The former is defined in sub-clause 5.3.3. The latter is defined by the specifications authors using this <<dataType>> model element.

The user-defined data types supports the modelling of structured data types (see <<dataType>> notations in 5.2.4.2). When user-defined or predefined data type is used to apply type information to a class attribute (see 5.1.1), the data type name is shown along with the class attribute. See user example of <<dataType>> in 5.2.4.2.

#### Example

The following examples are two user-defined data types. The left-most is named PLMN_Id that consists of Mobile Country Code (MCC) and Mobile Network Code (MNC), whose types are the predefined data types in 5.3.3. The right-most is named Xyz that consists of two predefined data types (i.e., String, Integer and one user-defined data type PLMN_Id.

Figure 15: <<dataType>> notations

The following example shows a ZClass using two user-defined data types and two predefined data types.

Figure 16: Usage example of <<dataType>>

#### Name style

For <<dataType>> name, use the same style as <<InformationObjectClass>> (see 5.2.2).

For <<dataType>> attribute, use the same style as Attribute (see 5.1.1).

### <<enumeration>>

#### Description

An enumeration is a data type. It contains sets of named literals that represent the values of the enumeration. An enumeration literal can represent a predefined data type (see 5.3.3) or a user-defined data type (see 5.2.4).

An enumeration has a name.

See 10.3.2 Enumeration of [1].

#### Example

This example shows an enumeration model element whose name is Day and it has seven enumeration literals. The upper compartment contains the keyword <<enumeration>> and the name of the enumeration. The lower compartment contains an ordered list of enumeration literals.

Figure 17: <<enumeration>> notation

#### Name style

For <<enumeration>> name, use the same style as <<InformationObjectClass>> (see 5.2.2).

Enumeration literal is composed of one or more words of upper case characters. Words are separated by the underscore character.

## Others

### Association class

#### Description

Editor’s Note: Use of Association classes needs further discussion.  Can cardinality be used? Can it be replaced by already-existing relation classes?

An association class is an association that also has class properties (or a class that has association properties).
Even though it is drawn as an association and a class, it is really just a single model element.

See 7.3.4 AssociationClass of [1].

Association classes are appropriate for use when an «InformationObjectClass» needs to maintain associations to several other «InformationObjectClass»es and there are relationships between the members of the associations within the scope of the "containing" «InformationObjectClass». For example, a namespace maintains a set of bindings, a binding ties a name to an object. A Binding «IOC» can be modelled as an Association Class that provides the binding semantics to the relationship between a name and some other «InformationObjectClass». This is depicted in the following figure.

#### Example

Figure 18: Association class notation

#### Name style

TBD

### Abstract class

#### Description

It specifies a special kind of <<InformationObjectClass>> as the general model element involved in a generalization relationship (see 5.1.4). An abstract class can not be instantiated.

#### Example

This shows that TP_ is an abstract class. It is the base class for SpecializedTP.

Figure 19: Abstract class notation

#### Name style

For abstract class name, use the same style as <<InformationObjectClass>> (see 5.2.2) and its last character shall be an underscore. Furthermore, the name shall be in italics.

### Predefined data types

#### Description

Predefined data types shall be modelled as «Primitive» data types. These data types cannot use other data types.

#### Examples

- Boolean

- Integer

- Float

- Double

- String

- GeneralizedTime.

#### Name style

Shall follow Upper CamelCase (UCC).

# Qualifier

Editor’s Note: Qualifier Invariant (TMF-definition) will be added.  Stereotypes/model elements for these qualifiers will be added.  Need for CO is for further study.

This sub-clause defines the qualifiers applicable for one specific model elements specified in this document, e.g., the class attribute. The class attribute must be qualified with M, O, CM or CO, with regards to their Read and Write capabilities.

The Read and Write capabilities are defined as follows.

- The model elements are made visible across an interface, separating the client from server. The Read capability means that a client can read the qualified class attribute information.  The Write capability means that a client can modify the qualified class attribute information.

Definition of qualifier M (Mandatory):

- The capability (e.g. Write capability) shall be supported.

Definition of qualifier O (Optional):

- The capability may or may not be supported.

Definition of qualifier CM (Conditional-Mandatory):

- The capability shall be supported under certain conditions, specifically:

- The class attribute qualified as CM shall have a corresponding constraint defined in the specification. If the specified constraint is met then the capability shall be supported.

Definition of qualifier CO (Conditional-Optional):

- The capability shall be may be supported under certain conditions, specifically:

- The class attribute qualified as CO shall have a corresponding constraint defined in the specification. If the specified constraint is met then the capability may be supported.

- (informative):
Examples of using <<ProxyClass>> to model Link related IOCs

# First Example

This shows a <<ProxyClass>> named YyyFunction. It represents all IOCs listed in the Note under the UML diagram. All the listed IOCs, in the context of this example, inherit from ManagedFunction IOC.

The use of <<ProxyClass>> eliminates the need to draw multiple UML <<InformationObjectClass>> boxes, i.e. those whose names are listed in the Note, in the UML diagram.

Figure 20: <<ProxyClass>> Notation Example A.1

Note: The YyyFunction <<ProxyClass>> represents AsFunction, AucFunction, BgFunction etc.

# Second Example

This shows a <<ProxyClass>> named YyyFunction. It represents all IOCs listed in the Note right under the UML diagram.  All the listed IOCs, in the context of this example, have link (internal and external) relations.

The actual names of the IOC represented by InternalYyyFunction <<ProxyClass>> and by the ExternalYyyFunction <<ProxyClass>> are listed under the subsection of X.Y of the associated YyyFunction.  For example, under X.Y.1 for AsFunction, two paragraphs are added to list all peer internal entities and external entities that are linked with AsFunction.  See example in quotation below that is using AsFunction as an example for YyyFunction.

The actual names of the IOC represented by Link_a_z <<ProxyClass>> and by ExternalLink_a_z <<ProxyClass>> are listed under the subsection of X.Y of the associated YyyFunction.  For example, under X.Y.1 for AsFunction, two paragraphs are added to list the names of the IOCs represented by Link_a_z and by ExternalLink_a_z. See the quoted text below that is using AsFunction as a example for YyyFunction.

“

X.Y.1	AsFunction

X.Y.1.1	Definition

This IOC represents As functionality. For more information about the As, see 3GPP TS 23.002 [4].

The linked InternalYyyFunction <<ProxyClass>> represents SlsFunction, CscfFunction, HlrFunction ...

The linked ExternalYyyFunction <<ProxyClass>> represents …

The Link_a_z <<ProxyClass>> represents Link_As_Scscf, Link_Bgcf_Scscf …

The ExternalLink_a_z <<ProxyClass>> represents …

“

Figure 21: <<ProxyClass>> Notation Example A.2

NOTE:	The ‘Yyy’ of YyyFunction <<ProxyClass>> represents AsFunction, AucFunction, etc.