3GPPSA5-TM Forum F2F Model Alignment meeting,

Darmstadt, Germany, Nov 30 – Dec 2, 2011 	S5vTMFa204

Source:	Ericsson

Title:	Changes to Model Repertoire v1.3

Document for:	Approval

1.   	Decision/action requested

Suggested changes to Model Repertoire v1.3. No intention to change the meaning. Changes should be strictly for clarification.

2. 	References

None

1st Modified Section

# Scope

The JWG on Model Alignment work has chosen UML to capture behaviour of systems/entities under management.

UML provides a rich set of concepts, notations and model elements to model distributive systems. Usage of all UML notations and model elements is not necessary for the purpose of JWG Model Alignment work. This paper documents the necessary and sufficient set of UML notations and model elements, including the ones built by the UML extension mechanism <<stereotype>>, for use by JWG Model Alignment work. Collectively, this set of notations and model elements is called the FNM (developed by the Converged Management of Fixed/Mobile Networks project) modelling repertoire.

JWG Model Alignment specifications shall employ the UML notation and model elements of this repertoire.  In the course of the JWG Model Alignment work, JWG Model Alignment group may modify (add, delete, modify) UML notation and model elements of this repertoire when necessary.

# References

[1]	OMG Unified Modelling Language (OMG UML), Infrastructure, Version 2.3.

[2]	OMG Unified Modelling Language (OMG UML), Superstructure, Version 2.3.

[3]	3GPP TS 32.300: 3rd Generation Partnership Projects; Technical Specification Group Services and System Aspects; Telecommunication management; Configuration Management (CM); Name convention for Managed Objects.

[4]	3GPP TS 23.002: 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network architecture.

# Definitions and abbreviations

## Definitions

Distinguished Name: See 3GPP TS 32.300 [3].

Naming attribute: See 3GPP TS 32.300 [3].

Lower Camel Case: It is the practice of writing compound words in which the words are joined without spaces. Initial letter of all except the first word shall be capitalized.  A word can be represented in abbreviated form. The abbreviation must be documented.  Examples: ‘mNIdentity’ and ‘minorDetails’ are the LCC for “managed node identity” and “minor details” respectively.

Upper Camel Case: It is the Lower Camel Case except that the first letter is capitalised.  Examples: ‘MNIdentity’ and ‘MinorDetails’ are the UCC for “managed node identity” and “minor details” respectively.

## Abbreviations

CM		Conditional Mandatory

CO	Conditional Optional

DN	Distinguished Name

FNM	Federated Network Model

IOC	Information Object Class

IRP	Integration Reference Point

JWG	(3GPP/TM Forum) Joint Working Group

LCC	Lower Camel Case

M	Mandatory

N/A	Not Applicable

NRM	Network Resource Model

O	Optional

OMG	Object Management Group

UCC	Upper Camel Case

UML	Unified Modelling Language (OMG)

# Requirements

The UML notations and model elements captured in this repertoire shall be used to model behaviours of the systems/entities specified by the JWG Model Alignment work such as the Umbrella model of the FNM discussed in Converged Management of Fixed/Mobile Network project.

Text may be marked in yellow colour. If so, the text is considered not-yet-agreed and needs revision.

# Model Elements and Notations

## General

Note that the graphical notation in this document is only used to represent particular model elements. Although the graphical notation is a correct representation of the model element, it may not be a valid representation of a UML class diagram.

The examples used in this document are for illustration purposes only and may or may not exist in specifications.

UML properties not described in this document shall not be used in specifications based on this repertoire.

## Basic model elements

UML has defined a number of basic model elements. This sub-clause lists the subset selected for use in specifications based on this repertoire. The semantics of these selected basic model elements are defined in [1].

For each basic model element listed, there are three parts. The first part contains its description. The second part contains its graphical notation examples and the third part contains the rule, if any, recommended for labelling or naming it.

The graphical notation has the following characteristics:

- Section 7.2.7 of [2] specifies "A class is often shown with three compartments. The middle compartment holds a list of attributes while the bottom compartment holds a list of operations" and "Additional compartments may be supplied to show other details…" This repertoire only allows the use of the name (top) and attribute (middle) compartments. The operation (bottom) compartment may be present but is always empty, as shown in figure below.

- Classes may or may not have attributes. The graphical notation of a class may show an empty attribute (middle) compartment even if the class has attributes, as shown in figure below.

- The attribute compartment shall show all attributes of the classes involved in an inheritance relationship.

- The visibility symbol shall not appear along with the class attribute, as shown below.

### Attribute

#### Description

- It is a typed element representing a property of a class. See 10.2.5 Property of [1].

- An element that is typed implies that the element can only refer to a constrained set of values.

- See 10.1.4 Type of [1] for more information on type.

- See 5.3.4 and 5.4.3 for predefined data types and user-defined data types that can apply type information to an element.

- The following table captures the properties of this modelled element.

Table 1: Attribute properties

| Property name | Description | Legal values | Qualification (See section 6) |
| --- | --- | --- | --- |
| Documentation | Contains a textual description of the attribute. Should refer (to enable traceability) to the specific requirement. | N/A | M; |
| Ordered | For a multi-valued multiplicity; this specifies whether the values of this attribute instance are sequentially ordered. See section 7.3.44 and its Table 7.1 of [2]. | True, False (default); | CM (Condition: The attribute is multi-valued); |
| Unique | For a multi-valued multiplicity, this specifies whether the values of this attribute instance are unique (i.e., no duplicate attribute values). See section 7.3.44 and its Table 7.1 of [2]. | True (default), False; | CM (Condition: The attribute is multi-valued); |
| Read | Specifies that this attribute can be read by the Manager. | True, False; | O, M, CM; |
| Write | Specifies that this attribute can be written by the Manager. | True, False; | O, M, CM; |
| Type | Refers to a predefined (see section 5.4.3) or user defined data type (see section 5.3.4). | N/A | M; |
| Invariant | Identifies whether the value of the attribute can be changed. If this value is True and the Property:Write is True, the attribute value is provided by the Manager at class instance creation time; else it is provided by the Agent at class instance creation time. | True, False (default). | M; |
| Value Range | Identifies the allowed values that the attribute can have (see section ?? of [?]). | True, False; | O, M, CM; |
| Attribute Notifications | Identifies whether a notification has to be sent in case of a value change. | True (default), False; | O, M, CM; |

#### Example

This example shows three attributes, i.e., a, b and c, listed in the attribute (the second) compartment of the class Xyz.

Figure 1: Attribute notation

#### Name style

An attribute name shall use the LCC style.

### Association relationship

#### Description

It shows a relationship between two classes and describes the reasons for the relationship and the rules that might govern that relationship.

It has ends. Its end, the association end(s), specifies the role that the object at one end of a relationship performs. Each end of a relationship has properties that specify the role (see 5.1.9), multiplicity (see 5.1.8), visibility and navigability (see the arrow symbol used in 5.1.2.2) and may have constraints. Note that visibility would not be used (see bullet 4 of 5.1).

See 7.3.3 Association of [2].

Three examples below show a binary association between two model elements. The association can include the possibility of relating a model element to itself.

The first example (Figure 2) shows a bi-directional navigable association in that each model element has a pointer to the other. The second example (Figure 3) shows a unidirectional association (shown with an open arrow at the target model element end) in that only the source model element has a pointer to the target model element and not vice-versa. The third example (Figure 4) shows a bi-directional non-navigable association in that each model element does not have a pointer to the other; i.e., such associations are just illustrative.

#### Example

An association shall have an indication of cardinality (not shown in the examples below; see 5.1.8) and shall, except the case of non-navigable association, have an indication of the role name (see 5.1.9).

The model element involved in an association is said to be “playing a role” in that association. The role has a name such as +theAClass in the examples below. See 5.1.9. Note that the "+" character in front of the role name, indicating the visibility, is ignored.

Figure 2: Bidirectional association relationship notation

Figure 3: Unidirectional association relationship notation

Figure 4: Non-navigable association relationship notation

Note that some tools do not use arrows in the UML graphical representation for bidirectional associations. Therefore, absence of arrows is not, but absence of role names is, an indication of a non-navigable association.

#### Name style

A role name shall use the LCC style.

### Aggregation association relationship

#### Description

It shows a class as a part of or subordinate to another class.

An aggregation is a special type of association in which objects are assembled or configured together to create a more complex object. Aggregation protects the integrity of an assembly of objects by defining a single point of control called aggregate, in the object that represents the assembly.

An aggregation shall contain a description of its use.

See 7.3.2 AggregationKind (from Kernel) of [2].

#### Example

A hollow diamond attached to the end of a relationship is used to indicate an aggregation. The diamond is attached to the class that is the aggregate. The aggregation association shall have an indication of cardinality at each end of the relationship (see 5.1.8).

Figure 5: Aggregation association relationship notation

#### Name style

It has no name so there is no name style.

### Composite aggregation association relationship

#### Description

A composite aggregation association is a strong form of aggregation that requires a part instance be included in at most one composite at a time. If a composite is deleted, all of its parts are deleted as well.

A composite aggregation shall contain a description of its use.

See 7.3.3 Association (from Kernel) of [2].

#### Example

A filled diamond attached to the end of a relationship is used to indicate a composite aggregation. The diamond is attached to the class that is the composite. The composition association shall have an indication of cardinality at each end of the relationship (see 5.1.8).
A composite aggregation association can e.g., be used to model an optional set of attributes of the composite class.

Figure 6: Composite aggregation association relationship notation

#### Name style

It has no name so there is no name style.

### Generalization relationship

#### Description

It indicates a relationship in which one class (the child) inherits from another class (the parent).

See 7.3.20 Generalization of [2].

#### Example

This example shows a generalization relationship between a more general model element (the IRPAgent) and a more specific model element (the IRPAgentVendorA) that is fully consistent with the first element and that adds additional information.

Figure 7: Generalization relationship notation

#### Name style

It has no name so there is no name style.

### Dependency relationship

#### Description

“A dependency is a relationship that signifies that a single or a set of model elements requires other model elements for their specification or implementation. This means that the complete semantics of the depending elements is either semantically or structurally dependent on the definition of the supplier element(s)...“, an extract from 7.3.12 Dependency of [2].

#### Example

This example shows that the BClass instances have a semantic relationship with the AClass instances. It indicates a situation in which a change to the target element (the AClass in the example) will require a change to the source element (the BClass in the example) in the dependency.

Figure 8: Dependency relationship notation

#### Name style

It has no name so there is no name style.

### Comment

#### Description

- A comment is a textual annotation that can be attached to a set of elements.

- See 7.3.9 Comment (from Kernel) from [2].

#### Example

This example shows a comment, as a rectangle with a "bent corner" in the upper right corner. It contains text. It appears on a particular diagram and may be attached to zero or more modelling elements by dashed lines.

Figure 9: Comment notation

#### Name style

It has no name so there is no name style.

### Multiplicity, a.k.a. cardinality in relationships

#### Description

- “A multiplicity is a definition of an inclusive interval of non-negative integers beginning with a lower bound and ending with a (possibly infinite) upper bound. A multiplicity element embeds this information to specify the allowable cardinalities for an instantiation of this element…“, an extract from 7.3.32 MultiplicityElement of [2].

#### Example

This example shows a multiplicity attached to the end of an association path. The meaning of this multiplicity is one to many. One Network instance is associated with zero, one or more SubNetwork instances. Other valid examples can show the “many to many” relationship.

The cardinality zero is not used to indicate the IOC’s so-called “transient state” characteristic. For example, it is not used to indicate that the instance is not yet created but it is in the process of being created. The cardinality zero will not be used to indicate this characteristic since such characteristic is considered inherent in all IOCs. All IOCs defined are considered to have such inherent “transient state” characteristics.

Note that the use of “0..*”, "0..n" or ‘*’ means “zero to many”. The use of ‘*’ is recommended.

Note that there is no default value for cardinality. If cardinality one is intended, then ‘1’ shall be used in the graphical notation (as shown in the example below).

Figure 10: Cardinality notation

#### Name style

It has no name so there is no name style.

### Role

#### Description

It indicates a navigation capability between two classes involved in an association relationship (see 5.1.2). A role is named. The direction of navigation is to the class attached to the end of the association relationship with (or near) the role name.

The use of role name in the graphical representation is mandatory for bidirectional and unidirectional association relationship notations (see Figure 2: Bidirectional association relationship notation and Figure 3: Unidirectional association relationship notation). Role name shall not be used in non-navigable association relationship notation (see Figure 4: Non-navigable association relationship notation).

- A role at the navigable end of a relationship becomes (or is mapped into) an attribute in the source class of the relationship. Therefore roles have the same behaviour (or properties) as attributes. See Table 1: Attribute properties

#### Example

This example shows that a Person (say instance John) is associated with a Company (say whose DN is “Company=XYZ”). We navigate the association by using the opposite association-end such that John’s Person.theCompany would hold the DN, i.e. "Company=XYZ".

Figure 11: Role notation

#### Name style

A role has a name. Use noun for the name. The name style follows the attribute name style; see section 5.2.1.3.
Corresponding naming rules for the representation of the role in the protocol specific specification need to be added.

### Xor constraint

#### Description

“A Constraint represents additional semantic information attached to the constrained elements. A constraint is an assertion that indicates a restriction that must be satisfied by a correct design of the system. The constrained elements are those elements required to evaluate the constraint specification…“, an extract from 7.3.10 Constraint (from Kernel) of [2].

For a constraint that applies to two elements such as two associations, the constraint shall be shown as a dashed line between the elements labeled by the constraint string (in braces). The constraint string, in this case, is xor.

Constraints attached to Comment and attached to an attribute are TBD.

#### Example

This example shows an Account (e.g. account 0960) that is associated with a Person (e.g. John Smith) or a Corporation (e.g. ABC Inc).

Figure 12: {xor} notation

#### Name style

It has no name so there is no name style.

## Stereotype

Sub-clause 5.1 listed the UML defined basic model elements. UML defined a stereotype concept allowing the specification of user-defined model elements.

This sub-clause lists all allowable stereotypes for this repertoire.

For each stereotype model element listed, there are three parts. The first part contains its description. The second part contains its graphical notation examples and the third part contains the rule, if any, recommended for labelling or naming it.

### <<ProxyClass>>

#### Description

It is a form or template representing a number of <<InformationObjectClass>>. It encapsulates attributes, links, methods (or operations), and interactions that are present in the represented <<InformationObjectClass>>.

The semantics of a <<ProxyClass>> is that all behaviour of the <<ProxyClass>> is present in the represented <<InformationObjectClass>>. Since this class is simply a representation of other classes, this class cannot define its own behaviour other than those already defined by the represented <<InformationObjectClass>>.

A particular <<InformationObjectClass>> can be represented by zero, one or more <<ProxyClass>>. For example, the ManagedElement <<InformationObjectClass>> can have MonitoredEntity <<ProxyClass>> and ManagedEntity <<ProxyClass>>.

The attributes of the <<ProxyClass>> are accessible by the source entity that has an association with the <<ProxyClass>>.

#### Example

This shows a <<ProxyClass>> named MonitoredEntity. It represents all NRM <<InformationObjectClass>> (e.g. GgsnFunction <<InformationObjectClass>>) whose instances are being monitored for alarm conditions.

Figure 13: <<ProxyClass>> notation

See Annex A for more examples that use <<ProxyClass>>.

#### Name style

For <<ProxyClass>> name, use the same style as <<InformationObjectClass>> (see 5.3.2).

### <<InformationObjectClass>>

#### Description

Editor’s Note: There is differences between this stereotype definition and the normal (e.g. OMG-defined) UML class definition.  Will decide if we a) would use normal class definition with implicit differences or b) use this stereotype.

The <<InformationObjectClass>> is identical to UML class except that it does not include/define methods or operations.

A UML class represents a capability or concept within the system being modelled. Classes have data structure and behaviour and relationships to other elements.

See more on UML class in 10.2.1 of [1].

- The following table captures the properties of this modelled element.

- Table 2: <<InformationObjectClass>>.properties

| Property name | Description | Legal values | Qualification (See section 6) |
| --- | --- | --- | --- |
| Documentation | Contains a textual description of this modelled element. Should refer (to enable traceability) to a specific requirement. | N/A | O, M, CM; |

#### Example

This example shows an RncFunction <<InformationObjectClass>>.

Figure 14: <<InformationObjectClass>> notation

#### Name style

TBD but need to mention that the name cannot end with an underscore (less it would conflict with names for abstract class).

### <<names>>

#### Description

It specifies a unidirectional composition. The target instance is uniquely identifiable, within the namespace of the source entity, among all other targeted instances of the same target class and among other targeted instances of other classes that have the same <<names>> composition with the source.

The source class and target class shall each has its own naming attribute.

The composition aggregation association relationship is used as the act of name containment providing a semantic of a whole-part relationship between the domain and the named elements that are contained, even if only by name. From the management perspective access to the part is through the whole. Multiplicity shall be indicated at both ends of the relationship.

A target instance can not have multiple <<names>> with multiple sources, i.e. a target instance can not participate in or belong to multiple namespaces.

#### Example

This shows that all instances of MscFunction are uniquely identifiable within a ManagedElement instance's namespace.

Figure 15: <<names>> notation

#### Name style

It has no name so there is no name style.

### <<dataType>>

#### Description

It represents the general notion of being a data type (i.e. a type whose instances are identified only by their values) whose definition is defined by user (e.g. specification authors).

This repertoire uses two kinds of data types: predefined data types and user-defined data types. The former is defined in sub-clause 5.4.3. The latter is defined by the specifications authors using this <<dataType>> model element.

The user-defined data types supports the modelling of structured data types (see <<dataType>> notations in 5.3.4.2). When user-defined or predefined data type is used to apply type information to a class attribute (see 5.2.1), the data type name is shown along with the class attribute. See user example of <<dataType>> in 5.3.4.2.

#### Example

The following examples are two user-defined data types. The left-most is named PLMN_Id that consists of Mobile Country Code (MCC) and Mobile Network Code (MNC), whose types are the predefined data types in 5.4.3. The right-most is named Xyz that consists of two predefined data types (i.e., String, Integer and one user-defined data type PLMN_Id.

Figure 16: <<dataType>> notations

The following example shows a ZClass using two user-defined data types and two predefined data types.

Figure 17: Usage example of <<dataType>>

#### Name style

For <<dataType>> name, use the same style as <<InformationObjectClass>> (see 5.3.2).

For <<dataType>> attribute, use the same style as Attribute (see 5.2.1).

### <<enumeration>>

#### Description

An enumeration is a data type. It contains sets of named literals that represent the values of the enumerationAn enumeration has a name.

See 10.3.2 Enumeration of [1].

#### Example

This example shows an enumeration model element whose name is Account and it has four enumeration literals. The upper compartment contains the keyword <<enumeration>> and the name of the enumeration. The lower compartment contains a list of enumeration literals.

Figure 18: <<enumeration>> notation

#### Name style

For <<enumeration>> name, use the same style as <<InformationObjectClass>> (see 5.3.2).

For Enumeration literal name, use the LCC style.

## Others

### Association class

#### Description

Editor’s Note: Use of Association classes needs further discussion.  Can cardinality be used? Can it be replaced by already-existing relation classes?

An association class is an association that also has class properties (or a class that has association properties).
Even though it is drawn as an association and a class, it is really just a single model element.

See 7.3.4 AssociationClass of [2].

Association classes are appropriate for use when an «InformationObjectClass» needs to maintain associations to several other «InformationObjectClass»es and there are relationships between the members of the associations within the scope of the "containing" «InformationObjectClass». For example, a namespace maintains a set of bindings, a binding ties a name to an object. A Binding «IOC» can be modelled as an Association Class that provides the binding semantics to the relationship between a name and some other «InformationObjectClass». This is depicted in the following figure.

#### Example

Figure 19: Association class notation

#### Name style

TBD

### Abstract class

#### Description

It specifies a special kind of <<InformationObjectClass>> as the general model element involved in a generalization relationship (see 5.1.5). An abstract class can not be instantiated.

This modelled element has the same properties as class. See Table 2: <<InformationObjectClass>>.properties.

#### Example

This shows that TP_ is an abstract class. It is the base class for SpecializedTP.

Figure 20: Abstract class notation

#### Name style

For abstract class name, use the same style as <<InformationObjectClass>> (see 5.3.2) and its last character shall be an underscore. Furthermore, the name shall be in italics.

### Predefined data types

#### Description

It represents the general notion of being a data type (i.e. a type whose instances are identified only by their values) whose definition is defined by this specification and not by the user (e.g. specification authors).

This repertoire uses two kinds of data types: predefined data types and user-defined data types.  The latter are defined in 5.3.4 <<dataType>> and 5.3.5 <<enumeration>>.

The following table lists the UML data types selected for use as predefined data type.

Table 3: UML defined data types

| Name | Description and reference |
| --- | --- |
| Boolean | See 7.3.43 of [2]. |
| Integer | See 7.3.43 of [2]. |
| String | See 7.3.43 of [2]. |

The following table lists data types that are defined by this repertoire.

Table 4: Non-UML defined data types

| Name | Description and reference |
| --- | --- |
| DateTime | Check if we can use ITU-T M.3020 Annex E GeneralizedTime for this. |
| ObjectName | Check if we can use ITU-T M.3020 Annex E definition of same name. |
| Real | Check if we can use ITU-T M.3020 Annex E definition of same name and use it to replace/accommodate NGCOR’s Float, Double. |

#### Examples

TBD

#### Name style

It shall use the UCC style.

# Qualifier

Need for CO is for further study.

This sub-clause defines the qualifiers applicable for one specific model elements specified in this document, e.g., the class attribute. The class attribute must be qualified with M, O, CM or CO, with regards to their Read and Write capabilities.

Definition of qualifier M (Mandatory):

- The capability (e.g. Write capability) shall be supported.

Definition of qualifier O (Optional):

- The capability may or may not be supported.

Definition of qualifier CM (Conditional-Mandatory):

- The capability shall be supported under certain conditions, specifically:

- The class attribute qualified as CM shall have a corresponding constraint defined in the specification. If the specified constraint is met then the capability shall be supported.

Definition of qualifier CO (Conditional-Optional):

- The capability shall be may be supported under certain conditions, specifically:

- The class attribute qualified as CO shall have a corresponding constraint defined in the specification. If the specified constraint is met then the capability may be supported.

# Design patterns

## Use of association class

## Use of “intervening class”

## Use of “ExternalXyz class”

# Annex A (informative):
Examples of using <<ProxyClass>> to model Link related IOCs

  - First Example

This shows a <<ProxyClass>> named YyyFunction. It represents all IOCs listed in the Note under the UML diagram. All the listed IOCs, in the context of this example, inherit from ManagedFunction IOC.

The use of <<ProxyClass>> eliminates the need to draw multiple UML <<InformationObjectClass>> boxes, i.e. those whose names are listed in the Note, in the UML diagram.

Figure 21: <<ProxyClass>> Notation Example A.1

Note: The YyyFunction <<ProxyClass>> represents AsFunction, AucFunction, BgFunction etc.

  - Second Example

This shows a <<ProxyClass>> named YyyFunction. It represents all IOCs listed in the Note right under the UML diagram.  All the listed IOCs, in the context of this example, have link (internal and external) relations.

The actual names of the IOC represented by InternalYyyFunction <<ProxyClass>> and by the ExternalYyyFunction <<ProxyClass>> are listed under the subsection of X.Y of the associated YyyFunction.  For example, under X.Y.1 for AsFunction, two paragraphs are added to list all peer internal entities and external entities that are linked with AsFunction.  See example in quotation below that is using AsFunction as an example for YyyFunction.

The actual names of the IOC represented by Link_a_z <<ProxyClass>> and by ExternalLink_a_z <<ProxyClass>> are listed under the subsection of X.Y of the associated YyyFunction.  For example, under X.Y.1 for AsFunction, two paragraphs are added to list the names of the IOCs represented by Link_a_z and by ExternalLink_a_z. See the quoted text below that is using AsFunction as a example for YyyFunction.

“

X.Y.1	AsFunction

X.Y.1.1	Definition

This IOC represents As functionality. For more information about the As, see 3GPP TS 23.002 [4].

The linked InternalYyyFunction <<ProxyClass>> represents SlsFunction, CscfFunction, HlrFunction ...

The linked ExternalYyyFunction <<ProxyClass>> represents …

The Link_a_z <<ProxyClass>> represents Link_As_Scscf, Link_Bgcf_Scscf …

The ExternalLink_a_z <<ProxyClass>> represents …

“

Figure 22: <<ProxyClass>> Notation Example A.2

NOTE:	The ‘Yyy’ of YyyFunction <<ProxyClass>> represents AsFunction, AucFunction, etc.

End of Modified Section