| 3GPP - TM Forum Model Alignment Study |  | Document: S5vTMFa225 |
| --- | --- | --- |
| Author: | Nigel Davis, Ciena. |  |
| Title: | Final comments on Umbrella (S5vTMFa219) |  |

Introduction

This document provides final comments on “S5vTMFa219 FNM Umbrella V1.5”. This document is an update of and supersedes “S5vTMFa221” and assumes that 221 has been fully implemented in 219. The document accounts for all comments not covered to conclusion during the face to face meeting in Darmstadt (30 Nov – 2 Dec 2011).

Specific Comments

The following sections are headed with the title or aspect of S5vTMFa219 and provide a brief description of the intent and explanation in italics followed by necessary adjustment in normal type. This pattern of intent in italics followed by detail adjustment is repeated through each section as appropriate.

Margin comments have been used to provide direction to the editor of S5vTMFa219 on changes required to that document.

Comments

Extract of existing text with comments etc

4.	Umbrella

The wording “Umbrella classes are partitioned.  Classes in “Partition operational” are used to capture run-time behaviour of managed resources.  Classes in “Partition inventory” are used to capture static behaviour of managed resources.” is not accepted. There is no reason to partition the classes in this way. Indeed such a partition will be likely to lead to unnecessary churn. It is proposed that the attributes of more coherent classes be marked with their expected lifecycle behavior with respect to the viewpoint of the interface concerned and those that are read only simply be marked as such. It is reasonable for any organization to rearrange the classes as they see fit to suit their local mechanism. See new contribution S5vTMFa191-CommentsOnInventory.

It is proposed that the wording be changed to:

“Converged classes are arranged in the model based upon the domain of concern and the degree of abstraction (principles, patterns, architectures, purpose specific views). A class may have attributes that are read only and read/write. The lifecycle of each attribute will be stated.”

4.1.1	Class diagram

Clearly the entities on the class diagram need to be aligned with the agreements on the text sections. In addition:

Note that the term “concrete” has been removed here due to the implication of the following (from Wikipedia) “An abstract class … is a class that cannot be instantiated. Such a class is only meaningful if the language supports inheritance.” as it has been agreed that inheritance is not the (only) adoption mechanism. Indeed there is no reason why a body could not adopt a class and use it directly. A class may be adopted as concrete or abstract. Whether the class is concrete or abstract is up to the user. It is accepted however that areas such as naming may cause the need for local refinement of naming rules but even here mechanisms other than basic inheritance may be chosen. For example JOSIF have chosen an injection approach.

The editor’s notes in this section do not convey sufficiently the concern raised in Nanjing that there may be a perception that the classes identified are all that is needed. It is suggested that a stronger text be constructed prior to delivery.

The editor’s note is as follows:

“

Editor Notes:

- UML classes above are not sufficient for FNM NM purposes.  Their class names may not be appropriate.

- These Umbrella classes are for inheritance only.

- Other Umbrella classes may be used for relation other than inheritance.

“

It is suggested that the note be refined to the following:

“

Editor Notes:

- The Umbrella model described in this document provides the set of classes etc that have been agreed for convergence to strengthen consistency of representation in the fixed and mobile environment. For full management of an FNM solution many other classes will be required in addition to those in the Umbrella.

- The Umbrella model cannot be used directly for implementation. Implementation classes must be derived from the Umbrella by Inheritance or some other appropriate mechanism.

- The model for the fixed environment and the model for the mobile environment must use different names for classes etc from those described in the Umbrella.

- Where an implementation class is essentially identical to the abstract class described in the Umbrella the name of the implementation class should be the same as that of the Umbrella class minus the underscore, e.g. the Umbrella class “Function_” would become “Function”.

“

4.1.2.6	TerminationPoint_

.

4.1.2.6.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| tpeType | M | M | - |
| parameters | O | M | O |

It is proposed that further sections be added. The text in these sections was extracted from text provided in the previous document version under the heading “description” and agreed as beyond description.

“4.1.2.6.3	Rationale

The encapsulation of the TPE is performed to both reduce the instances of objects required to represent a given transport assembly and to also simplify the translation from traditional environments where layering is not fully represented. The encapsulation may be opaque, not exposing the layering, or semi-transparent, exposing the explicit layering but compacted into a single instance. Where the encapsulation is semi-transparent the management system client and/or management system server can potentially expand the model back to fully layered as desired. In the semi-transparent case the TerminationPointEncapsulation is composed of many “Layer Terminations”.

4.1.2.6.4	Usage

The TerminatioPointEncapsulation provides a place against which to raise alarms, display parameters and set attributes associated with the signal flow. Where the encapsulation is opaque the definition of the parameters etc will need to be such as to distinguish the encapsulated protocols/layers. The TerminationPointEncapsulation can carry generalized parameters such as name and userLabel.

The TerminationPointEncapsulation can be related:

- Directly to one or more physical ports (i.e. that the signal is associated directly with an externally visible connector)

  - Note that a physical port could also be related to more than one TerminationPointEncapsulation

- To a logical functions that anchor the signal flow (i.e. it is floating between flexible functions in the equipment with no externally visible connector).

- To another TerminationPointEncapsulation to represent a client signal of the supporting TerminationPointEncapsulation where there may be many instances of client.

  - Note that there may be many instances of server TerminationPointEncapsulation that feed a single client (e,g, in the case of VCAT)

The intention is that this class be used directly or with minimal sub-classing, i.e. NOT per port type, however it is recommended that an attribute that represents the port type specification is filled out with a specification value.

For further background see SD1-18 Functional Modelling Concepts and naming refer to SD1-25 Object Naming.”

4.3.1	Definitions and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| aEnd | The value of this attribute shall be the Distinguished Name of one instance of link end. The other instance shall be named in the zEnd. . | Values to be conformant with TS 32.300 [3] |
| dnPrefix | It carries the DN Prefix information as defined in Annex C of 32.300 [2] or no information. |  |
| Id | Uniquely identifies the object instance within the scope of the naming authority. |  |
| managedElementType | The type of managed element. It conveys the type of one or more managed functions. Thus, it may represent one ME functionality or a combination of more than one functionality. | Two examples of values are: - NodeB; - HLR,VLR - STM16 Mux The specification developing organisation may specify rules for the structure and value ranges of this attribute.. |
| linkType | This attribute defines the type of the link. | Signalling, Bearer, OAM&P, Other or multiple combinations of the above types. |
| locationName | The physical location of this entity (e.g. an address). |  |
| managedElements | Models the role Manager. This attribute contains a list of the DN(s) of the related ManagedElement instance(s). |  |
| managedBy | Models the role subordinate. This attribute contains a list of the DN(s) of the related ManagementNode instance(s). |  |
| layerProtocolNameList | Name(s) and additional descriptive information for the protocol(s)/layer(s) used for the associated communication link. Syntax and semantic is not specified. |  |
| protocolVersion | Versions(s) and additional descriptive information for the protocol(s) used for the associated communication link. Syntax and semantic is not specified. |  |
| userDefinedNetworkType | Textual information regarding the type of network, e.g. UTRAN. |  |
| userLabel | A user-friendly (and user assignable) name of this object. |  |
| zEnd | The value of this attribute shall be the Distinguished Name of the alphabetically second instance in the Link subclass name to which this link/relation is associated (i.e., pointing to the instance of <Y> as described in the definition of Link IOC in the present document). As an example, with Link_As_Slf, aEnd would contain the Distinguished Name of the AsFunction instance, and the zEnd would contain the Distinguished Name of SlfFunction instance. | Values to be conformant with TS 32.300 [3] |

Additional attributes proposed.

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| direction | Client-Server, Server-Client, Bidirectional |  |
| ltType | Name of specification of construction of the LT (CP only…) |  |
| tpeType | Name of specification of construction of the TPE |  |
| index | Provides any relevant indexing of the LT (channel number etc for example 373 see SDH spec for detail) |  |

End of specific comments