TSG-RAN Working Group 3, O&M adhoc Meeting #3 	TSGR3#5(99) 606

Münster, GERMANY, 29-30 June 1999

# Agenda Item:	7

# Source: 		Motorola

# Title: 		Network Element ownership of logical cell parameters

Document for:	Decision for I3.05

Introduction

At the RAN-WG3 meeting #4 O&M adhoc session, a question arose as to which network element, the CRNC or the Node B, would be the owner of the logical cell parameters.  After debate, a working assumption was established which states any parameter configuration relating to the logical cell model as defined by RAN-WG3 should be performed within the CRNC.  The CRNC will then control the configuration of these logical cell parameters at Node B via the Iub interface (reference: tdoc-572).

It will be shown below that the working assumption for CRNC control of logical cell parameters should not apply to all parameters.  Node B control of certain logical cell parameters is needed to help facilitate RNC – Node B multi-vendor interoperability.

It is the intention of this contribution to do the following:

- To start a list of logical cell parameters needed for the definition of a cell.  Note that this list may not be exhaustive.

- To indicate whether the value of the logical cell parameter is actively used or must be known by the CRNC, the Node B, or (shared) by both the CRNC and Node B for cell operation and / or control.

- To indicate the owner of each logical cell parameter.  The owner of the parameter is the network element where the value should be configured via the management system or defaulted within the network element itself.  The parameter owner communicates the value to the other network element via an NBAP procedure (not specified in this contribution).  For some parameters, there is no clear owner; thus ownership can be arbitrarily assigned to the CRNC or Node B.

Discussion

Description of table columns:

| Cell Parameters | Textual name of the cell parameter |
| --- | --- |
| Description | Description of the purpose of the cell parameter |
| CRNC Reqd. | Indicates whether this parameter needs to be known / used by the CRNC |
| Node B Reqd. | Indicates whether this parameter needs to be known / used by the Node B |
| Per Cell / Per Carrier | Indicates whether this parameter can be applicable on a per carrier per cell basis, in addition to just a per cell basis |
| Owner | Network element (NE) where the parameter should be configured via the management system or defaulted within the NE |
| Reason | Where applicable, justification for NE ownership |

List of Cell Configuration Parameters

| Cell Parameters | Description | CRNC Reqd. | Node B Reqd. | Per Carrier / Per Cell | Owner | Reason |
| --- | --- | --- | --- | --- | --- | --- |
| Node B Gain | Maximum Gain Capability of the Node B | X | X | X | Node B | See NOTE 1 |
| Forward Frequency | Frequencies to be used in the cell in the DL direction | X | X | X | Node B | See NOTE 2 |
| Reverse Frequency | Frequencies to be used in the cell in the UL direction | X | X | X | Node B | See NOTE 2 |
| DL Scrambling Code | Unique scrambling code defined per cell. All the mobiles in the cell will determine the scrambling code during the cell search | X | X |  | CRNC |  |
| BCH Data Gain | Gain to be used for the BCH Data part. |  | X |  | Node B | See NOTE 3 |
| BCH Pilot Gain | Gain to be used for the BCH Pilot part. |  | X |  | Node B | See NOTE 3 |
| FACH Code | Channelization code to be used by the FACH. | X | X |  | CRNC |  |
| FACH Data Gain | Gain to be used for the FACH Data part. |  | X |  | Node B | See NOTE 3 |
| FACH Pilot Gain | Gain to be used for the FACH Pilot part. |  | X |  | Node B | See NOTE 3 |
| FACH Transmit Offset | The delay of transmission of FACH Transport blocks from the Node B reference as modified by T Cell. | X | X |  | CRNC |  |
| PCH Code | Channelization code to be used by the PCH. | X | X |  | CRNC |  |
| PCH Data Gain | Gain to be used for the PCH Data part. |  | X |  | Node B | See NOTE 3 |
| PCH Pilot Gain | Gain to be used for the PCH Pilot part. |  | X |  | Node B | See NOTE 3 |
| PCH Transmit Offset | The delay of transmission of PCH Transport blocks from the Node B reference as modified by T Cell. | X | X |  | CRNC |  |
| DSCH Code | Channelization code to be used by the DSCH. | X | X |  | CRNC |  |
| DSCH Data Gain | Gain to be used for the DSCH Data part. | X | X |  | CRNC |  |
| DSCH Pilot Gain | Gain to be used for the DSCH Pilot part. | X | X |  | CRNC |  |
| DSCH Transmit Offset | The delay of transmission of DSCH Transport blocks from the Node B reference as modified by T Cell. | X | X |  | CRNC |  |
| SCH Gain | The Gain to be applied to SCH |  | X |  | Node B | See NOTE 3 |
| PRACH Spreading Factor | Specifies the spreading factor to be used for the Data part | X | X |  | CRNC |  |
| PRACH Preamble signature | Preamble signature to be used in the PRACH | X | X |  | CRNC |  |
| PRACH Window Detection Range | Length in chips for detecting the PRACH preamble |  | X |  | Node B |  |
| PRACH Spreading Code | Spreading code to be used for each of the timeslots in a PRACH frame | X | X |  | CRNC |  |
| RACH Control Parameters | The number of retransmissions between successive retransmissions and the maximum number of times a channel can be requested by an UE | X |  |  | CRNC |  |
| Pilot Add threshold | Threshold to be used in soft handover procedures. | X |  |  | CRNC |  |
| Pilot Drop threshold | Threshold to be used in soft handover procedures | X |  |  | CRNC |  |
| Downlink Power Control Parameters | Parameters that control the behavior of PC for the downlink direction |  | X |  | Node B |  |
| Uplink Power Control Parameters | Parameters that control the behavior of PC for the uplink direction | X |  |  | CRNC |  |
| Rake Configuration Parameters | Search window length parameters used by the rake receiver |  | X |  | Node B |  |
| Cell Range Parameters | Access range parameters for calls |  | X |  | Node B |  |
| T Cell | The primitive specifies the time offset from the Node B superframe rollover to start of frame 1. | X | X |  | CRNC |  |
| System Frame Number Modulo | This Specifies the number of 72 frame multiframes occur prior to a frame number rollover to 0 | X | X |  | CRNC |  |
| IBTS Update Interval | The number of frame periods between the updates of IBTS in the System information |  | X |  | Node B |  |
| IBTS Averaging Length | The number of IBTS samples to be averaged and used for updating the System information |  | X |  | Node B |  |
| Neighbors | The DL scrambling codes of the neighbors of this cell. | X |  |  | CRNC |  |

NOTE 1:  (Node B Gain) The CRNC needs to understand Node B power to perform admission control.  The implementation of the PA is vendor specific and thus may have limitations and restrictions beyond what the RNC, and the management model for the RNC, should be expected to understand.

NOTE 2:  (Forward Frequency and Reverse Frequency) There are two issues in regards to assigning frequency parameters:

- Issue 1 – the time to tune to a frequency will vary for each vendor’s Node B.  If the CRNC is assumed to own the frequency parameters, then there is no set time for CRNC to know if the frequency allocation was successful at the Node B.  It is better for the Node B to own the frequency parameters and to notify the CRNC when the Node B has finished applying its logical parameter set.

- Issue 2 – vendor specific hardware implementation may limit the frequencies usable.  Support for this limitation should be in the Node B management system and hence the Node B should own the parameters.

Any potential physical limitations associated with a parameter at a NE should result in that NE owning that parameter.

NOTE 3: The Node B should own the parameters because it is the Node B that is aware of the power capability of the cell. Do not expect these parameters to be changed dynamically, but rather by O&M applications.

Proposal

- To change the RAN-WG3 current working assumption stated in I3.05 section 4.2 that configuration of parameters in the logical cell model is performed within the CRNC, which then controls the configuration of the logical cell parameters for the Node B.  The working assumption should state that a division of the logical cell parameters is required, where ownership of each logical cell parameter is given to the NE where it makes the most sense (e.g., where hardware dependencies exist, etc…).