# 3GPP TSG SA WG5 (Telecom Management) Meeting #102	S5-154144

# 24-28 August 2015, Beijing (China)	revision of S5-15xxxx

# Source:	NTT DOCOMO

# Title:	pCR TR 32.842 Add FM procedure flow for correlation in VNFM

# Document for:	Approval

# Agenda Item:	6.5.3 Study on Network Management of Virtualized Networks

# 1	Decision/action requested

Discuss and approve on the text proposal.

# 2	References

[REF-A]	TR 32.842 Telecommunication management; Study on network management of Virtualized Networks.

# 3	Rationale

Based on the use case of failure management when the alarm is generated by VNFC described in subclause 5.1.5 in TR 32.842 [REF-A], it is proposed to add a new FM procedure flow for fault correlation done in VNFM similar to clause 7.3.6 FM procedure flow for correlation done in EM.

# 4	Detailed proposal

It is proposed to make the following changes to TR 32.842 v121 [REF-A].

1st proposed change

### 7.3.X	FM procedure flow for correlation done in VNFM

#### 7.3.X.1 	Introduction

In the present FM procedure flow, there are two categories of fault reports that can be correlated at the VNFM:

-	Virtual resource (VR) fault report

-	VNF/VNFC fault report

#### 7.3.X.2 	VR fault report procedure

1.	The NFVI detects VR failure(s) and reports it to the VIM.

2. 	When the VIM receives the failure report, it will supplement/amend the failure related VR information, create a VR fault report, and send it to the corresponding VNFM.

3. 	The VNFM identifies which VNFs are impacted based on the VR fault report.

#### 7.3.X.3 	VNF/VNFC fault report procedure

4.	The VNF detects failure(s) caused by VNF software failure(s) and corresponding VNFC deployment on the virtualized infrastructure (e.g., configuration of the VNF, runtime communication of a VNFC with another VNFC) and reports the fault to the VNFM.

#### 7.3.X.4 	Fault correlation done in VNFM

5. 	Based on the VR fault report from the VNFM (see step 2 in clause 7.3.X.2) and the VNF/VNFC fault report (see step 4 in clause 7.3.X.3) from the VNF, the VNFM performs a correlation and analysis of the reported faults and their impact. If necessary, the VNFM triggers corrective actions.

6. 	The VNFM creates a VNF-related fault report and sends it to corresponding EM.

EM

NFVI

VNF

VNFM

2. VR fault report

VIM

1. NFVI failure report

6. VNF-related fault report

3. Identify related VNF

5. Fault correlation based on reports from 2 and 4

4. VNF/VNFC fault report

Figure 7.3.X-1: FM procedure flow for correlation done in VNFM.

End